3GPP TSG RAN WG1#102-e		R1-2005229

e-Meeting, August 17th – 28th, 2020

3GPP RAN WG4 Meeting #95-e	R4-2009260

Electronic meeting, 25th May – 5th June, 2020

Title:	LS on CSI-RS based intra-frequency and inter-frequency measurement definition

Release:	Release 16

Work Item:	NR_CSIRS_L3meas

Source:	RAN4

To:	RAN2

Cc:	RAN1

Contact Person:

Name:				Xuhua Tao

E-mail Address:	taoxuhua@catt.cn

- Overall Description:

RAN4 has discussed the definitions of CSI-RS based intra-frequency and inter-frequency measurements in Rel-16 WI of “RRM requirements for CSI-RS based L3 measurement” and has reached the following conclusion on their definitions:

  - CSI-RS based intra-frequency measurement: a measurement is defined as a CSI-RS based intra-frequency measurement provided that:

    - the SCS of CSI-RS resources on the neighbour cell configured for measurement is the same as the SCS of CSI-RS resources on the serving cell indicated for measurement, and

    - the CP type of CSI-RS resources on neighbour cell configured for measurement is the same as the CP type of CSI-RS resources on the serving cell indicated for measurement, and

  - It is applied for SCS = 60KHz

    - the centre frequency of CSI-RS resources on the neighbour cell configured for measurement is the same as the centre frequency of CSI-RS resource on the serving cell indicated for measurement

  - CSI-RS based inter-frequency measurement: a measurement is defined as a CSI-RS based inter-frequency measurement if it is not a CSI-RS based intra-frequency measurement

Actions:

To RAN WG2: RAN4 kindly asks RAN2 to take the above conclusions into consideration in future work.

- Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #96e						24-28 Aug, 2020, e-meeting

TSG RAN WG4 Meeting #96-ebis					26 Oct - 13 Nov, 2020, e-meeting