3GPP TSG RAN WG1 Meeting #102-e	R1-2007203

e-Meeting, August 17th – 28th, 2020

Title:	LS on multi-CC simultaneous TCI activation with multi-TRP/panel transmission

Response to:	-

Release:	Release 16

Work Item:	NR_eMIMO-Core

Source:	RAN WG1

To:	RAN WG2

Contact Person:

Name:	Jiwon Kang

E-mail Address:	jw.kang@lge.com

Attachments:	None

Overall Description:

In RAN1#102-e meeting, RAN1 discussed on whether/how to support the feature of multi-CC simultaneous TCI activation for PDSCH with the two features specified for multi-TRP/panel transmission, i.e. single DCI based and multi-DCI based multi-TRP/panel transmission, respectively. In the case of multi-DCI, a sentence ‘Network should not configure serving cells that are configured with CORESETPoolID=1 in these lists.’ is found from CellGroupConfig IE in TS38.331 and made a following conclusion accordingly.

Conclusion (RAN1#102-e) - By RRC configuration, each CC list cannot include a CC/BWP in which two CORESET pools are configured.

In the case of single DCI based multi-TRP/panel transmission, companies think that clarification from RAN2 is needed on whether/how this can be operated together with the feature of multi-CC simultaneous TCI activation. Switching between single DCI based multi-TRP/panel and single TRP/panel is controlled by MAC-CE, not by RRC. From RRC configuration perspective, therefore, it seems possible to include a CC/BWP being operated with single DCI based multi-TRP/panel in the CC list, and it seems that the new MAC-CE introduced for supporting single DCI based multi-TRP/panel, i.e. Enhanced TCI States Activation/Deactivation for UE-specific PDSCH MAC CE, applies to one specific CC/BWP even in the case when the CC/BWP is included in the CC list by RRC configuration.

Question: Is the following understanding is correct?

- By current RAN2 specification, it is not precluded to enable single DCI based multi-TRP/panel operation in one or more CC(s)/BWP(s) included in simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2 by using Enhanced TCI States Activation/Deactivation for UE-specific PDSCH MAC CE.

Actions to RAN WG2

RAN1 respectfully ask RAN2 to answer the above question.

Date of next TSG-RAN WG1 meetings:

TSG-RAN WG1 Meeting#103		 26th OCT to 13th NOV 2020