3GPP TSG RAN WG1 #103-e			R1-2007505

e-Meeting, October 26th – November 13th, 2020

3GPP TSG-RAN WG4 Meeting #96-e                 	                                                                                       R4-2011931

Electronic Meeting, 17 Aug – 28 Aug, 2020

Title:	LS reply to RAN1on UE capability on wideband carrier operation for NR-U

Response to:	LS on UE capability on wideband carrier operation for NR-U (R1-2004965)

Release:	Rel-16

Work Item:	NR_unlic-core

Source:	RAN WG4

To:	RAN WG1, RAN WG2

Cc:	-

Contact Person:

#### Name:	Tsang-wei Yu

E-mail Address:	ato.yu@mediatek.com

Attachments:	-

1. Overall Description:

RAN4 would like to thank RAN1 for the reply LS on UE capability on wideband carrier operation for NR-U. In RAN4#96-e meeting, RAN4 has discussed RAN1’s questions and agreed on the following response.

Question 1: Is there any difference in DL reception among DL Cases 1, 2a, 2b, 2, and 3 with respect to AGC when at least one of the sub-bands of a [BW or carrier] is not part of gNB’s acquired channel occupancy and contains interference from devices other than the UE’s serving gNB e.g. near-by WiFi AP? Does RAN4 think AGC issues may prevent UE to meet RAN4 requirements for Mode 2 and Mode 3?

- RAN4 response: Performance degradation is expected if the subband is occupied by interference from devices other than the UE’s serving gNB, e.g., near-by WiFi AP, for Case 2a/2b/3/4. RAN4 has not defined corresponding RF in-channel selectivity requirements nor RRM requirements for filter adaptation for Case 2a/2b/3/4 in Rel-16.

Question 2a: Is there a difference in UE capability between any of DL Cases 2a/2b and DL Case 3?

- RAN4 response: Yes, as indicated by the UE feature 4-1 in R4-2011680. UE could support DL Case 3 only if 4-1 is supported when DL intra-band guard bands are configured.

Question 2b: Is there a difference in UE capability between any of DL Cases 2a/2b/3 and DL Case 4?

- RAN4 response: RAN4 has not reached consensus yet.

Question 2c: Is there a difference in UE capability between any of DL Cases 2a/2b/3/4 and DL Case 1?

- RAN4 response: Yes. DL Case 1 (CA) is an independent UE capability.

Question 3: From RAN4 point of view, does “all LBT sub-bands” for Mode 1 refer to LBT sub-bands of configured carrier or BWP?

- RAN4 response: Current NR considers requirements related to the carrier and not the BWP. RAN4 are of the understanding that it shall be all LBT sub-bands per configured carrier for DL.

Question 4: Is change of transmit filtering required (as shown in Figure 1 for WB Mode 2B) to meet RAN4 requirements for any of UL Cases 1-3?

- RAN4 response: No for all cases

Question 5: Is there any difference if intra-cell GBs between scheduled contiguous sub-bands are scheduled or not?

- RAN4 response: There is no difference in RF requirement if intra-cell GBs between scheduled contiguous sub-bands are scheduled or not. RAN4 has removed the capability for UE transmission in UL intra-cell GBs. It can be assumed that there is no restriction in scheduling within the intra-cell GB between two scheduled adjacent RB-sets.

Finally, if the answer to any of Questions 2a/2b/2c/4/5 is yes and capabilities for any of the cases are deemed needed, RAN1 would like to request RAN4 to define the corresponding UE capabilities.

- RAN4 response: RAN4 would like to further understand RAN1’s intention of the action in the LS: whether RAN1 is asking RAN4 to confirm the feature groups [10-19a], [10-19b], [10-19c], [10-19d], [10-19e], [10-19f] in RAN1 feature list R1-2004970 or RAN1 is asking RAN4 to define new UE capabilities in RAN4 feature list, if needed.

2. To RAN WG1 and WG2 group.

ACTION: RAN4 kindly ask RAN1 and RAN2 take above information in to consideration.

3. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #97-e                	26 Oct – 13 Nov 2020         			 E-meeting