3GPP TSG-RAN WG1 #103-e		  	 R1-2009586

e-Meeting, October 26th – November 13th, 2020

Title:	LS on updated Rel-16 RAN1 UE features lists for NR

Response to:

Release:	Rel-16

Work Items:	NR_2step_RACH-Core, NR_unlic-Core, NR_IAB-Core, 5G_V2X_NRSL-Core, NR_L1enh_URLLC-Core, NR_IIOT-Core, NR_eMIMO-Core, NR_UE_pow_sav-Core, NR_pos-Core, NR_Mob_enh-Core, LTE_NR_DC_CA_enh-Core, TEI-16, NR_CLI_RIM-Core

Source:	RAN WG1

To:	RAN WG2, RAN WG4

CC:

Contact Person:

Name:	Hiroki Harada, Ralf Bendlin

E-mail Address:	hiroki.harada@docomo-lab.com, ralf_bendlin@labs.att.com

Attachment:	             R1-2009585.zip (Updated version of RAN1 UE features list for Rel-16 NR)

1. Overall Description:

Based on the agreements made in the RAN1#103-e meeting, RAN1 agreed to send the updated version of NR UE features list to RAN2/4 to facilitate their work on defining capability signaling. RAN1 kindly would like to ask RAN2 and RAN4 to take into account the list of RAN1 NR UE features for designing corresponding capability signalling in Rel.16.

Following are some notes regarding the NR features list:

- Regarding NR-U UE features 10-1 to 10-2e

  - RAN1 has defined basic feature groups according to NR-U deployment scenarios in TS38.300 [1] and channel access modes, i.e. dynamic channel access mode or semi-static channel access mode.

  - If a feature group is agreed to be a basic feature group, it is clarified in the “mandatory/optional” column in the attached RAN1 UE feature that this FG is part of a basic operation for which deployment scenario(s) in which channel access mode.

  - RAN1 respectfully asks RAN2 to adopt the following text proposal to Annex B.3 of TS38.300 [1].

# TS38.300, Annex B (informative): Deployment Scenarios # B.3 NR Operation with Shared Spectrum NR Radio Access operating with shared spectrum channel access can support the following deployment scenarios: - Scenario A: Carrier aggregation between NR in licensed spectrum (PCell) and NR in shared spectrum (SCell); - Scenario A.1: SCell is not configured with uplink (DL only). - Scenario A.2: SCell is configured with uplink (DL+UL). - Scenario B: Dual connectivity between LTE in licensed spectrum and NR in shared spectrum (PSCell); - Scenario C: NR in shared spectrum (PCell); - Scenario D: NR cell in shared spectrum and uplink in licensed spectrum; - Scenario E: Dual connectivity between NR in licensed spectrum (PCell)and NR in shared spectrum (PSCell). Carrier aggregation of cells in shared spectrum is applicable to all deployment scenarios.

RAN1 will share further updated version of the UE features list if any with RAN2/4 according to the further RAN1 discussion in next meeting.

2. Actions:

To RAN WG2

ACTION:

- RAN WG1 kindly would like to ask RAN WG2 to take into account the updated list of RAN1 NR UE features for designing corresponding capability signalling in Rel.16.

- RAN WG1 respectfully asks RAN WG2 to adopt the provided text proposal to Annex B.3 of TS38.300.

To RAN WG4

ACTION:

- RAN WG1 kindly would like to ask RAN WG4 to take into account the updated list of RAN1 NR UE features for designing corresponding capability signalling in Rel.16.

3. Date of Next RAN WG1 Meetings:

TSG-RAN WG1 Meeting #104-e			25th January – 5th February 2021		E-meeting.