3GPP TSG RAN WG1 #103-e		R1-2009848

e-Meeting, 26th October – 13th November, 2020

Title:	LS on signalling method for TRS/CSI-RS occasion(s) for idle/inactive UE(s)

Response to:

Release:	Release 17

Work Item:	NR_UE_pow_sav_enh-Core

Source:	RAN1

To:	RAN2

Contact Person:

Name: Taehyoung Kim

E-mail Address:	th86819.kim@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

In RAN1#103-e, RAN1 agrees the high level principle on signalling method for the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE (s) in Rel-17 UE power saving enhancements. RAN1 would like to inform RAN2 the following related agreements:

Agreements:

- SIB signalling provides the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s).

  - Up to RAN2 to decide which SIB is to be used.

  - Whether or not to additionally support other high-layer signalling methods (e.g., dedicated RRC, RRC release message, etc.) is up to RAN2

For the additional information, RAN1 would like to inform RAN2 the following related agreements made in RAN1#102-e:

Agreements:

The TRS/CSI-RS occasion(s) that may be for connected mode UEs can be shared to idle/inactive mode UEs.

-  Note: It is understood that gNB can potentially share the occasions to idle/inactive (which would just mean it up to NW whether to share or not share).

-  Note: It is understood that TRS/CSI-RS in the TRS/CSI-RS occasion(s) may or may not be transmitted.

-  Note: Always-on TRS/CSI-RS transmission by gNodeB is not required

-  At least TRS/CSI-RS occasion(s) corresponding to periodic TRS is supported

- FFS for other RS types

-  FFS: Whether UE blind detection is required or not.

Agreements:

Further study whether and how to inform the availability of TRS/CSI-RS to idle/inactive mode UE (implicitly or explicitly).

- Note: Availability corresponds to the information for whether TRS/CSI-RS is actually transmitted or not.

RAN1 would like to inform RAN2 that based on agreements in RAN1#102-e and RAN1#103-e, RAN1 is still discussing the availability of the potential TRS/CSI-RS occasion(s) after the TRS/CSI-RS occasion(s) are configured by SIB.

2. Actions:

To RAN2:

RAN1 respectfully asks RAN2 to take the signalling method for the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s) into account in the corresponding RAN2 study.

3. Date of Next RAN1 Meetings:

TSG-RAN WG1 Meeting 104-e	25 Jan. – 05 Feb. 2021 	 		Online

TSG-RAN WG1 Meeting 104bis-e	12 – 20 April 2021 	 		Online