3GPP TSG RAN WG1 #104-e		R1-2100010

e-Meeting, January 25th – February 5th, 2021

3GPP RAN WG4 Meeting #97e	R4-2017847

Electronic, November 2 – 13, 2020

# Title:	LS response on cell-grouping UE capability for synchronous NR-DC

# Response to:	(R2-2008662) LS on cell-grouping UE capability for synchronous NR-DC from RAN2

# Release:	Rel-16

# Work Item:	LTE_NR_DC_CA_enh-Core

Source:	RAN4

To:	RAN2

Cc:	RAN1

Contact Person:

Name:	Anatoliy Ioffe

E-mail Address:	aioffe@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN4 thanks RAN2 for the information related to the agreement on the release-16 UE capabilities for NR-DC and for the request for a view on the need of cell grouping UE capability signaling, including the need of differentiation between MCG and SCG.

RAN4 has discussed this issue and determined the following:

- Inter-band NR-DC configurations with three or more bands, including at least one FR2 band, are already defined in the Rel-16 version of TS38.101-3, with more configurations requested as part of the Rel-17 work item “Rel-17 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL” [RP-201541]

  - The UE may not support FR2 MCG and thus there might be need to indicate whether FR2 MCG is supported in NR-DC if RAN4 has such potential band combinations.

  - The UE may not support FR1-FR2 CA and thus there might be need to differentiate whether it is a FR1-FR2 CA or a FR1-FR2 DC

- Configurations for inter-band NR-DC with three or more bands, where all bands are within FR1, are also requested as part of the Rel-17 work item “Rel-17 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL” [RP-201541]

  - The UE may not support inter-CG power sharing for a given frequency range.

  - The UE may not support PUCCH group across serving cells with different numerologies

- RAN4 considers cell grouping UE capability signaling for synchronous NR-DC beneficial in these scenarios

RAN4 considers that it is beneficial to define cell-grouping UE capability also for synchronous NR-DC so as not to limit support of deployment options. RAN4 further considers that differentiation between MCG and SCG is also beneficial in the related signaling design.  The same cell grouping signaling structure can apply for both synchronous and asynchronous NR-DC for a given band combination.

2. Actions:

To RAN2

ACTION: 	RAN4 respectfully asks RAN2 to take the above information into consideration.

3. Date of Next RAN4 Meetings:

RAN4#98e	25 January – 5 February, 2021		Electronic

RAN4#98bis-e	12 April – 20 April, 2021		Electronic