3GPP TSG RAN WG1 #104-e			R1-2100024

e-Meeting, January 25th – February 5th, 2021

3GPP TSG-RAN WG2 Meeting #112 Electronic	R2-2010837

Elbonia, 02 – 13 November 2020

Title:	Reply LS on propagation delay compensation enhancements

Response to:	-

Release:	Release 17

Work Item:	NR_IIOT_URLLC_enh-Core

Source:	TSG RAN WG2

To:	TSG RAN WG1

Cc:

Contact Person:

#### Name:	Ping-Heng Wallace KUO

E-mail Address:	Ping-Heng.Kuo@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 would like to thank RAN1 for providing information relating the agreements on time-synchronization, as well as asking RAN2 for feedback on synchronicity budget of Uu interface in R1-2007446. Based on the RAN1 agreements, RAN2 has discussed and agreed to focus on the scenarios from the two representative use cases agreed in RAN1 (control-to-control and smart grid):

- Control-to-Control: In the control-to-control communication use case, where TSC devices behind a target UE are synchronized to any TD, from a GM behind the UE. The 5GS introduced error is caused by the relative time-stamping inaccuracies at the involved DS-TTs.

- Smart Grid: In the smart grid use case, where the TSC devices behind a target UE are synchronized to the 5G GM TD. The 5GS introduced error is caused by the synchronization of the 5G clock to the DS-TT.

It is noted that two Uu interfaces are involved in the Control-to-control scenario and one Uu interface is involved in the Smart Grid scenario. The agreed synchronicity budget per Uu interface is tabulated below:

| Scenario | Single Uu interface Budget |
| --- | --- |
| Control-to-Control | ±145ns to ±275ns |
| Smart Grid | ±795ns to ±845ns |

These values are determined with assumptions such that network-side synchronization for Control-to-control is based on gPTP and Smart Grid is based on GNSS. The Uu interface time synchronization budget can be interpreted as the maximum 5GS time synchronization error between the UE and the gNB-DU. RAN2 agrees that the decision on PDC option is up to RAN1, and RAN1 should aim to meet the most stringent requirement among these scenarios, but a number within the range is also acceptable. RAN2 would like to point out that the error caused by the limited granularity of referenceTimeInfo-r16 IE (±5ns) is already included in the network part budget, and RAN1 should not include this error in Uu interface again.

2. Actions:

To RAN1

ACTION: 	RAN2 respectfully asks RAN1 to take the above into account for the future work on propagation delay compensation for time synchronization.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP RAN2#113e		25 January – 5 February 2021			Electronic Meeting