3GPP TSG RAN WG1 Meeting #104-e			R1-2102209

e-Meeting, January 25th – February 5th, 2021

Title:	LS on Agreements Pertaining to L1/L2-Centric Inter-Cell Mobility

Response to:

Release:	Rel-17

Work Item:	NR_feMIMO-Core

Source:	RAN1, Samsung

To:	RAN2

Cc:	RAN3, RAN4

Contact Person:

#### Name:	Eko Onggosanusi

E-mail Address:	eko.o@samsung.com

1. Overall Description:

RAN1 discussed the support of L1/L2-centric inter-cell mobility.

The following agreements were made in RAN1 #102-e, #103-e, and 104-e meetings related to the support of L1/L2-centric inter-cell mobility.

- [Issue 2] For Rel.17 NR FeMIMO, on L1/L2-centric inter-cell mobility:

  - In RAN1#103-e, finalize scope and use cases for L1/L2-centric inter-cell mobility, including:

    - Applicability in various non-CA and CA setups such as intra-band and inter-band CA

    - Use cases in comparison to Rel.15 L3-based handover (HO) taking into account potential extension of DAPS-based Rel.16 mobility enhancement to FR2-FR2 HO

    - The extent of RAN2 impact (MAC CE, RRC, user plane protocols)

    - Network architecture, e.g. NSA vs. SA, inter-RAT scenarios

  - In RAN1#103-e, depending on the outcome of 2a), further identify additional components –along with the associated alternatives –required for supporting inter-cell mobility based on the same unified TCI framework as that for intra-cell mobility (including dynamic TCI state update signaling), including

    - Method(s) for incorporating non-serving cell information associated with TCI

    - Method(s) for DL measurements and UE reporting (e.g. L1-RSRP) associated with non-serving cell(s)

    - UE behavior for reception of signals and non-UE-specific control and data channels associated with non-serving cell(s)

    - UL-related enhancements, e.g. related to RA procedure including TA

    - Beam-level event-driven mechanism for L1/L2-centric inter-cell mobility

On Rel-17 enhancements to enable L1/L2-centric inter-cell mobility:

- The following use cases are assumed:

  - Network architecture:

    - NSA, i.e. LTE PCell and NR-PSCell

    - SA

  - Intra-band CA

    - FFS: If inter-band CA is also included

  - Intra- RAT (excluding inter-RAT)

  - Intra-frequency scenario:

    - The SSBs of non-serving cells have the same center frequency and SCS as the SSBs of the serving cell

    - An SSB of a non-serving cell is associated with a PCI different from the PCI of the serving cell

    - FFS: Support for inter-frequency scenario

  - FFS: Whether to support intra-DU only operation, or whether inter-DU is also allowed

- The following enhancement scope is assumed:

  - Facilitating measurement and reporting of non-serving RSs via incorporating non-serving cell info with some TCI(s), along with the necessary measurement and reporting scheme(s)

    - FFS: Detailed/exact method(s)

    - FFS: Whether this also implies the support of beam indication (TCI state update along with the necessary TCI state activation) for TCI(s) associated with non-serving cell RS(s)

    - FFS: Metric for the measurement and reporting, e.g. L1-RSRP or L3-RSRP or time- or spatial-domain-filtered L1-RSRP

    - FFS: Beam-level event-driven mechanism, using serving cell RS and/or non-serving cell RS

  - Facilitate serving cell to provide configurations for non-serving cell SSBs via RRC

    - FFS: details for the configurations, e.g. time/frequency location, transmission power, etc.

    - FFS: other information needed for inter-cell mobility

  - Note: In RAN1's understanding, non-serving cell SSB and non-serving cell RS can be part of the serving cell configuration

- FFS: The following enhancement scope is assumed by RAN1:

  - Whether RRC reconfiguration signaling is needed or not when a TCI associated with non-serving cell RS is indicated

    - A non-serving cell RS is an RS that is or has an SSB of a non-serving cell as direct or indirect QCL source

    - This implies no C-RNTI update when UE receives DL channel RS associated to non-serving cell RS as QCL source.

    - FFS whether TCI associated with non-serving cell can be indicated to or are applicable for all channels.

  - Whether some RRC parameters need to be updated without additional RRC signaling, e.g. some RRC parameters are pre-configured, which are associated with TCI states with neighbor cell RS as QCL source

  - Whether UE needs/can change serving cell during L1/L2-centric inter-cell mobility.

  - The above assumption to be verified by RAN2

On Rel.17 multi beam measurement/reporting enhancements for L1/L2-centric inter-cell mobility and inter-cell mTRP:

- A quality of up to K beams associated at least with non-serving cell(s) can be reported in a single CSI reporting instance

  - For each beam, the UE can report at least: (1) a Measured RS Indicator, and (2) a Beam Metric associated with the Measured RS Indicator

  - FFS: Maximum value of K

  - FFS: If K is fixed, configured, reported by UE capability, or dynamically selected

  - FFS: The type of beam metric (e.g. L1-RSRP, L3-RSRP, or hybrid L1/L3-RSRP) and related measurement behavior

  - FFS: Whether or not beam reporting associated with non-serving cell(s) can be mixed with that with serving-cell in one reporting instance

At the end of RAN1#104-e, send an LS to RAN2 with all the RAN1-related inter-cell mobility agreements done so far during Rel17.

On Rel.17 multi beam measurement/reporting enhancements for L1/L2-centric inter-cell mobility and inter-cell mTRP:

- Rel.15 L1-RSRP is used as reporting quantity for measurement and reporting of non-serving-cell(s)

  - Support SSB as a measurement RS for L1/L2-centric inter-cell mobility and inter-cell mTRP, and Rel.15 SS-RSRP calculated from SSB of non-serving cell(s)

    - FFS: Whether the measurement for SS-RSRP is limited within SMTC

    - FFS: Detailed reporting method, e.g. via including existing L1-RSRP report, UE-initiated report etc.

  - FFS: Whether or not to support CSI-RS (for e.g. mobility and/or tracking) of non-serving cell(s) as a measurement RS for L1/L2-centric inter-cell mobility and inter-cell mTRP. If the support of CSI-RS (for e.g. mobility and/or tracking) of non-serving cell(s) as a measurement RS for L1/L2-centric inter-cell mobility and inter-cell mTRP is confirmed, Rel.15 CSI-RSRP is also supported

    - Whether the support applies to CSI-RS with or without QCL source, or both

  - FFS: The number of non-serving cell(s) for measurement/reporting

  - FFS: time behavior of the reporting, i.e. periodic, semi-persistent, aperiodic, or UE-initiated

- FFS: If other reporting quantities are supported, e.g. L3-RSRP, hybrid L1/L3-RSRP

- FFS: Dynamic activation/deactivation/selection of the beam measurement on the RS(s) associated with non-serving cell(s) via MAC CE

- FFS: Timing assumption (e.g. time of arrival and time of the measurement) for measurement of non-serving cell RS measurement

On Rel.17 enhancements for L1/L2-centric inter-cell mobility,

- Discuss whether to support at least the source RS types already agreed for intra-cell mobility for the purpose of referencing to non-serving cell(s). Note: This implies the following source RS(s):

  - CSI-RS for BM configured for non-serving cell(s) for DL QCL and UL TX spatial references

  - CSI-RS for tracking (TRS) configured for non-serving cell(s) for DL QCL and UL TX spatial references

  - SSB configured for non-serving cell(s) for UL TX spatial references

  - SRS for BM configured for non-serving cell(s) for UL TX spatial references

  - FFS: whether to support CSI-RS for mobility

  - FFS: whether to support other source RS(s) potentially agreed later for intra-cell mobility

  - FFS: whether to support CSI-RS for BM and tracking configured for non-serving cell(s) and without non-serving cell SSB as QCL-TypeD source

2. Actions:

To: RAN2

ACTION: 	RAN1 respectfully asks RAN2 to take the above into account.

3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #104bis-e                       12th – 20th April, 2021		E-meeting