3GPP TSG RAN WG1 Meeting #104bis-e	R1-2102359

E-meeting, April 12th – April 20th, 2021

Agenda Item:	8.9.3

Source:	Huawei, HiSilicon

Title:	Support of a max DL TBS of 1736 bits in LTE-MTC

Document for:	Discussion and Decision

# Introduction

In RAN1#104-e, the following agreement has been  reached as shown below:

Agreement The number of soft channel bits is calculated based on the equation: Working Assumption: N=8

In this contribution, we discuss the soft bits for max DL TBS of 1736 bits regarding the above working assumption.

# Discussion

In RAN1#104-e, the calculation of soft channel bits of maximum TBS of 1736 bits has been agreed following the Rel-13. Cat. M1 method. In Rel-13, the soft channel bits of Cat. M1 was calculated based on 8 HARQ processes and the maximum TBS of 1000 bits [1]. In addition, in Rel-14, with the introduction of 10 HARQ processes, the soft buffer size is still calculated assuming eight HARQ processes. It has been captured in TS 36.306 as below.

Table 4.1A-1: Downlink physical layer parameter values set by the field ue-CategoryDL

| UE DL Category | Maximum number of DL-SCH transport block bits received within a TTI (Note 1) | Maximum number of bits of a DL-SCH transport block received within a TTI | Total number of soft channel bits | Maximum number of supported layers for spatial multiplexing in DL |
| --- | --- | --- | --- | --- |
| DL Category M1 | 1000 | 1000 | 25344 | 1 |

From our point of view, N = 8 is sufficient even for 14 HARQ processes. There is very low probability that more than 8 HARQ processes fail successively, and assuming N > 8 increases UE complexity. Therefore, we propose to confirm the working assumption.

Proposal 1：Confirm the working assumption as below.

The number of soft channel bits is calculated based on the equation:

Where N=8

Thus, the soft buffer size to support maximum TBS of 1736 bits can be calculated as: (1736+24+4)/32*32*3*8=43008 bits.

Proposal 2: The soft channel bits for UEs supporting maximum DL TBS of 1736 bits is 43008 bits.

# Conclusions

In this contribution, we provide discussions the soft channel bits to support maximum DL TBS of 1736 bits for HD-FDD Cat. M1 UEs in CE mode A, and the following proposals are given:

Proposal 1：Confirm the working assumption as below.

The number of soft channel bits is calculated based on the equation:

Where N=8

Proposal 2: The soft channel bits for UEs supporting maximum DL TBS of 1736 bits is 43008 bits.

# References

- [1]   R1-132877, Analysis on the reduction of peak rate, Huawei, HiSilicon, RAN1#74, Aug., 2013.