3GPP TSG RAN WG1#104b-e													R1-2102362

e-Meeting, April 12th – 20th, 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	Inter-UE coordination in sidelink resource allocation mode 2

Agenda item:	8.11.1.2

Document for:	Discussion and Decision

# Introduction

At RAN#86, a new work item “NR Sidelink enhancement” (NR_SL_enh) was approved ‎[1]. One of the objectives is relevant for the present agenda item:

2. Resource allocation enhancement: - Study the feasibility and benefit of solution(s) on the enhancement(s) in mode 2 for enhanced reliability and reduced latency in consideration of both PRR and PIR defined in TR37.885 (by RAN#91), and specify the identified solution(s) if deemed feasible and beneficial [RAN1, RAN2] - Inter-UE coordination with the following. - A set of resources is determined at UE-A. This set is sent to UE-B in mode 2, and UE-B takes this into account in the resource selection for its own transmission. - Note: The solution should be able to operate in-coverage, partial coverage, and out-of-coverage and to address consecutive packet loss in all coverage scenarios. - Note: RAN2 work will start after RAN#89.

In this contribution, we provide our views on enhancements in sidelink resource allocation mode 2.

# Discussion

In RAN1#103-e the following conclusions were reached in relation to the present agenda item:

Conclusion: - The schemes of inter-UE coordination in Mode 2 are categorized as being based on the following types of “A set of resources” sent by UE-A to UE-B: - UE-A sends to UE-B the set of resources preferred for UE-B’s transmission - e.g., based on its sensing result - UE-A sends to UE-B the set of resources not preferred for UE-B’s transmission - e.g., based on its sensing result and/or expected/potential resource conflict - UE-A sends to UE-B the set of resource where the resource conflict is detected - FFS: details of resource conflict, e.g., including type of resource conflict - FFS: details of sensing operation at UE-A side - FFS: which type(s) of resource set information is(are) beneficial/feasible to which cast type(s) - Note: these different types may be used in combination with each other - From RAN1 perspective, further study on the feasibility/benefit of inter-UE coordination is required - Send an LS to RAN plenary

Conclusion: - For the schemes of inter-UE coordination identified as feasible/beneficial, at least the following aspects are further discussed. - How/when UE-A determines the contents of “A set of resources”, including consideration of UL scheduling - When UE-A sends “A set of resources” to UE-B, including which UE(s) sends it - How UE-A and UE-B are determined - How UE-A sends “A set of resources” to UE-B, including container used for carrying it, implicitly or explicitly or both - How/when/whether UE-B receives “A set of resources” and takes it into account in the resource selection for its own transmission - How/whether to define the relationship between support/signaling of inter-UE coordination and cast type

## How UE-A determines “a set of resources”

### On details of non-preferred resources

The acquisition of information about non-preferred resources on UE-A’s side depends on which UE(s) is/are the intended receiver(s) of UE-B’s transmission, specifically:

- UE-A is the only intended receiver of UE-B’s transmission; or

- At least one other UE (e.g., UE-C) is an intended receiver of UE-B’s transmission.

In both cases, the type of non-preferred resource can at least include:

- Half-duplex – Resources where the intended receiver is expected to be transmitting;

- Unavailability – Resources where the intended receiver is unavailable, e.g., during receiving UE DRX Off period, etc.;

- Reception resources of the intended receiver – Resources where it is known that the intended receiver will be receiving transmissions as the intended receiver, e.g., due to known periodic transmissions;

- Interference – Resources where it is known that the intended receiver will be interfered by other UEs’ transmissions.

The resources affected by I, II, III and IV are expected to be known at the intended receiver, which for case a) does not require UE-A to acquire any additional information from an external source. However, for case b), UE-A needs to be made aware of the non-preferred resources at the intended receiver.

Observation 1: When UE-A is not the intended receiver (or not the only intended receiver), then it needs to acquire information about the non-preferred resources at the (other) intended receiver(s).

The information of the non-preferred resources can be requested by UE-A from the intended receiver or it can be provided directly without request to UE-A. The case where this information is made available to UE-B directly from the intended receiver corresponds in practice to application of inter-UE coordination case a).

We note that, if UE-A is made aware of the identity used by the intended receiver(s), then it can in principle infer some of the resource conflicts type I and III by active monitoring of the resource pool (i.e., by detecting SCIs containing that identity). The main benefit of this approach is that it can lead to more efficient signaling, as only the ID of the intended receiver needs to be provided compared to signaling the non-preferred resources type I and III.

Observation 2: The acquisition of the ID(s) used by the intended receiver(s) allows UE-A to obtain from its sensing of the resource pool (including decoding of 2nd stage SCI), transmissions of the intended receiver as well as transmissions from other UEs to the intended receiver.

Note that a UE can use different IDs when communicating with different UEs and cast types. However, it is expected that these IDs will change on a much longer time scale than that of inter-UE coordination.

Additionally, if the UE-A is also made aware of UE-B’s ID(s) then it can also use this information to detect, based on pool monitoring, which resources the UE-B would not be able to use for a transmission to its intended receiver. For example, detection of future reserved SL communications with a high priority between UE-B and another UE other than the intended receiver. This information could then be used by UE-A to optimize the set of non-preferred resources to UE-B, e.g. by excluding the resources that would anyway not be used by UE-B due to half duplex or simultaneous SL transmissions.

Observation 3: The acquisition of the ID(s) of UE-B allows UE-A to obtain from its sensing of the resource pool (including decoding of 2nd stage SCI), ongoing communications between UE-B and another UE other than the UE-B intended receiver.

Proposal 1: RAN1 to consider the indication to UE-A of the ID(s) used by UE-B and the intended receiver(s) of UE-B’s transmission to enable UE-A to identify non-preferred resources directly from its resource pool monitoring.

### On details of preferred resources

As discussed in 2.1, the acquisition of information about the “preferred resources” depends on which UE is the intended receiver of UE-B’s transmission, specifically:

- UE-A is the only intended receiver of UE-B’s transmission; or

- At least one other UE (e.g., UE-C) is an intended receiver of UE-B’s transmission.

In case a), the preferred resources are identified from UE-A’s own sensing. In case b), UE-A should be made aware of UE-C’s preferred resources.

For case b), UE-C’s preferred resources can be indicated explicitly by UE-C to UE-A or instead UE-C can indicate its used IDs to UE-A. In the latter case, UE-A can then obtain, directly from its sensing of the resource pool, based on the indicated IDs of UE-C, what are the transmissions that UE-C will be performing and then exclude the corresponding resources from the preferred resource set. As the preferred resources from UE-C may be quite dynamic, whereas UE-C’s IDs are updated less frequently, it is more efficient in terms of signaling overhead to allow the intended receiver to indicate to UE-A all the IDs instead of the preferred resources.

Proposal 2: RAN1 to consider the indication to UE-A of the ID(s) of the intended receiver, to allow UE-A to determine the preferred resource set.

UE-B can also experience the following types of resource conflict, where it is not able to perform a transmission indicated by UE-A:

- Concurrent transmission on Uu – Slots in which UE-B has a concurrent higher priority transmission on the Uu interface and due to UE capability or power limitations is unable to perform the PC5 transmission;

- Concurrent transmission on PC5 – Slots in which UE-B has a concurrent transmission (e.g., from an existing pre-reserved resource) and hence it is unable to perform an additional PC5 transmission (i.e., in another sub-channel);

- Half-duplex due to reception – Slots in which UE-B has to perform SL reception.

When UE-A indicates the preferred resources, UE-B then has to exclude all the resources in which it has detected a conflict (e.g., due to its own concurrent transmissions or receptions, defined respectively as resource conflict type V, VI and VII). This, of course, is not very signaling-efficient; therefore, it is desirable that UE-A be made aware of the resource conflicts at UE-B and then exclude the corresponding resources from the preferred resource set prior to informing UE-B.

The information of UE-B’s conflicts can be indicated explicitly by UE-B to UE-A or instead UE-B can indicate its used IDs to UE-A. UE-A can then, at least for resource conflicts type VI and VII, obtain directly from its sensing of the resource pool what are the transmissions that UE-B will be performing as well as in which resources UE-B will be receiving; and from that, exclude the corresponding resources from the preferred resource set. Again, the change of conflicting resources is more dynamic than the update of UE-B’s IDs. Therefore, it is more signaling-efficient to indicate to UE-A the relevant IDs of UE-B instead of the list of conflicting resources.

Proposal 3: RAN1 to consider the indication to UE-A of the ID(s) of UE-B, to allow UE-A to optimize the preferred resource set.

It is noted that UE-B indicating its ID(s) to UE-A may also be beneficial in other aspects besides inter-UE coordination, e.g., in terms of SL DRX. For example, based on the obtained ID(s), UE-A may detect UE-B is active in receiving from a third UE, upon monitoring a transmission from the third UE to UE-B with the obtained ID(s). Thus, if the data towards UE-B is generated at UE-A outside the SL DRX On-duration of UE-B for unicast communication from UE-A to UE-B, UE-A may attempt to transmit to UE-B immediately, without waiting for the next SL DRX On-duration of UE-B. It is noted that there may be different reasons for UE-B to be active outside the configured SL DRX On-duration between UE-B and UE-A, e.g., the DRX inactivity timer is running at UE-B due to its communication with the third UE, or if UE-B applies a different SL DRX configuration to communicate with the third UE.

### On details of sensing operation at UE-A

For UE-A to be able to provide the best set of preferred resources, UE-A will need to perform full sensing of the resource pool. In case UE-A only performs partial sensing, then the resources that are not sensed will not be part of the preferred resources indicated by UE-A.

Furthermore, in order to be able to detect non-preferred and conflicting resources directly from the sensing procedure, it is desirable that UE-A be aware of UE-B’s and UE-C’s (in case UE-A is not the intended receiver of UE-B’s transmission) ID(s). This implies that, during the sensing procedure of UE-A, UE-A should monitor both PSCCH and PSSCH so that it is able to decode 1st and 2nd stage SCIs.

Proposal 4: RAN1 to consider that during UE-A’s sensing for the purpose of inter-UE coordination, UE-A decodes both 1st and 2nd stage SCIs and from these infers potential resource conflicts at UE-B as well as non-preferred resources at UE-B’s intended receiver.

## When UE-A sends “a set of resources”, including which UE sends it

### Information carried by explicit coordination request

A. Set of resources (preferred or non-preferred) determined at UE-B

As shown in Figure 1, UEs within a group (e.g., a platoon) may coordinate to select and reserve resources for a certain time duration. For example, as shown in Figure 1(a), a group leader (UE-A) may propose a “set of resources” to be allocated to the (other) group members (UE-B1 and UE-B2), e.g., based on coordination requests received from the group members. Similarly, as shown in Figure 1(b), a group leader (UE-B) may send a groupcast coordination request and receive coordination messages from the (other) group members (UE-A1 and UE-A2).

In order to optimize the “set of resources” determined at UE-A, a UE-B may provide to UE-A in its coordination request message a set of preferred or non-preferred resources determined at UE-B. In this way, the “set of resources” subsequently determined at UE-A may already be optimized from UE-B’s perspective. For example, if UE-A determines a set of preferred resources, it may exclude resources that have been indicated as non-preferred by UE-B in the first place.

Proposal 5: The coordination request message from UE-B to UE-A may include a set of preferred or non-preferred resources determined at UE-B.

Figure 1. Inter-UE coordination within a group of UEs

B. Set of resources (preferred or non-preferred) for receiving the coordination message

In order to avoid the half-duplex issue at UE-B in the inter-UE coordination procedure, UE-B should not transmit at the same time as UE-A transmits its coordination message towards UE-B. As a simple solution, UE-B may indicate to UE-A a set of time resources in its coordination request message, where the set of time resources is then taken into account by UE-A to select a resource for transmitting the coordination message to UE-B. As one example, UE-B may indicate a set of time resources where UE-B will monitor the PC5 interface, such that UE-A may select a resource from the indicated set to avoid the half-duplex problem at UE-B.

Proposal 6: The coordination request message from UE-B to UE-A may include a set of preferred or non-preferred time resources for receiving the coordination message, in order to assist UE-A in mitigating the half-duplex issue at UE-B.

### Density/distance-based resource conflict indication triggering

For UE-B’s transmission to its intended receivers, multiple UE-As (that are not the intended receivers) may have detected a resource conflict on a set of resources or may have acquired information on the non-preferred resources. However, multiple UE-As sending to UE-B “a set of resources” about the same resource conflict or non-preferred resource may cause resource wastage since it is sufficient if only one (or a few) UE-A(s) send(s) the conflict indication. It is also worth noting that sending a conflict indication may cause those UE-As to not be able to receive sidelink transmissions concurrently due to the half-duplex constraint, which may not be desirable for many UE-As.

Observation 4: Multiple UE-As sending to UE-B a conflict indication about the same resource conflict may cause resource wastage.

A circumstance where a non-recipient UE-A’s assistance on resource conflicts or non-preferred resources is particularly more likely to be an overhead than an advantage is when there is a large number (or high density) of UEs in the surrounding area of UE-B. Here, it is very likely that many UEs may have the information on the same resource conflict or non-preferred resources for UE-B’s transmission and they may choose to act as UE-As, causing resource wastage. Therefore, to inhibit too many non-recipient UE-As from sending “a set of resources” to UE-B regarding the same resource conflict or non-preferred resource, a probability that depends on UE density and/or distance (to UE-B) can be used to trigger assistance from the UEs. For instance, a UE can be configured with a higher probability to send a conflict indication when there are fewer UEs around, while the UE is configured with a lower probability when the UE density is higher to reduce the likelihood of multiple UEs sending the same assistance information to UE-B. In addition, distance can also be considered as a triggering criterion, e.g., further UEs (from UE-B’s perspective) may be configured with a lower probability of sending the conflict indication.

Proposal 7: RAN1 to consider UE density and/or distance dependent probability for triggering UE-A to send a conflict indication.

## How UE-A sends “a set of resources”

### Signaling format

This section addresses the question of how to efficiently encode the “set of resources” determined by UE-A so as to minimize control signaling overhead. Figure 2 shows an example of a “set of resources” consisting of k=2 selected resources (e.g., preferred or non-preferred) within a resource selection window (RSW) of length T logical slots of a resource pool (RP) consisting of NsubchannelSL subchannels.

Figure 2. Example showing k=2 selected resources within a RSW and RP

We can distinguish the following four cases (A, B, C, D), depending on:

- whether or not the resources in the “set of resources” may overlap in time, and

- whether or not the resource size LsubCH (i.e., number of contiguously allocated subchannels) needs to be signaled to UE-B (e.g., if the resource size has been indicated as a sensing parameter by UE-B to UE-A when requesting the “set of resources”, then UE-B already has this information).

Case A. Resources may not overlap in time, resource size is not signaled

If the selected resources are assumed not to overlap in time (i.e., they always occur in different slots), UE-A may determine a time RIV (TRIV) as follows

|  | TRIV=j=1ktjj=t11+t22+…+tkk | (1) |
| --- | --- | --- |

where 0≤TRIV<Tk and 0≤t1<t2<…<tk<T are the logical slot offsets (in strictly increasing order) of the selected resources with respect to the first logical slot of the RSW. (Note: By convention, uv=0 if u<v.)

In the example shown in Figure 2, the logical slot offsets of the selected resources are t1,t2=3, 7, thus TRIV=31+72=24. This may be seen as a generalization of the TRIV used in Rel-16 SCI format 1-A (see TS 38.214, Section 8.1.5) for an arbitrary k. (Note: A similar “combinatorial index”, albeit in the frequency domain, was introduced in LTE Rel-10 for UL resource allocation type 1, see TS 36.213, Section 8.1.2.)

As there are Tk possible values for the TRIV, the overhead incurred is ΦTRIV=log2Tk bits. Using a combinatorial number system allows UE-B to rapidly compute the k-combination t1,t2,…,tk from the received TRIV without having to explicitly list all preceding k-combinations in the lexicographic ordering.

The TRIV may be used in a standalone manner (e.g., to indicate a set of preferred or non-preferred slots to UE-B), or it may be accompanied by a frequency RIV (FRIV) indicating the starting subchannel indices nsubCH,jstart of the selected resources as follows

|  | FRIV=j=1knsubCH,jstart NsubchannelSL+1-LsubCHj-1 | (2) |
| --- | --- | --- |

where the resource size LsubCH is assumed to be known by UE-B. This results in an overhead ΦFRIV=klog2NsubchannelSL+1-LsubCH bits.

Case B. Resources may not overlap in time, resource size is signaled

If the resource size LsubCH needs to be signaled to UE-B, the FRIV formula used in Rel-16 SCI format 1-A (see TS 38.214, Section 8.1.5) may be generalized for an arbitrary k as follows

|  | FRIV=j=1knsubCH,jstart NsubchannelSL+1-LsubCHj-1+i=1LsubCH-1NsubchannelSL+1-ik | (3) |
| --- | --- | --- |

where the second sum term is used to convey the resource size LsubCH. This incurs an overhead ΦFRIV'=log2i=1NsubchannelSLik bits.

In some scenarios, the selected resources may have different resource sizes LsubCH,1,…,LsubCH,k (e.g., when the “set of resources” is sent to multiple UE-Bs). In this case, the FRIV formula may be further generalized as follows

|  | FRIV=j=1knsubCH,jstart i=1j-1NsubchannelSL+1-LsubCH,i+l<Lj=1kNsubchannelSL+1-lj | (4) |
| --- | --- | --- |

where l=l1,…,lk and L=LsubCH,1,…,LsubCH,k may be compared, for example, according to the lexicographic ordering, e.g., (1,2)<(2,1).

(The TRIV for this case is calculated as in Case A.)

Case C. Resources may overlap in time, resource size is not signaled

If the resources are allowed to occur in the same slot (see Figure 3), UE-A may determine a resource index rj for each selected resource such that 0≤r1<r2<…<rk<n, where n=T×NsubchannelSL+1-LsubCH is the total number of candidate single-slot resources consisting of LsubCH contiguous subchannels.

For example, the resource index for the jth selected resource may be calculated as follows

|  | rj=tj+nsubCH,jstart T | (5) |
| --- | --- | --- |

In this case, the candidate resources are enumerated by considering the time domain first, as shown on the left side of Figure 3. Alternatively, the resource index for the jth selected resource may be calculated as follows

|  | rj=nsubCH,jstart+tj NsubchannelSL+1-LsubCH | (6) |
| --- | --- | --- |

In this case, the candidate resources are enumerated by considering the frequency domain first, as shown on the right side of Figure 3.

Figure 3. Different ways of enumerating candidate resources (n=40, k=5, LsubCH=1)

UE-A may determine a combinatorial RIV (CRIV) for indicating the corresponding k-combination r1,r2,…,rk as follows

|  | CRIV=j=1krjj=r11+r22+…+rkk | (7) |
| --- | --- | --- |

where 0≤CRIV<nk and 0≤r1<r2<…<rk<n are the resource indices (in strictly increasing order) of the selected resources. For the example on the left side of Figure 3, CRIV=01+172+223+304+375=464978. Note that, unlike the TRIV or FRIV, the CRIV conveys both time- and frequency-domain information.

As there are nk possible values for the CRIV, the overhead is ΦCRIV=log2nk bits.

Case D. Resources may overlap in time, resource size is signaled

If the resource size LsubCH needs to be signaled to UE-B, UE-A may jointly encode the resource indices r1,r2,…,rk and resource size LsubCH as follows

|  | CRIV=j=1krjj+i=1LsubCH-1nik | (8) |
| --- | --- | --- |

where ni=T×NsubchannelSL+1-i is the total number of candidate single-slot resources consisting of i contiguous subchannels. The second sum term (similar to the second sum term in the FRIV formulas of Case B) is used to convey the resource size information efficiently: the lowest n1k values signal LsubCH=1, followed by n2k values for LsubCH=2, and so on. In this case, the overhead is ΦCRIV'=log2i=1NsubchannelSL nik bits.

Table 1 provides the overhead (bits) incurred for signaling the “set of resources” in each of the different cases (A, B, C, D) for the following parameters: T=100, NsubchannelSL=27, LsubCH=10.

Table 1. Overhead (bits) incurred for signaling the “set of resources”

Proposal 8: RAN1 to clarify whether (any of) the resources in the “set of resources” determined by UE-A may occur in the same slot.

Proposal 9: RAN1 to clarify whether the resource size LsubCH needs to be signaled to UE-B.

### Dedicated resource pool for inter-UE coordination

Inter-UE coordination can help to increase the reliability of data transmissions in the sidelink shared channel. However, the transmission of coordination messages is itself uncoordinated, and thus may be unreliable due to hidden nodes, half-duplex conflicts, etc. Especially under high system load, coordination messages may frequently collide with data messages (Data-Coordination conflicts) as well as with other coordination messages (Coordination-Coordination conflicts). Without reliable transmission of coordination messages, the expected gains of inter-UE coordination may not be fully realized.

Data-Coordination conflicts may be prevented by imposing a strict separation, in time or frequency, between data message transmissions and coordination message transmissions, similar to what is found in military ad hoc networks [2][3]. As shown in Figure 4, this may be achieved by configuring a dedicated resource pool for Rel-17 inter-UE coordination, used exclusively for transmission of coordination messages. This Coordination Pool may be TDMed or FDMed with the resource pool(s) used for transmission of data messages, i.e., Data Pool(s). In the FDM case, half-duplex conflicts may still occur between data and coordination transmissions (e.g., a UE may not be able to receive a coordination message in a slot if it is transmitting a data message in the same slot). On the other hand, the TDM case may introduce latency in inter-UE coordination, as coordination transmissions can only occur occasionally (e.g., every n-th slot).

Figure 4. Dedicated resource pool for Rel-17 inter-UE coordination

Coordination-Coordination conflicts may also occur within the Coordination Pool (e.g., a UE may be unable to receive a coordination message if it is itself transmitting one). These may be addressed by over-dimensioning the Coordination Pool (e.g., small n in the TDM case), thus reducing the probability of such conflicts. This makes sense because, unlike the Data Pool, which is robust as a result of inter-UE coordination, the Coordination Pool itself, as explained above, is uncoordinated. For example, assume 99% of the total traffic is data and 1% is coordination. Then, instead of dimensioning the Coordination Pool proportionally (i.e., 1% of the total capacity: n=100 slots), one could, for example, over-dimension it by a factor of 10 (i.e., 10% of the total capacity: n=10 slots). In this way, the inter-UE coordination becomes robust (due to low probability of Coordination-Coordination conflicts) even under high system load. In addition, a small n is beneficial in terms of latency for inter-UE coordination. For example, for SCS=60kHz, n=10 slots results in one coordination opportunity every 2.5ms.

Robust coordination may also be achieved without a dedicated Coordination Pool. For example, if coordination messages and data messages share the same pool, robust coordination may be achieved by duplicating coordination message transmissions (i.e., blind retransmission) and/or using more robust MCS. Either way, a price must be paid in terms of spectral efficiency loss in order to maintain robustness. It is not clear which approach (dedicated or shared pool) leads to a higher system capacity or throughput for a given reliability target. Furthermore, it is not clear whether the shared pool approach can really maintain high reliability under high system load.

Observation 5: Transmission of coordination messages may be unreliable (prone to hidden nodes, half-duplex conflicts, etc.). It is unclear whether transmitting coordination messages and data messages in a shared pool can fulfill the reliability requirements under high system load.

Proposal 10: RAN1 to study whether transmission of coordination messages and data messages in a shared pool can fulfill the reliability requirements under high system load.

Observation 6: Robust coordination (and therefore reliability) will come at a price (capacity loss) regardless of whether coordination messages and data messages are strictly separated (dedicated pool) or not (shared pool). It is unclear which approach will achieve higher throughput for a given reliability target.

Proposal 11: RAN1 to study which approach (dedicated or shared pool) achieves higher throughput for a given reliability target.

## Retransmission of decoded SCI by RxUE

According to the RAN1#103-e conclusion, inter-UE coordination also includes resource conflict indication from UE-A to UE-B. As a form of resource conflict indication, a RxUE (acting as UE-A) can retransmit information indicating resources reserved by a TxUE (UE-B) (e.g., when the RxUE detects a resource conflict involving the reserved resources). On the one hand, the retransmission of resource reservation information by UE-A can extend the information transmission range and thus solve the hidden-node problem. On the other hand, as the retransmission is performed in a different slot than that in which UE-B transmitted the SCI reserving the resources, persistent resource conflicts due to the half-duplex issue can be resolved too.

Observation 7: Resource reservation conflicts due to the hidden-node and/or half-duplex problem can be resolved if a RxUE retransmits the resources reserved by a TxUE (e.g., when the RxUE detects a resource reservation conflict).

The NR SL design in Rel-16 uses a 1st-stage SCI to transmit the reserved resources to facilitate SL reception as well as sensing-based mode 2 resource allocation. Considering backward compatibility, in order to allow Rel-16 UEs to also utilize the resource reservation conflict indication, it’s convenient to use a 1st stage SCI to retransmit the resource reservation information. In this way, other UEs (regardless of whether they be Rel-16 or Rel-17) can avoid the resource conflict as part of the normal mode 2 sensing mechanism.

Proposal 12: RAN1 to consider whether a RxUE can retransmit an SCI received from a TxUE (e.g., as a resource reservation conflict indication).

The content of the retransmitted SCI should be adapted in order to reflect the same reserved resources in the time domain. For instance, if the RxUE retransmits the SCI n slots later than the original SCI from the TxUE, the “Time resource assignment” bits of SCI (i.e., the TRIV) should be adapted to indicate the same slots pertaining to the reserved resources as in the original SCI from the TxUE.

Proposal 13: When a RxUE retransmits the original SCI from a TxUE, the RxUE needs to adapt the content of SCI to reflect the same reserved resources as in the original SCI.

# Conclusions

In this contribution, we discussed enhancements in sidelink resource allocation mode 2 and make the following observations and proposals:

Regarding enhanced sensing by UE-A:

Observation 1: When UE-A is not the intended receiver (or not the only intended receiver), then it needs to acquire information about the non-preferred resources at the (other) intended receiver(s).

Observation 2: The acquisition of the ID(s) used by the intended receiver(s) allows UE-A to obtain from its sensing of the resource pool (including decoding of 2nd stage SCI), transmissions of the intended receiver as well as transmissions from other UEs to the intended receiver.

Observation 3: The acquisition of the ID(s) of UE-B allows UE-A to obtain from its sensing of the resource pool (including decoding of 2nd stage SCI), ongoing communications between UE-B and another UE other than the UE-B intended receiver.

Proposal 1: RAN1 to consider the indication to UE-A of the ID(s) used by UE-B and the intended receiver(s) of UE-B’s transmission to enable UE-A to identify non-preferred resources directly from its resource pool monitoring.

Proposal 2: RAN1 to consider the indication to UE-A of the ID(s) of the intended receiver, to allow UE-A to determine the preferred resource set.

Proposal 3: RAN1 to consider the indication to UE-A of the ID(s) of UE-B, to allow UE-A to optimize the preferred resource set.

Proposal 4: RAN1 to consider that during UE-A’s sensing for the purpose of inter-UE coordination, UE-A decodes both 1st and 2nd stage SCIs and from these infers potential resource conflicts at UE-B as well as non-preferred resources at UE-B’s intended receiver.

Regarding contents of the resource coordination request message:

Proposal 5: The coordination request message from UE-B to UE-A may include a set of preferred or non-preferred resources determined at UE-B.

Proposal 6: The coordination request message from UE-B to UE-A may include a set of preferred or non-preferred time resources for receiving the coordination message, in order to assist UE-A in mitigating the half-duplex issue at UE-B.

Regarding resource conflict indication:

Observation 4: Multiple UE-As sending to UE-B a conflict indication about the same resource conflict may cause resource wastage.

Proposal 7: RAN1 to consider UE density and/or distance dependent probability for triggering UE-A to send a conflict indication.

Regarding signalling of “set of resources”:

Proposal 8: RAN1 to clarify whether (any of) the resources in the “set of resources” determined by UE-A may occur in the same slot.

Proposal 9: RAN1 to clarify whether the resource size LsubCH needs to be signaled to UE-B.

Regarding reliability of coordination signaling:

Observation 5: Transmission of coordination messages may be unreliable (prone to hidden nodes, half-duplex conflicts, etc.). It is unclear whether transmitting coordination messages and data messages in a shared pool can fulfill the reliability requirements under high system load.

Proposal 10: RAN1 to study whether transmission of coordination messages and data messages in a shared pool can fulfill the reliability requirements under high system load.

Observation 6: Robust coordination (and therefore reliability) will come at a price (capacity loss) regardless of whether coordination messages and data messages are strictly separated (dedicated pool) or not (shared pool). It is unclear which approach will achieve higher throughput for a given reliability target.

Proposal 11: RAN1 to study which approach (dedicated or shared pool) achieves higher throughput for a given reliability target.

Regarding retransmission of resource reservation:

Observation 7: Resource reservation conflicts due to the hidden-node and/or half-duplex problem can be resolved if a RxUE retransmits the resources reserved by a TxUE (e.g., when the RxUE detects a resource reservation conflict).

Proposal 12: RAN1 to consider whether a RxUE can retransmit an SCI received from a TxUE (e.g., as a resource reservation conflict indication).

Proposal 13: When a RxUE retransmits the original SCI from a TxUE, the RxUE needs to adapt the content of SCI to reflect the same reserved resources as in the original SCI.

# References

- RP-202846, “Revised WID on NR Sidelink enhancement”

- C.D. Young (Rockwell Collins, Inc.) (1999). “USAP Multiple Access: Dynamic Resource Allocation for Mobile Multihop Multichannel Wireless Networking”, Proceedings of IEEE Military Communications Conference (MILCOM), 31 Oct - 3 Nov 1999, Atlantic City, NJ, USA.

- J.B. Cain, T. Billhartz, L. Foore, E. Althouse, and J. Schlorff (Harris Corporation, Naval Research Laboratory) (2003). “A Link Scheduling and Ad Hoc Networking Approach using Directional Antennas”, Proceedings of IEEE Military Communications Conference (MILCOM), 13-16 Oct 2003, Boston, MA, USA.