3GPP TSG RAN WG1 #104b-e	R1-2102488

e-Meeting, April 12th – 20th, 2021

Source: 	ZTE

Title: 	Corrections on issues related to DMRS configuration for DCI format 1_2

Agenda item:	7.2.5

Document for:	Discussion/Decision

Introduction

In Rel-16 URLLC, separate DMRS configuration for PDSCH mapping type A and type B is introduced for DCI format 1_2 as follows [1].

dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2-r16 SetupRelease { DMRS-DownlinkConfig } OPTIONAL, -- Need M dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2-r16 SetupRelease { DMRS-DownlinkConfig } OPTIONAL, -- Need M

In this contribution, the following two issues related to the new introduced DMRS configuration are discussed.

- Issue #1: RRC parameter correction for DM-RS reception procedure

- Issue #2: UE PDSCH processing time

Discussion

Issue #1: RRC parameter correction for DM-RS reception procedure

In Section 5.1.6.2 in TS 38.214, the highlighted RRC parameter for DMRS configuration for DCI format 1_2 is not aligned with that defined in TS 38.331, and should be corrected.

5.1.6.2 DM-RS reception procedure <Unchanged parts are omitted> If a UE receiving PDSCH scheduled by DCI format 1_2 is configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA-ForDCI-Format1-2 or dmrs-DownlinkForPDSCH-MappingTypeB-ForDCI-Format1-2 or a UE receiving PDSCH scheduled by DCI format 1_0 or DCI format 1_1 is configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA or dmrs-DownlinkForPDSCH-MappingTypeB, the UE may assume that the following configurations are not occurring simultaneously for the received PDSCH: - any DM-RS ports among 1004-1007 or 1006-1011 for DM-RS configurations type 1 and type 2, respectively are scheduled for the UE and the other UE(s) sharing the DM-RS REs on the same CDM group(s), and - PT-RS is transmitted to the UE. <Unchanged parts are omitted>

Given the discussing UE behavior applies to all receiving PDSCH configured with phaseTrackingRS in DMRS-DownlinkConfig, the specification description could simply revised back to Rel-15 version. This also aligns with the corresponding description for PUSCH in Section 6.2.2.

Therefore, we propose the following text proposal for Issue#1:

Proposal 1: Adopt Text Proposal #1 below for DM-RS reception procedure.

----------------------------------------Text Proposal #1 for Section 5.1.6.2 in TS 38.214 g50-------------------------------------

- 5.1.6.2 DM-RS reception procedure <Unchanged parts are omitted> If a UE receiving PDSCH scheduled by DCI format 1_2 is configured with the higher layer parameter phaseTrackingRS in DMRS-DownlinkConfig dmrs-DownlinkForPDSCH-MappingTypeA-ForDCI-Format1-2 or dmrs-DownlinkForPDSCH-MappingTypeB-ForDCI-Format1-2 or a UE receiving PDSCH scheduled by DCI format 1_0 or DCI format 1_1 is configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA or dmrs-DownlinkForPDSCH-MappingTypeB, the UE may assume that the following configurations are not occurring simultaneously for the received PDSCH: - any DM-RS ports among 1004-1007 or 1006-1011 for DM-RS configurations type 1 and type 2, respectively are scheduled for the UE and the other UE(s) sharing the DM-RS REs on the same CDM group(s), and - PT-RS is transmitted to the UE. <Unchanged parts are omitted>

Issue #2: UE PDSCH processing time

In Rel-15, PDSCH processing time for PDSCH processing capability 1 and capability 2 depends on whether additional DMRS is configured. However, new DMRS parameters dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 are introduced for DCI format 1_2 in Rel-16, and it hasn’t been reflected in current Rel-16 specification.

Note that, the legacy RRC parameter dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingType only apply to DCI format 1_1. For DCI format 1_0, our understanding is that UE will always use PDSCH processing capability 1 with a longer processing time (right column in Table 5.3-1) since dmrs-AdditionalPosition = 'pos2' is assumed for PDSCH scheduled by DCI format 1_0.

Table 5.3-1: PDSCH processing time for PDSCH processing capability 1 PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB; dmrs-AdditionalPosition ≠ 'pos0' in DMRS-DownlinkConfig in either of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB or if the higher layer parameter is not configured. 0; 8; N1,0. 1; 10; 13. 2; 17; 20. 3; 20; 24. Table 5.3-2: PDSCH processing time for PDSCH processing capability 2 PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB. 0; 3. 1; 4.5. 2; 9 for frequency range 1.

For PDSCH processing capability 1, there could be two options to include the new introduced RRC parameters for DCI format 1_2.

- Option 1: The PDSCH processing time is independent from DCI formats. That is, only when dmrs-AdditionalPosition = 'pos0' is configured in DMRS-DownlinkConfig in all of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2, the PDSCH decoding time N1 follows the shorter processing time (left column in Table 5.3-1). Otherwise it follows the longer processing time for PDSCH processing capability 1 (right column in Table 5.3-1). This would cause a larger processing time even when a PDSCH is scheduled without additional DMRS for PDSCH processing capability 1.

- Option 2: The PDSCH processing time is defined per DCI format. In other words, the PDSCH processing time can be different for PDSCH scheduled by DCI format DCI 1_1 and DCI format 1_2. This is more flexible while it requires UE can dynamically change the UE PDSCH processing time for different PDSCHs scheduled by different DCI formats. It seems not a big issue since the processing time for PDSCH scheduled by DCI format 1_0 may also different from the PDSCH scheduled by DCI format 1_1 in legacy Rel-15.

- For PDSCH processing capability 2, Option 2 seems the only feasible option. Because a UE may be only configured with DCI format DCI 1_1/0_1 or only DCI format 1_2/0_2. In such case, only the DMRS configuration associated with the configured DCI format matters. Option 1 would make the processing time dependent on all DMRS configurations from both two non-fallback DCI formats even when only one non-fallback DCI format is configured.

- Based on above analysis, the following Text proposal #2 is proposed.

Proposal 2: Adopt Text Proposal #2 below for UE PDSCH processing procedure time.

----------------------------------------Text Proposal #3 for Section 5.3 in TS 38.214 g50------------------------------------

- 5.3 UE PDSCH processing procedure time <Unchanged parts are omitted> Table 5.3-1: PDSCH processing time for PDSCH processing capability 1 PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB for DCI format 1_1, or in both of dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 for DCI format 1_2; dmrs-AdditionalPosition ≠ 'pos0' in DMRS-DownlinkConfig in either of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB for DCI format 1_1, or in either of dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 for DCI format 1_2, or if the higher layer parameter is not configured. 0; 8; N1,0. 1; 10; 13. 2; 17; 20. 3; 20; 24. Table 5.3-2: PDSCH processing time for PDSCH processing capability 2 PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB for DCI format 1_1, or in both of dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 for DCI format 1_2. 0; 3. 1; 4.5. 2; 9 for frequency range 1.

Conclusion

In this contribution, two issues related to the new introduced DMRS configuration are discussed, and we have the following proposals, where the corresponding text proposals can be found in Section 2.

Proposal 1: Adopt Text Proposal #1 below for DM-RS reception procedure.

Proposal 2: Adopt Text Proposal #2 below for UE PDSCH processing procedure time.

Reference

TS 38.213-g50, Physical layer procedures for data, Release 16.

TS 38.331-g41, Radio Resource Control (RRC) protocol specification, Release 16.