3GPP TSG RAN WG1 #104bis-e                                        R1-2102496

e-Meeting, April 12th – April 20th, 2021

Source:	ZTE

Title:          Discussion on enhanced intra-UE multiplexing

Agenda item:	8.3.3

Document for: Discussion and Decision

- Introduction

In the RAN1 #104-e meeting, the potential enhancements for intra-UE multiplexing between different priorities were discussed, and the following agreements were reached [1].

In this contribution, we further discuss the potential techniques and provide our views.

Agreements: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, - Use a PUCCH resource in the second PUCCH-Config (the PUCCH-config containing the PUCCH resource of the HP HARQ-ACK) at least in case the total number of LP and HP HARQ-ACK bits is more than 2. - FFS: The PUCCH resource is configured dedicated for multiplexing of HP HARQ-ACK and LP HARQ-ACK. - FFS in case the total number of LP and HP HARQ-ACK bits is 2. - FFS details Working assumption: Reuse Rel-15 intra-UE PUCCH/PUSCH multiplexing timeline requirements for Rel-17 intra-UE PUCCH/PUSCH multiplexing with different priorities - FFS whether or not to specify a different behavior than Rel-15 when the timeline requirements are not met Agreements: When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, further study the following options (proponents are encouraged to provide more details and analysis): - Opt.1: The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource. - Opt.1a: The UE does not transmit negative SR. - Opt.1b: For negative SR, the UE transmit only HARQ-ACK on the HARQ-ACK resource. - Opt.1c: For negative SR, the UE transmits SR and HARQ-ACK on the SR resource - FFS: whether with power boost to transmit multiplexed payload or not. - Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource. - Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource. - Opt.2b: Using 4 CS values as for SR+1-bit HARQ-ACK in Rel-15/16. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit. - Opt.2c: If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource. - Opt.3: No enhancement over Rel-16. - Other options not excluded. - FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK? Agreements: When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF1, further study the following options (proponents are encouraged to provide more details and analysis): - Opt.1: The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource. - Opt.1a: The UE does not transmit negative SR. - Opt.1b: For negative SR, the UE transmit only HARQ-ACK on the HARQ-ACK resource. - Opt.1c: For negative SR, the UE transmits SR and HARQ-ACK on the SR resource - FFS: whether with power boost to transmit multiplexed payload or not. - Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource. - Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource. - Opt.2b: Applying QPSK for SR+1-bit HARQ-ACK. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit. - FFS on conditions of multiplexing. - Opt.3: For positive SR, transmit HARQ-ACK on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource. - Opt.4: For positive SR, transmit SR on the SR resource and drop HARQ-ACK. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource. - Opt.5: No enhancement over Rel-16. - Other options not excluded. - FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK? Agreements: When a PUCCH carrying HP SR with PF1 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, further study the following options (proponents are encouraged to provide more details and analysis): - Opt.1: The SR and HARQ-ACK are multiplexed and transmitted on the SR resource. - Opt.1a: For positive SR, the UE transmits the PUCCH in the resource using PUCCH format 1 for SR. The value of cyclic shift of sequence, i.e., , of this PUCCH format 1 is determined by HARQ-ACK, and the bit, i.e., b(0), of this PUCCH format 1 is determined by SR. For negative SR, the UE transmits only a PUCCH with HARQ-ACK information and drops the PUCCH with negative SR. - Opt.1b: SR and HARQ-ACK are multiplexed and modulated to be transmitted on the SR resource - Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource. - Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource. - Opt.2b: Using 4 CS values as for SR+1-bit HARQ-ACK in Rel-15/16. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit. - Opt.2c: If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource. - Opt.2d: HP SR and LP HARQ-ACK are multiplexed by the Rel-15 cyclic shift only if latency requirement for HP SR is met. Otherwise, drop the LP HARQ-ACK and only transmit the HP SR on its resource. - Opt.3: For positive SR, transmit HARQ-ACK on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource. - Opt.4: No enhancement over Rel-16. - Other options not excluded. - FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK? Agreements: For multiplexing LP HARQ-ACK in a HP PUSCH, support 0< beta-offset <1. - FFS value(s) - FFS to additionally support beta-offset =0 or a value disabling the multiplexing - Aim to NOT increase the corresponding bitwidth in the DCI (compared to Rel-16) Agreements: Per UE with the capability of inter-band CA, simultaneous PUCCH/PUSCH transmission of different PHY priorities over different cells can be RRC configured within the same PUCCH group - FFS: dynamic indication

- Multiplexing UCIs of different priorities in a PUCCH

  - Coding for UCIs with different priorities (e.g. separate coding vs. joint coding)

In RAN1#104e meeting, it took much time to discuss whether separate coding or joint coding should be adopted when the two UCIs with different priorities are transmitted in a same PUCCH resource. Companies are encouraged to provide the simulation results for separate coding and joint coding to help to identify which one is better.

Only the cases of total number of LP and HP HARQ-ACK bits >2 are considered in our simulation. For the case of total number of LP and HP HARQ-ACK bits =2, no coding enhancement is needed as the principle of 2bits multiplexing as that in Rel-15 can be easily applied.

The details of our simulation assumptions and results are attached in appendix. Six cases of different UCI payload combination are considered in our simulations as shown in Table-1. The RE allocation (in terms of encoded bit length) for HP and LP HARQ-ACK for separate coding is also shown in Table-1.

Table 1. The payload of HP and LP and the number of RBs allocated

| UCI payload |  | Joint coding |  |  |  | Separate coding |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Total |  | HP UCI |  |  | LP_UCI |  |  |
| UCI_H | UCI_L | N_RB | EncBit | CRC | CR | N_RB | EncBit | CRC_H | EncBit_H | CR_H | CRC_L | EncBit_L | CR_L |
| 1 | 8 | 8 | 256 | 0 | 0.0352 |  |  |  |  |  |  |  |  |
|  |  | 5 | 160 | 0 | 0.0563 | 5 | 160 | 0 | 80 | 0.0125 | 0 | 80 | 0.1000 |
| 4 | 6 | 8 | 256 | 0 | 0.0391 |  |  |  |  |  |  |  |  |
|  |  | 7 | 224 | 0 | 0.0446 | 7 | 224 | 0 | 160 | 0.0250 | 0 | 64 | 0.0938 |
| 4 | 16 | 8 | 256 | 11 | 0.1211 |  |  |  |  |  |  |  |  |
|  |  | 6 | 256 | 11 | 0.1615 | 6 | 192 | 6 | 96 | 0.0417 | 6 | 96 | 0.2292 |
| 10 | 10 | 8 | 256 | 11 | 0.1211 |  |  |  |  |  |  |  |  |
|  |  | 7 | 224 | 11 | 0.1384 | 7 | 224 | 0 | 160 | 0.0625 | 0 | 64 | 0.1563 |
| 12 | 12 | 8 | 256 | 11 | 0.1367 |  |  |  |  |  |  |  |  |
|  |  | 7 | 224 | 11 | 0.1563 | 7 | 224 | 6 | 160 | 0.1125 | 6 | 64 | 0.2813 |
| 8 | 48 | 8 | 256 | 11 | 0.2617 |  |  |  |  |  |  |  |  |
|  |  | 6 | 256 | 11 | 0.3490 | 6 | 192 | 0 | 72 | 0.1111 | 11 | 120 | 0.4917 |

Resource efficiency is a main criteria to evaluate different coding schemes. Based on the simulation results, we can observe that, in all the cases, to satisfy the requirement of BER, the total number of RBs needed by separate coding is less than that of by joint coding, which means the resource efficiency of separate coding is better than joint coding. The benefit is always valid even in case of different coding ways, e.g., repetition coding (for 1 bit payload), RM coding (for payload no more than 11 bits) and Polar coding (for payload more than 11 bit).

The trend matches the theoretical analysis. When a UE determines to transmit a HP UCI and a LP UCI in a same PUCCH with joint coding, a lower coding rate would be assumed to satisfy the higher reliability requirement for the HP UCI. However, due to the nature of joint coding, same coding rate would also be assumed for the LP UCI, which is over-dimensioned for LP UCI and thus results in resource waste. On the other hand, for separate coding, different coding rate can be set for HP UCI and LP UCI respectively due to their different performance requirements. In general, the coding rate of LP UCI would be higher than that of HP UCI. Thus, the coding rate for LP UCI in separate coding is generally higher than that in joint coding. Therefore, the resource allocated for LP UCI in separate coding can be less than that in joint coding. The resource saving is obvious especially when the payload of HP UCI is much less than that of LP UCI. This is also observed in our simulation results. For the case with 1bit HP HARQ-ACK and 8 bits LP, the performance of separate coding with 5RBs is still better than the performance of joint coding with 8RBs, which means 3RBs can be saved for separate coding to achieved the same performance requirements compared to joint coding. Similar observation can also be observed for the case of (4bits HP+16bits LP) and (8bits HP +48 bits LP). And for the case with 4bits HP UCI and 6bits UCI, 1RB can be saved by separate coding compared to joint coding.

Observation 1: The separate coding is beneficial in terms of resource efficiency when the two UCIs with different priorities will be transmitted in a same PUCCH.

Proposal 1: Adopt separate coding for the multiplexing of high priority UCI and low priority UCI on a PUCCH format 2/3/4 when the total payload of the UCIs is more than 2 bits.

In current TS 38.212[2], when a UCI will be transmitted in the PUCCH format 2/3/4, the UCI can be encoded by two schemes, RM coding or Polar coding, depending on the number of bits of the UCI, i.e., if the number of the UCI bits is more than 11 bits, the UCI will be encoded by Polar coding, otherwise it will be encoded by RM coding.

In Rel-17, when the two UCIs with different priorities will be multiplexed on a PUCCH format 2/3/4 by separate coding, the coding schemes should be considered for the different number of bits of the two UCIs. Considering the relationship between the efficiency of coding scheme and the number of bits, the existing coding schemes for a certain payload do not need to be changed, even if they will be transmitted on the same PUCCH.

Proposal 2: When the two UCIs with different priorities will be multiplexed on a PUCCH format 2/3/4 by separate coding, for a certain priority UCI,

- If its number of bits is more than 2 bits but no more than 11 bit, RM coding is performed for this UCI.

- If its number of bits is more than 11 bits, Polar coding is performed for this UCI.

For separate coding, more things should be considered, for example, the RE mapping scheme of HP and LP UCI coded bits, the compression/bundling/Partial dropping for LP UCI when no enough resource is left for LP UCI. These can be further studied after the coding scheme is determined.

Proposal 3: More things should be further studied, for example, the RE mapping scheme of HP and LP UCI coded bit, the compression/bundling/Partial dropping for LP UCI, after the separate coding scheme is determined.

  - Multiplexing enable/disable mechanism

An important principle for intra-UE multiplexing UCI with different priorities is that the latency and reliability of high priority transmission should not be affected. Even if the multiplexing timelines are met, whether the UE actually multiplexes or prioritizes the transmission of PUCCH/PUSCH with high priority should be determined and configured by the network. Therefore, it is necessary to indicate whether a high priority channel can be multiplexed with a low priority channel by dynamic scheduling DCI or RRC configuration. If the multiplexing indicator is disabled, the UE will prioritize the transmission(s) of PUCCH/PUSCH with high priority as Rel-16. Otherwise, the UE will use the enhanced Rel-17 methods for multiplexing.

When a high priority UCI is overlapping with the low priority UCI, since the multiplexing indicator is determined by the network, if the multiplexing indicator is existed for both the high priority UCI and the low priority UCI, these two indicators will indicate a same operation, i.e., multiplexing or not multiplexing, accordingly the UE is repetitive indicated by two same indicators. Therefore, there is unnecessary to configure the multiplexing indicator in both DCI field or RRC parameter for high priority transmission and DCI field or RRC parameter for low priority transmission. Moreover, the multiplexing indicator should exist in the DCI field or RRC parameter for the high priority transmission which is more reliable than the other one.

Proposal 4: The indicator of intra-UE multiplexing UCI with different priorities should be in the scheduling DCI or RRC parameter for the high priority transmission.

  - PUCCH resource determination and mapping for multiplexing between HARQ-ACKs with different priorities

In RAN1#104e meeting, agreement has been achieved as:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, - Use a PUCCH resource in the second PUCCH-Config (the PUCCH-config containing the PUCCH resource of the HP HARQ-ACK) at least in case the total number of LP and HP HARQ-ACK bits is more than 2. - FFS: The PUCCH resource is configured dedicated for multiplexing of HP HARQ-ACK and LP HARQ-ACK. - FFS in case the total number of LP and HP HARQ-ACK bits is 2. - FFS details

For the first FFS, it is not necessary to configure the dedicated resource for multiplexing of HP HARQ-ACK and LP HARQ-ACK. The ambiguity due to the uncertainty of LP HARQ-ACK multiplexing with HP HARQ-ACK can be solved by gNB implementation.

In case the total number of LP and HP HARQ-ACK bits is more than 2, PUCCH format 2, 3, or 4 is used, if the DCI corresponding to LP PUCCH is missing, the gNB can blind decode the PUCCH based on the hypothesis of different payload size under the condition that whether the LP HARQ-ACK is multiplexed with HP HARQ-ACK or not. gNB will determine correctly the retransmission of the PDSCH correspond to the LP PUCCH, and obtain the HP UCI without ambiguity.

For the second FFS, in case the total number of LP and HP HARQ-ACK bits is no more than 2, the same agreement should be adopted, i.e., using a PUCCH resource in the second PUCCH-Config (the PUCCH-config containing the PUCCH resource of the HP HARQ-ACK).

But there may be ambiguity for gNB on determining whether the LP HARQ-ACK is multiplexed with HP HARQ-ACK or not. In the case that the total number of UCI bits is no more than 2 bits and for LP HARQ-ACK and HP HARQ-ACK multiplexing, PUCCH format 0 may be used, if the DCI corresponding to LP PUCCH is missing, since the single HP HARQ-ACK and HARQ-ACK obtained by multiplexing HP and LP map to the same mcs, it will be ambiguous for gNB to detect the LP UCI if it is absent in multiplexed PUCCH. For example, according to the current specification in section 9.2.3 in TS38.213, for PUCCH format 0, if the DCI corresponding to LP PUCCH is missing, UE will transmit only 1 bit HP UCI with mcs =0 for NACK or 6 for ACK, and gNB will assume 2 bits UCI and consider the result as {HP=NACK, LP=NACK} or{ HP=ACK, LP=ACK}, obviously when mcs =6, the gNB misunderstands the LP UCI. Hypothesis decoding on payload size doesn’t work for this case, to overcome this problem, different initial sequences can be configured for multiplexed UCI and single HP UCI respectively, or the extended PUCCH resource can also be configured for multiplexed UCI.

The extended PUCCH resource for multiplexed UCI can be implicitly indicated to UE, for example, the PRI in the HP DCI indicates the PUCCH resource from the PUCCH set in the PUCCH-config with high priority for HP UCI only if no LP UCI need to be multiplexed. And the PUCCH resource corresponding to PRI+x is the extended PUCCH resource from the PUCCH set in the PUCCH-config with high priority for multiplexed UCI, the x is predefined, e.g., x=1. Obviously, the extended PUCCH resource are still the PUCCH resource from the PUCCH set in the PUCCH-Config with high priority and consistent with the previous agreement.

Between the two schemes of different initial sequences and the extended PUCCH resource, we slightly prefer the extended PUCCH resource in second PUCCH-Config (the PUCCH-config containing the PUCCH resource of the HP HARQ-ACK) for the multiplexed UCI as the minor specification impact.

Proposal 5: For the case that the total number of bits is no more than 2 bits, the PRI+x in the HP DCI is used to implicitly determine an extended PUCCH resource from the PUCCH set in the PUCCH-config with high priority for the multiplexed UCI,, x is predefined, e.g., x=1.

  - Multiplexing HARQ-ACK and SR with different priorities

    - PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF0 or PF1

If a UE would transmit positive SR in a resource using PUCCH format 0 and at most two HARQ-ACK information bits in a resource using PUCCH format 0/1 in a slot, the UE transmits the PUCCH in the resource using PUCCH format 0 in PRB(s) for SR. The same way as Table 9.2.5-1 and Table 9.2.5-2 in TS 38.213[3] can be reused for the UE to determine a value of  and (based on HARQ-ACK) for computing a value of cyclic shift.

Otherwise, the UE transmits only the PUCCH with HARQ-ACK information bits in a resource using PUCCH format 0/1 and drops the PUCCH for negative SR.

    - PUCCH carrying HP SR with PF1 overlaps with a PUCCH carrying LP HARQ-ACK with PF0

If a UE would transmit positive SR in a resource using PUCCH format 1 and HARQ-ACK information bits in a resource using PUCCH format 0 in a slot, the UE transmits the PUCCH in the resource using PUCCH format 1 in PRB(s) for SR. The value of cyclic shift of sequence, i.e.,, of this PUCCH format 1 is determined by HARQ-ACK, and the bit, i.e., b(0), of this PUCCH format 1 is determined by SR. For example, if HARQ-ACK value is 0, the UE transmits the PUCCH by setting b(0) = 0 and = 0. If HARQ-ACK value is 1, the UE transmits the PUCCH by setting b(0) = 0 and = 6.

Else, the UE transmits only the PUCCH with HARQ-ACK information bits in a resource using PUCCH format 0 and drops the PUCCH with negative SR.

    - PUCCH carrying HP SR with PF0/1 overlaps with a PUCCH carrying LP HARQ-ACK with PF2/3/4

If a UE would transmit positive SR in a resource using PUCCH format 0/1 and HARQ-ACK information bits in a resource using PUCCH format 2/3/4, the UE reuses Rel-15 rules for multiplexing.

Else, the UE transmits only the PUCCH with HARQ-ACK information and drops the PUCCH with negative SR.

The only difference with the principle of Rel-15 multiplexing is the dropping of SR if SR is negative.

    - PUCCH carrying HP SR with PF1 overlaps with a PUCCH carrying LP HARQ-ACK with PF1

If a UE would transmit SR in a resource using PUCCH format 1 and HARQ-ACK information bits in a resource using PUCCH format 1 in a slot, the UE reuses Rel-15 rules for multiplexing.

Proposal 6: Adopt the following rules to multiplex high priority SR and low priority HARQ-ACK.

| HARQ-ACK SR | PUCCH format 0 | PUCCH format 1 | PUCCH format 2/3/4 |
| --- | --- | --- | --- |
| PUCCH format 0 | For positive SR, the UE transmits the PUCCH in the resource using PUCCH format 0 in PRB(s) for SR. The same way in Rel-15 can be reused for the UE to determine the value of and for computing the value of cyclic shift . For negative SR, the UE transmits only a PUCCH with HARQ-ACK information. |  | For positive SR, the UE Reuse Rel-15 rules. For negative SR, the UE transmits only a PUCCH with HARQ-ACK information and drops the PUCCH with negative SR. |
| PUCCH format 1 | For positive SR, the UE transmits the PUCCH in the resource using PUCCH format 1 in PRB(s) for SR. The value of cyclic shift of sequence, i.e., , of this PUCCH format 1 is determined by HARQ-ACK, and the bit, i.e., b(0), of this PUCCH format 1 is determined by SR For negative SR, the UE transmits only a PUCCH with HARQ-ACK information and drops the PUCCH with negative SR. | Reuse Rel-15 rules. |  |

  - Multiplexing in case a PUCCH overlaps with more than one PUCCH

In the RAN1 #104-e meeting, the case that a PUCCH overlaps with more than one PUCCHs containing slot based resources or sub-slot based resources is discussed, most companies think this case could be discussed after the solution of one PUCCH overlapping with the other PUCCH have been determined.

There are some problem mentioned to be further studied, for example:

- Determination of the PUCCH time unit for handling the PUCCH collision case when different PUCCH time units of different priorities are configured, when A LP HARQ-ACK PUCCH overlaps with more than one HP sub-slot

- How to handle the cases when a LP HARQ-ACK PUCCH overlaps with multiple HP SR PUCCHs.

- When a LP HARQ-ACK overlaps with HP HARQ-ACK PUCCH and HP SR PUCCH simultaneously, how to handle the case if the LP HARQ-ACK can be multiplexed with HP HARQ-ACK but the LP HARQ-ACK cannot be multiplexed with HP SR.

Proposal 7: Determine the multiplexing rules for the UCI multiplexing rules of more than two overlapping PUCCHs with different priorities after the UCI multiplexing rules for two overlapping PUCCHs is clear.

- Multiplexing UCIs of different priorities in a PUSCH

  - Coding for UCIs with different priorities (e.g. separate coding vs. joint coding)

For multiplexing HARQ-ACKs of different priorities in a PUSCH, it is nature to use separate coding for UCIs multiplexed in PUSCH. Separate coding of UCIs has less impact for the specification. And separate coding is useful for different reliability requirements to different maximal coding rate. The separate beta-offsets can also be supported for different priority combinations.

Proposal 8: For multiplexing HARQ-ACKs of different priorities in a PUSCH in R17, support separate coding for the HARQ-ACKs with different priorities.

  - Enhancements for multiplexing parameters

    - Beta-offset and multiplexing enable/disable mechanism

According to the agreement of RAN1#104-e meeting, for multiplexing LP HARQ-ACK in a HP PUSCH, 0< beta-offset <1 should be supported, and beta-offset =0 or a value disabling the multiplexing is FFS.

From the aspect of signaling to enable or disable multiplexing, the same mechanism of intra-UE multiplexing UCI should be applied to intra-UE multiplexing UCI with data. Moreover, the indication on multiplexing enable/disable should exist in the scheduling DCI or RRC parameter for the high priority transmission, but not implicitly be set by beta_offset=0 or other values.

Proposal 9: The beta-offset should not be used to disable the intra-UE multiplexing UCI with data.

Proposal 10: The indicator of intra-UE multiplexing UCI with data exists in the scheduling DCI or RRC parameter for the high priority transmission.

When the gNB allows a UE to multiplex a high priority HARQ-ACK into a low priority PUSCH, a bigger beta_offset value can be configured to ensure the reliability of HARQ-ACK. The existing values of beta_offset in Table 9.3-1 in TS 38.213 are sufficient to ensure that the high priority HARQ-ACK can be allocated more REs in low priority PUSCH.

  - Resource mapping

When the LP UCI is multiplexed in HP PUCCH or HP PUSCH, the LP UCI may not be accommodated totally since the HP UCI or HP PUSCH should not be affected as possible and the LP UCI can only be mapped to the leftover PUCCH resource. In this case, the payload size for the LP UCI should be reduced until it can be carried by the remaining resource. There are two options for the payload size reduction, i.e., LP UCI is dropped partially and LP UCI compression

- Partially LP UCI dropping

This is the most straightforward way. The UE just drop some or all UCI depending on the available resource. For the dropped ACK/NACK, the network has to assume NACK and retransmit the corresponding PDSCH. This may reduce the resource efficiency since the UE may have already decode the PDSCH correctly. Anyway, it is better than the operation in Rel-16, where the LP UCI is totally dropped.

- LP UCI compression

Another solution is LP UCI compression. For example, HARQ-ACK bundling is widely used to reduce the HARQ-ACK codebook overhead in LTE and NR. After bundling, if the NACK is transmitted, the network still does not know the exact HARQ-ACK for each PDSCH. This situation is the same as the partial LP UCI dropping. However, if ACK is obtained by bundling, it means all the corresponding PDSCH is ACK and the network can get the exact feedback for each PDSCH. Since the possibility of ACK is much higher than NACK in practice, it can further avoid the unnecessary retransmission, which can further improve the resource efficiency compared to partial LP UCI dropping. Therefore, it is slightly preferred

Proposal 11: LP UCI compression is slightly preferred in case there is no enough resource left for LP UCI.

  - Timeline and latency requirements

For the latency requirement, the HP HARQ-ACK information should be mapped to the LP PUSCH resources no later than the last symbol of PUCCH resource for HARQ-ACK which is shown in Figure 1.

Figure 1. High priority HARQ-ACK multiplex with low priority PUSCH

Proposal 12: For the overlapping between high priority HARQ-ACK and low priority PUSCH, if the gNB allows a UE to multiplex the HARQ-ACK on PUSCH, the UE maps this HARQ-ACK to PUSCH resource elements no later than the last symbol of PUCCH resource for HARQ-ACK.

- Discussion on the overlapping between dynamic grant PUSCH and configured grant PUSCH

In Rel-16, the overlapping between DG PUSCH and CG PUSCH with different priorities was discussed. The potential solutions are shown below based on the latest discussion.

Proposal from Feature Lead: For collision handling between high priority CG and low priority DG, down-select following options. - Option 1: define a UE capability for collision handling between the CG and DG with different priorities in PHY layer. - If UE supports the capability, PHY layer can make the prioritization so that the UE is expected to transmit the PUSCH corresponding to the configured grant, and cancel the PUSCH transmission scheduled by the PDCCH at latest starting at the first symbol of the PUSCH corresponding to the configured grant. - Otherwise, MAC layer should make the prioritization so that only one MAC PDU is delivered to PHY layer. - Option 2: re-use Rel.15 timeline, MAC layer should make the prioritization so that only one MAC PDU (e.g. the one with higher priority) is delivered to PHY layer. - Supported by QC, Intel, LG, Apple - Option 3: PHY layer can make the prioritization so that the UE is expected to transmit the PUSCH corresponding to the configured grant, and cancel the overlapping low priority PUSCH scheduled by the PDCCH at latest starting at the first symbol of the PUSCH corresponding to the configured grant. - Supported by Nokia, NSB, Huawei/HiSilicon, CATT, NEC, MTK, ZTE - No PHY collision handling necessary if MAC does not generate a PDU for the CG. - PHY does not expect MAC to generate a PDU for a later, lower-priority, CG PUSCH, which overlaps with an earlier, higher-priority, DG PUSCH. Proposal from Feature Lead: For collision handling between high priority DG and low priority CG, down-select following options: - Option 1: Define a UE capability for collision handling between the CG and DG with different priorities in PHY layer. - If a UE supports the capability, the UE is expected to cancel the overlapping low priority CG by the first overlapping symbol at the latest. Further, a UE expects that the first [overlapping] symbol of the high priority DG is not earlier than Tproc,2+d1 after the last symbol of the PDCCH with the DCI format scheduling the high priority DG. - Otherwise, the UE can only cancel the entire PUSCH transmission corresponding to the configured grant starting in a symbol 𝑗, if the end of symbol 𝑖 for PDCCH scheduling the PUSCH is at least 𝑁2 symbols before the beginning of symbol 𝑗. - Option 2: Rel.15 timeline is reused to support cancellation of the low priority CG PUSCH. - A UE is not expected to be scheduled by a PDCCH ending in symbol i to transmit a high priority DG PUSCH on a given serving cell overlapping in time with a transmission occasion, where the UE is allowed to transmit a CG PUSCH with low priority, starting in a symbol j on the same serving cell if the end of symbol i is not at least N2 symbols before the beginning of symbol j. - Option 3: PHY layer can make the prioritization so that the UE is expected to cancel the overlapping low priority CG PUSCH by the first overlapping symbol at the latest. Further, a UE expects that the first [overlapping] symbol of the high priority DG PUSCH is not earlier than Tproc,2+d1 after the last symbol of the PDCCH with the DCI format scheduling the high priority channel. - No PHY collision handling necessary if MAC does not generate a PDU for the CG.

In the RAN1 #102-e meeting, the following agreement was achieved.

Agreements: Support PHY prioritization for the case where low-priority DG-PUSCH collides with high-priority CG-PUSCH in R17. - FFS details - Clarify R16 baseline if needed.

- HP CG vs LP DG

If the UE processes the CG PUSCH first, the UE will only transmit the CG PUSCH. If the start of the DG PUSCH is earlier than the CG PUSCH and the UE processes the DG PUSCH first, the UE should stop processing the DG PUSCH and then prepare the CG PUSCH. In this case, more time is needed for the CG PUSCH. Since the UE has already been aware of the CG PUSCH, available time for processing depends on when the high priority arrives. However, the high priority data could arrive at any time. If the UE has enough time to cancel the DG PUSCH and transmits CG PUSCH, e.g. the data is arrived before Tproc,2+d1 before the first symbol of the CG PUSCH, the UE should cancel the DG PUSCH before the first symbol of the CG PUSCH and transmit the CG PUSCH. If the UE does not have enough for such processing, the CG PUSCH cannot be transmitted on the current occasion. Anyway, it should be up to the UE implementation. Therefore, we support the option 3 above to resolve this issue.

Proposal 13: For the overlapping between HP CG and LP DG, PHY layer can make the prioritization so that the UE is expected to transmit the PUSCH corresponding to the configured grant, and cancel the overlapping low priority PUSCH scheduled by the PDCCH at latest starting at the first symbol of the PUSCH corresponding to the configured grant.

- LP CG vs HP DG

Similarly, if the UE processes the CG PUSCH first, more time is needed for the UE to cancel the CG PUSCH and prepare the DG PUSCH. It means the time interval between the DG PUSCH and the corresponding scheduling PDCCH should be large enough. The timeline defined in Rel-16 can be reused. Regarding the UE capability, it can be discussed after the solution is stable.

Proposal 14: For the overlapping between LP CG and HP DG, PHY layer can make the prioritization so that the UE is expected to cancel the overlapping low priority CG PUSCH by the first overlapping symbol at the latest. Further, a UE expects that the first [overlapping] symbol of the high priority DG PUSCH is not earlier than Tproc,2+d1 after the last symbol of the PDCCH with the DCI format scheduling the high priority channel.

- Conclusion

According to the analysis given above, we have the following observations and proposals:

Observation 1: The separate coding is beneficial in terms of resource efficiency when the two UCIs with different priorities will be transmitted in a same PUCCH.

Proposal 1: Adopt separate coding for the multiplexing of high priority UCI and low priority UCI on a PUCCH format 2/3/4 when the total payload of the UCIs is more than 2 bits.

Proposal 2: When the two UCIs with different priorities will be multiplexed on a PUCCH format 2/3/4 by separate coding, for a certain priority UCI,

- If its number of bits is more than 2 bits but no more than 11 bit, RM coding is performed for this UCI.

- If its number of bits is more than 11 bits, Polar coding is performed for this UCI.

Proposal 3: More things should be further studied, for example, the RE mapping scheme of HP and LP UCI coded bit, the compression/bundling/Partial dropping for LP UCI, after the separate coding scheme is determined.

Proposal 4: The indicator of intra-UE multiplexing UCI with different priorities should be in the scheduling DCI or RRC parameter for the high priority transmission.

Proposal 5: For the case that the total number of bits is no more than 2 bits, the PRI+x in the HP DCI is used to implicitly determine an extended PUCCH resource from the PUCCH set in the PUCCH-config with high priority for the multiplexed UCI,, x is predefined, e.g., x=1.

Proposal 6: Adopt the following rules to multiplex high priority SR and low priority HARQ-ACK.

| HARQ-ACK SR | PUCCH format 0 | PUCCH format 1 | PUCCH format 2/3/4 |
| --- | --- | --- | --- |
| PUCCH format 0 | For positive SR, the UE transmits the PUCCH in the resource using PUCCH format 0 in PRB(s) for SR. The same way in Rel-15 can be reused for the UE to determine the value of and for computing the value of cyclic shift . For negative SR, the UE transmits only a PUCCH with HARQ-ACK information. |  | For positive SR, the UE Reuse Rel-15 rules. For negative SR, the UE transmits only a PUCCH with HARQ-ACK information and drops the PUCCH with negative SR. |
| PUCCH format 1 | For positive SR, the UE transmits the PUCCH in the resource using PUCCH format 1 in PRB(s) for SR. The value of cyclic shift of sequence, i.e., , of this PUCCH format 1 is determined by HARQ-ACK, and the bit, i.e., b(0), of this PUCCH format 1 is determined by SR For negative SR, the UE transmits only a PUCCH with HARQ-ACK information and drops the PUCCH with negative SR. | Reuse Rel-15 rules. |  |

Proposal 7: Determine the multiplexing rules for the UCI multiplexing rules of more than two overlapping PUCCHs with different priorities after the UCI multiplexing rules for two overlapping PUCCHs is clear.

Proposal 8: For multiplexing HARQ-ACKs of different priorities in a PUSCH in R17, support separate coding for the HARQ-ACKs with different priorities.

Proposal 9: The beta-offset should not be used to disable the intra-UE multiplexing UCI with data.

Proposal 10: The indicator of intra-UE multiplexing UCI with data exists in the scheduling DCI or RRC parameter for the high priority transmission.

Proposal 11: LP UCI compression is slightly preferred in case there is no enough resource left for LP UCI.

Proposal 12: For the overlapping between high priority HARQ-ACK and low priority PUSCH, if the gNB allows a UE to multiplex the HARQ-ACK on PUSCH, the UE maps this HARQ-ACK to PUSCH resource elements no later than the last symbol of PUCCH resource for HARQ-ACK.

Proposal 13: For the overlapping between HP CG and LP DG, PHY layer can make the prioritization so that the UE is expected to transmit the PUSCH corresponding to the configured grant, and cancel the overlapping low priority PUSCH scheduled by the PDCCH at latest starting at the first symbol of the PUSCH corresponding to the configured grant.

Proposal 14: For the overlapping between LP CG and HP DG, PHY layer can make the prioritization so that the UE is expected to cancel the overlapping low priority CG PUSCH by the first overlapping symbol at the latest. Further, a UE expects that the first [overlapping] symbol of the high priority DG PUSCH is not earlier than Tproc,2+d1 after the last symbol of the PDCCH with the DCI format scheduling the high priority channel.

- Reference

- 3GPP, Chairman's Notes RAN1#104-e final.

- 3GPP TS 38.212-g50, Multiplexing and channel coding, Release 16.

- 3GPP TS 38.213-g50, Physical layer procedures for control, Release 16.

- Appendix

Simulation assumption

Table A-1. Simulation assumptions:

| Parameters | value |
| --- | --- |
| Fc | 2GHz |
| Channel | TDL-C |
| Mobile speed | 3km/h, 60km/h |
| Antenna configuration | 1Tx, 4Rx |
| PUCCH format | Format 2, with 2 Symbols |
| FH | Disabled |

Simulation results and observations

Figure A-1

Observations:

For 1bit HP UCI and 8bits LP UCI, the performance of separate coding with 5RBs is better than that of joint coding with 8RBs.

- For separate coding with 5RBs: the required SNR to satisfy BER@10^-4 for HP UCI is about -5.92dB, while the required SNR to satisfy BER@10^-2 for LP UCI is about -6.2dB.

- For joint coding with 5RBs: the required SNR to satisfy BER@10^-4 for HP UCI and BER@10^-2 for LP UCI is about -5.67dB.

- For joint coding with 5RBs: the required SNR to satisfy BER@10^-4 for HP UCI and BER@10^-2 for LP UCI is about -2.96dB.

Figure A-2

Observations:

For 4bits HP UCI and 6bits LP UCI, the performance of separate coding with 6RBs is still better than that of joint coding with 8RBs.

- For separate coding with 7RBs: the required SNR to satisfy BER@10^-4 for HP UCI is about -5.9dB while the required SNR to satisfy BER@10^-2 for LP UCI is about -6.83dB.

- For joint coding with 8RBs: the required SNR to satisfy BER@10^-4 for HP UCI and BER@10^-2 for LP UCI is about -5.43dB.

- For joint coding with 7RBs: the required SNR to satisfy BER@10^-4 for HP UCI and BER@10^-2 for LP UCI is about -4.80dB.

Figure A-3

Observation 3:

For 4bits HP UCI and 16bits LP UCI, the performance of separate coding with 6RBs is still better than that of joint coding with 8RBs.

- For separate coding with 6RBs: the required SNR to satisfy BER@10^-4 for HP UCI is about -4.0dB while the required SNR to satisfy BER@10^-2 for LP UCI is about -4.43dB.

- For joint coding with 8RBs: the required SNR to satisfy BER@10^-4 for HP UCI and BER@10^-2 for LP UCI is about -2.76dB.

- For joint coding with 6RBs: the required SNR to satisfy BER@10^-4 for HP UCI and BER@10^-2 for LP UCI is about -1.5dB.

Figure A-4

Observations:

For 10bits HP UCI and 10bits LP UCI, the performance of separate coding with 7RBs is still better than that of joint coding with 8RBs.

- For separate coding with 7RBs: the required SNR to satisfy BER@10^-4 for HP UCI is about -3.57dB while the required SNR to satisfy BER@10^-2 for LP UCI is about -4.84dB.

- For joint coding with 8RBs: the required SNR to satisfy BER@10^-4 for HP UCI and BER@10^-2 for LP UCI is about -2.81dB.

- For joint coding with 7RBs: the required SNR to satisfy BER@10^-4 for HP UCI and BER@10^-2 for LP UCI is about -2.05dB.

Figure A-5

Observations:

For 12bits HP UCI and 12bits LP UCI, the performance of separate coding with 7RBs is still better than that of joint coding with 8RBs.

- For separate coding with 6RBs: the required SNR to satisfy BER@10^-4 for HP UCI is about -2.95dB while the required SNR to satisfy BER@10^-2 for LP UCI is about -3.69dB.

- For joint coding with 8RBs: the required SNR to satisfy BER@10^-4 for HP UCI and BER@10^-2 for LP UCI is about -2.60dB.

- For joint coding with 7RBs: the required SNR to satisfy BER@10^-4 for HP UCI and BER@10^-2 for LP UCI is about -1.64dB.

Figure A-6

Observations:

For 8bits HP UCI and 48bits LP UCI, the performance of separate coding with 6RBs is still better than that of joint coding with 8RBs.

- For separate coding with 6RBs: the required SNR to satisfy BER@10^-4 for HP UCI is about -0.89dB while the required SNR to satisfy BER@10^-2 for LP UCI is about -0.38dB.

- For joint coding with 8RBs: the required SNR to satisfy BER@10^-4 for HP UCI and BER@10^-2 for LP UCI is about 0.23dB.

- For joint coding with 6RBs: the required SNR to satisfy BER@10^-4 for HP UCI and BER@10^-2 for LP UCI is about 2dB.