3GPP TSG RAN WG1 #104b-e                                             R1-2102499

e-Meeting, April 12th – 20th, 2021

Source: 	ZTE Corporation

Title: 	Discussion on joint channel estimation for PUSCH

Agenda item:	8.8.1.3

Document for:	Discussion/Decision

Introduction

In RAN1#104-e, joint channel estimation for PUSCH was discussed with reaching the following agreements [1][2].

Agreements: - Following potential use cases are considered for joint channel estimation for PUSCH: - Use case 1: back-to-back PUSCH transmissions within one slot. - Use case 2: non-back-to-back PUSCH transmissions within one slot. - Use case 3: back-to-back PUSCH transmissions across consecutive slots. - Use case 4: non-back-to-back PUSCH transmissions across consecutive slots. - Use case 5: PUSCH transmissions across non-consecutive slots. Note: RAN1 assumes “back-to-back PUSCH transmission” has zero gap in-between adjacent PUSCH transmissions. Agreements: - For back-to-back PUSCH transmissions across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation at least for the following case: - Over back-to-back PUSCH transmissions (of the same TB) for repetition type A scheduled by dynamic grant or configured grant - FFS details (including possible other cases) Agreements: - For joint channel estimation, a time domain window is introduced to facilitate further discussion, during which UE is expected to maintain power consistency and phase continuity among PUSCH transmissions subject to power consistency and phase continuity requirements. - FFS: whether the window should be specified - FFS: the length of the time domain window is defined by a set of repetitions/slots/symbols - FFS: single or multiple time domain windows - FFS: relation with UE capability - FFS: the time domain window may or may not be configured. - FFS: whether the term "time domain window" is used in the specification or replaced by other technical terms - FFS: Whether the window is determined by the power consistency and phase continuity requirements and/or by other factors is to be decided. Agreements: - Companies are encouraged to study optimization of DMRS granularity in time domain with joint channel estimation, including: - Use cases - Simulations results - Enhanced schemes, e.g., - Different DMRS density for different PUSCH transmissions - No DMRS for some PUSCH transmissions - If applicable, impact of dynamic changes, e.g., cancellation of a repetition and companies report the evaluation method. - Companies are encouraged to study optimization of DMRS location in time domain with joint channel estimation, including: - Use cases - Simulations results - Enhanced schemes, e.g., - DMRS equally spaced among PUSCH transmissions - DMRS located in special slots - Orphan symbol used for DMRS - If applicable, impact of dynamic changes, e.g., cancellation of a repetition and companies report the evaluation method. - Note: the simulation assumptions for DM-RS in TR 38.830 are used as baseline for performance evaluation on optimization of DMRS location/granularity in time domain. - Take into account impairments such as frequency offset, and report corresponding parametrization together with the results. Further discuss impairment details. Working assumption: - For back-to-back PUSCH transmissions across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following case: - Over back-to-back PUSCH transmissions for one TB processed over multiple slots - It’s subject to UE capability Agreements: - For joint channel estimation. - Take into account the residual frequency error, e.g., +/- 0.1 ppm as upper bound. - Companies can report other values and frequency error model.

In this contribution, we provide our further analysis on joint channel estimation for PUSCH.

Discussion

Use cases

In the RAN1#104e meeting, 5 potential use cases were agreed to be considered for joint channel estimation for PUSCH. For use case 3 (back-to-back PUSCH transmissions across consecutive slots), RAN1 has agreed to support joint channel estimation in case of over back-to-back PUSCH transmissions (of the same TB) across consecutive slots for repetition type A scheduled by dynamic grant or configured grant, or over back-to-back PUSCH transmissions for one TB processed over consecutive slots (as a working assumption).

Regarding use case 1 (back-to-back PUSCH transmissions within one slot), the essence is whether to support joint channel estimation for PUSCH repetition type B. In our view, PUSCH repetition type B could be beneficial for coverage improvement in case of TDD operation by using the symbols in special slots. On the other hand, we need to be cautious that the support of joint channel estimation should be based on Rel-16 PUSCH repetition type B functionality, since it was concluded in SI that enhancements to PUSCH repetition type B itself is not supported. In other words, joint channel estimation for PUSCH repetition type B is supported while optimization specific for PUSCH repetition type B is not considered. Thus, use case 1 can be supported for joint channel estimation for PUSCH

Proposal 1: Support use case 1 (back-to-back PUSCH transmissions within one slot) for joint channel estimation for PUSCH.

- Joint channel estimation for PUSCH repetition type B is supported while optimization specific for PUSCH repetition type B is not considered.

Based on RAN4 reply LS in [3], it is possible that phase continuity can be maintained in case of no more than X un-scheduled OFDM symbols in-between the PUSCH repetition or in case of other physical signals/channels in-between PUCCH or PUSCH repetitions, as shown below. From RAN1 perspective, it would extend the benefits from joint channel estimation if use case 2 and use case 4 with non-back-to-back PUSCH transmission are also supported. Thus, contingent on RAN4 further discussion, joint channel estimation for use case 2 and use case 4 should be supported.

- For back-to-back transmissions with zero gap in-between adjacent transmissions, the conditions under Q1 need to be met to maintain phase continuity. - For non-back-to-back transmission with non-zero gap in-between adjacent transmissions, RAN4 concluded that at least following additional condition also need to be met in addition to the conditions under Q1: - No downlink reception in-between the PUSCH or PUCCH repetition in the same band for TDD case - In scenario of no more than X un-scheduled OFDM symbols in-between the PUSCH or PUCCH repetition (e.g., X = 0, 1, 2, …, 14), and scenario of other physical signals/channels in-between PUCCH or PUSCH repetitions from the UE perspective, e.g., SRS or PUCCH transmission in-between the PUSCH repetition for the UE, RAN4 is still discussing if X can be non-zero value and UE can maintain phase continuity

Proposal 2: As long as the condition of power consistency and phase continuity defined by RAN4 can be met, support use case 2 (non-back-to-back PUSCH transmissions within one slot) and use case 4 (non-back-to-back PUSCH transmissions across consecutive slots) for joint channel estimation for PUSCH.

As for use case 5 (PUSCH transmissions across non-consecutive slots), it expected to be very difficult to maintain phase continuity for UE according to RAN4 discussion. Thus, use case 5 could be deprioritized.

Proposal 3: De-prioritize use case 5 for joint channel estimation for PUSCH.

Inter-slot FH bundling for cross-slot channel estimation

During SI phase, cross-slot channel estimation has been proved beneficial for coverage enhancement due to improved channel estimation accuracy. On top of it, inter-slot FH with inter-slot bundling may provide additional performance gain. Basically, a smaller bundle size could achieve higher diversity gain while a lower joint channel estimation gain and vice versa. To achieve a good balance between FH diversity gain and joint channel estimation gain, the inter-slot FH pattern in the time domain should be studied.

In Figure 1, different inter-slot FH patterns in the time domain for PUSCH with 8 repetitions are provided. In the example, FDD operation is assumed and the bundling size is one of the divisors of the maximum number of repetitions.

Figure 1 Different inter-slot FH patterns in the time domain for PUSCH with 8 repetitions in FDD

In Figure 2, the performance of different inter-slot FH patterns in the time domain for PUSCH with 8 repetitions is provided.

Figure 2. Performance of different inter-slot FH patterns in the time domain for PUSCH with 8 repetitions

As can be observed in Figure 2, the case with inter-slot FH with inter-slot bundling of size 4 can provide additional 2.66 dB, 1.38dB, 0.88dB and 0.78 dB gain over the case with baseline performance, legacy inter-slot FH, joint channel estimation while no inter-slot FH and inter-slot FH with inter-slot bundling of size 2 respectively in 700MHz rural scenario.

Observation 1: Inter-slot FH with inter-slot bundling to enable joint channel estimation can provide up to 2.66 dB gain for PUSCH with 8 repetitions in 700MHz rural scenario.

According to Figure 2, the case with FH for only one time, i.e., bundling size 4, provides the best performance. Thus, the bundling size can be implicitly determined by the number of repetitions K, e.g., floor (K/2) or cell(K/2). However, under different scenarios, e.g., different UE speed or different number of receiving antennas, it may need to use different bundling sizes for better performance. Thus, it may also desirable to make the bundling size configurable by RRC or even dynamically indicated by DCI.

Proposal 4: For the determination of inter-slot bundling size for inter-slot FH, RAN1 down-selects from the two options below.

- Option 1: Inter-slot bundling size is implicitly determined by the number of repetitions K, e.g., floor (K/2) or cell(K/2).

- Option 2: Inter-slot bundling size is RRC configured or dynamically indicated to a UE.

For TDD operation, joint channel estimation may be only performed among PUSCH transmissions in consecutive UL slots. Depending on TDD configuration and definition of one FH bundle, the inter-slot FH pattern could be different. Take Figure 3 for instance, two inter-slot FH patterns are given based on bundling size of 2, and FH happens between each FH bundle.

Figure 3 Different inter-slot FH patterns in the time domain for PUSCH with 8 repetitions in TDD, and inter-slot FH bundling size is 2.

- Regarding Pattern 1, the inter-slot FH bundling is based on consecutive slots. That is, one bundle contains consecutive slots which could be either DL or UL slots. For Pattern 2, the inter-slot FH bundling is based on available slots. That is, one bundle can only include the slots for actual PUSCH transmission(s). It seems no much difference among above two patterns, since they all have three hops while joint channel estimation can only be done in one hop. However, there could be differences if different TDD configuration or different bundling size is assumed. As a result, the inter-slot FH bundling pattern for TDD should be further studied.

Proposal 5: FFS the inter-slot FH bundling pattern for TDD operation.

Optimization of DMRS location/granularity in the time domain

Potential optimization of DMRS location/granularity in the time domain was discussed in SI phase. For instance, DMRS may only exist in some of the repetitions, i.e., DMRS-less scheme. The main concern in SI study is from the marginal performance gain and potential unmanageable specification impacts. In the following, we first evaluate whether there is clear performance gain for DMRS optimization, and then provide our views for minimizing the specification impacts.

In Figure 4, different DMRS patterns in the time domain for PUSCH with 8 repetitions are provided.

- DMRS Pattern 1: 1 DMRS symbol in each repetition

- DMRS Pattern 2: 2 DMRS symbols in each repetition

- DMRS Pattern 3: 1 DMRS symbol in every two repetitions

- DMRS Pattern 4: 2 DMRS symbols in every two repetitions

Figure 4 Different DMRS patterns in the time domain for PUSCH with 8 repetitions

Based on the four DMRS patterns above, the following 6 cases are considered depending on whether cross-slot joint channel estimation is applied or not. The detailed simulation assumptions are provided in Table A-2 in the appendix.

- Case 1: DMRS pattern 1 without joint channel estimation.

- Case 2: DMRS pattern 2 without joint channel estimation.

- Case 3: DMRS pattern 1 with joint channel estimation.

- Case 4: DMRS pattern 2 with joint channel estimation.

- Case 5: DMRS pattern 3 with joint channel estimation.

- Case 6: DMRS pattern 4 with joint channel estimation.

In Figure 5, the performance impact of frequency offset (FO) is provided, where the FO follows Uniform distribution with [-0.1, +0.1] ppm, and one FO is generated for each PUSCH repetition.

Figure 5. Performance of different DMRS patterns in the time domain for PUSCH with 8 repetitions w/ or w/o FO

As can be observed in Figure 5, we compared the performance with FO and without FO in 700MHz Rural scenario，and the performance difference is very small. That is, the performance impact due to FO is negligible in 700MHz Rural scenario.

Observation 2: The performance impact due to frequency offset error is negligible in 700MHz Rural scenario.

In Figure 6, the performance of different DMRS patterns w/ or w/o joint channel estimation is given. FO is considered in all the simulation cases in Figure 6.

Figure 6. Performance of different DMRS patterns in the time domain for PUSCH with 8 repetitions

As can be observed in Figure 6, Case 6 can provide additional 2.52 dB, 2.43 dB, 0.15 dB, 0.81 dB and 0.87 dB gain over the Case 1, Case 2, Case 3, Case4 and Case 5 respectively in 700MHz Rural scenario at target BLER 0.1.

Observation 3: Optimization of DMRS location/granularity in the time domain can provide 0.15~2.52 dB gain for PUSCH repetitions in 700MHz Rural scenario.

To minimize the specification efforts, DMRS optimization should be limited to certain UE features. For instance, DMRS optimization is only applied for PUSCH repetition type A. This could avoid lots of discussion on how to design the DMRS due to different segmentation cases for PUSCH repetition type B. In addition, DMRS pattern should not be changed in each repetition. In other words, it should only focus on DMRS optimization among different repetitions, e.g., DMRS may not present in all repetitions. Other limitations, if any, could also be considered.

In addition, the above discussed PUSCH repetition bundling for inter-slot FH could be considered to apply for DMRS optimization. For instance, DMRS optimization is based on each bundle, and DMRS pattern is the same among different bundles.

With discussion above, we have the following proposal.

Proposal 6: Support optimization of DMRS location/granularity in the time domain with minimized specification impacts by at last the following conditions.

- DMRS optimization is only applied for PUSCH repetition type A.

- DMRS pattern in each repetition is not changed.

- Consider to reuse the repetition bundle defined for inter-slot FH for DMRS optimization.

Conclusion

According to the analysis given above, we have the following observations and proposals:

Proposal 1: Support use case 1 (back-to-back PUSCH transmissions within one slot) for joint channel estimation for PUSCH.

- Joint channel estimation for PUSCH repetition type B is supported while optimization specific for PUSCH repetition type B is not considered.

Proposal 2: As long as the condition of power consistency and phase continuity defined by RAN4 can be met, support use case 2 (non-back-to-back PUSCH transmissions within one slot) and use case 4 (non-back-to-back PUSCH transmissions across consecutive slots) for joint channel estimation for PUSCH.

Proposal 3: De-prioritize use case 5 for joint channel estimation for PUSCH.

Observation 1: Inter-slot FH with inter-slot bundling to enable joint channel estimation can provide up to 2.66 dB gain for PUSCH with 8 repetitions in 700MHz rural scenario.

Proposal 4: For the determination of inter-slot bundling size for inter-slot FH, RAN1 down-selects from the two options below.

- Option 1: Inter-slot bundling size is implicitly determined by the number of repetitions K, e.g., floor (K/2) or cell(K/2).

- Option 2: Inter-slot bundling size is RRC configured or dynamically indicated to a UE.

Proposal 5: FFS the inter-slot FH bundling pattern for TDD operation.

Observation 2: The performance impact due to frequency offset error is negligible in 700MHz Rural scenario.

Observation 3: Optimization of DMRS location/granularity in the time domain can provide 0.15~2.52 dB gain for PUSCH repetitions in 700MHz Rural scenario.

Proposal 6: Support optimization of DMRS location/granularity in the time domain with minimized specification impacts by at last the following conditions.

- DMRS optimization is only applied for PUSCH repetition type A.

- DMRS pattern in each repetition is not changed.

- Consider to reuse the repetition bundle defined for inter-slot FH for DMRS optimization.

Reference

3GPP RAN1#104-e, Chairman’s note.

3GPP RAN1#104-e, R1-2102161 [104-e-NR-CovEnh-03] Summary of email discussion on joint channel estimation for PUSCH, Moderator (China Telecom).

3GPP RAN4#98-e, R4-2103393, “Reply on LS on PUCCH and PUSCH repetition’’.

- Appendix

- Table A-1 Simulation assumption for inter-slot FH bundling for cross-slot channel estimation

| Parameter | Value |
| --- | --- |
| Carrier frequency | 700MHz |
| Channel model | TDL-C (delay spread: 300ns) as in 38.901 for Rural scenario |
| UE speed | 3 km/h |
| BS antenna configuration | 4 Rx |
| UE antenna configuration | 1 Tx |
| System bandwidth | 20 MHz |
| Sub-carrier spacing | 15kHz |
| Occupied RB | 4 |
| MCS | 0 |
| Waveform | DFT-s-OFDM |
| Number of OS per repetition | 14 (PUSCH mapping type A) |
| DMRS overhead | 2 DMRS symbols |
| Number of repetitions | 8 |
| Frequency hopping | Enabled |
| Channel estimation | Practical |
| Receiver type | MMSE |
| Hopping RB offset | 50RBs offset for inter-slot hopping |

- Table A-2 Simulation assumption for different DMRS patterns in the time domain

| Parameter | Value |
| --- | --- |
| Carrier frequency | 700MHz |
| Channel model | TDL-C (delay spread: 300ns) as in 38.901 for Rural scenario |
| UE speed | 3 km/h |
| BS antenna configuration | 4Rx |
| UE antenna configuration | 1 Tx |
| System bandwidth | 20 MHz |
| Sub-carrier spacing | 15 kHz |
| Occupied RB | 4 |
| MCS | 0/1 |
| Waveform | DFT-s-OFDM |
| Number of OS per repetition | 14 (PUSCH mapping type A) |
| DMRS overhead | 1/2 DMRS symbols |
| Number of repetitions | 8 |
| Frequency hopping | Enabled |
| Channel estimation | Practical |
| Receiver type | MMSE |
| Frequency offset model | Uniform distribution with [-0.1, +0.1] ppm, and one FO is generated for each PUSCH repetition. |