3GPP TSG RAN WG1 #104b-e		R1-2102526

e-Meeting, April 12th –  20th, 2021

Source:	vivo

Title:	Discussion on methods for Rx/Tx timing delay mitigating

Agenda Item:	8.5.1

Document for:	Discussion and Decision

Introduction

In RAN1#104e meeting [1], some issues for Rx/Tx timing error mitigating were discussed and some agreements and conclusions were achieved. In this contribution, we present our views on potential enhancements for mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays.

‘Inter-TRP timing error’ clarification

In RAN1 meeting #104e, it was concluded that the Rx timing error and Tx timing error are internal timing errors. However, when we calculate RSTD from different gNBs/TRPs, the ‘inter-TRP timing error’ is generated, which may be simply defined as the difference of the gNB/TRP Tx/Rx timing errors between TRPs. We would like to get clarification whether ‘inter-TRP timing error’ mitigation will be handled in the scope of Rx/Tx timing error mitigation or it can be regarded as part of ‘synchronization error’ between TRPs, which is not within the scope of the WI.

- The issues of ‘inter-TRP timing error’ in DL-TDOA/UL-TDOA method should be clarified.

- e.g., whether to regard ‘inter-TRP timing error’ as synchronization error beween TRPs.

Rx/Tx timing delay mitigating for positioning methods

Rx/Tx timing delay mitigating for DL-TDOA method

Conclusion: Study the following options for mitigating TRP Tx timing errors and/or UE Rx timing errors for DL TDOA: - Option 1: - Support a TRP to provide the association information of DL PRS resources with Tx TEGs to LMF - Option 2: - Support LMF to provide the association information of DL PRS resources with Tx TEGs to UE for UE-based positioning - Option 3: - Support a TRP to provide the Tx timing errors per Tx TEG to LMF - Option 4: - Support LMF to provide the Tx timing errors per Tx TEG of TRP to a UE for UE-based positioning - Option 5: - Support a UE to provide the association information of RSTD measurements with UE Rx TEG(s) to LMF when the UE reports the RSTD measurements to LMF - Option 6: - Support LMF to provide Rx timing errors per Rx TEG to a UE for UE-based positioning - Option7: - Support a UE to provide Rx timing errors per Rx TEG to LMF for UE-assisted positioning - Option 8: - Support a TRP to provide the Tx timing error differences between Tx TEGs of the TRP to LMF - Option 9: - Support LMF to provide the Tx timing error differences between Tx TEGs of a TRP to a UE for UE-based positioning - Option10: - Support a UE to provide Rx timing error differences between Rx TEGs to LMF for UE-assisted positioning - FFS: details of the signalling, procedures, and UE capability - FFS: How the TEGs are determined by the UE or TRP (could be by implementation, i.e., no specification impact) - Note: Other options are not precluded. - Note: Depending on the discussion results, none/one/multiple of the above options may be adopted in Rel-17.

In SI stage, regarding UE/gNB Rx and Tx timing error, we have made an agreement as follows. It can be seen that Rx and Tx timing errors are generated per panel independently. In addition, as far as we know, at least in positioning, one TRP will not deploy more than one antenna panel. Thus, for a common understanding, a TRP should be associated with an Rx TEG or Tx TEG. Even in some scenarios, multiple antenna panels sharing the same location are deployed, but different antenna panels can also be distinguished by different TRP IDs, so that in these scenarios, different Rx/Tx TEGs can also be distinguished by different TRP IDs. Therefore, we think there will not be multiple groups of ‘Rx/Tx timing delay’. Then, a TRP does not need to indicate the association information with Rx TEG or Tx TEG to the LMF.

Agreement: Optional: The UE/gNB RX and TX timing error, in FR1/FR2, can be modeled as a truncated Gaussian distribution with zero mean and standard deviation of T1 ns, with truncation of the distribution to the [-T2, T2] range, and with T2=2*T1: - T1: [X] ns for gNB and [Y] ns for UE - FFS: X, Y - Note: RX and TX timing errors are generated per panel independently - FFS: how the Rx and Tx timing errors are applied

- Support 1 Tx TEG and 1 Rx TEG on the TRP side for positioning, so that a TRP does not need to provide the association information with Rx/Tx TEG to the LMF.

In DL-TDOA method, RSTD measurement is easily affected by UE Rx timing error. For example, when calculating RSTD, PRS Resource A is associated with Rx TEG 0, but ‘the reference PRS resource’ is associated with RX TEG1, this will introduce additional ‘Rx timing error difference’ cause by Rx TEG0 and Rx TEG1 in RSTD measurement results. In general, the UE can provide the association information of RSTD measurements with UE Rx TEG(s) to LMF in RSTD measurements report, which helps the LMF further exclude some of the measurement results containing additional ‘Rx timing error difference’ for higher accuracy.

However, in some cases, there is no need for UEs to report the association information of RSTD measurements with UE Rx TEG(s). For UEs that only support 1 Rx TEG, ‘Rx timing error difference’ natural does not exist. For scenarios with not very high positioning accuracy requirement, the ‘ns-level timing error’ does not have too much impact on the positioning service. Therefore, we think the UE can be requested to provide the association information of RSTD measurements with UE Rx TEG(s) to LMF when the UE supports more than 1 UE Rx TEG when needed.

- The UE can be requested to provide the association information of RSTD measurements with UE Rx TEG(s) to LMF when the UE supports more than 1 UE Rx TEG.

Moreover, even if the LMF can obtain the association information between RSTD measurements and Rx TEGs, it is also difficult to mitigate the impact of UE Rx timing error without the estimation of ‘Rx timing error difference’.  However, when measuring PRS(s) from a TRP, different UE Rx TEGs are associated, it is possible to estimate ‘Rx timing error difference’. In this case, ‘Rx timing error difference’ between different Rx TEGs will result in mainly differences between ToA measurements. For example, for a UE measuring PRS(s) from a TRP with 2 Rx panels (or 2 Rx TEGs), e.g. a PRS resource from a TRP associated with with 2 Rx panels (or 2 Rx TEGs) or 2 PRS resources from a TRP associated with with 2 Rx panels (or 2 Rx TEGs) respectively, the difference between ToA measurements from each Rx panel (or Rx TEG) can be simply written as:

ToA_difference=(t1+e1)-(t2+e2)=(t1-t2)+(e1-e2)

where t1, t2 are propagation delays between Rx panels and the TRP respectively, and e1, e2 are Rx timing errors for 2 Rx panels respectively. The ToA_difference can be divided to 2 parts, the first part is (t1-t2) which represents the difference in propagation delay, and the second part is (e1-e2) which represents the  ‘Rx timing error difference’. Moreover, since the propagation distance difference of (t1-t2) shall be less than the distance between the 2 Rx panels, when the distance between the 2 Rx panels is assumed as 10 cm, the value of (t1-t2) can be considered less than 0.33 ns, which is almost negligible compared to ‘Rx timing error difference’ (e1-e2). Therefore, we can treat ‘ToA_difference’ as (e1-e2) approximately, that is, the difference between ToA measurements associated with different Rx TEGs can be regarded as ‘Rx timing error difference’. The related evaluation results are shown below.

Figure1 The performance of ‘Rx timing error differences’ estimation in SH scenario

Figure2 The performance of ‘Rx timing error differences’ estimation in DH scenario

Figure 1 shows the performance of ‘Rx timing error differences’ estimation in SH scenario. In the left figure, it is observed that the ‘estimated Rx timing error differences’ by calculating ‘ToA_difference’ according to a UE measuring PRS(s) from a TRP with 2 Rx panels (or 2 Rx TEGs) are very close to the real Rx timing error differences. Since in SH scenario, the probability of both Rx panels (or 2 Rx TEGs) being in LOS is high, so the estimation of ‘Rx timing error differences’ is more accurate. In the right figure, we compare the performance for the cases of ‘Rx timing error elimination by timing error estimation’, ‘Rx timing error elimination by extracting measurements from same TEG’ and ‘no Rx timing error elimination’. For the case of ‘Rx timing error elimination by timing error estimation’, the UE location is derived after compensating for the ‘estimated Rx timing error differences’. For the case of ‘Rx timing error elimination by extracting measurements from same Rx TEG’, we first extracted the RSTD measurement results associated with the same Rx panel (or Rx TEG), then calculated the UE position based on these measurement results. While for the case of ‘no Rx timing error elimination’, we assume that the LMF does not know any Rx TEG information of RSTD measurements and still calculates the UE position in a conventional manner. It can be seen that after compensating for the ‘estimated Rx timing error differences’, the accuracy is largely improved compared with the case of ‘no Rx timing error elimination’, for example: from 3.2m@90% to 0.3m@90%; however, compared with the case of ‘Rx timing error elimination by extracting measurements from same Rx TEG’, the accuracy improvement is relatively small, e.g., from 1.9m@90% to 0.3m@90%. This is because in SH scenario, LOS path is relatively abundant, even if we use the measurements from only one Rx panel (or Rx TEG), the accuracy will not be reduced too much.

Figure 2 shows the performance of ‘Rx timing error differences’ estimation in DH scenario. In the left figure, it is observed that the ‘estimated Rx timing error differences’ by calculating ‘ToA_difference’ according to a UE measuring PRS(s) from a TRP with 2 Rx panels (or 2 Rx TEGs) are close to the real Rx timing error differences, but compared with the SH scenario, the error difference estimation accuracy is reduced. This is because in DH scenario, the probability of both two Rx panels (or 2 Rx TEGs) being in LOS is smaller than in the SH scenario. In addition, in the right figure, it is observed that after compensating for the ‘estimated Rx timing error differences’, the accuracy is greatly improved compared with the case of ‘no Rx timing error elimination’, for example: from 30m@90% to 3m@90%; furthermore, compared with the case of ‘Rx timing error elimination by extracting measurements from same Rx TEG’, the accuracy is also significantly improved, e.g. from 13m@90% to 3m@90%. The reason is that in the DH scenario, the probability of LOS is lower. For the case of ‘Rx timing error elimination by extracting measurements from same Rx TEG’, if we use the measurements from only one Rx panel (or Rx TEG) to calculate the UE position, the number of LOS paths cannot be guaranteed, so the accuracy is greatly reduced.

- When the UE is able to measure PRS(s) from a TRP associated with different UE Rx TEGs, the difference between ToA measurements associated with different UE Rx TEGs can be regarded as ‘Rx timing error difference’, which can be used for mitigating UE Rx timing errors in DL-TDOA method.

- After compensating for the ‘estimated Rx timing error differences’, the accuracy is improved, compared with the case of ‘Rx timing error elimination by extracting measurements from same Rx TEG’ in which only using the measurements associated with only one Rx TEG to calculate the UE position.

- The accuracy is improved in both SH and DH scenarios, especially in the DH scenario, the accuracy improvement is large.

We think there are 2 alternatives for a UE to measure PRS(s) from a TRP associated with different UE Rx TEGs.

- Alt1: The UE can measure one PRS resource associated with different Rx TEGs from the TRP. For example, the UE can measure the PRS resource associating with different Rx TEGs at the same time, or measure the PRS resource of different repetitions associating with different Rx TEGs, or measure the PRS resource of different time stamp.

- Alt2: The UE can measure different PRS resources associated with different Rx TEGs from the TRP. For example, the UE can measure at least 2 PRS resources, and different PRS resources are associated with different Rx TEGs.

- There are 2 alternatives for a UE to measure PRS(s) from a TRP associated with different UE Rx TEGs.

- Alt1: The UE can measure one PRS resource associated with different Rx TEGs from the TRP

- Alt2: The UE can measure different PRS resources associated with different Rx TEGs from the TRP

Furthermore, when the UE is able to measure PRS(s) from a TRP associated with different UE Rx TEGs, it can compensate ‘Rx timing error difference’ by itself first, and then report compensated RSTD measurements to the LMF without Rx TEG association information, or it can report original RSTD measurements (uncompensated measurements) and related Rx TEG information to the LMF. The latter is our preference, since we think that the UE compensates for RSTD measurements by UE implementation is unreliable, and the LMF should be eligible for more information to ensure positioning accuracy.

- When the UE is able to measure PRS(s) from a TRP associated with different UE Rx TEGs, support the UE to report original RSTD measurements (uncompensated measurements) and related Rx TEGs information to the LMF, rather than compensating ‘Rx timing error difference’ by UE implementation and then reporting compensated RSTD measurements to the LMF.

Therefore, when the UE is able to measure PRS(s) from a TRP associated with different UE Rx TEGs, the reported measurements related to different UE Rx TEGs  is the key information for the LMF to mitigate UE Rx timing errors. However, in some cases, the key information may be missed. For example, one UE is able to measure multiple PRS resources from a TRP associated with different UE Rx TEGs. However, in current specification, the UE can only select up to 4 RSTD measurements to report, and each RSTD measurement corresponds to a PRS resource. Once there is no rule to restrict UE behavior, the UE may report up to 4 RSTD measurements only associated with the same UE Rx TEG, even if there are other PRS resources associated with another UE Rx TEG and the corresponding RSTD measurements have high quality, so that the key information is missed. In addition, for a UE is able to measure a PRS resource associated with different Rx TEGs, it is also natural for the UE to associate only 1 Rx TEG with the RSTD measurement for this PRS resource, e.g. the UE only provides the Rx TEG information of the first path used to determine the RSTD measurement.  Similarly, the key information is missed. Thus, to ensure that the UE can report the key information to the LMF, some reporting rules should be defined. For example, for a UE is able to measure multiple PRS resources from a TRP associated with different UE Rx TEGs, when the RSTD measurements for PRS resources associated with a Rx TEG meet certain conditions, the UE should report at least one RSTD measurement associated with the Rx TEG; for a UE is able to measure a PRS resource associated with different UE Rx TEGs, when the measurements associated with a Rx TEG meet certain conditions, the UE should report the measurements associated with the Rx TEG. Therefore, we propose

- In DL-TDOA method, to eliminate the positioning error caused by the UE Rx timing errors of more than one UE Rx TEGs, the RSTD measurement report of a certain TRP for more than one UE Rx TEGs needs to be guaranteed if the UE is able to measure PRS(s) associated with different UE Rx TEGs.

- FFS the UE reporting rules to guarantee the RSTD measurement report for more than one UE Rx TEGs.

Rx/Tx timing delay mitigating for UL-TDOA method

Conclusion: Study the following option(s) for mitigating UE Tx and TRP Rx timing errors for UL TDOA: - Option 1: - Support a TRP to provide the association information of RTOA measurements with Rx TEGs to LMF when the TRP reports the RTOA measurements - Option 2: - Support a UE to provide the association information of SRS resources for positioning with UE Tx TEG(s) to LMF for UL TDOA positioning. - Option 3: - Support a UE to provide Tx timing errors per Tx TEG to LMF for UL TDOA positioning. - Option 4: - Support a UE to provide Tx timing error differences between Tx TEGs to LMF for UL TDOA positioning. - FFS: the details of the signalling, procedures, and UE capability - FFS: How the TEGs are determined by the UE or TRP (could be by implementation, i.e., no specification impact) - Note: Other options are not precluded. - Note: Depending on the discussion results, none/one/multiple of the above options may be adopted in Rel-17.

In UL-TDOA method, positioning accuracy is also easily affected by UE Tx timing error. For example, when the LMF is calculating the difference of the RTOA measured by different TRPs for a UE, when the UE Tx TEG associated with the RTOA is different, an extra ‘UE Tx timing error difference’ will be generated in the ‘RTOA difference results’, so that positioning accuracy is impacted. In general, the UE can provide the association information of SRS resources with UE Tx TEG(s) to LMF, which helps the LMF further exclude some of the ‘RTOA difference results’ containing additional ‘Tx timing error difference’ for higher accuracy.

Similar to DL-TDOA method, in some cases, there is also no need for UEs to provide the association information of SRS resources with UE Tx TEG(s), such as scenarios with not very high positioning accuracy requirement. Therefore, we propose

- The UE can be requested to provide the association information of SRS resources for positioning with UE Tx TEG(s) to LMF when the UE supports more than 1 UE Tx TEG.

In addition, during the time when the UE transmits the SRS resources, as the UE flips or is blocked, it is up to UE implementation to switch Tx panel for better uplink transmission. Thus, for the same SRS resources for positioning, the associated Tx panel and Tx TEG will also change accordingly.  From our point of view, the information of the UE Tx TEG(s) change should also be provided to the LMF. Therefore, we propose

- The UE should provide the information of the UE Tx TEG(s) change associated with SRS resource(s) to the LMF, when the UE Tx TEG associated with SRS resource(s) changes, e.g. due to switching of UE antenna panel to avoid blockage.

- FFS how to provide this information to the LMF by the UE.

Furthermore, we think the information of the UE Tx TEG(s) change should also be provided to the gNB which performs SRS measurements. For example, after one gNB measuring SRS, it may report RTOA measurements to the LMF. To obtain the RTOA measurements of a SRS resource, the gNB may derive SRS measurements from multiple SRS occasions/instances for joint processing (e.g. averaging). However, without the information of the UE Tx TEG(s) change, it is possible for the gNB to perform joint processing on multiple SRS occasions associated with different UE Tx TEGs, which will introduce unnecessary errors. So we propose that

- The information of the UE Tx TEG and Tx TEG change information associated with SRS resource(s) should also be provided to the gNB performing SRS measurements, to prevent the gNB performing joint processing on different SRS measurement time occasions associated with different UE Tx TEGs for the same SRS resource(s).

- FFS how to provide this information to the gNB performing SRS measurements.

Moreover, in current TS38.455[2], for TRP measurement results containing UL RTOA as below, there is no indication related to ‘SRS resource ID’ and the indicated ‘time stamp’ also cannot clearly indicate which SRS resource the RTOA measurement is associated with. Then even if the UE can provide UE Tx TEG information associated with SRS resource(s), if the gNB does not provide the information of ‘SRS resource ID’ related to RTOA measurements, the LMF also cannot further process the RTOA for UE Tx timing error mitigating.

9.2.37	TRP Measurement Result [2]

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  |
| >CHOICE Measured Results Value | M |  |  |  |
| >>UL Angle of Arrival | M |  | 9.2.38 |  |
| >>UL SRS-RSRP | M |  | INTEGER (0..126) |  |
| >>UL RTOA | M |  | 9.2.39 |  |
| >>gNB Rx-Tx Time Difference | M |  | 9.2.40 |  |
| >Time Stamp | M |  | 9.2.42 |  |
| >Measurement Quality | O |  | 9.2.43 |  |
| >Measurement Beam Information | O |  | 9.2.57 |  |

So we propose

- Support the gNB to provide information related to SRS resource(s) ID to the LMF in RTOA measurement result, to help the LMF obtain the relationship between the RTOA measurement result and the Tx TEGs of SRS resource(s).

Then, if a gNB is able to measure SRS resoures related to different UE Tx TEGs, the reported RTOA measurements related to different UE Tx TEGs is the key information for the LMF to migitate UE Tx timing errors. Therefore, it is better to define gNB reporting rules to guarantee the key information to be reported, which is similar to what we mentioned in DL-TDOA method.

- In UL-TDOA method, to eliminate the positioning error caused by the UE Tx timing errors of more than one UE Tx TEGs, the RTOA measurement report for more than one UE Tx TEGs needs to be guaranteed if the gNB is able to measure SRS resoures associated different UE Tx TEGs.

- FFS the gNB reporting rules to guarantee the RTOA measurement report for more than one UE Tx TEGs.

Rx/Tx timing delay mitigating for DL and UL positioning method

For Multi-RTT method, the round-trip-time is calculated by the use of the UE Rx-Tx time difference measurements and gNB Rx-Tx time difference measurements. The measurement accuracy of UE Rx-Tx time difference measurements may be impacted by UE Rx and/or Tx timing errors, and the measurement accuracy of gNB Rx-Tx time difference measurements may be impacted by gNB Rx and/or Tx timing errors. If UE Tx and Rx timing errors and gNB Tx and Rx timing errors cannot be compensated, the accuracy of the multi-RTT positioning may be significantly impacted. The similar problem will also appear in the ‘DL TDOA+UL-TDOA’ method, which can be supported by implementation to reduce some common errors from TRPs and UEs.

In RAN1# 104e, the issues related Rx/Tx mitigating for DL and UL method were discussed intensely. For mitigating UE Rx/Tx timing errors, the following conclusion is achieved.

Conclusion: Study the following options for mitigating UE Rx/Tx timing errors in DL+UL positioning: - Option 1: - Support UE to provide the association information of UE Rx-Tx time difference measurements with UE Rx TEGs in the measurement report to LMF - Option 2: - Support UE to provide the association information of UE Rx-Tx time difference measurements with UE Tx TEGs in the measurement report to LMF - Option 3: - Combination of Option 1 and Option 2; - Option 4: - Support UE to provide the association information of UE Rx-Tx time difference measurements with UE RxTx TEGs in a measurement report to LMF for multi-RTT positioning - FFS: the definition of UE RxTxTEG. It includes both UE Rx timing and Tx timing errors. - Option 5: - Support UE to provide the association information of DL-RSTD measurements with UE RxTx TEGs in a measurement report to LMF for simultaneous DL-TDOA and UL-TDOA configuration for positioning - Option 6: - Support UE to provide the timing errors per Rx/Tx TEG, or the timing error differences between the Tx/Rx TEGs to LMF - Option 7: - Support UE to provide the timing errors per RxTx TEG, or the Tx timing error differences between the RxTx TEGs to LMF - FFS: the details of signalling, procedures and UE capability - FFS: How the TEGs are determined by the UE or TRP (could be by implementation, i.e., no specification impact) - Note: Other options are not precluded. Note: Depending on the discussion results, none/one/multiple of the above options may be adopted in Rel-17.

The main discussion focused on whether to support UE to provide the association information of UE Rx-Tx time difference measurements with UE ‘Rx TEGs’, or UE ‘{Rx TEGs, Tx TEGs}’, or UE ‘RxTx TEGs’. Among these three options, providing the association information of UE Rx-Tx time difference measurements with UE ‘Rx TEGs’ is our preference. The reasons are as follows.

- In DL and UL positioning, the DL measurements ‘Rx-Tx time difference’ is decoupled from SRS transmission. The ‘Rx time’ in ‘Rx-Tx time difference’ is determined by the receiving time of PRS resource, while the ‘Tx time’ in ‘Rx-Tx time difference’ is determined by the time of the UL subframe that is closest in time to the receiving time of PRS resource instead of the time of SRS transmission. If support UE to provide the association information of UE Rx-Tx time difference measurements with ‘{Rx TEGs, Tx TEGs}’ or ‘RxTx TEGs’, it is needed to indicate the association information of the ‘SRS resource ID/set ID’ with Rx-Tx time difference measurements, since different pair of {PRS resources, SRS resources} may be associated with different Tx TEGs in ‘{Rx TEGs, Tx TEGs}’ or different ‘RxTx TEGs’. However, this makes it difficult to decouple the DL measurements and SRS transmission in DL and UL positioning.

- In addition, we also cannot guarantee that Tx TEG(s) information associated with the ‘Tx time’ in ‘Rx-Tx time difference’ is always the same as the Tx TEG(s) information associated with the actual SRS transmission. For example, the information of the UE Tx TEG(s) associated with SRS resource(s) may be changed during SRS transmission time, which is up to UE implementation. If support UE to provide the association information of UE Rx-Tx time difference measurements with ‘RxTx TEGs’, once the information of the UE Tx TEG(s) (or Tx panel(s)) associated with SRS resource(s) is changed, it is difficult to apply the change of Tx TEG(s) (or Tx panel(s)) to certain ‘RxTx TEG’, as one ‘RxTx TEG’ is composed of both DL measurements and SRS transmission. So we think in DL and UL positioning, it is better to provide the information of the UE Tx TEG(s) associated with SRS resource(s) (as in UL-TDOA method) rather than the DL measurements of ‘Rx-Tx time difference’.

Therefore, we propose

- In DL and UL positioning, support the UE to provide the association information of UE Rx-Tx time difference measurements with UE Rx TEGs in the measurement report to LMF.

Furthermore, the UE can provide additional information of UE ‘Rx timing error+Tx timing error’ associated with the combination of Rx TEGs and Tx TEGs to the LMF, for Rx/Tx timing error mitigating in DL and UL positioning. For example, when Rx TEGs and Tx TEGs are combined, the ‘Rx timing error+Tx timing error’ of some combinations is within a certain margin, then the UE should also provide this information to the LMF. We think this additional information not only applies to the multi-RTT method, but also applies to the ‘DL-TDOA+UL-TDOA’ method. For ‘DL-TDOA+UL-TDOA’ method which can be supported by combining DL-TDOA method and UL-TDOA method, in general, the UE may only provide separate Rx TEG information and/or separate Tx TEG information to the LMF. The additional information of ‘Rx timing error+Tx timing error’ information can work when eliminating the timing error caused by DL and UL combination.

- In DL and UL positioning, support the UE to provide additional information of UE ‘Rx timing error+Tx timing error’ associated with some combinations of Rx TEGs and Tx TEGs to the LMF.

- e.g., when Rx TEGs and Tx TEGs are combined, if the ‘Rx timing error+Tx timing error’ of some combinations is within a certain margin, the UE provides this information to the LMF

Reference device for positioning error mitigating

In RAN1#104e, ‘the reference device’ for Rx/Tx timing error mitigating was discussed and the following agreements were achieved.

Agreement: - Study specification impact for enabling a reference device with known location to support the following functionalities: - Measure DL PRS and report associated measurements (e.g., RSTD, Rx-Tx time difference, RSRP) to the LMF; - Transmit SRS and enable TRPs to measure and report measurements (e.g., RTOA, Rx-Tx time difference, AOA) associated with the reference device to the LMF; - FFS: The details of the signalling, the measurements, the parameters related to the Rx and Tx timing delays, AoD and AOA enhancements and measurement calibrations; - FFS: The report of device location coordinate information to the LMF if the LMF does not have the information - FFS: The device with the known location being a UE and/or a gNB - FFS: Precision to which location of reference device is known - Note: RAN1 assumes using these enhancements for the purpose of network synchronization is NOT within the scope of the WI

Reference device should be introduced at least for Rx/Tx timing error mitigating. From our point of view, the reference devices may be divided into 2 types as follows.

- Pre-deployed gNB/UE. This type of device can be a gNB or a UE or a UE attached to a gNB, which are pre-deployed with known location to the LMF. This type of device can be stationary or have mobility. In addition, it can be strongly controlled by the LMF for the purpose of error calibration.

- Normal UE. This type of device can be a normal UE, whose location is unknown to the LMF. However, it should have the ability to obtain its own location such as via RAT-independent techniques and be able to report the location to the LMF. In addition, this type of device may generally have mobility. In addition, it can also be controlled by the LMF for the purpose of timing error calibration. For example, a simple procedure for the LMF to determine a UE as a reference UE is as follows:

- Firstly, the LMF sends a request to some UEs, requesting them to report capability information and related location information. The capability information may include the ability to obtain its own location, the accuracy and reliability of the location, the positioning measurement/ processing ability, or error calibration ability (e.g. ability related Rx/Tx timing error calibration or Rx/Tx TEG), etc. The location information may include the absolute location or relative location of the UE, location uncertainty, the related time stamp, etc.

- Optionally, at the same time, the LMF can also request whether these UEs are willing to become reference devices for assisting network timing error calibration.

- Then some UEs that are willing to become reference devices report these capabilities and related location information to the LMF.

- The LMF selects the appropriate UEs as the ‘reference UE’ according to UE capability and related location. Then the LMF indicates that these UEs are ‘reference UE’, for example, through implicit indication, such as sending positioning assistance data/configuration information.

- Hence, ‘reference UEs’ perform DL positioning measurements, and then report the measurements to the LMF. Optionally, the latest location information is included in the measurement report.

In addition, since current gNB is not allowed to measure PRS or transmit SRS, once gNB is required to assist in calibration work, all the gNBs or TRPs in a certain scenario are required to measure PRS or transmit SRS, and the overhead of signaling interaction is relatively large. Furthermore, since the sector of the gNB has a certain downward tilt angle which is assumed to be basically fixed, we are not sure when the ‘reference gNB’ is required to measure/transmit PRS/SRS, whether an effective air interface connection can be established between it and other normal gNBs, especially the air interface connection of LOS, which reduces the calibration accuracy. Therefore, for pre-deployed devices as reference, our preference is pre-deployed UE rather than pre-deployed gNB.

Therefore, we propose

- Support reference devices of two types: pre-deployed UEs, normal UEs.

- Specify the procedure for the LMF to determine the ‘reference UEs’ from normal UEs.

- e.g. including LMF requesting UE capability information and/or related location information, UE reporting capability information and/or related location information, etc.

Moreover, for better assisting error calibration on the network side, it is proposed to enable the ‘reference UE’ as the UEs which can be controlled by the LMF, both for the pre-deployed UEs and normal UEs. For example, support the LMF to indicate the use of Rx TEGs or Tx TEGs of the ‘reference UE’ such as indicating the use of only 1 Rx TEGs or Tx TEGs, which makes the LMF to ignore the intra-UE timing error when calibrating the gNB timing error. In addition, the LMF may also indicate the mobility or the motion trajectory of the ‘reference UE’, so that the ‘reference UE’ can establish LOS connections with more TRPs and increase effective coverage, which is beneficial for the scenarios where many TRPs are deployed or LOS links are insufficient. Further, it can reduce the number of ‘reference UE’ and the overhead on the network side. Therefore, the ‘reference UE’ can be strongly controlled d by the LMF for the purpose of error calibration.

- Support the ‘reference UE’ being controlled by the LMF for better assisting network calibration, e.g., including

- support the LMF to indicate the use of Rx TEGs or Tx TEGs of the ‘reference UE’

- support the LMF to indicate the mobility or the motion trajectory of the ‘reference UE’

Furthermore, ‘the reference UE’ can also be used for angle error calibration. There are some factors that can cause angle estimation errors in angle-based methods. For example, in DL-AoD positioning, the gNB may report the azimuth/zenith angle of the beams characterized with unknown errors, which can eventually lead to a decrease in location estimation as observed in previous contribution [6]. Therefore, these ‘beam orientation errors’ should also be calibrated to increase the performance of DL-AoD method. From our point of view, ‘reference UE’ based method can also be considered, since the ideal UE location and angle information can help the LMF or gNB to estimate the error or the error model, and further eliminate or mitigate the impact of this error in other measurements. Similar angle errors also exist in AoA measurement, and we can still use ‘reference UE’ based methods to solve such problems. When using ‘the reference UE’ to calibrate the angle error of the gNB, a simple way is to indicate the location of ‘the reference UE’ to the gNB, so that the gNB can calibrate the angle error by itself.

- The location information of ‘reference UE’  can be provided to the gNB for angle error calibration by itself.

UE and TRP measurements enhancement

In RAN1#104e, the following agreements related to UE and TRP measurements enhancement were achieved.

Agreement: Support enabling - A UE to report one or more measurement instances (of RSTD, DL RSRP, and/or UE Rx-Tx time difference measurements) in a single measurement report to LMF for UE-assisted positioning, and - A TRP to report one or more measurement instances (of RTOA, UL RSRP, and/or gNB Rx-Tx time difference measurements) in a single measurement report to LMF, and - Each measurement instance is reported with its own timestamp - FFS: The measurement instances are within a [configured] measurement time window - FFS: Each UE measurement instance can be configured with N instances of the DL-PRS Resource Set - FFS: N (including N=1) - FFS: Each TRP measurement instance can be configured with M SRS measurement time occasions - FFS: M (including M=1) - FFS: details of signalling, procedures, and UE capability if any - FFS: whether and how to consider the additional enhancement related to measurement reporting of multi-paths and quality metric - Note 1: A measurement instance refers to one or more measurements, which can either be the same or different types, which are obtained from the same DL PRS resource(s), or the same UL SRS resource(s). - Note 2: This enhancement has no intention to change the mapping of measurement types to Rel-16 positioning techniques and no intention to introduce new positioning techniques either.

In current specification, when UE to report DL measurements to the LMF, it is up to UE implementation to derive PRS instances to obtain RSTD, RSRP and/or Rx-Tx time difference measurements for reporting. For better matching measurements of UE and TRP or tracking non-idealities related to time-drifts for the LMF, we support a UE or a TRP to report one or more measurement instances in a single measurement report.

However, in some cases, there is no need to support above reporting behavior. For example, when the UE is configured to periodically report DL measurements with small period, it is automatically equivalent to ‘reporting one or more measurement in a single report’. In addition, for DL or UL only positioning without combination of DL and UL measurements, UE reporting one or more measurement instances in a single report may not always be needed. Thus, this reporting behavior should be treated as an optional reporting behavior, which is suitable for both UE and gNB.

- The UE or the TRP can be configured to report one or more measurement instances in a single measurement report to the LMF.

In addition, about ‘FFS: The measurement instances are within a [configured] measurement time window’, our preference is not to introduce a ‘measurement time window’, as this window can be replaced by other ways. Take the UE as an example, the UE can be configured to report all the measurement instances with corresponding time stamps, or the UE can be configured to report the most recent measurement instance before measurement report. The same method can also be applied to gNB. We think these two methods are sufficient for positioning report, and it is also clear enough for the LMF to match the measurement results from the UE and the gNB.

Regarding UE to report the most recent measurement instances before measurement report, we think it is benefit for low latency positioning scenarios, as in these scenarios, the latest UE location is strongly required. Furthermore, considering PRS processing time, the most recent measurement instance should be in advance of a certain time before the measurement report, which is related to PRS processing capability.

Figure3 The most recent measurement instance to be report

- Support to enable the UE to report PRS measurements derived from the most recent measurement instances in advance of a certain time before the measurement report.

- The certain time before the measurement report is related to PRS processing capability.

Moreover, regarding ‘FFS: Each UE measurement instance can be configured with N instances of the DL-PRS Resource Set’, we think its relationship with ‘Nsample’ defined for measurement period requirement by RAN4 needs to be clarified. In TS38.133[3][4][5], in RSTD measurement requirement, ‘Nsample’ is defined as the number of PRS RSTD samples (or instances of DL-PRS Resource Set here) and  ‘Nsample’= 4; in Rx-Tx time difference measurement requirement,  ‘Nsample’ is the number of UE Rx-Tx time difference measurement samples (or instances of DL-PRS Resource Set here) and  ‘Nsample’= [4]; in PRS-RSRP measurement requirement, ‘Nsample’ is the number of PRS-RSRP measurement samples  (or instances of DL-PRS Resource Set here) and Nsample= 4. The motivation behind allowing more than one PRS sample in defining the measurement period is to ensure that with channel variations or deep fading, there is at least one PRS sample that satisfies the side conditions. That is, the UE needs to measure 4 PRS samples to get a measurement result. Similarly, we understand that one ‘measurement instance’ also corresponds one instance for measurement results to be report. However, if N<4, the measurement results cannot be ensured in one measurement instance. Compared with the requirement of RAN4, RAN1 seems to be more inclined to configure the number of DL-PRS Resource Set instances in a measurement instance flexibly. Therefore, we suggest asking RAN4 to reach a consensus, and then we can determine the value of N and whether N can be configured by the network.

- The relationship between ‘the number of DL-PRS Resources Set instances related to each UE measurement instance’ and ‘the number of PRS samples for RSTD/Rx-Tx time difference/PRS-RSRP measurements’ defined by RAN4’ should be clarified.

- Send an LS to RAN4 for consistent understanding.

Conclusion

In this contribution, we discuss methods for Rx/Tx timing mitigating with the following observations and proposals.

- When the UE is able to measure PRS(s) from a TRP associated with different UE Rx TEGs, the difference between ToA measurements associated with different UE Rx TEGs can be regarded as ‘Rx timing error difference’, which can be used for mitigating UE Rx timing errors in DL-TDOA method.

- After compensating for the ‘estimated Rx timing error differences’, the accuracy is improved, compared with the case of ‘Rx timing error elimination by extracting measurements from same Rx TEG’ in which only using the measurements associated with only one Rx TEG to calculate the UE position.

- The accuracy is improved in both SH and DH scenarios, especially in the DH scenario, the accuracy improvement is large.

- There are 2 alternatives for a UE to measure PRS(s) from a TRP associated with different UE Rx TEGs.

- Alt1: The UE can measure one PRS resource associated with different Rx TEGs from the TRP

- Alt2: The UE can measure different PRS resources associated with different Rx TEGs from the TRP

- The issues of ‘inter-TRP timing error’ in DL-TDOA/UL-TDOA method should be clarified.

- e.g., whether to regard ‘inter-TRP timing error’ as synchronization error beween TRPs.

- Support 1 Tx TEG and 1 Rx TEG on the TRP side for positioning, so that a TRP does not need to provide the association information with Rx/Tx TEG to the LMF.

- The UE can be requested to provide the association information of RSTD measurements with UE Rx TEG(s) to LMF when the UE supports more than 1 UE Rx TEG.

- When the UE is able to measure PRS(s) from a TRP associated with different UE Rx TEGs, support the UE to report original RSTD measurements (uncompensated measurements) and related Rx TEGs information to the LMF, rather than compensating ‘Rx timing error difference’ by UE implementation and then reporting compensated RSTD measurements to the LMF.

- In DL-TDOA method, to eliminate the positioning error caused by the UE Rx timing errors of more than one UE Rx TEGs, the RSTD measurement report of a certain TRP for more than one UE Rx TEGs needs to be guaranteed if the UE is able to measure PRS(s) associated with different UE Rx TEGs.

- FFS the UE reporting rules to guarantee the RSTD measurement report for more than one UE Rx TEGs.

- The UE can be requested to provide the association information of SRS resources for positioning with UE Tx TEG(s) to LMF when the UE supports more than 1 UE Tx TEG.

- The UE should provide the information of the UE Tx TEG(s) change associated with SRS resource(s) to the LMF, when the UE Tx TEG associated with SRS resource(s) changes, e.g., due to switching of UE antenna panel to avoid blockage.

- FFS how to provide this information to the LMF by the UE.

- The information of the UE Tx TEG and Tx TEG change information associated with SRS resource(s) should also be provided to the gNB performing SRS measurements, to prevent the gNB performing joint processing on different SRS measurement time occasions associated with different UE Tx TEGs for the same SRS resource(s).

- FFS how to provide this information to the gNB performing SRS measurements.

- Support the gNB to provide information related to SRS resource(s) ID to the LMF in RTOA measurement result, to help the LMF obtain the relationship between the RTOA measurement result and the Tx TEGs of SRS resource(s).

- In UL-TDOA method, to eliminate the positioning error caused by the UE Tx timing errors of more than one UE Tx TEGs, the RTOA measurement report for more than one UE Tx TEGs needs to be guaranteed if the gNB is able to measure SRS resoures associated with different UE Tx TEGs.

- FFS the gNB reporting rules to guarantee the RTOA measurement report for more than one UE Tx TEG(s).

- In DL and UL positioning, support the UE to provide the association information of UE Rx-Tx time difference measurements with UE Rx TEGs in the measurement report to LMF.

- In DL and UL positioning, support the UE to provide additional information of UE ‘Rx timing error+Tx timing error’ associated with some combinations of Rx TEGs and Tx TEGs to the LMF.

- e.g., when Rx TEGs and Tx TEGs are combined, if the ‘Rx timing error+Tx timing error’ of some combinations is within a certain margin, the UE provides this information to the LMF.

- Support reference devices of two types: pre-deployed UEs, normal UEs.

- Specify the procedure for the LMF to determine the ‘reference UEs’ from normal UEs.

- e.g. including LMF requesting UE capability information and/or related location information, UE reporting capability information and/or related location information, etc.

- Support the ‘reference UE’ being controlled by the LMF for better assisting network calibration, e.g., including

- support the LMF to indicate the use of Rx TEGs or Tx TEGs of the ‘reference UE’

- support the LMF to indicate the mobility or the motion trajectory of the ‘reference UE’

- The location information of ‘reference UE’  can be provided to the gNB for angle error calibration by itself.

- The UE or the TRP can be configured to report one or more measurement instances in a single measurement report to the LMF.

- Support to enable the UE to report PRS measurements derived from the most recent measurement instances in advance of a certain time before the measurement report.

- The certain time before the measurement report is related to PRS processing capability.

- The relationship between ‘the number of DL-PRS Resources Set instances related to each UE measurement instance’ and ‘the number of PRS samples for RSTD/Rx-Tx time difference/PRS-RSRP measurements’ defined by RAN4 should be clarified.

- Send an LS to RAN4 for consistent understanding.

- References

- Chairman’s notes for 3GPP TSG RAN WG1 Meeting 104, e-Meeting, Junuary 25th –Febuary 5th, 2021

- TS 38.455, “NR Positioning Protocol A (LPPa)”

- R4-2103578, CR on RSTD measurement requirements, RAN4#98e

- R4-2103581, CR to TS 38.133 on UE Rx-Tx time difference measurements (section 9.9.4), RAN4#98e

- R4-2104074, PRS-RSRP measurement requirements, RAN4#98e

- R1-2001447, Discussion on potential enhancements for DL-AoD method, vivo, RAN1#104e, Jan 25th-Feb 5th ,2021