3GPP TSG RAN WG1 #104b-e	                                              R1-2102548

e-Meeting, April 12th – 20th, 2021

Source:	vivo

Title:	Initial performance evaluation results of XR

Agenda Item:	8.14.3

Document for:	Discussion and Decision

- Introduction

In RAN1#104-e meeting, evaluation for capacity and power consumption were discussed and most evaluation assumptions have been agreed[1]. The overall simulation assumptions can be found in Appendix A.

In this contribution, we provide our initial round simulation results of capacity, power consumption and coverage for Cloud Gaming and XR (i.e. VR/AR) traffic respectively for Indoor Hotspot, Dense Urban and Urban Macro scenarios.

- Performance evaluation results of capacity

Capacity is one of the most important metrics for XR traffic. In this section, the results of downlink and uplink capacity for both Cloud Gaming and XR are presented and summarized in the form of table separately, where the resource utilization (RU) is also provided to assist the analysis of system capacity. Note that, in these tables, the value of #UEs per cell represents the system capacity defined as the maximum number of UEs per cell with that at least 90% among them are satisfied UEs. A UE will be declared as the satisfied UE if more than X% of packets are successfully delivered within a given air interface PDB. Besides, UE% denotes the specific percentage of satisfied UE per cell. The traffic model and scenario combinations in Table 1 are evaluated for system capacity in this section.

Table 1. Traffic model and scenario combinations used for system capacity evaluation

| DL FR1 - Scenario: - Indoor Hotspot/Dense Urban/Urban Macro - Traffic model: - CG: 30Mbps, 60/120FPS, PDB=15ms - VR/AR: 45/60Mbps, 60/120FPS, PDB=10ms | DL FR2 - Scenario: - Indoor Hotspot/Dense Urban - Traffic model: - CG: 30Mbps, 60/120FPS, PDB=15ms - VR/AR: 45/60Mbps, 60/120FPS, PDB=10ms |
| --- | --- |
| UL FR1 - Scenario: - Indoor Hotspot/Dense Urban/Urban Macro - Traffic model: - Scene information - Pose information | UL FR2 - Scenario: - Indoor Hotspot/Dense Urban - Traffic model: - Scene information - Pose information |

  - DL

Based on the agreements of the last meeting, Table 2 and Table 3 provide the traffic models for different traffics (i.e., CG and VR/AR). The packets arrival interval and packets size are determined by the corresponding data rate and the frame rate. The video frame rate is assumed to be 60 FPS when two eye buffers are bundled transmitted by the network. And the video frame rate is assumed to be 120 FPS when two eye buffers are sent with staggered in time. For simplicity, one video frame is modeled as one packet and the packet size distribution follows the truncated Gaussian distribution. Besides, the jitter is also considered to model the random delay caused by different encoding delays and core network transmission delays. For F FPS (e.g., F = 60), the arrival time of packet k is (k / F + J), where the jitter size J is a random variable drawn from a truncated Gaussian distribution. Considering that the key difference between different traffic models is data rate, frame rate and PDB, we will use the combination of data-rate_frame-rate_PDB to identify different traffic models in the following sections.

Table 2. Traffic models of Cloud Gaming (CG) and XR with 60 FPS

| Traffic model | CG | VR/AR | VR/AR |
| --- | --- | --- | --- |
| Data rate (Mbps) | 30 | 45 | 60 |
| Packet size distribution | Truncated Gaussian distribution |  |  |
| Mean packet size (Bytes) | 62500 | 93750 | 125000 |
| STD of packet sizes (Bytes) | 9375 | 14062 | 18750 |
| Maximum packet size (Bytes) | 93750 | 140625 | 187500 |
| Minimum packet size (Bytes) | 6250 | 9375 | 12500 |
| Packet arrival interval (ms) | 16.67 |  |  |
| Packet delay budget (ms) | 15 | 10 |  |
| Jitter distribution | Truncated Gaussian distribution |  |  |
| Jitter Mean (ms) | 0 |  |  |
| Jitter STD (ms) | 2 |  |  |
| Jitter Range (ms) | [-4, 4] |  |  |

Table 3. Traffic models of Cloud Gaming (CG) and XR with 120 FPS

| Traffic model | CG | VR/AR | VR/AR |
| --- | --- | --- | --- |
| Data rate (Mbps) | 30 | 45 | 60 |
| Packet size distribution | Truncated Gaussian distribution |  |  |
| Mean packet size (Bytes) | 31250 | 46875 | 62500 |
| STD of packet sizes (Bytes) | 4687 | 7031 | 9375 |
| Maximum packet size (Bytes) | 46875 | 70312 | 93750 |
| Minimum packet size (Bytes) | 3125 | 4687 | 6250 |
| Packet arrival interval (ms) | 8.33 |  |  |
| Packet delay budget (ms) | 15 | 10 |  |
| Jitter distribution | Truncated Gaussian distribution |  |  |
| Jitter Mean (ms) | 0 |  |  |
| Jitter STD (ms) | 2 |  |  |
| Jitter Range (ms) | [-4, 4] |  |  |

    - FR1

The simulation results of DL capacity for FR1 in Indoor Hotspot, Dense Urban and Urban Macro deployment scenarios are presented in Table 4, Table 5  and Table 6 respectively. Based on the notes given at the beginning of section 2, a UE will be declared as a satisfied UE if X% or more of the downlink packets are successfully delivered within PDB, where X = 95 and X = 99 are evaluated.

- Indoor Hotspot

Table 4. FR1 DL capacity simulation results in Indoor Hotspot scenario

| Traffic Model |  | X=95 |  |  | X=99 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | #UEs per cell | UE% | RU | #UEs per cell | UE% | RU |
| CG | 30Mbps_60FPS_15ms | 18 | 92.28% | 92.01% | 16 | 95.83% | 86.95% |
|  | 30Mbps _120FPS_15ms | 22 | 90.34% | 96.11% | 20 | 92.43% | 91.83% |
| VR/AR | 45Mbps _60FPS_10ms | 7 | 91.27% | 65.80% | 6 | 93.52% | 57.71% |
|  | 45Mbps _120FPS_10ms | 10 | 95.28% | 83.36% | 10 | 93.33% | 83.36% |
|  | 60Mbps _60FPS_10ms | 4 | 94.44% | 51.89% | 3 | 99.07% | 39.26% |
|  | 60Mbps _120FPS_10ms | 8 | 93.75% | 87.10% | 7 | 91.32% | 79.72% |

Observation 1: When the capacity requirement per UE is more than 99% of packets are successfully delivered within a given air interface PDB,

- For DL CG with 30Mbps and 60FPS in FR1 Indoor Hotspot, system capacity is 16 UEs per cell.

- For DL CG with 30Mbps and 120FPS in FR1 Indoor Hotspot, system capacity is 20 UEs per cell.

- For DL VR/AR with 45Mbps and 60FPS in FR1 Indoor Hotspot, system capacity is 6 UEs per cell.

- For DL VR/AR with 45Mbps and 120FPS in FR1 Indoor Hotspot, system capacity is 10 UEs per cell.

- For DL VR/AR with 60Mbps and 60FPS in FR1 Indoor Hotspot, system capacity is 3 UEs per cell.

- For DL VR/AR with 60Mbps and 120FPS in FR1 Indoor Hotspot, system capacity is 7 UEs per cell.

Observation 2: When the capacity requirement per UE is more than 95% of packets are successfully delivered within a given air interface PDB,

- For DL CG with 30Mbps and 60FPS in FR1 Indoor Hotspot, system capacity is 18 UEs per cell.

- For DL CG with 30Mbps and 120FPS in FR1 Indoor Hotspot, system capacity is 22 UEs per cell.

- For DL VR/AR with 45Mbps and 60FPS in FR1 Indoor Hotspot, system capacity is 7 UEs per cell.

- For DL VR/AR with 45Mbps and 120FPS in FR1 Indoor Hotspot, system capacity is 10 UEs per cell.

- For DL VR/AR with 60Mbps and 60FPS in FR1 Indoor Hotspot, system capacity is 4 UEs per cell.

- For DL VR/AR with 60Mbps and 120FPS in FR1 Indoor Hotspot, system capacity is 8 UEs per cell.

- Dense Urban

Table 5. FR1 DL capacity simulation results in Dense Urban scenario

| Traffic Model |  | X=95 |  |  | X=99 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | #UEs per cell | UE% | RU | #UEs per cell | UE% | RU |
| CG | 30Mbps_60FPS_15ms | 25 | 91.37% | 97.23% | 24 | 92.50% | 96.39% |
|  | 30Mbps _120FPS_15ms | 32 | 90.13% | 99.80% | 30 | 91.43% | 99.57% |
| VR/AR | 45Mbps _60FPS_10ms | 9 | 93.65% | 72.94% | 9 | 92.83% | 72.94% |
|  | 45Mbps _120FPS_10ms | 15 | 90.16% | 94.10% | 14 | 91.84% | 92.30% |
|  | 60Mbps _60FPS_10ms | 6 | 91.92% | 67.69% | 5 | 93.33% | 57.60% |
|  | 60Mbps _120FPS_10ms | 10 | 93.10% | 92.00% | 9 | 92.70% | 86.68% |

Observation 3:  When the capacity requirement per UE is more than 99% of packets are successfully delivered within a given air interface PDB,

- For DL CG with 30Mbps and 60FPS in FR1 Dense Urban, system capacity is 24 UEs per cell.

- For DL CG with 30Mbps and 120FPS in FR1 Dense Urban, system capacity is 30 UEs per cell.

- For DL VR/AR with 45Mbps and 60FPS in FR1 Dense Urban, system capacity is 9 UEs per cell.

- For DL VR/AR with 45Mbps and 120FPS in FR1 Dense Urban, system capacity is 14 UEs per cell.

- For DL VR/AR with 60Mbps and 60FPS in FR1 Dense Urban, system capacity is 5 UEs per cell.

- For DL VR/AR with 60Mbps and 120FPS in FR1 Dense Urban, system capacity is 9 UEs per cell.

Observation 4:  When the capacity requirement per UE is more than 95% of packets are successfully delivered within a given air interface PDB,

- For DL CG with 30Mbps and 60FPS in FR1 Dense Urban, system capacity is 25 UEs per cell.

- For DL CG with 30Mbps and 120FPS in FR1 Dense Urban, system capacity is 32 UEs per cell.

- For DL VR/AR with 45Mbps and 60FPS in FR1 Dense Urban, system capacity is 9 UEs per cell.

- For DL VR/AR with 45Mbps and 120FPS in FR1 Dense Urban, system capacity is 15 UEs per cell.

- For DL VR/AR with 60Mbps and 60FPS in FR1 Dense Urban, system capacity is 6 UEs per cell.

- For DL VR/AR with 60Mbps and 120FPS in FR1 Dense Urban, system capacity is 10 UEs per cell.

- Urban Macro

Table 6. FR1 DL capacity simulation results in Urban Macro scenario

| Traffic Model |  | X=95 |  |  | X=99 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | #UEs per cell | UE% | RU | #UEs per cell | UE% | RU |
| CG | 30Mbps_60FPS_15ms | 18 | 90.74% | 93.80% | 16 | 94.55% | 89.84% |
|  | 30Mbps _120FPS_15ms | 22 | 91.24% | 97.36% | 20 | 91.09% | 95.04% |
| VR/AR | 45Mbps _60FPS_10ms | 6 | 94.39% | 59.32% | 6 | 92.27% | 59.32% |
|  | 45Mbps _120FPS_10ms | 10 | 90.32% | 86.38% | 9 | 92.77% | 80.80% |
|  | 60Mbps _60FPS_10ms | 4 | 90.08% | 52.78% | 3 | 93.48% | 42.54% |
|  | 60Mbps _120FPS_10ms | 7 | 92.03% | 83.45% | 6 | 93.65% | 75.97% |

Observation 5:  When the capacity requirement per UE is more than 99% of packets are successfully delivered within a given air interface PDB,

- For DL CG with 30Mbps and 60FPS in FR1 Urban Macro, system capacity is 16 UEs per cell.

- For DL CG with 30Mbps and 120FPS in FR1 Urban Macro, system capacity is 20 UEs per cell.

- For DL VR/AR with 45Mbps and 60FPS in FR1 Urban Macro, system capacity is 6 UEs per cell.

- For DL VR/AR with 45Mbps and 120FPS in FR1 Urban Macro, system capacity is 9 UEs per cell.

- For DL VR/AR with 60Mbps and 60FPS in FR1 Urban Macro, system capacity is 3 UEs per cell.

- For DL VR/AR with 60Mbps and 120FPS in FR1 Urban Macro, system capacity is 6 UEs per cell.

Observation 6:  When the capacity requirement per UE is more than 95% of packets are successfully delivered within a given air interface PDB,

- For DL CG with 30Mbps and 60FPS in FR1 Urban Macro, system capacity is 18 UEs per cell.

- For DL CG with 30Mbps and 120FPS in FR1 Urban Macro, system capacity is 22 UEs per cell.

- For DL VR/AR with 45Mbps and 60FPS in FR1 Urban Macro, system capacity is 6 UEs per cell.

- For DL VR/AR with 45Mbps and 120FPS in FR1 Urban Macro, system capacity is 10 UEs per cell.

- For DL VR/AR with 60Mbps and 60FPS in FR1 Urban Macro, system capacity is 4 UEs per cell.

- For DL VR/AR with 60Mbps and 120FPS in FR1 Urban Macro, system capacity is 7 UEs per cell.

From the above tables, it can be seen that with a more relaxed reliability requirement, i.e., 95%, the system capacity could increase by 0 to 2 in FR1 DL. Sending two eye buffers with staggered in time is another way to increase the system capacity, because the PDB of each frame transmission is independent, implying that the total PDB for two-eye frame transmission is longer. Besides, by increasing the frame rate from 60FPS to 120FPS, the system capacity could increase by 3 to 7 in FR1 DL. Comparing different traffic models, the system capacity degrades with the increasing of data rate due to the larger packet size. For an instance, for VR/AR traffic, the system capacity could decrease by 2 to 5 as the data rate increasing from 45Mbps to 60Mbps in FR1 DL. And the capacity of Cloud Gaming traffic is better than that of VR/AR traffic because of the smaller data rate and the relatively relaxed PDB requirement.

Observation 7: When the reliability requirement is relaxed from 99% to 95%, the system capacity could increase by 0 to 2 UE per cell for FR1 DL traffics.

Observation 8: When increasing the frame rate from 60FPS to 120FPS, the system capacity could increase by 3 to 7 UEs per cell for FR1 DL traffics.

    - FR2

In this section, the system capacity of CG and VR/AR traffic is evaluated in FR2 with 100MHz and 400MHz bandwidth, respectively. And the results in Indoor Hotspot and Dense Urban deployment scenarios are provided in Table 7 to Table 10. For Dense Urban scenario with 400MHz bandwidth, only 35 UEs per cell are simulated at most for CG traffic and VR/AR traffic with 45Mbps, and 25 UEs per cell are simulated at most for VR/AR traffic with 60Mbps. Under the maximum number of UEs per cell in the simulation, UE% is still close to 100%. So, ‘>’ indicates that the system capacity is larger than the value listed in #UEs per cell in Table 10, and UE% and RU correspond to #UEs per cell listed in Table 10.

- Indoor Hotspot

Table 7. FR2 DL capacity simulation results in Indoor Hotspot scenario with 100M bandwidth

| Traffic Model |  | X=95 |  |  | X=99 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | #UEs per cell | UE% | RU | #UEs per cell | UE% | RU |
| CG | 30Mbps_60FPS_15ms | 10 | 92.28% | 63.04% | 9 | 92.90% | 57.57% |
|  | 30Mbps _120FPS_15ms | 12 | 92.34% | 78.56% | 10 | 93.97% | 63.36% |
| VR/AR | 45Mbps _60FPS_10ms | 6 | 91.96% | 52.66% | 5 | 90.56% | 41.49% |
|  | 45Mbps _120FPS_10ms | 7 | 93.78% | 67.45% | 6 | 96.30% | 55.89% |
|  | 60Mbps _60FPS_10ms | 4 | 95.14% | 42.19% | 3 | 90.74% | 31.39% |
|  | 60Mbps _120FPS_10ms | 5 | 93.09% | 61.22% | 4 | 95.14% | 45.98% |

Observation 9:  When the capacity requirement per UE is more than 99% of packets are successfully delivered within a given air interface PDB,

- For DL CG with 30Mbps and 60FPS in FR2 Indoor Hotspot, system capacity is 9 UEs per cell with 100MHz bandwidth.

- For DL CG with 30Mbps and 120FPS in FR2 Indoor Hotspot, system capacity is 10 UEs per cell with 100MHz bandwidth.

- For DL VR/AR with 45Mbps and 60FPS in FR2 Indoor Hotspot, system capacity is 5 UEs per cell with 100MHz bandwidth.

- For DL VR/AR with 45Mbps and 120FPS in FR2 Indoor Hotspot, system capacity is 6 UEs per cell with 100MHz bandwidth.

- For DL VR/AR with 60Mbps and 60FPS in FR2 Indoor Hotspot, system capacity is 3 UEs per cell with 100MHz bandwidth.

- For DL VR/AR with 60Mbps and 120FPS in FR2 Indoor Hotspot, system capacity is 4 UEs per cell with 100MHz bandwidth.

Observation 10:  When the capacity requirement per UE is more than 95% of packets are successfully delivered within a given air interface PDB,

- For DL CG with 30Mbps and 60FPS in FR2 Indoor Hotspot, system capacity is 10 UEs per cell with 100MHz bandwidth.

- For DL CG with 30Mbps and 120FPS in FR2 Indoor Hotspot, system capacity is 12 UEs per cell with 100MHz bandwidth.

- For DL VR/AR with 45Mbps and 60FPS in FR2 Indoor Hotspot, system capacity is 6 UEs per cell with 100MHz bandwidth.

- For DL VR/AR with 45Mbps and 120FPS in FR2 Indoor Hotspot, system capacity is 7 UEs per cell with 100MHz bandwidth.

- For DL VR/AR with 60Mbps and 60FPS in FR2 Indoor Hotspot, system capacity is 4 UEs per cell with 100MHz bandwidth.

- For DL VR/AR with 60Mbps and 120FPS in FR2 Indoor Hotspot, system capacity is 5 UEs per cell with 100MHz bandwidth.

Table 8. FR2 DL capacity simulation results in Indoor Hotspot scenario with 400M bandwidth

| Traffic Model |  | X=95 |  |  | X=99 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | #UEs per cell | UE% | RU | #UEs per cell | UE% | RU |
| CG | 30Mbps_60FPS_15ms | 50 | 92.67% | 84.85% | 46 | 93.45% | 72.78% |
|  | 30Mbps _120FPS_15ms | 53 | 92.11% | 87.39% | 50 | 93.06% | 83.39% |
| VR/AR | 45Mbps _60FPS_10ms | 30 | 94.26% | 72.92% | 27 | 92.35% | 63.62% |
|  | 45Mbps _120FPS_10ms | 33 | 92.85% | 82.48% | 31 | 92.13% | 75.847% |
|  | 60Mbps _60FPS_10ms | 22 | 94.19% | 68.36% | 20 | 93.61% | 62.36% |
|  | 60Mbps _120FPS_10ms | 25 | 90.22% | 83.05% | 23 | 93.31% | 68.52% |

Observation 11:  When the capacity requirement per UE is more than 99% of packets are successfully delivered within a given air interface PDB,

- For DL CG with 30Mbps and 60FPS in FR2 Indoor Hotspot, system capacity is 46 UEs per cell with 400MHz bandwidth.

- For DL CG with 30Mbps and 120FPS in FR2 Indoor Hotspot, system capacity is 50 UEs per cell with 400MHz bandwidth.

- For DL VR/AR with 45Mbps and 60FPS in FR2 Indoor Hotspot, system capacity is 27 UEs per cell with 400MHz bandwidth.

- For DL VR/AR with 45Mbps and 120FPS in FR2 Indoor Hotspot, system capacity is 31 UEs per cell with 400MHz bandwidth.

- For DL VR/AR with 60Mbps and 60FPS in FR2 Indoor Hotspot, system capacity is 20 UEs per cell with 400MHz bandwidth.

- For DL VR/AR with 60Mbps and 120FPS in FR2 Indoor Hotspot, system capacity is 23 UEs per cell with 400MHz bandwidth.

Observation 12:  When the capacity requirement per UE is more than 95% of packets are successfully delivered within a given air interface PDB,

- For DL CG with 30Mbps and 60FPS in FR2 Indoor Hotspot, system capacity is 50 UEs per cell with 400MHz bandwidth.

- For DL CG with 30Mbps and 120FPS in FR2 Indoor Hotspot, system capacity is 53 UEs per cell with 400MHz bandwidth.

- For DL VR/AR with 45Mbps and 60FPS in FR2 Indoor Hotspot, system capacity is 30 UEs per cell with 400MHz bandwidth.

- For DL VR/AR with 45Mbps and 120FPS in FR2 Indoor Hotspot, system capacity is 33 UEs per cell with 400MHz bandwidth.

- For DL VR/AR with 60Mbps and 60FPS in FR2 Indoor Hotspot, system capacity is 22 UEs per cell with 400MHz bandwidth.

- For DL VR/AR with 60Mbps and 120FPS in FR2 Indoor Hotspot, system capacity is 25 UEs per cell with 400MHz bandwidth.

- Dense Urban

Table 9. FR2 DL capacity simulation results in Dense Urban scenario with 100M bandwidth

| Traffic Model |  | X=95 |  |  | X=99 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | #UEs per cell | UE% | RU | #UEs per cell | UE% | RU |
| CG | 30Mbps_60FPS_15ms | 22 | 92.92% | 78.69% | 20 | 92.65% | 75.33% |
|  | 30Mbps _120FPS_15ms | 26 | 93.37% | 87.23% | 24 | 91.50% | 83.39% |
| VR/AR | 45Mbps _60FPS_10ms | 10 | 93.33% | 63.53% | 9 | 94.18% | 56.84% |
|  | 45Mbps _120FPS_10ms | 13 | 93.96% | 82.24% | 12 | 93.47% | 77.29% |
|  | 60Mbps _60FPS_10ms | 7 | 93.60% | 59.28% | 6 | 91.27% | 50.47% |
|  | 60Mbps _120FPS_10ms | 9 | 94.83% | 77.35% | 8 | 95.83% | 69.18% |

Observation 13:  When the capacity requirement per UE is more than 99% of packets are successfully delivered within a given air interface PDB,

- For DL CG with 30Mbps and 60FPS in FR2 Dense Urban, system capacity is 20 UEs per cell with 100MHz bandwidth.

- For DL CG with 30Mbps and 120FPS in FR2 Dense Urban, system capacity is 24 UEs per cell with 100MHz bandwidth.

- For DL VR/AR with 45Mbps and 60FPS in FR2 Dense Urban, system capacity is 9 UEs per cell with 100MHz bandwidth.

- For DL VR/AR with 45Mbps and 120FPS in FR2 Dense Urban, system capacity is 12 UEs per cell with 100MHz bandwidth.

- For DL VR/AR with 60Mbps and 60FPS in FR2 Dense Urban, system capacity is 6 UEs per cell with 100MHz bandwidth.

- For DL VR/AR with 60Mbps and 120FPS in FR2 Dense Urban, system capacity is 8 UEs per cell with 100MHz bandwidth.

Observation 14:  When the capacity requirement per UE is more than 95% of packets are successfully delivered within a given air interface PDB,

- For DL CG with 30Mbps and 60FPS in FR2 Dense Urban, system capacity is 22 UEs per cell with 100MHz bandwidth.

- For DL CG with 30Mbps and 120FPS in FR2 Dense Urban, system capacity is 26 UEs per cell with 100MHz bandwidth.

- For DL VR/AR with 45Mbps and 60FPS in FR2 Dense Urban, system capacity is 10 UEs per cell with 100MHz bandwidth.

- For DL VR/AR with 45Mbps and 120FPS in FR2 Dense Urban, system capacity is 13 UEs per cell with 100MHz bandwidth.

- For DL VR/AR with 60Mbps and 60FPS in FR2 Dense Urban, system capacity is 7 UEs per cell with 100MHz bandwidth.

- For DL VR/AR with 60Mbps and 120FPS in FR2 Dense Urban, system capacity is 9 UEs per cell with 100MHz bandwidth.

Table 10. FR2 DL capacity simulation results in Dense Urban scenario with 400M bandwidth

| Traffic Model |  | X=95 |  |  | X=99 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | #UEs per cell | UE% | RU | #UEs per cell | UE% | RU |
| CG | 30Mbps_60FPS_15ms | >35 | 100% | 45.36% | >35 | 100% | 45.36% |
|  | 30Mbps _120FPS_15ms | >35 | 100% | 42.05% | >35 | 100% | 42.05% |
| VR/AR | 45Mbps _60FPS_10ms | >35 | 100% | 53.89% | >35 | 99.37% | 53.89% |
|  | 45Mbps _120FPS_10ms | >35 | 99.99% | 54.27% | >35 | 99.72% | 54.27 |
|  | 60Mbps _60FPS_10ms | >25 | 100% | 50.73% | >25 | 99.24% | 50.73% |
|  | 60Mbps _120FPS_10ms | >25 | 100% | 51.18% | >25 | 98.86% | 51.18% |

Observation 15:  When the capacity requirement per UE is more than 99% of packets are successfully delivered within a given air interface PDB,

- For DL CG with 30Mbps and 60FPS in FR2 Dense Urban, system capacity is greater than 35 UEs per cell with 400MHz bandwidth.

- For DL CG with 30Mbps and 120FPS in FR2 Dense Urban, system capacity is greater than 35 UEs per cell with 400MHz bandwidth.

- For DL VR/AR with 45Mbps and 60FPS in FR2 Dense Urban, system capacity is greater than 35 UEs per cell with 400MHz bandwidth.

- For DL VR/AR with 45Mbps and 120FPS in FR2 Dense Urban, system capacity is greater than 35 UEs per cell with 400MHz bandwidth.

- For DL VR/AR with 60Mbps and 60FPS in FR2 Dense Urban, system capacity is greater than 25 UEs per cell with 400MHz bandwidth.

- For DL VR/AR with 60Mbps and 120FPS in FR2 Dense Urban, system capacity is greater than 25 UEs per cell with 400MHz bandwidth.

Observation 16:  When the capacity requirement per UE is more than 95% of packets are successfully delivered within a given air interface PDB,

- For DL CG with 30Mbps and 60FPS in FR2 Dense Urban, system capacity is greater than 35 UEs per cell with 400MHz bandwidth.

- For DL CG with 30Mbps and 120FPS in FR2 Dense Urban, system capacity is greater than 35 UEs per cell with 400MHz bandwidth.

- For DL VR/AR with 45Mbps and 60FPS in FR2 Dense Urban, system capacity is greater than 35 UEs per cell with 400MHz bandwidth.

- For DL VR/AR with 45Mbps and 120FPS in FR2 Dense Urban, system capacity is greater than 35 UEs per cell with 400MHz bandwidth.

- For DL VR/AR with 60Mbps and 60FPS in FR2 Dense Urban, system capacity is greater than 25 UEs per cell with 400MHz bandwidth.

- For DL VR/AR with 60Mbps and 120FPS in FR2 Dense Urban, system capacity is greater than 25 UEs per cell with 400MHz bandwidth.

From the above tables, it can be observed that compared to100MHz bandwidth, much more users will be satisfied with 400MHz bandwidth due to wider bandwidth. For an instance, for VR/AR traffic with 60Mbps and 60 FPS with 400MHz bandwidth, when reliability requirement is 99%, the capacity in Indoor Hotspot scenarios is 20, and the capacity in Dense Urban scenarios is larger than 25. But under the same conditions, with 100MHz bandwidth, the capacity is only 4 and 8 in Indoor Hotspot scenario and Dense Urban scenario, respectively.

  - UL

For the UL evaluations, the packet size distribution of Scene information is assumed to follow truncated Gaussian distribution and the data rate is assumed as 20 Mbps. Pose information is periodically sampled and delivered with a fixed size. No jitter is assumed for UL evaluations. The traffic characteristics and requirements for Scene information and Pose information are presented in Table 11 and Table 12 separately.

Table 11. Traffic models of UL Scene information

| Traffic model | Packet Mean | Packet STD | Packet Max | Packet Min | Packet arrival interval | PDB | Jitter |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Scene information | 41667Bytes | 6250Bytes | 62500Bytes | 4167Bytes | 16.67ms | 60ms | No |

Table 12. Traffic models of UL Pose information

| Traffic model | Packet Mean | Packet arrival interval | PDB | Jitter |
| --- | --- | --- | --- | --- |
| Pose information | 100 Bytes | 4ms | 10ms | No |

    - FR1

- Scene information

Table 13. FR1 UL capacity simulation results for Scene information

| Scenarios | X=95 |  |  | X=99 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | #UEs per cell | UE% | RU | #UEs per cell | UE% | RU |
| Indoor Hotspot | 6 | 100% | 62.37% | 6 | 99.07% | 62.37% |
| Dense Urban | 3 | 93.86% | 51.45% | 3 | 92.30% | 51.45% |
| Urban Macro | 1 | 74.60% | 39.36% | 1 | 71.43% | 39.36% |

The simulation results of system capacity for Scene information are provided in Table 13. For FR1 UL transmission, the capacity in Indoor Hotspot scenario is superior to that in Dense Urban and Urban Macro scenario. With “DDDSU” TDD slot allocation, 6 and 3 UEs per cell can satisfy the UL capacity requirement for Indoor Hotspot and Dense Urban scenarios, respectively. But for UL Scene information in FR1 Urban Macro scenario with 1 UE per cell, only 74.60% of UEs can satisfy system capacity requirement even with 95% reliability requirement. Besides, there is no effect on capacity by relaxing reliability requirement from 99% to 95%.

Observation 17: When the capacity requirement per UE is more than 99% of packets are successfully delivered within a given air interface PDB,

- For UL Scene information in FR1 Indoor Hotspot, system capacity is 6 UEs per cell.

- For UL Scene information in FR1 Dense Urban, system capacity is 3 UEs per cell.

- For UL Scene information in FR1 Urban Macro with 1 UE per cell, there is less than 90% of UEs that can satisfy system capacity requirement.

Observation 18: When the capacity requirement per UE is more than 95% of packets are successfully delivered within a given air interface PDB,

- For UL Scene information in FR1 Indoor Hotspot, system capacity is 6 UEs per cell.

- For UL Scene information in FR1 Dense Urban, system capacity is 3 UEs per cell.

- For UL Scene information in FR1 Urban Macro with 1 UE per cell, there is less than 90% of UEs that can satisfy system capacity requirement.

Observation 19: For Scene information in FR1 UL, system capacity cannot be improved by relaxing reliability requirement from 99% to 95%.

- Pose information

Table 14. FR1 UL capacity simulation results for Pose information

| Scenarios | #UEs per cell | UE% | RU | Latency |
| --- | --- | --- | --- | --- |
| Indoor Hotspot | >15 | 100% | 20.47% | 2.86ms |
| Dense Urban | >15 | 100% | 20.76% | 2.94ms |
| Urban Macro | >15 | 95.99% | 22.46% | 3.82ms |

| - UE% | - Latency |
| --- | --- |

Figure 1. UL simulation results for Pose information in FR1

The simulation results of system capacity for Pose information are provided in Table 14 and Figure 1. For the case of Pose information, UE can achieve tight transmission reliability due to the smaller traffic size. Hence, only 99% transmission reliability is assumed for Pose information. In addition to the percentage of UEs with ≥99% packets successfully delivered within PDB, the value of the 95th percentile on the CDF curve for user interaction delay is also the key metric for UL capacity evaluation. Considering that the system capacity of VR/AR traffic is no more than 15 in FR1 DL transmission, 15 UEs at most are simulated in FR1 UL for Pose information.

As shown in Figure 1, with less than or equal to 15 UEs per cell, the percentage of satisfied UEs (reliability requirement equals 99%) could reach 100% for Indoor Hotspot and Dense Urban scenario, and reach 95.99% for Urban Macro scenarios, because of smaller packet sizes for Pose information. So ‘>’ indicates that the system capacity is greater than the value listed in #UEs per cell in Table 14. Moreover, the 95th percentile user interaction delay can absolutely satisfy the uplink PDB requirement.

Observation 20:  When the capacity requirement per UE is more than 99% of packets are successfully delivered within a given air interface PDB,

- For UL Pose information in FR1 Indoor Hotspot, system capacity is greater than 15 UEs per cell.

- For UL Pose information in FR1 Dense Urban, system capacity is greater than 15 UEs per cell.

- For UL Pose information in FR1 Urban Macro, system capacity is greater than 15 UEs per cell.

Observation 21: For Pose information in FR1 for UL, 95th percentile user interaction delay is far below 10ms with the number of UEs per cell up to 15.

    - FR2

The simulation results of system capacity for Scene information and Pose information are provided in Table 15 and Table 16 respectively, where only 400MHz bandwidth is evaluated in FR2 UL. For Scene information and Pose information, only 30UEs per cell are simulated at most in Indoor Hotspot and Dense Urban deployment scenarios. Under the maximum number of UEs per cell in the simulation, UE% is still larger than 90%. So, ‘>’ indicates that the system capacity is larger than the value listed in #UEs per cell in Table 15 and Table 16, and UE%, RU and Latency correspond to #UEs per cell listed in Table 15 and Table 16.

- Scene information

Table 15. FR2 UL capacity simulation results for Scene information with 400MHz bandwidth

| Scenarios | X=95 |  |  | X=99 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | #UEs per cell | UE% | RU | #UEs per cell | UE% | RU |
| Indoor Hotspot | >30 | 100% | 8.19% | >30 | 97.69% | 8.19% |
| Dense Urban | >30 | 98.67% | 12.26% | >30 | 96.98% | 12.26% |

Observation 22:  When the capacity requirement per UE is more than 99% of packets are successfully delivered within a given air interface PDB,

- For UL Scene information in FR2 Indoor Hotspot, system capacity is greater than 30 UEs per cell with 400MHz bandwidth.

- For UL Scene information in FR2 Dense Urban, system capacity is greater than 30 UEs per cell with 400MHz bandwidth.

Observation 23:  When the capacity requirement per UE is more than 95% of packets are successfully delivered within a given air interface PDB,

- For UL Scene information in FR2 Indoor Hotspot, system capacity is greater than 30 UEs per cell with 400MHz bandwidth.

- For UL Scene information in FR2 Dense Urban, system capacity is greater than 30 UEs per cell with 400MHz bandwidth.

- Pose information

Table 16. FR2 UL capacity simulation results for Pose information with 400MHz bandwidth

| Scenarios | #UEs per cell | UE% | RU | Latency |
| --- | --- | --- | --- | --- |
| Indoor Hotspot | >30 | 98.98% | 7.35% | 4.50ms |
| Dense Urban | >30 | 97.69% | 7.26% | 5.28ms |

Observation 24:  When the capacity requirement per UE is more than 99% of packets are successfully delivered within a given air interface PDB,

- For UL Pose information in FR2 Indoor Hotspot, system capacity is greater than 30 UEs per cell with 400MHz bandwidth.

- For UL Pose information in FR2 Dense Urban, system capacity is greater than 30 UEs per cell with 400MHz bandwidth.

- Performance evaluation results of power consumption

As analyzed in our companion’s contribution [2], power consumption is the crucial issue for XR traffic and it is another key metric for XR evaluation, since the specific characteristics of XR traffic i.e., dense traffic period, high traffic data rates and low latency requirement etc. which is different from eMBB and URLLC. For power consumption evaluation, UE capacity performance should be jointly taken into consideration.

According to the last meeting’s agreements, no DRX configuration (i.e. without adopting any power saving mechanism) which can make no capacity loss is the baseline scheme. And CDRX configuration adopted in R15 should be evaluated as a power saving scheme. In addition, the potential enhancement of CDRX and the ongoing power saving schemes in R17 power saving WI, such as PDCCH skipping, search space set group switching mechanisms are also evaluated to contrastive analysis the potential impact on power consumption and corresponding system capacity. In this section, power consumption of different schemes is evaluated by using downlink and uplink traffic models with 60 FPS. Table 17 gives the adopted traffic models and corresponding scenarios for power consumption evaluation.

Table 17. Traffic model and scenario combinations used for power consumption evaluation

| DL FR1 - Scenario: - Indoor Hotspot/Dense Urban/Urban Macro - Traffic model: - CG: 30Mbps, 60FPS, PDB=15ms - VR/AR: 45/60Mbps, 60FPS, PDB=10ms | UL FR1 - Scenario: - Indoor Hotspot/Dense Urban - Traffic model: - Scene information - Pose information |
| --- | --- |

  - DL

For downlink CG traffic and XR traffic in FR1, in addition to no DRX configuration (Case1) as the baseline, DRX configurations (Case2) with two sets of parameters are evaluated, which are denoted as Case2_1 and Case2_2 shown in Table 18. Note that 10ms is the minimum DRX cycle configuration supported by the current specification and 4ms of DRX cycle is not yet introduced.

Table 18. The DRX configuration for Case2 and Case 3

| Cases | DRX cycle (ms) | drx-onDurationTimer (ms) | drx-InactivityTimer(ms) |
| --- | --- | --- | --- |
| Case2_1 | 10 | 8 | 4 |
| Case2_2 | 4 | 3 | 3 |
| Case3 | 20 | 6 | 4 |

Due to non-integer XR traffic periodicity, the existing integer DRX cycle cannot align with the traffic arrival and it will increase data transmission delay. To solve this issue, a straightforward method is to introduce some non-integer values of DRX cycle to align with the XR traffic periodicity. But it is not flexible because of the various traffic period and the jitter effect of DL burst arrival. To solve the impact of jitter and align DRX cycle with traffic arrival, the adjustment to the start offset of DRX ON Duration could be a potential method (Case 3), which is an enhancement of the CDRX scheme and is equivalent to aligning DRX cycle with the period of XR traffic. The parameters of Case 3 are also given in Table 18. Additionally, PDCCH skipping mechanism with the setting of jitter window (Case4) can also make a better trade-off between power saving and capacity. During the jitter window, gNB will not send the PDCCH skipping command until the XR traffic arrive. And the length of the jitter window equals to the maximum positive jitter range i.e. +4ms.

Based on the assumptions and configurations mentioned above, the UE capacity and power consumption results in FR1 DL are shown in Table 19 to Table 27, where the power saving gain (PSG) is calculated compared to the baseline scheme (Case 1, i.e. no DRX configuration). The number of UEs per cell is configured as the maximum number of UEs per cell with that at least 90 % among them are satisfied UEs (i.e. the system capacity defined in section 2.1). And the capacity requirement per UE is more than 99% of packets are successfully delivered within a given air interface PDB. The mean power consumption shown in the following tables is the average power consumption within each slot.

- Case1(Baseline): Without adopting any power saving mechanism.

- Case2_1 and 2_2: adopting R15/R16 DRX configuration.

- Case3: aligning the start time of DRX ON Duration with the arrival of XR traffic.

- Case4: PDCCH skipping with jitter handling.

    - CG 30Mbps_60FPS_15ms

- Indoor Hotspot

Table 19. FR1 DL UE capacity and power consumption results in Indoor Hotspot scenario

| Metrics Cases | Satisfied #UE / total #UE per cell | RU | Mean_power | Power Saving Gain (PSG) compared to Case 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Mean PSG | PSG of 5%-tile UE in PSG CDF | PSG of 50%-tile UE in PSG CDF | PSG of 95%-tile UE in PSG CDF |
| Case1 | 15.33/16 | 86.95% | 122.59 | - | - | - | - |
| Case2_1 | 15.08/16 | 86.10% | 119.03 | 3.22% | 1.57% | 3.28% | 5.25% |
| Case2_2 | 15.25/16 | 86.90% | 116.71 | 5.10% | 3.12% | 5.43% | 7.37% |
| Case3 | 15.08/16 | 86.93% | 92.23 | 24.76% | 11.65% | 26.14% | 35.95% |
| Case4 | 15.25/16 | 86.96% | 86.15 | 29.73% | 19.87% | 31.30% | 36.64% |

Observation 25: For DL CG with 30Mbps and 60FPS in FR1 Indoor Hotspot,

- 3.22% mean PSG can be achieved without capacity loss by using DRX scheme with 10ms DRX cycle.

- 5.10% mean PSG can be achieved without capacity loss by using DRX scheme with 4ms DRX cycle.

- 24.76% mean PSG can be achieved without capacity loss by using aligning DRX ON Duration scheme.

- 29.73% mean PSG can be achieved without capacity loss by using PDCCH skipping with jitter handling scheme.

- Dense Urban

Table 20. FR1 DL UE capacity and power consumption results in Dense Urban scenario

| Metrics Cases | Satisfied #UE / total #UE per cell | RU | Mean_power | Power Saving Gain (PSG) compared to Case 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Mean PSG | PSG of 5%-tile UE in PSG CDF | PSG of 50%-tile UE in PSG CDF | PSG of 95%-tile UE in PSG CDF |
| Case1 | 22.20/24 | 96.39% | 131.49 | - | - | - | - |
| Case2_1 | 21.97/24 | 95.31% | 128.77 | 2.07% | 0.35% | 2.92% | 4.93% |
| Case2_2 | 22.10/24 | 95.96% | 126.73 | 3.62% | 2.05% | 3.72% | 5.32% |
| Case3 | 22.08/24 | 96.58% | 105.68 | 19.63% | 5.63% | 21.70% | 34.90% |
| Case4 | 22.14/24 | 96.54% | 97.58 | 25.79% | 12.85% | 28.62% | 36.00% |

Observation 26: For DL CG with 30Mbps and 60FPS in FR1 Dense Urban,

- 2.07% mean PSG can be achieved without capacity loss by using DRX scheme with 10ms DRX cycle.

- 3.62% mean PSG can be achieved without capacity loss by using DRX scheme with 4ms DRX cycle.

- 19.63% mean PSG can be achieved without capacity loss by using aligning DRX ON Duration scheme.

- 25.79% mean PSG can be achieved without capacity loss by using PDCCH skipping with jitter handling scheme.

- Urban Macro

Table 21. FR1 DL UE capacity and power consumption results in Urban Macro scenario

| Metrics Cases | Satisfied #UE / total #UE per cell | RU | Mean_power | Power Saving Gain (PSG) compared to Case 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Mean PSG | PSG of 5%-tile UE in PSG CDF | PSG of 50%-tile UE in PSG CDF | PSG of 95%-tile UE in PSG CDF |
| Case1 | 15.13/16 | 89.84% | 126.72 | - | - | - | - |
| Case2_1 | 14.84/16 | 89.54% | 123.65 | 2.42% | 0.39% | 3.07% | 5.22% |
| Case2_2 | 15.00/16 | 90.37% | 122.05 | 3.69% | 2.25% | 3.88% | 5.56% |
| Case3 | 14.98/16 | 90.75% | 98.73 | 22.09% | 7.06% | 24.48% | 36.04% |
| Case4 | 14.98/16 | 90.83% | 92.16 | 27.27% | 13.39% | 30.19% | 36.84% |

Observation 27: For DL CG with 30Mbps and 60FPS in FR1 Urban Macro,

- 2.42% mean PSG can be achieved without capacity loss by using DRX scheme with 10ms DRX cycle.

- 3.69% mean PSG can be achieved without capacity loss by using DRX scheme with 4ms DRX cycle.

- 22.09% mean PSG can be achieved without capacity loss by using aligning DRX ON Duration scheme.

- 27.27% mean PSG can be achieved without capacity loss by using PDCCH skipping with jitter handling scheme.

    - VR/AR 45Mbps_60FPS_10ms

- Indoor Hotspot

Table 22. FR1 DL UE capacity and power consumption results in Indoor Hotspot scenario

| Metrics Cases | Satisfied #UE / total #UE per cell | RU | Mean_power | Power Saving Gain (PSG) compared to Case 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Mean PSG | PSG of 5%-tile UE in PSG CDF | PSG of 50%-tile UE in PSG CDF | PSG of 95%-tile UE in PSG CDF |
| Case1 | 5.61/6 | 57.71% | 117.37 | - | - | - | - |
| Case2_1 | 5.44/6 | 57.44% | 112.60 | 4.06% | 2.40% | 4.27% | 5.63% |
| Case2_2 | 5.50/6 | 57.58% | 110.36 | 5.97% | 4.35% | 6.14% | 7.71% |
| Case3 | 5.42/6 | 57.24% | 84.04 | 28.40% | 18.71% | 29.57% | 37.32% |
| Case4 | 5.50/6 | 57.27% | 79.54 | 32.23% | 26.29% | 32.78% | 37.39% |

Observation 28: For DL VR/AR with 45Mbps and 60FPS in FR1 Indoor Hotspot,

- 4.06% mean PSG can be achieved without capacity loss by using DRX scheme with 10ms DRX cycle.

- 5.97% mean PSG can be achieved without capacity loss by using DRX scheme with 4ms DRX cycle.

- 28.40% mean PSG can be achieved without capacity loss by using aligning DRX ON Duration scheme.

- 32.23% mean PSG can be achieved without capacity loss by using PDCCH skipping with jitter handling scheme.

- Dense Urban

Table 23. FR1 DL UE capacity and power consumption results in Dense Urban scenario

| Metrics Cases | Satisfied #UE / total #UE per cell | RU | Mean_power | Power Saving Gain (PSG) compared to Case 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Mean PSG | PSG of 5%-tile UE in PSG CDF | PSG of 50%-tile UE in PSG CDF | PSG of 95%-tile UE in PSG CDF |
| Case1 | 8.35/9 | 72.94% | 120.12 | - | - | - | - |
| Case2_1 | 8.18/9 | 71.84% | 115.49 | 3.86% | 2.31% | 4.05% | 5.86% |
| Case2_2 | 8.23/9 | 72.29% | 113.27 | 5.70% | 3.82% | 5.90% | 7.90% |
| Case3 | 8.24/9 | 72.62% | 87.96 | 26.77% | 16.56% | 27.85% | 37.38% |
| Case4 | 8.21/9 | 70.53% | 82.60 | 31.24% | 13.03% | 31.91% | 45.99% |

Observation 29: For DL VR/AR with 45Mbps and 60FPS in FR1 Dense Urban,

- 3.86% mean PSG can be achieved without capacity loss by using DRX scheme with 10ms DRX cycle.

- 5.70% mean PSG can be achieved without capacity loss by using DRX scheme with 4ms DRX cycle.

- 26.77% mean PSG can be achieved without capacity loss by using aligning DRX ON Duration scheme.

- 31.24% mean PSG can be achieved without capacity loss by using PDCCH skipping with jitter handling scheme.

- Urban Macro

Table 24. FR1 DL UE capacity and power consumption results in Urban Macro scenario

| Metrics Cases | Satisfied #UE / total #UE per cell | RU | Mean_power | Power Saving Gain (PSG) compared to Case 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Mean PSG | PSG of 5%-tile UE in PSG CDF | PSG of 50%-tile UE in PSG CDF | PSG of 95%-tile UE in PSG CDF |
| Case1 | 5.54/6 | 59.32% | 118.17 | - | - | - | - |
| Case2_1 | 5.47/6 | 59.67% | 113.74 | 3.75% | 1.25% | 4.30% | 6.06% |
| Case2_2 | 5.51/6 | 59.80% | 111.54 | 5.61% | 2.58% | 6.30% | 8.07% |
| Case3 | 5.52/6 | 59.56% | 86.08 | 27.15% | 12.29% | 29.15% | 37.54% |
| Case4 | 5.46/6 | 58.65% | 81.42 | 31.10% | 7.92% | 33.49% | 47.24% |

Observation 30: For DL VR/AR with 45Mbps and 60FPS in FR1 Urban Macro,

- 3.75% mean PSG can be achieved without capacity loss by using DRX scheme with 10ms DRX cycle.

- 5.61% mean PSG can be achieved without capacity loss by using DRX scheme with 4ms DRX cycle.

- 27.15% mean PSG can be achieved without capacity loss by using aligning DRX ON Duration scheme.

- 31.10% mean PSG can be achieved without capacity loss by using PDCCH skipping with jitter handling scheme.

    - VR/AR 60Mbps_60FPS_10ms

- Indoor Hotspot

Table 25. FR1 DL UE capacity and power consumption results in Indoor Hotspot scenario

| Metrics Cases | Satisfied #UE / total #UE per cell | RU | Mean_power | Power Saving Gain (PSG) compared to Case 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Mean PSG | PSG of 5%-tile UE in PSG CDF | PSG of 50%-tile UE in PSG CDF | PSG of 95%-tile UE in PSG CDF |
| Case1 | 2.97/3 | 39.26% | 116.75 | - | - | - | - |
| Case2_1 | 2.89/3 | 39.40% | 111.74 | 4.29% | 2.86% | 4.32% | 5.80% |
| Case2_2 | 2.92/3 | 39.33% | 109.54 | 6.17% | 4.43% | 6.33% | 7.84% |
| Case3 | 2.83/3 | 38.87% | 82.81 | 29.07% | 19.28% | 30.49% | 35.83% |
| Case4 | 2.84/3 | 38.86% | 78.57 | 32.70% | 26.82% | 33.53% | 36.18% |

Observation 31: For DL VR/AR with 60Mbps and 60FPS in FR1 Indoor Hotspot,

- 4.29% mean PSG can be achieved without capacity loss by using DRX scheme with 10ms DRX cycle.

- 6.17% mean PSG can be achieved without capacity loss by using DRX scheme with 4ms DRX cycle.

- 29.07% mean PSG can be achieved without capacity loss by using aligning DRX ON Duration scheme.

- 32.70% mean PSG can be achieved without capacity loss by using PDCCH skipping with jitter handling scheme.

- Dense Urban

Table 26. FR1 DL UE capacity and power consumption results in Dense Urban scenario

| Metrics Cases | Satisfied #UE / total #UE per cell | RU | Mean_power | Power Saving Gain (PSG) compared to Case 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Mean PSG | PSG of 5%-tile UE in PSG CDF | PSG of 50%-tile UE in PSG CDF | PSG of 95%-tile UE in PSG CDF |
| Case1 | 4.67/5 | 57.60% | 120.15 | - | - | - | - |
| Case2_1 | 4.52/5 | 57.32% | 115.34 | 4.01% | 2.56% | 4.12% | 5.67% |
| Case2_2 | 4.59/5 | 57.36% | 113.11 | 5.86% | 4.28% | 5.98% | 7.71% |
| Case3 | 4.57/5 | 57.18% | 88.09 | 26.68% | 17.88% | 27.36% | 35.32% |
| Case4 | 4.57/5 | 57.23% | 82.68 | 31.18% | 25.13% | 32.02% | 35.92% |

Observation 32: For DL VR/AR with 60Mbps and 60FPS in FR1 Dense Urban,

- 4.01% mean PSG can be achieved without capacity loss by using DRX scheme with 10ms DRX cycle.

- 5.86% mean PSG can be achieved without capacity loss by using DRX scheme with 4ms DRX cycle.

- 26.68% mean PSG can be achieved without capacity loss by using aligning DRX ON Duration scheme.

- 31.18% mean PSG can be achieved without capacity loss by using PDCCH skipping with jitter handling scheme.

- Urban Macro

Table 27. FR1 DL UE capacity and power consumption results in Urban Macro scenario

| Metrics Cases | Satisfied #UE / total #UE per cell | RU | Mean_power | Power Saving Gain (PSG) compared to Case 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Mean PSG | PSG of 5%-tile UE in PSG CDF | PSG of 50%-tile UE in PSG CDF | PSG of 95%-tile UE in PSG CDF |
| Case1 | 2.80/3 | 42.54% | 119.88 | - | - | - | - |
| Case2_1 | 2.73/3 | 43.44% | 115.61 | 3.56% | 0.33% | 4.17% | 5.76% |
| Case2_2 | 2.74/3 | 43.37% | 113.42 | 5.39% | 2.10% | 6.00% | 7.79% |
| Case3 | 2.74/3 | 44.67% | 88.55 | 26.14% | 10.11% | 28.58% | 35.50% |
| Case4 | 2.73/3 | 42.63% | 83.36 | 30.46% | 18.20% | 32.62% | 36.01% |

Observation 33: For DL VR/AR with 60Mbps and 60FPS in FR1 Urban Macro,

- 3.56% mean PSG can be achieved without capacity loss by using DRX scheme with 10ms DRX cycle.

- 5.39% mean PSG can be achieved without capacity loss by using DRX scheme with 4ms DRX cycle.

- 26.14% mean PSG can be achieved without capacity loss by using aligning DRX ON Duration scheme.

- 30.46% mean PSG can be achieved without capacity loss by using PDCCH skipping with jitter handling scheme.

Based on the observations given above, for Case 2, to ensure achieving the requirement of capacity, the values of drx-onDurationTimer and drx-InactivityTimer should be larger enough to provide sufficient scheduling, transmission opportunities and cover the jitter range (i.e. [-4, 4]ms). Thus, only about 6% PSG can be obtained by Case2 (i.e., using R15/R16 DRX configuration). Compared to Case2_1, Case2_2 with a shorter DRX cycle will attain a bit more PSG.

Regarding to Case3 which can align the start time of DRX ON Duration with the ideal arrival of XR traffic dynamically, the traffic bursts will be scheduled in time and the power consumption due to mismatching will be reduced. Also, a longer DRX cycle (i.e. 20ms) can be configured without impacting any system capacity. Additionally, to handle jitter, the length of drx-onDurationTimer should be no less than the maximum positive jitter range (i.e. +4ms) in our evaluation. Similar effects can be also achieved by adopting Case4, i.e. PDCCH skipping with jitter handling scheme. Compared to Case3, Case4 shows a little more PSG, since the PDCCH skipping command can be sent and applied immediately without waiting for the expiry of drx-InactivityTimer, when there is no traffic buffered in the gNB side. However, it is no doubt that Case4 will bring more signaling overhead than Case3, since the PDCCH skipping command is required at least once per DRX cycle.

As observed from the above tables, compared to Case2 (i.e. the R15/R16 DRX configuration), both Case3 (i.e. aligning the start time of DRX ON Duration with the arrival of XR traffic) and Case4 (i.e. PDCCH skipping scheme) can achieve about 30% mean PSG without capacity loss, which is five times as much as the mean PSG of Case 2.

Observation 34: By adopting the R15/R16 DRX configuration, only 2.07%~6.17% mean PSG can be obtained without the loss of the system capacity for FR1 DL.

Observation 35: 19.63%~32.7% mean PSG will be achieved by aligning the start time of DRX ON Duration with the arrival of XR traffic or PDCCH skipping scheme without capacity loss, which is five times as much as the power saving gain obtained by the R15/R16 DRX configuration.

  - UL

    - Scene information

Similar to the characteristics of XR downlink traffic, XR uplink traffic such as Scene information also has the non-integer period (e.g., 16.67ms). Differently, there is no need to model jitter for uplink traffic. Therefore, the four power saving schemes without jitter handling and evaluation method evaluated by using DL XR traffic can be reused for uplink Scene information. And the DRX parameters of Case2_1, Case 2_2 and Case3 are shown in Table 28.

- Case1(Baseline): without adopting any power saving mechanism.

- Case2_1 and 2_2: adopting R15/R16 DRX configuration.

- Case3: aligning the start time of DRX ON Duration with the arrival of XR traffic.

- Case4: PDCCH skipping.

Table 28. The DRX configuration for Case2 and Case 3

| Cases | DRX cycle (ms) | drx-onDurationTimer (ms) | drx-InactivityTimer(ms) |
| --- | --- | --- | --- |
| Case2_1 | 10 | 8 | 4 |
| Case2_2 | 4 | 3 | 3 |
| Case3 | 20 | 6 | 4 |

- Indoor Hotspot

Table 29. FR1 UL capacity and power consumption results in Indoor Hotspot scenario

| Metrics Cases | Satisfied #UE / total #UE per cell | RU | Mean_power | Power Saving Gain (PSG) compared to Case 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Mean PSG | PSG of 5%-tile UE in PSG CDF | PSG of 50%-tile UE in PSG CDF | PSG of 95%-tile UE in PSG CDF |
| Case1 | 5.94/6 | 62.37% | 107.00 | - | - | - | - |
| Case2_1 | 5.78/6 | 66.11% | 101.41 | 5.22% | 2.77% | 5.25% | 7.48% |
| Case2_2 | 5.78/6 | 66.37% | 98.79 | 7.67% | 5.27% | 7.95% | 9.66% |
| Case3 | 5.69/6 | 63.27% | 66.78 | 37.59% | 26.09% | 39.10% | 44.16% |
| Case4 | 5.83/6 | 63.30% | 51.31 | 52.04% | 35.64% | 54.47% | 60.87% |

Observation 36: For UL Scene information in FR1 Indoor Hotspot,

- 5.22% mean PSG can be achieved without capacity loss by using DRX scheme with 10ms DRX cycle.

- 7.67% mean PSG can be achieved without capacity loss by using DRX scheme with 4ms DRX cycle.

- 37.59% mean PSG can be achieved without capacity loss by using aligning DRX ON Duration scheme.

- 52.04% mean PSG can be achieved without capacity loss by using PDCCH skipping scheme.

- Dense Urban

Table 30. FR1 UL capacity and power consumption results in Dense Urban scenario

| Metrics Cases | Satisfied #UE / total #UE per cell | RU | Mean_power | Power Saving Gain (PSG) compared to Case 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Mean PSG | PSG of 5%-tile UE in PSG CDF | PSG of 50%-tile UE in PSG CDF | PSG of 95%-tile UE in PSG CDF |
| Case1 | 2.77/3 | 51.45% | 121.06 | - | - | - | - |
| Case2_1 | 2.75/3 | 51.24% | 113.79 | 6.01% | 1.28% | 7.54% | 8.61% |
| Case2_2 | 2.77/3 | 51.26% | 112.16 | 7.35% | 1.62% | 9.11% | 10.18% |
| Case3 | 2.75/3 | 51.34% | 80.91 | 31.13% | 1.53% | 39.44% | 57.79% |
| Case4 | 2.77/3 | 51.44% | 71.57 | 40.88% | 0.16% | 53.76% | 61.58% |

Observation 37: For UL Scene information in FR1 Dense Urban,

- 6.01% mean PSG can be achieved without capacity loss by using DRX scheme with 10ms DRX cycle.

- 7.35% mean PSG can be achieved without capacity loss by using DRX scheme with 4ms DRX cycle.

- 31.13% mean PSG can be achieved without capacity loss by using aligning DRX ON Duration scheme.

- 40.88% mean PSG can be achieved without capacity loss by using PDCCH skipping scheme.

Similar as the results and analyses for downlink traffic, as shown in Table 29 and Table 30, only 5.22%~7.67% mean PSG can be achieved by using the R15/R16 DRX configuration. And 31.13%~37.59% mean PSG can be obtained without loss of the system capacity for Case3. In order to ensure the performance of cell edge UEs, Case3 has to configure a longer drx-onDurationTimer and drx-InactivityTimer, which increases unnecessary power consumption for High SINR UEs. In fact, gNB can configure per UE DRX configuration to achieve a larger mean PSG for Case3. For Case4, there is no need to configure the jitter window and thus, 40.88%~52.04% mean PSG can be obtained without loss of the system capacity.

Observation 38: By adopting the R15/R16 DRX configuration, only 5.22%~7.67% mean PSG can be obtained without the loss of the system capacity for FR1 uplink Scene information.

Observation 39: For FR1 uplink Scene information, 31.13%~37.59% PSG and 40.88%~52.04% mean PSG can be obtained without capacity loss by aligning the start time of DRX ON Duration with the arrival of XR traffic and adopting PDCCH skipping scheme, respectively.

    - Pose information

In the last meeting, the period of UL Pose information is assumed as 4ms without jitter. So, an integer DRX cycle can be configured to match the UL traffic arrival. Note that the packet arrival interval of Pose information is far below the corresponding PDB requirement (i.e., 10ms), and the packet size is extremely small which can be finished by using only one TTI. Due to these reasons, to further avoid the transition between wake up and go to sleep states, several packets can be bundled together (i.e. Case3). In our evaluation, considering the potential impact on the system capacity, we assume that two UL Pose information packets can be bundled transmitted. Figure 2 illustrates the packets bundled transmission with corresponding DRX configurations compared to UL Pose information transmission without using packet bundling. As analyzed in Figure 2, the PDB of bundled packet should take the minimum PDB of the two single packets, i.e. the PDB of bundled packet is 6ms. And the corresponding DRX configuration parameters for Case2 and Case3 can be found in Table 31.

- Case1(Baseline): Without adopting any power saving mechanism.

- Case2: DRX configuration without packet bundling.

- Case3: packets bundled transmission (i.e., two packets are bundled together for transmission at a time)

Table 31. The DRX configuration for Case2 and Case3

| Cases | DRX cycle (ms) | drx-onDurationTimer (ms) | drx-InactivityTimer(ms) |
| --- | --- | --- | --- |
| Case2 | 4 | 2 | 1 |
| Case3 | 8 | 3 | 1 |

Figure 2. Packets bundled transmission with corresponding DRX configurations

The power saving gain of different schemes for FR1 UL Pose information in Indoor Hotspot scenario and Dense Urban scenario can be found in Table 32 and Table 33, respectively. Based on the capacity evaluation of UL Pose information in section 2.2.1, 15 UEs per cell are configured. And a UE will be declared as a satisfied UE if more than 99% of the uplink packets are successfully delivered within PDB.

- Indoor Hotspot

Table 32. FR1 UL capacity and power consumption results in Indoor Hotspot scenario

| Metrics Cases | Satisfied #UE / total #UE per cell | RU | Mean_power | Power Saving Gain (PSG) compared to Case 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Mean PSG | PSG of 5%-tile UE in PSG CDF | PSG of 50%-tile UE in PSG CDF | PSG of 95%-tile UE in PSG CDF |
| Case1 | 15.00/15 | 20.47% | 118.48 | - | - | - | - |
| Case2 | 15.00/15 | 18.59% | 91.43 | 22.83% | 22.82% | 22.83% | 22.85% |
| Case3 | 15.00/15 | 11.75% | 75.59 | 36.20% | 36.18% | 36.20% | 36.23% |

Observation 40: For UL Pose information in FR1 Indoor Hotspot,

- 22.83% mean PSG can be achieved without capacity loss by using DRX without considering packets bundling.

- 36.20% mean PSG can be achieved without capacity loss by using packets bundled transmission.

- Dense Urban

Table 33. FR1 UL capacity and power consumption results in Dense Urban scenario

| Metrics Cases | Satisfied #UE / total #UE per cell | RU | Mean_power | Power Saving Gain (PSG) compared to Case 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Mean PSG | PSG of 5%-tile UE in PSG CDF | PSG of 50%-tile UE in PSG CDF | PSG of 95%-tile UE in PSG CDF |
| Case1 | 15.00/15 | 20.76% | 119.97 | - | - | - | - |
| Case2 | 14.59/15 | 18.44% | 92.46 | 22.93% | 22.97% | 22.92% | 23.25% |
| Case3 | 13.66/15 | 12.88% | 76.38 | 36.34% | 35.45% | 36.22% | 37.54% |

Observation 41: For UL Pose information in FR1 Dense Urban,

- 22.93% mean PSG can be achieved without capacity loss by using DRX without considering packets bundling.

- 36.34% mean PSG can be achieved without capacity loss by using packets bundled transmission.

Compared with the DRX configuration without packets bundling, the packets bundled transmission scheme can obtain additional 13.37% and 13.41% PSG in Indoor Hotspot and Dense Urban scenarios, respectively.

Observation 42: Compared with the DRX configuration without packets bundling, the packets bundled transmission scheme can obtain additional 13.37% and 13.41% PSG for Indoor Hotspot scenario and Dense Urban scenario, respectively.

- Performance evaluation results of coverage

In our companion’s contribution, we propose to use max isotropic loss (MIL) as the XR coverage evaluation metric[3]. The MIL can be deduced based on link budget template. Most of simulation assumptions for capacity evaluation can be reused, and the channel-specific parameters of PDSCH and PUSCH are shown in appendix B, the channel-specific parameters of other channels are described in TS 38.830 [4]. The evaluation results of MIL for different scenarios are shown below.

  - FR1

Following carrier frequencies and frame structures are considered in FR1, and only NLOS outdoor-to-indoor is considered for Dense Urban scenario and Urban Macro scenario. In coverage evaluation of XR traffic, the target data rate of PDSCH is 30Mbps, 45Mbps and 60Mbps. And the data rate requirements 0.2Mbps and 20Mbps for PUSCH are evaluated, which correspond to two cases for Pose information and Scene information respectively. The evaluation results of FR1 Dense Urban and Urban Macro scenario are shown in Figure 3 and Figure 4. It can be observed that PUSCH is the bottleneck channel among all channels, based on MIL, especially the PUSCH with high data-rate requirement.

- 4GHz, DDDSU, Dense Urban

Figure 3. MIL for 4GHz, DDDSU, Dense Urban

- 4GHz, DDDSU, Urban Macro

Figure 4. MIL for 4GHz, DDDSU, Urban Macro

Observation 43: For both Dense Urban and Urban Macro scenarios in FR1, for VR/AR/CG traffic with 100MHz bandwidth, PUSCH is the bottleneck channel with frame structure DDDSU in terms of MIL.

  - FR2

Following carrier frequency and frame structures with 100MHz and 400MHz bandwidth are evaluated in FR2, respectively. For Dense Urban scenario, only NLOS outdoor-to-indoor is evaluated. Only NLOS indoor-to-indoor is considered for Indoor Hotspot scenario. The target data rate of PDSCH is 30Mbps, 45Mbps and 60Mbps. And the data rate requirements 0.2Mbps and 20Mbps for PUSCH are evaluated, which correspond to two cases for Pose information and Scene information respectively. And the evaluation results for Dense Urban and Indoor Hotspot scenarios in FR2 are shown in Figure 5 to Figure 8.

- 30GHz, DDDSU, 400MHz, Dense Urban

Figure 5. MIL for 30GHz, DDDSU, 400MHz, Dense Urban

- 30GHz, DDDSU, 100MHz, Dense Urban

Figure 6. MIL for 30GHz, DDDSU, 100MHz, Dense Urban

Observation 44: For Dense Urban scenario in FR2, for VR/AR/CG traffic with 400MHz and 100MHz bandwidth, PUSCH is the bottleneck channel with frame structure DDDSU in terms of MIL.

- 30GHz, DDDSU, 400MHz, Indoor Hotspot

Figure 7. MIL for 30GHz, DDDSU, 400MHz, Indoor Hotspot

- 30GHz, DDDSU, 100MHz, Indoor Hotspot

Figure 8. MIL for 30GHz, DDDSU, 100MHz, Indoor Hotspot

Observation 45: For Indoor Hotspot scenario in FR2, for VR/CG traffic with 400MHz bandwidth, and VR/AR /CG traffic with 100MHz bandwidth, PDSCH is the bottleneck channel with frame structure DDDSU in terms of MIL.

Observation 46: For Indoor Hotspot scenario in FR2, for AR traffic with 400MHz bandwidth, PUSCH is the bottleneck channel with frame structure DDDSU in terms of MIL.

- Conclusion

In this contribution, we provide our initial simulation results on XR capacity, power consumption and coverage with the following observations:

Observation 1: When the capacity requirement per UE is more than 99% of packets are successfully delivered within a given air interface PDB,

- For DL CG with 30Mbps and 60FPS in FR1 Indoor Hotspot, system capacity is 16 UEs per cell.

- For DL CG with 30Mbps and 120FPS in FR1 Indoor Hotspot, system capacity is 20 UEs per cell.

- For DL VR/AR with 45Mbps and 60FPS in FR1 Indoor Hotspot, system capacity is 6 UEs per cell.

- For DL VR/AR with 45Mbps and 120FPS in FR1 Indoor Hotspot, system capacity is 10 UEs per cell.

- For DL VR/AR with 60Mbps and 60FPS in FR1 Indoor Hotspot, system capacity is 3 UEs per cell.

- For DL VR/AR with 60Mbps and 120FPS in FR1 Indoor Hotspot, system capacity is 7 UEs per cell.

Observation 2: When the capacity requirement per UE is more than 95% of packets are successfully delivered within a given air interface PDB,

- For DL CG with 30Mbps and 60FPS in FR1 Indoor Hotspot, system capacity is 18 UEs per cell.

- For DL CG with 30Mbps and 120FPS in FR1 Indoor Hotspot, system capacity is 22 UEs per cell.

- For DL VR/AR with 45Mbps and 60FPS in FR1 Indoor Hotspot, system capacity is 7 UEs per cell.

- For DL VR/AR with 45Mbps and 120FPS in FR1 Indoor Hotspot, system capacity is 10 UEs per cell.

- For DL VR/AR with 60Mbps and 60FPS in FR1 Indoor Hotspot, system capacity is 4 UEs per cell.

- For DL VR/AR with 60Mbps and 120FPS in FR1 Indoor Hotspot, system capacity is 8 UEs per cell.

Observation 3:  When the capacity requirement per UE is more than 99% of packets are successfully delivered within a given air interface PDB,

- For DL CG with 30Mbps and 60FPS in FR1 Dense Urban, system capacity is 24 UEs per cell.

- For DL CG with 30Mbps and 120FPS in FR1 Dense Urban, system capacity is 30 UEs per cell.

- For DL VR/AR with 45Mbps and 60FPS in FR1 Dense Urban, system capacity is 9 UEs per cell.

- For DL VR/AR with 45Mbps and 120FPS in FR1 Dense Urban, system capacity is 14 UEs per cell.

- For DL VR/AR with 60Mbps and 60FPS in FR1 Dense Urban, system capacity is 5 UEs per cell.

- For DL VR/AR with 60Mbps and 120FPS in FR1 Dense Urban, system capacity is 9 UEs per cell.

Observation 4:  When the capacity requirement per UE is more than 95% of packets are successfully delivered within a given air interface PDB,

- For DL CG with 30Mbps and 60FPS in FR1 Dense Urban, system capacity is 25 UEs per cell.

- For DL CG with 30Mbps and 120FPS in FR1 Dense Urban, system capacity is 32 UEs per cell.

- For DL VR/AR with 45Mbps and 60FPS in FR1 Dense Urban, system capacity is 9 UEs per cell.

- For DL VR/AR with 45Mbps and 120FPS in FR1 Dense Urban, system capacity is 15 UEs per cell.

- For DL VR/AR with 60Mbps and 60FPS in FR1 Dense Urban, system capacity is 6 UEs per cell.

- For DL VR/AR with 60Mbps and 120FPS in FR1 Dense Urban, system capacity is 10 UEs per cell.

Observation 5:  When the capacity requirement per UE is more than 99% of packets are successfully delivered within a given air interface PDB,

- For DL CG with 30Mbps and 60FPS in FR1 Urban Macro, system capacity is 16 UEs per cell.

- For DL CG with 30Mbps and 120FPS in FR1 Urban Macro, system capacity is 20 UEs per cell.

- For DL VR/AR with 45Mbps and 60FPS in FR1 Urban Macro, system capacity is 6 UEs per cell.

- For DL VR/AR with 45Mbps and 120FPS in FR1 Urban Macro, system capacity is 9 UEs per cell.

- For DL VR/AR with 60Mbps and 60FPS in FR1 Urban Macro, system capacity is 3 UEs per cell.

- For DL VR/AR with 60Mbps and 120FPS in FR1 Urban Macro, system capacity is 6 UEs per cell.

Observation 6:  When the capacity requirement per UE is more than 95% of packets are successfully delivered within a given air interface PDB,

- For DL CG with 30Mbps and 60FPS in FR1 Urban Macro, system capacity is 18 UEs per cell.

- For DL CG with 30Mbps and 120FPS in FR1 Urban Macro, system capacity is 22 UEs per cell.

- For DL VR/AR with 45Mbps and 60FPS in FR1 Urban Macro, system capacity is 6 UEs per cell.

- For DL VR/AR with 45Mbps and 120FPS in FR1 Urban Macro, system capacity is 10 UEs per cell.

- For DL VR/AR with 60Mbps and 60FPS in FR1 Urban Macro, system capacity is 4 UEs per cell.

- For DL VR/AR with 60Mbps and 120FPS in FR1 Urban Macro, system capacity is 7 UEs per cell.

Observation 7: When the reliability requirement is relaxed from 99% to 95%, the system capacity could increase by 0 to 2 UE per cell for FR1 DL traffics.

Observation 8: When increasing the frame rate from 60FPS to 120FPS, the system capacity could increase by 3 to 7 UEs per cell for FR1 DL traffics.

Observation 9:  When the capacity requirement per UE is more than 99% of packets are successfully delivered within a given air interface PDB,

- For DL CG with 30Mbps and 60FPS in FR2 Indoor Hotspot, system capacity is 9 UEs per cell with 100MHz bandwidth.

- For DL CG with 30Mbps and 120FPS in FR2 Indoor Hotspot, system capacity is 10 UEs per cell with 100MHz bandwidth.

- For DL VR/AR with 45Mbps and 60FPS in FR2 Indoor Hotspot, system capacity is 5 UEs per cell with 100MHz bandwidth.

- For DL VR/AR with 45Mbps and 120FPS in FR2 Indoor Hotspot, system capacity is 6 UEs per cell with 100MHz bandwidth.

- For DL VR/AR with 60Mbps and 60FPS in FR2 Indoor Hotspot, system capacity is 3 UEs per cell with 100MHz bandwidth.

- For DL VR/AR with 60Mbps and 120FPS in FR2 Indoor Hotspot, system capacity is 4 UEs per cell with 100MHz bandwidth.

Observation 10:  When the capacity requirement per UE is more than 95% of packets are successfully delivered within a given air interface PDB,

- For DL CG with 30Mbps and 60FPS in FR2 Indoor Hotspot, system capacity is 10 UEs per cell with 100MHz bandwidth.

- For DL CG with 30Mbps and 120FPS in FR2 Indoor Hotspot, system capacity is 12 UEs per cell with 100MHz bandwidth.

- For DL VR/AR with 45Mbps and 60FPS in FR2 Indoor Hotspot, system capacity is 6 UEs per cell with 100MHz bandwidth.

- For DL VR/AR with 45Mbps and 120FPS in FR2 Indoor Hotspot, system capacity is 7 UEs per cell with 100MHz bandwidth.

- For DL VR/AR with 60Mbps and 60FPS in FR2 Indoor Hotspot, system capacity is 4 UEs per cell with 100MHz bandwidth.

- For DL VR/AR with 60Mbps and 120FPS in FR2 Indoor Hotspot, system capacity is 5 UEs per cell with 100MHz bandwidth.

Observation 11:  When the capacity requirement per UE is more than 99% of packets are successfully delivered within a given air interface PDB,

- For DL CG with 30Mbps and 60FPS in FR2 Indoor Hotspot, system capacity is 46 UEs per cell with 400MHz bandwidth.

- For DL CG with 30Mbps and 120FPS in FR2 Indoor Hotspot, system capacity is 50 UEs per cell with 400MHz bandwidth.

- For DL VR/AR with 45Mbps and 60FPS in FR2 Indoor Hotspot, system capacity is 27 UEs per cell with 400MHz bandwidth.

- For DL VR/AR with 45Mbps and 120FPS in FR2 Indoor Hotspot, system capacity is 31 UEs per cell with 400MHz bandwidth.

- For DL VR/AR with 60Mbps and 60FPS in FR2 Indoor Hotspot, system capacity is 20 UEs per cell with 400MHz bandwidth.

- For DL VR/AR with 60Mbps and 120FPS in FR2 Indoor Hotspot, system capacity is 23 UEs per cell with 400MHz bandwidth.

Observation 12:  When the capacity requirement per UE is more than 95% of packets are successfully delivered within a given air interface PDB,

- For DL CG with 30Mbps and 60FPS in FR2 Indoor Hotspot, system capacity is 50 UEs per cell with 400MHz bandwidth.

- For DL CG with 30Mbps and 120FPS in FR2 Indoor Hotspot, system capacity is 53 UEs per cell with 400MHz bandwidth.

- For DL VR/AR with 45Mbps and 60FPS in FR2 Indoor Hotspot, system capacity is 30 UEs per cell with 400MHz bandwidth.

- For DL VR/AR with 45Mbps and 120FPS in FR2 Indoor Hotspot, system capacity is 33 UEs per cell with 400MHz bandwidth.

- For DL VR/AR with 60Mbps and 60FPS in FR2 Indoor Hotspot, system capacity is 22 UEs per cell with 400MHz bandwidth.

- For DL VR/AR with 60Mbps and 120FPS in FR2 Indoor Hotspot, system capacity is 25 UEs per cell with 400MHz bandwidth.

Observation 13:  When the capacity requirement per UE is more than 99% of packets are successfully delivered within a given air interface PDB,

- For DL CG with 30Mbps and 60FPS in FR2 Dense Urban, system capacity is 20 UEs per cell with 100MHz bandwidth.

- For DL CG with 30Mbps and 120FPS in FR2 Dense Urban, system capacity is 24 UEs per cell with 100MHz bandwidth.

- For DL VR/AR with 45Mbps and 60FPS in FR2 Dense Urban, system capacity is 9 UEs per cell with 100MHz bandwidth.

- For DL VR/AR with 45Mbps and 120FPS in FR2 Dense Urban, system capacity is 12 UEs per cell with 100MHz bandwidth.

- For DL VR/AR with 60Mbps and 60FPS in FR2 Dense Urban, system capacity is 6 UEs per cell with 100MHz bandwidth.

- For DL VR/AR with 60Mbps and 120FPS in FR2 Dense Urban, system capacity is 8 UEs per cell with 100MHz bandwidth.

Observation 14:  When the capacity requirement per UE is more than 95% of packets are successfully delivered within a given air interface PDB,

- For DL CG with 30Mbps and 60FPS in FR2 Dense Urban, system capacity is 22 UEs per cell with 100MHz bandwidth.

- For DL CG with 30Mbps and 120FPS in FR2 Dense Urban, system capacity is 26 UEs per cell with 100MHz bandwidth.

- For DL VR/AR with 45Mbps and 60FPS in FR2 Dense Urban, system capacity is 10 UEs per cell with 100MHz bandwidth.

- For DL VR/AR with 45Mbps and 120FPS in FR2 Dense Urban, system capacity is 13 UEs per cell with 100MHz bandwidth.

- For DL VR/AR with 60Mbps and 60FPS in FR2 Dense Urban, system capacity is 7 UEs per cell with 100MHz bandwidth.

- For DL VR/AR with 60Mbps and 120FPS in FR2 Dense Urban, system capacity is 9 UEs per cell with 100MHz bandwidth.

Observation 15:  When the capacity requirement per UE is more than 99% of packets are successfully delivered within a given air interface PDB,

- For DL CG with 30Mbps and 60FPS in FR2 Dense Urban, system capacity is greater than 35 UEs per cell with 400MHz bandwidth.

- For DL CG with 30Mbps and 120FPS in FR2 Dense Urban, system capacity is greater than 35 UEs per cell with 400MHz bandwidth.

- For DL VR/AR with 45Mbps and 60FPS in FR2 Dense Urban, system capacity is greater than 35 UEs per cell with 400MHz bandwidth.

- For DL VR/AR with 45Mbps and 120FPS in FR2 Dense Urban, system capacity is greater than 35 UEs per cell with 400MHz bandwidth.

- For DL VR/AR with 60Mbps and 60FPS in FR2 Dense Urban, system capacity is greater than 25 UEs per cell with 400MHz bandwidth.

- For DL VR/AR with 60Mbps and 120FPS in FR2 Dense Urban, system capacity is greater than 25 UEs per cell with 400MHz bandwidth.

Observation 16:  When the capacity requirement per UE is more than 95% of packets are successfully delivered within a given air interface PDB,

- For DL CG with 30Mbps and 60FPS in FR2 Dense Urban, system capacity is greater than 35 UEs per cell with 400MHz bandwidth.

- For DL CG with 30Mbps and 120FPS in FR2 Dense Urban, system capacity is greater than 35 UEs per cell with 400MHz bandwidth.

- For DL VR/AR with 45Mbps and 60FPS in FR2 Dense Urban, system capacity is greater than 35 UEs per cell with 400MHz bandwidth.

- For DL VR/AR with 45Mbps and 120FPS in FR2 Dense Urban, system capacity is greater than 35 UEs per cell with 400MHz bandwidth.

- For DL VR/AR with 60Mbps and 60FPS in FR2 Dense Urban, system capacity is greater than 25 UEs per cell with 400MHz bandwidth.

- For DL VR/AR with 60Mbps and 120FPS in FR2 Dense Urban, system capacity is greater than 25 UEs per cell with 400MHz bandwidth.

Observation 17: When the capacity requirement per UE is more than 99% of packets are successfully delivered within a given air interface PDB,

- For UL Scene information in FR1 Indoor Hotspot, system capacity is 6 UEs per cell.

- For UL Scene information in FR1 Dense Urban, system capacity is 3 UEs per cell.

- For UL Scene information in FR1 Urban Macro with 1 UE per cell, there is less than 90% of UEs that can satisfy system capacity requirement.

Observation 18: When the capacity requirement per UE is more than 95% of packets are successfully delivered within a given air interface PDB,

- For UL Scene information in FR1 Indoor Hotspot, system capacity is 6 UEs per cell.

- For UL Scene information in FR1 Dense Urban, system capacity is 3 UEs per cell.

- For UL Scene information in FR1 Urban Macro with 1 UE per cell, there is less than 90% of UEs that can satisfy system capacity requirement.

Observation 19: For Scene information in FR1 UL, system capacity cannot be improved by relaxing reliability requirement from 99% to 95%.

Observation 20:  When the capacity requirement per UE is more than 99% of packets are successfully delivered within a given air interface PDB,

- For UL Pose information in FR1 Indoor Hotspot, system capacity is greater than 15 UEs per cell.

- For UL Pose information in FR1 Dense Urban, system capacity is greater than 15 UEs per cell.

- For UL Pose information in FR1 Urban Macro, system capacity is greater than 15 UEs per cell.

Observation 21: For Pose information in FR1 for UL, 95th percentile user interaction delay is far below 10ms with the number of UEs per cell up to 15.

Observation 22:  When the capacity requirement per UE is more than 99% of packets are successfully delivered within a given air interface PDB,

- For UL Scene information in FR2 Indoor Hotspot, system capacity is greater than 30 UEs per cell with 400MHz bandwidth.

- For UL Scene information in FR2 Dense Urban, system capacity is greater than 30 UEs per cell with 400MHz bandwidth.

Observation 23:  When the capacity requirement per UE is more than 95% of packets are successfully delivered within a given air interface PDB,

- For UL Scene information in FR2 Indoor Hotspot, system capacity is greater than 30 UEs per cell with 400MHz bandwidth.

- For UL Scene information in FR2 Dense Urban, system capacity is greater than 30 UEs per cell with 400MHz bandwidth.

Observation 24:  When the capacity requirement per UE is more than 99% of packets are successfully delivered within a given air interface PDB,

- For UL Pose information in FR2 Indoor Hotspot, system capacity is greater than 30 UEs per cell with 400MHz bandwidth.

- For UL Pose information in FR2 Dense Urban, system capacity is greater than 30 UEs per cell with 400MHz bandwidth.

Observation 25: For DL CG with 30Mbps and 60FPS in FR1 Indoor Hotspot,

- 3.22% mean PSG can be achieved without capacity loss by using DRX scheme with 10ms DRX cycle.

- 5.10% mean PSG can be achieved without capacity loss by using DRX scheme with 4ms DRX cycle.

- 24.76% mean PSG can be achieved without capacity loss by using aligning DRX ON Duration scheme.

- 29.73% mean PSG can be achieved without capacity loss by using PDCCH skipping with jitter handling scheme.

Observation 26: For DL CG with 30Mbps and 60FPS in FR1 Dense Urban,

- 2.07% mean PSG can be achieved without capacity loss by using DRX scheme with 10ms DRX cycle.

- 3.62% mean PSG can be achieved without capacity loss by using DRX scheme with 4ms DRX cycle.

- 19.63% mean PSG can be achieved without capacity loss by using aligning DRX ON Duration scheme.

- 25.79% mean PSG can be achieved without capacity loss by using PDCCH skipping with jitter handling scheme.

Observation 27: For DL CG with 30Mbps and 60FPS in FR1 Urban Macro,

- 2.42% mean PSG can be achieved without capacity loss by using DRX scheme with 10ms DRX cycle.

- 3.69% mean PSG can be achieved without capacity loss by using DRX scheme with 4ms DRX cycle.

- 22.09% mean PSG can be achieved without capacity loss by using aligning DRX ON Duration scheme.

- 27.27% mean PSG can be achieved without capacity loss by using PDCCH skipping with jitter handling scheme.

Observation 28: For DL VR/AR with 45Mbps and 60FPS in FR1 Indoor Hotspot,

- 4.06% mean PSG can be achieved without capacity loss by using DRX scheme with 10ms DRX cycle.

- 5.97% mean PSG can be achieved without capacity loss by using DRX scheme with 4ms DRX cycle.

- 28.40% mean PSG can be achieved without capacity loss by using aligning DRX ON Duration scheme.

- 32.23% mean PSG can be achieved without capacity loss by using PDCCH skipping with jitter handling scheme.

Observation 29: For DL VR/AR with 45Mbps and 60FPS in FR1 Dense Urban,

- 3.86% mean PSG can be achieved without capacity loss by using DRX scheme with 10ms DRX cycle.

- 5.70% mean PSG can be achieved without capacity loss by using DRX scheme with 4ms DRX cycle.

- 26.77% mean PSG can be achieved without capacity loss by using aligning DRX ON Duration scheme.

- 31.24% mean PSG can be achieved without capacity loss by using PDCCH skipping with jitter handling scheme.

Observation 30: For DL VR/AR with 45Mbps and 60FPS in FR1 Urban Macro,

- 3.75% mean PSG can be achieved without capacity loss by using DRX scheme with 10ms DRX cycle.

- 5.61% mean PSG can be achieved without capacity loss by using DRX scheme with 4ms DRX cycle.

- 27.15% mean PSG can be achieved without capacity loss by using aligning DRX ON Duration scheme.

- 31.10% mean PSG can be achieved without capacity loss by using PDCCH skipping with jitter handling scheme.

Observation 31: For DL VR/AR with 60Mbps and 60FPS in FR1 Indoor Hotspot,

- 4.29% mean PSG can be achieved without capacity loss by using DRX scheme with 10ms DRX cycle.

- 6.17% mean PSG can be achieved without capacity loss by using DRX scheme with 4ms DRX cycle.

- 29.07% mean PSG can be achieved without capacity loss by using aligning DRX ON Duration scheme.

- 32.70% mean PSG can be achieved without capacity loss by using PDCCH skipping with jitter handling scheme.

Observation 32: For DL VR/AR with 60Mbps and 60FPS in FR1 Dense Urban,

- 4.01% mean PSG can be achieved without capacity loss by using DRX scheme with 10ms DRX cycle.

- 5.86% mean PSG can be achieved without capacity loss by using DRX scheme with 4ms DRX cycle.

- 26.68% mean PSG can be achieved without capacity loss by using aligning DRX ON Duration scheme.

- 31.18% mean PSG can be achieved without capacity loss by using PDCCH skipping with jitter handling scheme.

Observation 33: For DL VR/AR with 60Mbps and 60FPS in FR1 Urban Macro,

- 3.56% mean PSG can be achieved without capacity loss by using DRX scheme with 10ms DRX cycle.

- 5.39% mean PSG can be achieved without capacity loss by using DRX scheme with 4ms DRX cycle.

- 26.14% mean PSG can be achieved without capacity loss by using aligning DRX ON Duration scheme.

- 30.46% mean PSG can be achieved without capacity loss by using PDCCH skipping with jitter handling scheme.

Observation 34: By adopting the R15/R16 DRX configuration, only 2.07%~6.17% mean PSG can be obtained without the loss of the system capacity for FR1 DL.

Observation 35: 19.63%~32.7% mean PSG will be achieved by aligning the start time of DRX ON Duration with the arrival of XR traffic or PDCCH skipping scheme without capacity loss, which is five times as much as the power saving gain obtained by the R15/R16 DRX configuration.

Observation 36: For UL Scene information in FR1 Indoor Hotspot,

- 5.22% mean PSG can be achieved without capacity loss by using DRX scheme with 10ms DRX cycle.

- 7.67% mean PSG can be achieved without capacity loss by using DRX scheme with 4ms DRX cycle.

- 37.59% mean PSG can be achieved without capacity loss by using aligning DRX ON Duration scheme.

- 52.04% mean PSG can be achieved without capacity loss by using PDCCH skipping scheme.

Observation 37: For UL Scene information in FR1 Dense Urban,

- 6.01% mean PSG can be achieved without capacity loss by using DRX scheme with 10ms DRX cycle.

- 7.35% mean PSG can be achieved without capacity loss by using DRX scheme with 4ms DRX cycle.

- 31.13% mean PSG can be achieved without capacity loss by using aligning DRX ON Duration scheme.

- 40.88% mean PSG can be achieved without capacity loss by using PDCCH skipping scheme.

Observation 38: By adopting the R15/R16 DRX configuration, only 5.22%~7.67% mean PSG can be obtained without the loss of the system capacity for FR1 uplink Scene information.

Observation 39: For FR1 uplink Scene information, 31.13%~37.59% PSG and 40.88%~52.04% mean PSG can be obtained without capacity loss by aligning the start time of DRX ON Duration with the arrival of XR traffic and adopting PDCCH skipping scheme, respectively.

Observation 40: For UL Pose information in FR1 Indoor Hotspot,

- 22.83% mean PSG can be achieved without capacity loss by using DRX without considering packets bundling.

- 36.20% mean PSG can be achieved without capacity loss by using packets bundled transmission.

Observation 41: For UL Pose information in FR1 Dense Urban,

- 22.93% mean PSG can be achieved without capacity loss by using DRX without considering packets bundling.

- 36.34% mean PSG can be achieved without capacity loss by using packets bundled transmission.

Observation 42: Compared with the DRX configuration without packets bundling, the packets bundled transmission scheme can obtain additional 13.37% and 13.41% PSG for Indoor Hotspot scenario and Dense Urban scenario, respectively.

Observation 43: For both Dense Urban and Urban Macro scenarios in FR1, for VR/AR/CG traffic with 100MHz bandwidth, PUSCH is the bottleneck channel with frame structure DDDSU in terms of MIL.

Observation 44: For Dense Urban scenario in FR2, for VR/AR/CG traffic with 400MHz and 100MHz bandwidth, PUSCH is the bottleneck channel with frame structure DDDSU in terms of MIL.

Observation 45: For Indoor Hotspot scenario in FR2, for VR/CG traffic with 400MHz bandwidth, and VR/AR /CG traffic with 100MHz bandwidth, PDSCH is the bottleneck channel with frame structure DDDSU in terms of MIL.

Observation 46: For Indoor Hotspot scenario in FR2, for AR traffic with 400MHz bandwidth, PUSCH is the bottleneck channel with frame structure DDDSU in terms of MIL.

# References

- Chairman’s Notes of RAN1 #104-e final (Online meeting, January 25th – February 5th, 2021).

- R1-2102549, Challenges and potential enhancements of XR, RAN1 #104e-bis.

- R1-2102547, Discussion on evaluation methodologies of XR, RAN1 #104e-bis.

- TR 38.830, “Study on NR coverage enhancements (Release 17)” V17.0.0 (2020-12).

# Appendix A – Simulation assumptions

Table I. Simulation assumption for FR1

| Parameter | value |  |  |  |
| --- | --- | --- | --- | --- |
| Scenarios | Scenario-1: Indoor Hotspot 12 nodes in 50 m x 120 m | Scenario-2: Dense Urban hexagonal layout with 7, 3 Sectors |  | Scenario-3: Urban Macro hexagonal layout with 7, 3 Sectors |
| Channel model | InH | Uma |  | Uma |
| Carrier frequency | 4GHz |  |  |  |
| Bandwidth | 100 MHz, 1.72% Guard Band |  |  |  |
| Subcarrier spacing | 30 KHz |  |  |  |
| Frame structure | DDDSU (S: 10D:2G:2U) |  |  |  |
| BS Antennas (M,N,P,Mg,Ng;Mp,Np) | For 32T: (4,4,2,1,1;4,4) (dH,dV) = (0.5, 0.5)λ |  | For 64T: (8,8,2,1,1;4,8) (dH, dV)=(0.5, 0.8)λ |  |
| UE Antennas (M,N,P,Mg,Ng;Mp,Np) | DL: For 4R: (1,2,2,1,1;1,2), (dH, dV)=(0.5, N/A)λ UL: For 2T: (1,1,2,1,1;1,1), (dH, dV)=( 0.5, N/A)λ |  |  |  |
| BS antenna pattern | Ceiling-mount pattern, 5 dBi | 3-TRxP pattern, 8 dBi |  | 3-TRxP pattern, 8 dBi |
| UE antenna pattern | Omnidirectional, 0 dBi |  |  |  |
| BS Power | 24 dBm per 20MHz | 44 dBm per 20MHz |  | 49 dBm/20 MHz |
| UE max Power | 23 dBm |  |  |  |
| UE Power | Max Tx power: 23 dBm, (P0 = -80, alpha = 0.8) | Max Tx power: 23 dBm, (P0 = -74, alpha = 0.6) |  | Max Tx power: 23 dBm, (P0 = -74, alpha = 0.6) |
| ISD | 20 m | 200 m |  | 500m |
| BS height | 3 m | 25 m |  | 25m |
| UE height | 1.5 m | Outdoor UEs: 1.5 m Indoor UTs: 3(nfl – 1) + 1.5; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) |  |  |
| Noise Figure | BS:5 dB, UE:9 dB |  |  |  |
| Scheduler | MU-MIMO Proportional Fair |  |  |  |
| Max MCS | 256QAM |  |  |  |
| Device deployment | 100% indoor | 20% outdoor, 80% indoor |  |  |
| Down-tilt | 90 degrees | 12 degrees |  | 6 degrees |
| BS receiver | MMSE-IRC |  |  |  |
| UE receiver | MMSE-IRC |  |  |  |
| Channel estimation | Realistic |  |  |  |
| Target BLER | 10% |  |  |  |
| UE speed | 3 km/h |  |  |  |

Table II. Simulation assumption for FR2

| Parameter | value |  |
| --- | --- | --- |
| Scenarios | Scenario-1: Indoor Hotspot 12 nodes in 50 m x 120 m | Scenario-2: Dense Urban hexagonal layout with 7, 3 Sectors |
| Carrier frequency | 30 GHz |  |
| Bandwidth | 100 MHz, 1.72% Guard Band 400 MHz, 4.96% Guard Band |  |
| Subcarrier spacing | 120 KHz |  |
| Frame structure | DDDSU (S: 10D:2G:2U) |  |
| BS Antennas (M,N,P,Mg,Ng;Mp,Np) | For 2T: (16,8,2,1,1;1,1) (dH, dV)=(0.5, 0.5)λ | For 2T/panel: (4,8,2,2,2;1,1) (dH, dV)=(0.5, 0.5)λ (dg,H,dg,V) = (4.0, 2.0) λ |
| UE Antennas (M,N,P,Mg,Ng;Mp,Np) | For 4R/panel: (1,4,2,1,3;1,2) (dH,dV) = (0.5, 0.5)λ (dg,V,dg,H) = (0, 0)λ |  |
| BS antenna pattern | Ceiling-mount pattern, 5 dBi | 3-TRxP pattern, 8dBi |
| UE antenna pattern | Directional antenna panel, 5 dBi |  |
| Min. BE-UT distance(2D) | 0 m | 10 m |
| BS height | 3 m | 25 m |
| UE height | 1.5 m | Outdoor UEs: 1.5 m Indoor UTs: 3(nfl – 1) + 1.5; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) |
| BS Power | 23dBm per 80MHz, EIRP should not exceed 58 dBm | 40dBm per 80MHz, EIRP should not exceed 73 dBm |
| UE Power | Max Tx power: 23 dBm, maximum EIRP 43 dBm, (P0 = -60, alpha = 0.6) | Max Tx power: 23 dBm, maximum EIRP 43 dBm, (P0 = -74, alpha = 0.6) |
| Noise Figure | BS:7 dB, UE:13 dB |  |
| Scheduler | SU-MIMO Proportional Fair |  |
| Max MCS | 256QAM |  |
| Device deployment | 100% indoor | 100% outdoor |
| BS receiver | MMSE-IRC |  |
| UE receiver | MMSE-IRC |  |
| Channel estimation | Realistic |  |
| Target BLER | 10% |  |
| UE speed | 3 km/h |  |

# Appendix B – Channel-specific parameters for Coverage

Table Ⅲ. Channel-specific parameters for PDSCH and PUSCH in FR1

| Parameter | value |  |
| --- | --- | --- |
| Channel model for LLS | TDL-C |  |
| Number of gNB antenna port in LLS | 2 |  |
| Number of UE antenna port in LLS | 1Tx/4Rx |  |
| Frequency hopping | w/o frequency hopping |  |
| Repetition | w/o repetition |
| BLER | w/o HARQ, 10% iBLER. |  |
| DMRS configuration | For 3km/h: Type I, 2 DMRS symbol, no multiplexing with data. |  |
| Waveform | DFT-s-OFDM for uplink CP-OFDM for downlink |  |
| PRBs/MCS for PDSCH (30Mbps) | 273RB, Qm = 2, Code rate = 308/1024 |  |
| PRBs/MCS for PDSCH (45Mbps) | 273RB, Qm = 2, Code rate = 449/1024 |  |
| PRBs/MCS for PDSCH (60Mbps) | 273RB, Qm = 2, Code rate = 602/1024 |  |
| PRBs/MCS for PUSCH (0.2Mbps) | 273RB, Qm = 2, Code rate = 30/1024 |  |
| PRBs/MCS for PUSCH (20Mbps) | 273RB, Qm = 2, Code rate = 679/1024 |  |

Table Ⅳ. Channel-specific parameters for PDSCH and PUSCH in FR2

| Parameter | value |
| --- | --- |
| Channel model for LLS | TDL-A |
| Number of gNB antenna port in LLS | 2 |
| Number of UE antenna port in LLS | 1Tx/2Rx |
| Frequency hopping | w/o frequency hopping |
| Repetition | w/o repetition |
| BLER | w/o HARQ, 10% iBLER. |
| DMRS configuration | For 3km/h: Type I, 2 DMRS symbol, no multiplexing with data. |
| Waveform | DFT-s-OFDM for uplink CP-OFDM for downlink |
| PRBs/MCS for PDSCH (30Mbps) | For 400MHz: 264RB, Qm = 2, Code rate = 120/1024 For 100MHz: 66RB, Qm = 2, Code rate = 308/1024 |
| PRBs/MCS for PDSCH (45Mbps) | For 400MHz: 264RB, Qm = 2, Code rate = 120/1024 For 100MHz: 66RB, Qm = 2, Code rate = 449/1024 |
| PRBs/MCS for PDSCH (60Mbps) | For 400MHz: 264RB, Qm = 2, Code rate = 193/1024 For 100MHz: 66RB, Qm = 2, Code rate = 602/1024 |
| PRBs/MCS for PUSCH (0.2Mbps) | For 400MHz: 264RB, Qm = 2, Code rate = 30/1024 For 100MHz: 66RB, Qm = 2, Code rate = 30/1024 |
| PRBs/MCS for PUSCH (20Mbps) | For 400MHz: 264RB, Qm = 2, Code rate = 193/1024 For 100MHz: 66RB, Qm = 2, Code rate = 679/1024 |