3GPP TSG RAN WG1 Meeting #104b-e				R1-2102571

e-Meeting, April 12th – April 25th, 2021

Source 	: CAICT

Title 	: UE feedback enhancements for HARQ-ACK

Agenda Item	: 8.3.1.1

Document for	: Discussion / Decision

Introduction

In the RAN1 #104e meeting, UE feedback enhancements for HARQ-ACK were discussed and the agreements are addressed in [1][2]. In this contribution, we will discuss more aspects about SPS HARQ-ACK dropping for TDD in Rel.17 URLLC.

Configuration of SPS HARQ-ACK deferral

The following agreements were reached about how to configure SPS HARQ-ACK deferral.

Agreements:

Further down-select between the following two options for SPS HARQ-ACK deferral:

- Option 1: Joint RRC configuration of the SPS HARQ-ACK deferral per PUCCH cell group

  - Note: any SPS HARQ-ACK within a PUCCH cell group in principle is subject to deferral

- Option 2: The SPS HARQ-ACK deferral is configured per SPS configuration

Note: part of sps-config, only HARQ-ACK of SPS PDSCH configurations configured for deferral is in principle subject to deferral

To our opinion, option 2 provides more flexibility and option 1 could be implemented by option 2. Considering different SPS configuration could be used for different service type. The latency and reliability requirements of each service type could be different. For some urgent service, deferred HARQ-ACK is meaningless. For some loose latency service, HARQ-ACK dropping due to TDD collision could be acceptable and HARQ-ACK deferring is not needed. SPS HARQ-ACK deferral configured per SPS configuration provides much differentiation handling according to the service type.

Proposal 1: SPS HARQ-ACK deferral is configured per SPS configuration.

SPS HARQ-ACK deferring in TDD

Conditions for SPS HARQ-ACK deferring

About the condition of SPS HARQ-ACK deferral, the following alternatives were discussed in the previous meeting:

- Alt. 1:  Deferral only, if the SPS HARQ-ACK in the initial slot/sub-slot cannot be transmitted as the resulting PUCCH resource for transmission using the PUCCH by SPS-PUCCH-AN-List-r16 or n1PUCCH-AN is not valid

  - Note: This means, that if SPS HARQ-ACK is multiplexed with any other UCI / dynamic PUCCH resource then it cannot be deferred!

  - FFS on the definition of ‘not valid’ (i.e. TDD configuration and semi-static flexible symbol handling)

- Alt. 2: Deferral only, if the SPS HARQ-ACK in the initial slot/sub-slot cannot be transmitted as the resulting PUCCH resource for transmission using SPS-PUCCH-AN-List-r16, n1PUCCH-AN or other configured PUCCH resource(s) is not valid

  - FFS: other configured PUCCH resource(s) (e.g., PUCCH-ResourceSet, multi-CSI-PUCCH-ResourceList)

  - FFS on the definition of ’not valid’ (i.e. TDD configuration and semi-static flexible symbol handling)

- Alt. 3 - Defer if there is no available symbol for an UL transmission in the initial slot/sub-slot

  - FFS on the definition of ‘no available symbol for an UL transmission’ (i.e. TDD configuration and semi-static flexible symbol handling)

According to the current UCI multiplexing behavior, HARQ-ACK with the same priority index within a slot/subslot is constructed into one HARQ-ACK codebook as defined in [4]. UE decides one PUCCH for HARQ-ACK from configured SPS-PUCCH-AN-List-r16, n1PUCCH-AN, or PUCCH-ResourceSet. If the determined PUCCH for HARQ-ACK overlaps with another PUCCH for SR or CSI, final determined PUCCH is based on UCI type, UCI payload size, and PRI for this slot/subslot in DCI if exists. As a result, SPS HARQ-ACK corresponds to “initial PUCCH” in the initial slot/subslot from the configured SPS-PUCCH-AN-List-r16 or n1PUCCH-AN for SPS HARQ-ACK, PUCCH-ResourceSet for dynamic scheduled PDSCH, or multi-CSI-PUCCH-ResourceList for CSI. In the current system, when some of the symbols in the “initial PUCCH” is invalid due to TDD configuration, the SPS HARQ-ACK and other UCI in this PUCCH would be dropped correspondingly.

Observation 1: In the initial slot/subslot, SPS HARQ-ACK corresponds to “initial PUCCH” which is one PUCCH from SPS-PUCCH-AN-List-r16 or n1PUCCH-AN for SPS HARQ-ACK, PUCCH-ResourceSet for dynamic scheduled PDSCH, or multi-CSI-PUCCH-ResourceList for CSI.

With deferring, dropping SPS HARQ-ACK could be avoided.

If the “initial PUCCH” is based on semi-static configuration as SPS-PUCCH-AN-List-r16, n1PUCCH-AN, or multi-CSI-PUCCH-ResourceList, gNB and UE could have identical understanding about whether “initial PUCCH” is available and whether SPS HARQ-ACK would be deferred or not.

If gNB indicates one PUCCH from PUCCH-ResourceSet in the initial slot/subslot through DCI, gNB may assume “initial PUCCH” is decided from the configured PUCCH-ResourceSet. At the UE side, however, it is possible this DCI is missed and assumes “initial PUCCH” is from configured SPS-PUCCH-AN-List-r16, n1PUCCH-AN, or multi-CSI-PUCCH-ResourceList.

- If the gNB “initial PUCCH” from PUCCH-ResourceSet is valid while UE “initial PUCCH” from SPS-PUCCH-AN-List-r16, n1PUCCH-AN, or multi-CSI-PUCCH-ResourceList is invalid, UE assumes the SPS HARQ is deferred while gNB would assume SPS HARQ-ACK is transmitted in the DCI indicated gNB “initial PUCCH”. Certainly, gNB could not detect the gNB “initial PUCCH” since it is not transmitted at all. gNB could decide SPS HARQ-ACK is deferred conditioned that gNB “initial PUCCH” is not detected.

- If the gNB “initial PUCCH” from PUCCH-ResourceSet is invalid while UE “initial PUCCH” from SPS-PUCCH-AN-List-r16, n1PUCCH-AN, or multi-CSI-PUCCH-ResourceList is valid, UE transmits SPS HARQ-ACK in this PUCCH while gNB thinks the SPS HARQ-ACK is deferred since the gNB “initial PUCCH” is invalid. For this, gNB could blind detect the UE “initial PUCCH”. As long as the UE “initial PUCCH” is detected, gNB could be aware of SPS HARQ-ACK not deferred. If gNB fails to detect the UE “initial PUCCH”, there would be misalignment between gNB and UE, and the subsequent HARQ-ACK/UCI feedback would be impacted. Anyway, this could be a corner case since it only occurs when DCI is missed by UE and UE “initial PUCCH” is failure to be blind detected by gNB simultaneously.

Therefore, if gNB supports PUCCH blind detection assuming DCI is missed by UE, SPS HARQ-ACK needs not to be deferred if it is transmitted in the initial PUCCH which is decided from PUCCH-ResourceSet.

Proposal 2: SPS HARQ-ACK is deferred if initial PUCCH includes invalid symbol(s) which the initial PUCCH is decided according to the current UCI multiplexing behavior from the configuration of SPS-PUCCH-AN-List-r16 or n1PUCCH-AN, n1PUCCH-AN, multi-CSI-PUCCH-ResourceList.

Proposal 3: If gNB supports PUCCH blind detection assuming DCI is missed by UE, SPS HARQ-ACK needs not to be deferred if it is transmitted in the initial PUCCH which is decided from PUCCH-ResourceSet. Otherwise, SPS HARQ-ACK should be deferred even if it is transmitted in the PUCCH from PUCCH-ResourceSet.

Target PUCCH for deferral HARQ-ACK transmission

For one SPS PDSCH which HARQ-ACK deferral is enabled, the main procedure is illustrated in the Fig. 1 below. The initial slot/subslot is determined by the indicated K1 value in the SPS activation DCI. A target slot/subslot should be determined to transmit the deferred SPS HARQ-ACK. Intra-slot deferral was considered as one option in the previous meeting before inter-slot deferral. In the Fig.1, the candidate target slot/subslots is with the initial slot/subslot included or not included, and ends up before the deadline of deferring.

Fig. 1 Procedure of SPS HARQ-ACK deferring

When “initial PUCCH” fails to be transmitted, not only SPS HARQ-ACK in it would be deferred but also other UCIs in the “initial PUCCH” would be dropped. Excluding the deferred SPS HARQ-ACK to other slots, UCI payload size and/or UCI type within the initial slot/subslot would be different from before. It is possible another PUCCH in the initial slot/subslot could carry these UCIs. It is also possible another PUCCH could carry part of the deferred SPS HARQ-ACK. With this procedure, SPS HARQ-ACK deferral impacts on the UCIs in the initial slot/subslot could be decreased.

An example is shown in the Fig. 2. PUCCH-A, PUCCH-B, PUCCH-C, and PUCCH-D are configured in SPS-PUCCH-AN-List-r16. PUCCH-E, PUCCH-F, PUCCH-G, and PUCCH-H are configured in multi-CSI-PUCCH-ResourceList. There are N bits of SPS HARQ-ACK and M bits of CSI to be transmitted in the initial slot. The N bits of SPS HARQ-ACK correspond to PUCCH-B and the M bits of CSI correspond to PUCCH-E. Since PUCCH-B and PUCCH-E are overlapped, PUCCH-F which was configured of CSI and fits for the payload size of (N+M)bits would be determined as the “initial PUCCH”. Due to some symbols in the PUCCH-F is invalid, SPS HARQ-ACK is deferred from this PUCCH-F. Within the N bits of SPS HARQ-ACK, there are N1 bits of SPS HARQ-ACK is configured deferrable, where N1≤N. Without considering the deferred HARQ-ACK or part of the deferred HARQ-ACK, it is possible another PUCCH other than PUCCH-B could be used to carry SPS HARQ-ACK. For example, suppose N-N2 bits of SPS HARQ-ACK is to be transmitted in the initial slot/subslot, PUCCH-D would be determined according to the payload size, wherein N2≤N1. Since both PUCCH-D and PUCCH-E are valid and does not overlap with each other, the M bits of CSI could be transmitted in the PUCCH-E and N-N2 bits of SPS HARQ-ACK could be transmitted in the PUCCH-D. With this procedure, the M bits of CSI dropping is possible to be avoided which was influenced by deferred SPS HARQ-ACK. Also more opportunities are provided to transmit SPS HARQ-ACK in the initial slot/subslot. The value of N2 could be the minimum value that results in at least one valid PUCCH or PUSCH for the UCI in the “initial PUCCH” according to the existing UCI multiplexing rules with the assumption that N2 bits of SPS HARQ-ACK is deferred to other slots.

Fig. 2 Example of target slot determination

Proposal 4: SPS HARQ-ACK is possible to be deferred to one PUCCH other than the initial PUCCH in the initial slot/subslot.

Proposal 5: To decrease the deferred SPS HARQ-ACK impacts on the UCI in the initial PUCCH, if the initial PUCCH is invalid, UCI multiplexing procedure in the initial slot/subslot assumes N-N2 bits of SPS HARQ-ACK is to be transmitted while N2 bits of SPS HARQ-ACK is deferred to the following slots/subslots,

- wherein, N is the bit width of SPS HARQ-ACK in the initial PUCCH. N1 bits of SPS HARQ-ACK within the N bits is configured deferrable, N1≤N. The value of N2 is the minimum value that results in at least one valid PUCCH or PUSCH for the UCIs in the “initial PUCCH” according to the existing UCI multiplexing rules, N2≤N1.

According to the above discussion, a target PUCCH is needed for the N2 bits of deferred HARQ-ACK in the slots/subslots following the initial slot/subslot. It is preferred the deferred HARQ-ACK could be transmitted as early as possible. Then, the first available PUCCH is preferred.

To get available PUCCH, valid UL symbols are needed. It is no doubt that semi-static UL symbols is valid UL symbols. The semi-static flexible symbol(s) is possible to be dynamically transferred to DL or UL symbols by dynamic SFI. If the SFI is missed at the UE side while the gNB thinks the indication of SFI has been valid, there would be misalignments about the available UL symbols between gNB and UE. Then the reliability of deferred SPS HARQ would be impacted if the semi-static flexible symbol(s) is deemed as available symbols. This will harm the performance of URLLC. To satisfy high reliability of SPS HARQ, semi-static flexible symbols could be exempted from being valid symbols for available PUCCH. However, for most cases, SFI could be correctly detected and this restriction would delay the deferred HARQ-ACK to further later symbols. This is also detrimental to URLLC from the latency performance point of view.

Observation 2: Always exempting semi-static F symbols from being valid symbols is detrimental from the latency point of view for URLLC.

The performance of SFI could be controllable at gNB side to some extent. In addition, gNB is also aware of the reliability requirement of SPS. Then, it could be efficient gNB configures semi-static F symbols are valid symbols for “available” PUCCH when the probability of SFI misdetection is really low while invalid if the probability is unacceptable comparing with the reliability of SPS.

Proposal 6: gNB configures whether UL symbols indicated by SFI could be valid symbols when decide available PUCCH or not.

With valid UL symbols, there are two cases to decide the target PUCCH:

- Case A: Without the deferred SPS HARQ-ACK, the target PUCCH does not exist.

- Case B: The target PUCCH was to be transmitted even if there is no deferred SPS HARQ-ACK.

To our understanding, the deferred HARQ-ACK is expected to be transmitted as early as possible. It is proposed the next available PUCCH is the earliest one within Case A and Case B.

Proposal 7: The next available PUCCH is the earliest one within the PUCCHs decided within available symbols and the PUCCHs which was to be transmitted according to Rel.16 procedure.

For case A, a number of contiguous valid UL symbols could be used as the target PUCCH. The number of contiguous valid UL symbols for target PUCCH could be decided according to the configuration of n1PUCCH-AN/SPS-PUCCH-AN-List-r16 for SPS HARQ-ACK. Alternatively, special PUCCH configuration for deferred HARQ-ACK could be considered to decide the number of contiguous symbols. The target PUCCH includes the number of contiguous symbols where the location of these symbols could be accordance or not accordance with the corresponding PUCCH configuration.

Proposal 8: To decide the number of contiguous UL symbols for available PUCCH transmission, PUCCH parameters configured by n1PUCCH-AN/SPS-PUCCH-AN-List-r16 could be reused, or special PUCCH configuration for deferred HARQ-ACK could be considered.

For case B, the PUCCH could be one from configured SPS-PUCCH-AN-List-r16, multi-CSI-PUCCH-ResourceList, or PUCCH-ResourceSet in PUCCH-Config. According to Rel.16 procedure, PUCCH transmission according to SPS-PUCCH-AN-List-r16 or multi-CSI-PUCCH-ResourceList is based on semi-static configuration and could have consistent understanding between gNB and UE. For PUCCH configured by PUCCH-ResourceSet, the transmission depends on dynamic scheduling. Whether consistent PUCCH could be achieved by gNB and UE depends on the probability of DCI misdetection. Similar as the above discussion, it is proposed gNB configures whether PUCCH transmission scheduled for dynamic HARQ-ACK could be used as available PUCCH or not.

Proposal 9: Semi-static configured PUCCH transmission according to SPS-PUCCH-AN-List-r16 or multi-CSI-PUCCH-ResourceList could be used as available PUCCH.

Proposal 10: gNB configures whether PUCCH transmission scheduled for dynamic HARQ-ACK could be used as available PUCCH or not.

Conclusion

In this contribution, we discussed some aspects about SPS HARQ-ACK dropping for TDD in Rel.17 URLLC. The following observations and proposals are reached:

Proposal 1: SPS HARQ-ACK deferral is configured per SPS configuration.

Observation 1: In the initial slot/subslot, SPS HARQ-ACK corresponds to “initial PUCCH” which is one PUCCH from SPS-PUCCH-AN-List-r16 or n1PUCCH-AN for SPS HARQ-ACK, PUCCH-ResourceSet for dynamic scheduled PDSCH, or multi-CSI-PUCCH-ResourceList for CSI.

Proposal 2: SPS HARQ-ACK is deferred if initial PUCCH includes invalid symbol(s) which the initial PUCCH is decided according to the current UCI multiplexing behavior from the configuration of SPS-PUCCH-AN-List-r16 or n1PUCCH-AN, n1PUCCH-AN, multi-CSI-PUCCH-ResourceList.

Proposal 3: If gNB supports PUCCH blind detection assuming DCI is missed by UE, SPS HARQ-ACK needs not to be deferred if it is transmitted in the initial PUCCH which is decided from PUCCH-ResourceSet. Otherwise, SPS HARQ-ACK should be deferred even if it is transmitted in the PUCCH from PUCCH-ResourceSet.

Proposal 4: SPS HARQ-ACK is possible to be deferred to one PUCCH other than the initial PUCCH in the initial slot/subslot.

Proposal 5: To decrease the deferred SPS HARQ-ACK impacts on the UCI in the initial PUCCH, if the initial PUCCH is invalid, UCI multiplexing procedure in the initial slot/subslot assumes N-N2 bits of SPS HARQ-ACK is to be transmitted while N2 bits of SPS HARQ-ACK is deferred to the following slots/subslots,

- wherein, N is the bit width of SPS HARQ-ACK in the initial PUCCH. N1 bits of SPS HARQ-ACK within the N bits is configured deferrable, N1≤N. The value of N2 is the minimum value that results in at least one valid PUCCH or PUSCH for the UCIs in the “initial PUCCH” according to the existing UCI multiplexing rules, N2≤N1.

Observation 2: Always exempting semi-static F symbols from being valid symbols is detrimental from the latency point of view for URLLC.

Proposal 6: gNB configures whether UL symbols indicated by SFI could be valid symbols when decide available PUCCH or not.

Proposal 7: The next available PUCCH is the earliest one within the PUCCHs decided within available symbols and the PUCCHs which was to be transmitted according to Rel.16 procedure.

Proposal 8: To decide the number of contiguous UL symbols for available PUCCH transmission, PUCCH parameters configured by n1PUCCH-AN/SPS-PUCCH-AN-List-r16 could be reused, or special PUCCH configuration for deferred HARQ-ACK could be considered.

Proposal 9: Semi-static configured PUCCH transmission according to SPS-PUCCH-AN-List-r16 or multi-CSI-PUCCH-ResourceList could be used as available PUCCH.

Proposal 10: gNB configures whether PUCCH transmission scheduled for dynamic HARQ-ACK could be used as available PUCCH or not.

- Reference

- Chairman's Notes RAN1#104-e final

- R1-2101818, “Moderator summary #2 on HARQ-ACK feedback enhancements for NR Rel-17 URLLC/IIoT”

- 3GPP TS 38.212 V16.3.0, “Multiplexing and channel coding”

- 3GPP TS 38.213 V16.3.0, “Physical layer procedures for control”