3GPP TSG RAN WG1 #104b-e	R1-2102579

# e-Meeting, April 12th – 20th, 2021‎

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | - | rev | - | Current version: | 15.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on common PUCCH resource in initial UL BWP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on the current specification, common PUCCH resource in the initial UL BWP is used if a UE does not have dedicated PUCCH resource configuration. The intended UE behavior is not clear. There are three possible interpretations. - If a UE does not have dedicated PUCCH resource configuration for the active UL BWP, common PUCCH resource in initial UL BWP is used; - If a UE does not have dedicated PUCCH resource configuration for any of the UL BWP(s) of the PCell , common PUCCH resource in initial UL BWP is used; - If a UE does not have dedicated PUCCH resource configuration for the initial UL BWP and the initial UL BWP is the active BWP, common PUCCH resource in initial UL BWP is used. Accroding to interpretation 1, it leads to potential misunderstanding that a UE is expected to use common PUCCH resource in initial UL BWP if the UE does not have dedicated PUCCH resource configuration for the active UL BWP which is different from initial UL BWP. According to interpretation 2, when dedicated PUCCH resource configuration is configured for at least one of the UL BWP(s) of the PCell, UE is not expected to use common PUCCH resource in the initial UL BWP. However, it is possible that dedicated PUCCH resource configuration is configured for a dedicated BWP but such dedicated BWP is not activated for the UE and UE stays in initial UL BWP without dedicated PUCCH resource configuration for the initial UL BWP. In this case, common PUCCH resource should be still used. Interpretation 3 is the intended UE behavior. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the condition of using common PUCCH resource in the initial BWP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear UE behavior for PUCCH transmission with common PUCCH resource in the initial BWP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR has isolated impact on PUCCH transmission in initial UL BWP when there is no dedicated PUCCH resource and it is common understanding for both UE and gNB implimentations. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

9.2.1	PUCCH Resource Sets

If a UE does not have dedicated PUCCH resource configuration for an active UL BWP of initial UL BWP of  PRBs, provided by PUCCH-ResourceSet in PUCCH-Config, a PUCCH resource set is provided by pucch-ResourceCommon through an index to a row of Table 9.2.1-1 for transmission of HARQ-ACK information on PUCCH in the initial UL BWP of  PRBs. The PUCCH resource set includes sixteen resources, each corresponding to a PUCCH format, a first symbol, a duration, a PRB offset , and a cyclic shift index set for a PUCCH transmission. The UE transmits a PUCCH using frequency hopping. An orthogonal cover code with index 0 is used for a PUCCH resource with PUCCH format 1 in Table 9.2.1-1. The UE transmits the PUCCH using the same spatial domain transmission filter as for a PUSCH transmission scheduled by a RAR UL grant as described in Clause 8.3.

< Unchanged part is omitted >