3GPP TSG RAN WG1 Meeting #104b-e	     	   	                R1-2102597

e-Meeting, April 12th – 20th, 2021

Source:	CATT

Title:	Discussion and TP on remaining issues in NR positioning

Agenda item:	7.2.8

Document for:	Discussion and Decision

Introduction

In RAN1#104-e meeting, the following agreements were achieved on remaining issues on maintenance for Rel-16 NR positioning [1]. In this contribution, we discuss several issues related to SRS-Pos.

R1-2102251 (Rel-16, TS38.214, CR0171) CR on DL PRS resource indication in activation command for semi-persistent SRS for positioning and amount of DL PRS resource sets per TRP Moderator (Intel Corporation), Nokia, Nokia Shanghai Bell, CATT R1-2102252 (Rel-16, TS38.214, CR0172) CR on timestamp reference in NR positioning measurement report Moderator (Intel Corporation), vivo R1-2102253 (Rel-16, TS38.214, CR0173) CR on measurement gap configuration for NR positioning Moderator (Intel Corporation), vivo, Huawei R1-2102205 (Rel-16, TS38.214, CR0168) CR for the configuration of semi-persistent SRS Moderator (Ericsson), ZTE R1-2102206 (Rel-16, TS38.214, CR0169) CR for the configuration of DL-PRS as the spatial relation of periodic and semi-persistent positioning SRS R1-2102207 (Rel-16, TS38.214, CR0170) CR for the configuration of SRS for positioning in SRS carrier switching Final LS endorsed in R1-2101918 R1-2101882 (TS36.214, Rel-16, CR#0057, Cat F) Corrections to 36.214 for Reference Point for eNB Rx – Tx time difference CATT

Discussion

The pathloss estimates that the UE can simultaneously maintain for SRS-Pos

In section 7.3.1 of TS 38.213 [1], for the pathloss estimates that the UE can simultaneously maintain for all SRS-Pos, in addition to the up to four pathloss estimates that the UE maintains per serving cell for PUSCH/PUCCH/SRS-MIMO transmissions, the describptions about UE behavior for the indication of pathloss estimates for SRS-Pos are as follows.

- 7.3.1 UE behaviour ----------------------------------------------------- unrelated part omitted ------------------------------------------------ The UE indicates a capability for a number of pathloss estimates that the UE can simultaneously maintain for all SRS resource sets provided by SRS-PosResourceSet in addition to the up to four pathloss estimates that the UE maintains per serving cell for PUSCH/PUCCH transmissions and for SRS transmissions configured by SRS-Resource.

However, the UE may not indicate this capability for a band. If the UE does not indicate this capability for a band, the UE does not support any pathloss estimates in addition to the up to four pathloss estimates that the UE maintains per serving cell for the PUSCH/PUCCH/SRS-MIMO transmissions in that band. Therefore, we propose to add a word “may” before the word “indicates” to address this issue.

Moreover, we propose to change “a number of  pathloss estimates” into “up to sixteen pathloss estimates” to align the description with the “up to four pathloss estimates” in the following sentence and make it clear.

In addition, in fact, there are two capabilities for the pathloss estimates that the UE can simultaneously maintain for all SRS-Pos per serving cell or across all cells [3] and also shown in the table below, therefore, it would be better to add an additional paragraph in TS 38.213 [1] to describe these two capabilities separately.

| Features | Index | Feature group | Components | Type | Note | Mandatory /Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 13. NR Positioning | 13-9e | PathLoss estimate maintenance per serving cell | - Max number of pathloss estimates that the UE can simultaneously maintain for all the SRS resource sets for positioning per serving cell in addition to the up to four pathloss estimates that the UE maintains per serving cell for the PUSCH/PUCCH/SRS transmissions” - Candidate values are {1, 4, 8, 16} - Note: SRS in “PUSCH/PUCCH/SRS” refers to SRS configured by SRS-Resource | Per band | RAN1 kindly requests RAN2 to decide on the necessity for location server to know if the feature is supported SRS and SSB and/or PRS are in the same band Note: if the UE does not indicate this capability for a band, the UE does not support any pathloss estimates in addition to the up to four pathloss estimates that the UE maintains per serving cell for the PUSCH/PUCCH/SRS transmissions in that band. | Optional with capability signaling |
| 13. NR Positioning | 13-9f | PathLoss estimate maintenance across all cells | - Max number of pathloss estimates that the UE can simultaneously maintain for all the SRS resource sets for positioning across all cells in addition to the up to four pathloss estimates that the UE maintains per serving cell for the PUSCH/PUCCH/SRS transmissions” - Candidate values are {1, 4, 8, 16} - Note: SRS in “PUSCH/PUCCH/SRS” refers to SRS configured by SRS-Resource | Per UE | RAN1 kindly requests RAN2 to decide on the necessity for location server to know if the feature is supported SRS and SSB and/or PRS are in the same band | Optional with capability signaling |

In order to solve the above issues, one text proposal is proposed for section 7.3.1 in TS 38.213 as follows.

Proposal 1: Adopt the following text proposal (TP-A) for the pathloss estimates that the UE can simultaneously maintain for SRS-Pos in section 7.3.1 of 38.213:

# Proposed TP-A -----------------------------------------------Start of Text Proposal for 38.213---------------------------------------------- - 7.3.1 UE behaviour ----------------------------------------------------- unrelated part omitted ------------------------------------------------ The UE may indicate a capability for up to sixteen pathloss estimates that the UE can simultaneously maintain for all SRS resource sets provided by SRS-PosResourceSet per serving cell in addition to the up to four pathloss estimates that the UE maintains per serving cell for PUSCH/PUCCH transmissions and for SRS transmissions configured by SRS-Resource. The UE may indicate a capability for up to sixteen pathloss estimates that the UE can simultaneously maintain for all SRS resource sets provided by SRS-PosResourceSet across all cells in addition to the up to four pathloss estimates that the UE maintains per serving cell for PUSCH/PUCCH transmissions and for SRS transmissions configured by SRS-Resource. -------------------------------------------------------End of Text Proposal --------------------------------------------------

Conclusions

In this contribution, we discuss one TP related to the pathloss estimates that the UE can simultaneously maintain for SRS-Pos. Based on the discussion, we propose:

Proposal 1: Adopt the following text proposal (TP-A) for the pathloss estimates that the UE can simultaneously maintain for SRS-Pos in section 7.3.1 of 38.213:

# Proposed TP-A -----------------------------------------------Start of Text Proposal for 38.213---------------------------------------------- - 7.3.1 UE behaviour ----------------------------------------------------- unrelated part omitted ------------------------------------------------ The UE may indicate a capability for up to sixteen pathloss estimates that the UE can simultaneously maintain for all SRS resource sets provided by SRS-PosResourceSet per serving cell in addition to the up to four pathloss estimates that the UE maintains per serving cell for PUSCH/PUCCH transmissions and for SRS transmissions configured by SRS-Resource. The UE may indicate a capability for up to sixteen pathloss estimates that the UE can simultaneously maintain for all SRS resource sets provided by SRS-PosResourceSet across all cells in addition to the up to four pathloss estimates that the UE maintains per serving cell for PUSCH/PUCCH transmissions and for SRS transmissions configured by SRS-Resource. -------------------------------------------------------End of Text Proposal --------------------------------------------------

References

- 3GPP RAN1 Chairman’s Notes of TSG RAN WG1 Meeting #104-e.

- 3GPP TS 38.213, “NR; Physical layer procedures for control”，v16.5.0，2021-03.

- R1-2102006, “Updated RAN1 UE features list for Rel-16 NR after RAN1#104-e”, Moderators (AT&T, NTT DOCOMO, INC.).