3GPP TSG RAN WG1 #104b-e                                                                          R1-2102614

e-Meeting, April 12th –20th, 2021

Source:              CATT

Title:                   Evaluation results of XR performance

Agenda Item:     8.14.3

Document for:   Discussion

Introduction

In RAN#104e-meeting [1], general simulation assumption, traffic model and definition of Per UE KPI for XR/CG performance evaluation were agreed.

Agreements: RAN1 adopts a parameterized statistical traffic model for evaluation of XR and CG, and KPI with details as shown below (RAN1 strives to agree on the remaining details during RAN1 #104e, based on SA4 input):

- There are M1 and M2 streams in DL and UL respectively

  - At least adopt the case where M1=1 & M2=1

  - FFS the values of M1 and M2, including the possibility of being application-dependent

- DL

  - Bitrate for video streaming

    - VR/AR: [60 Mbps (mandatory), 30 Mbps (optional)]

    - CG: [30 Mbps (mandatory), 45 Mbps (optional)]

      - FFS: other optional values

  - Air interface Packet Delay budget (PDB)

    - Air interface delay is measured from the point when a packet arrives at gNB to the point when it is successfully delivered to UE

    - Air interface PDB for video streaming

      - VR/AR: [10ms (mandatory), 20ms (optional)]

      - CG: [15ms (mandatory), 30ms (optional)]

        - FFS: other optional values

  - FFS: Frame-level/IP packet-level modeling for packet arrival, latency measure, etc.

  - FFS: Packet size, including the possibility of varying packet sizes

  - FFS: Packet Inter arrival time including the possibility of modeling jitter

- UL

  - FFS: Bitrate

  - FFS: Air interface Packet Delay budget (PDB)

  - FFS: Frame-level/IP packet-level modeling for packet arrival, latency measure, etc.

  - FFS: Packet size

- Per UE KPI

  - Baseline: A UE is declared a satisfied UE if more than X (%) of packets are successfully transmitted within a given air interface PDB. The exact value of X is FFS.

  - FFS: In addition to the baseline, the following additional method is FFS

    - When determining a XR/CG user is satisfied or not, the following factors are considered. FFS how to use those factors.

      - Packet loss information

      - Packet delay information

      - Some XR/CG source related information if they can be available within RAN, e.g. the mapping between packet and slices or frames and the packet importance

      - Multiple data streams traffic model

  - FFS if there are multiple streams (if adopted)

- FFS additional aspects not addressed above.

- Note 1: Companies are encouraged to provide details such as parameters (e.g., mean, STD, etc.), distributions, etc., by analyzing SA4 input, e.g., V/S/P traces

- Note 2: All FFS points above are to be further discussed in RAN1 #104e

This contribution presents initial performance evaluation results, including capacity and power in Indoor Hotspot and Dense Urban scenarios.

Performance results

In this section, preliminary simulation results are provided based on agreed simulation assumption, including system capacity and power consumption.

Simulation assumptions

Traffic model

Traffic arrived at XR/CG sever/device periodically and transported jittery to RAN. Packet size is generated based on the Truncated Gaussian distribution. The additional delay is considered as a variable followed Truncated Gaussian distribution. The detailed traffic parameters for XR/CG evaluations are shown in Table 1. Note that these parameter values are those before the truncation. In SLS, the packet is obtained by intercepting the sample of Gaussian distribution within a certain range.

Table 1: Traffic parameters for XR evaluations

| Gaussian Parameters |  | XR | CG |
| --- | --- | --- | --- |
| Bit rate | - | 30Mbps | 8Mbps |
| Packet size | Mean packet size | 62500byte | 16667byte |
|  | Std. packet size value | 2600byte | 2600byte |
|  | Min packet size | 45000byte | 6667 |
|  | Max packet size | 93750byte | 33750 |
| Inter-arrival time | Packet generated period | 17ms | 17ms |
|  | Mean jitter value | 0 |  |
|  | STD. jitter value | 2 |  |
|  | Max jitter value | 4 |  |
|  | Min jitter value | 0 |  |

Simulation scenario

In RAN1#103e and RAN1#104e, most simulation parameters for XR/CG performance evaluation were agreed. The simulation assumption is shown in Table 2.

Table 2: Simulation parameters

| Scenario | Indoor Hotspot | Dense Urban |
| --- | --- | --- |
| Layout | 120m x 50m ISD: 20m TRP numbers: 12 | 21 cell with wraparound ISD：200m |
| Carrier frequency | FR1:4GHz | FR1:4GHz |
| Bandwidth | FR1:100MHz | FR1:100MHz |
| Subcarrier spacing | FR1: 30 kHz | FR1:30kHz |
| BS height | 3m | 25m |
| UE height | hUT=1.5 m |  |
| BS noise figure | FR1: 5 dB | FR1: 5 dB |
| UE noise figure | FR1: 9 dB | FR1: 9 dB |
| BS receiver | MMSE-IRC MMSE-IRC |  |
| UE receiver |  |  |
| UE speed | 3 km/h |  |
| Channel estimation | Realistic |  |
| MCS | Up to 256QAM |  |
| BS antenna pattern | Ceiling-mount antenna radiation pattern, 5 dBi | 3-sector antenna radiation pattern, 8dBi |
| UE antenna pattern | FR1: Omni-directional, 0 dBi, | FR1: Omni-directional, 0 dBi, |
| TX power | gNB: FR1: 24dBm/20MHz; | gNB: FR1:44dBm/20MHz UE: 23dBm |
| gNB antenna configuration | gNB: - FR1:32TxRU, (M,N,P,Mg,Ng; Mp,Np)=(4,4,2,1,1;4,4), (dH, dV) = (0.5, 0.5)λ | gNB: - FR1: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) , (dH, dV) = (0.5λ, 0.5λ) |
| UE antenna configuration | UE: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ |  |

Simulation assumptions for power

TR 38.840 is reused to assess the power consumption evaluation for XR performance evaluation. The power consumption model of FR1 for XR performance evaluation is shown in Table 3.

Table 3: Power consumption model for XR performance evaluation

| Power state | Relative Power |
| --- | --- |
| Micro Sleep | 45 |
| PDCCH_only | 100 |
| PDCCH+PDSCH | 300 |

KPI and DL latency model

The performance evaluation has considered requirements of latency, reliability and power. The Key Performance Indexes (KPIs) are the index of the system performance for XR services, including capacity and UE power consumption. In RAN1#103-e, it was agreed that:

Agreement:

System capacity is defined as the maximum number of users per cell with at least X % of UEs being satisfied.

- X=90 (baseline) or 95 (optional)

- Other values of X can also be evaluated optionally

Note: The exact ‘satisfied’ requirements will be discussed separately

FFS: how to calculate the percentage of satisfied users across multiple drops of simulations

In RAN1#104-e, the ‘satisfied’ requirements were decided:

Agreement:

- Per UE KPI

  - Baseline: A UE is declared a satisfied UE if more than X (%) of packets are successfully transmitted within a given air interface PDB.

    - The exact value of X is FFS, e.g., 99, 95

      - FFS different values for I-frame and P-frame if evaluation of them is agreed.

      - Other values can be optionally evaluated

In this contribution, system capacity is defined as the max number of UE per cell when 90% UEs satisfy requirements. A UE is declared a satisfied UE if more than 99% of the packets are successfully transmitted within 10ms PDB. DL latency considers the part of processing, waiting for DL TTI, ACK/NACK feedback and retransmission. Detailed DL latency model is described in Figure 1. In Figure 1, the black line with arrow means DL first transmission process and the red line with arrow means a retransmission process. The UE is assumed to have PDSCH processing capability 2.

Figure 1: DL latency model

Simulation results

In the section, we provide evaluation results of XR and CG service based on the simulation assumptions described in section 2.1.

Indoor Hotspot

We simulate the performance of XR and CG service base on traffic model in Table 1 in FR1 Indoor Hotspot scenario. The simulation adopts 1e-2 target BLER.

2.4.1.1    System capacity

Table 4 shows two detailed C-DRX configuration parameters. C-DRX 1 adopts common DRX cycle value for UE power saving and C-DRX 2 adopts the parameters matched the inter-arrival time of XR/CG traffic.

Table 4: C-DRX configuration parameters

| DRX configuration | DRX Cycle | OnDurationTimer | InactivityTimer |
| --- | --- | --- | --- |
| C-DRX 1 | 160ms | 100ms | 8ms |
| C-DRX 2 | 17ms | 10ms | 2ms |

Figure 2 and Figure 4 show the percentage of UEs that 99% of the packets successfully transmitted within 10ms for XR and CG. The percentage of satisfied XR UEs decreases when the system load increases to 12 UEs per cell. Figure 3 shows the resource utilization (RU) of XR service under different system loads.

- XR

Figure 2: The percentage of satisfied UE with XR applications under different system loads

Figure 3: RU under different system load for XR service

- CG

Figure 4: The percentage of satisfied UE with CG applications under different system loads

From the results, it could be observed that the system capacity without any power saving scheme is 6 UEs per cell for XR service of 60 FPS frame rate and 30Mbps bit rate, the system capacity without any power saving scheme is at least 12UEs per cell for CG service of 60 FPS frame rate and 8Mbps bit rate. The RU is from 35% to 83.3% for XR service. The DRX configuration reduces the percentage of satisfied UEs for XR service. The system capacity of XR with the DRX configuration could be reduced by 18.1%~75% and even up to 100% compared to the baseline.

For C-DRX 2, it seems that the DRX has greater impact on capacity when the system load is low. The reason might be that UEs in high load cell have longer active time in DRX cycle. In particular, the UE in high-load cell is more likely to be interfered by other UEs in the same cell, which causes lower SINR. The lower SINR leads to the higher probability of retransmission. Further, the higher the probability of retransmission, the higher the probability of UE receiving PDCCH and then entering DRX Inactivity state.

Observation 1: For XR service in Indoor Hotspot scenario, the system capacity without any power saving scheme is 6 UEs per cell for XR service with 60 FPS frame rate and 30Mbps bit rate.

Observation 2:  For CG service in Indoor Hotspot scenario, the system capacity without any power saving scheme is at least 12 UEs per cell for CG service with 60FPS and 8Mbps bit rate.

Observation 3: For XR service in Indoor Hotspot scenario, the system capacity of XR with the DRX configuration could be reduced by 18.1%~75% and even up to 100% compared to the system capacity of XR without any power saving scheme.

Observation 4: Compare to common DRX configuration parameter for R15 UE power saving, the DRX configuration matched the inter-arrival of XR traffic has less impact on capacity.

2.4.1.2 UE Power consumption

In the section, UE power consumption result of XR service is shown. Figure 5 describes the average UE power consumption under different system load without any power saving scheme. Figure 6 and Figure 7 describe the average UE power consumption under different system load with different DRX configurations. The power consumptions in Figure 5, 6 and 7 are the average value of all UEs.

  - Case1: baseline

Figure 5: UE power consumption with different system loads

  - Case2: C-DRX 1

Figure 6: UE power consumption over slot under different system load with C-DRX 1

  - Case 3: C-DRX 2

Figure 7: UE power consumption over slot under different system load with C-DRX 2

From these results, it could be observed that the baseline UE power consumption for XR service is around 96.2 units at 30 Mbps bit rate. Compared to average UE power consumption without any power saving scheme, the average UE power consumption with C-DRX 1 and C-DRX 2 is reduced by 3.9%~13% and 6%~12.3%, respectively, where the power saving gain is from the reduction of unnecessary PDCCH monitoring.

Table 5 collects the power saving performance results of DRX for 30Mbps XR service when the number of UE per cell is 6.

Table 5: Evaluation of UE power saving schemes for Indoor Hotspot

| Power Saving Scheme | Power Saving Gain (PSG) compared to Case 1 |  |  |  | #satisfied UEs per cell / #UEs per cell |
| --- | --- | --- | --- | --- | --- |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |
| Case 1 | - | - | - | - | 5 / 6 |
| C-DRX 1 | 7.3 % | 7.13% | 4.68% | 20.5% | 0 / 6 |
| C-DRX 2 | 10.37% | 4.9 % | 9.87 % | 17.28% | 2 / 6 |

Observation 5: The performance of DRX for 30Mbps XR service in Indoor Hotspot and detailed DRX configurations are shown as follows,

Table 4: C-DRX configuration parameters

| DRX configuration | DRX Cycle | OnDurationTimer | InactivityTimer |
| --- | --- | --- | --- |
| C-DRX 1 | 160ms | 100ms | 8ms |
| C-DRX 2 | 17ms | 10ms | 2ms |

Table 5: Evaluation of UE power saving schemes for Indoor Hotspot

| Power Saving Scheme | Power Saving Gain (PSG) compared to Case 1 |  |  |  | #satisfied UEs per cell / #UEs per cell |
| --- | --- | --- | --- | --- | --- |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |
| Case 1 | - | - | - | - | 5 / 6 |
| C-DRX 1 | 7.3 % | 7.13% | 4.68% | 20.5% | 0 / 6 |
| C-DRX 2 | 10.37% | 4.9 % | 9.87 % | 17.28% | 2 / 6 |

Dense Urban

We simulate the performance of XR service base on traffic model in Table 3 in FR1 Dense Urban scenario. The simulation adopts 1e-5 target BLER. C-DRX that matches the inter-arrival time of XR traffic time is adopted to evaluate the power saving gain and capacity loss, i.e., 17ms DRX cycle, 10ms drx-onDurationTimer, and 2ms drx-InactivityTimer.

      - System capacity

Figure 8 shows the percentage of UE that 99% of the packet successfully transmitted within 10ms for XR service in Dense Urban. The percentage of satisfied XR UEs decreases when the system load increases to 12 UEs per cell. The DRX configuration reduces the percentage of satisfied UEs for XR service. The system capacity without any power saving scheme is 10 UEs per cell for XR service of 60 FPS frame rate and 30Mbps bit rate. The system capacity with DRX configuration is reduced by 15.9%~52.4% compare to the system capacity without any power saving scheme. Figure 9 shows the RU of XR service under different system loads. The RU is from 36.2% to 88.7% for XR service when the number of UEs per cell is from 4 to 12.

Figure 8: The percentage of satisfied UE with XR applications under different system load in DU

Figure 9: RU under different system load for XR service at DU scenario

Observation 6: For XR service in Dense Urban scenario, the system capacity without any power saving scheme is 10 UEs per cell with XR service of 60 FPS frame rate and 30Mbps bit rate.

Observation 7: For XR service in Dense Urban scenario, the system capacity with DRX configuration is reduced by 15.9%~52.4% compare to the system capacity without any power saving scheme.

2.4.1.2 UE Power consumption

In the section, power performance result of XR service is shown. Figure 10 describes the average UE power consumption under different system load without any power saving scheme. Figure 11 shows the average UE power consumption under different system load with DRX configuration.

  - Case1: baseline

Figure 10: UE power consumption with different system loads

  - Case2: C-DRX 2

Figure 11: UE power consumption over slot under different system load with C-DRX 2

From the results, it could be observed that the baseline average UE power consumption over slot for XR service is around 100 units at 30 Mbps bit rate. Compared to baseline average UE power consumption, the average UE power consumption with C-DRX is reduced by 8.7%~14.4%.

Table 6 collects the power saving performance results of DRX for 30Mbps XR service in DU when the number of UE per cell is 10.

Table 6: Evaluation of UE power saving schemes for Dense Urban

| Power Saving Scheme | Power Saving Gain (PSG) compared to baseline |  |  |  | #satisfied UEs per cell / #UEs per cell |
| --- | --- | --- | --- | --- | --- |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |
| baseline | - | - | - | - | 9 / 10 |
| C-DRX 2 | 10.3 % | 4.7% | 8.9 % | 19% | 7 / 10 |

Observation 8: The performance of DRX for 30Mbps XR service in Dense Urban and detailed DRX configurations are shown as follows,

Table 4: C-DRX configuration parameters

| DRX configuration | DRX Cycle | OnDurationTimer | InactivityTimer |
| --- | --- | --- | --- |
| C-DRX 2 | 17ms | 10ms | 2ms |

Table 6: Evaluation of UE power saving schemes for Dense Urban

| Power Saving Scheme | Power Saving Gain (PSG) compared to baseline |  |  |  | #satisfied UEs per cell / #UEs per cell |
| --- | --- | --- | --- | --- | --- |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |
| baseline | - | - | - | - | 9 / 10 |
| C-DRX 2 | 10.3 % | 4.7% | 8.9 % | 19% | 7 / 10 |

Conclusion

In this contribution, the initial performance evaluation results are presented based on XR and CG traffic model and simulation assumption. Based on these results, we have the following observations:

Observation 1: For XR service in Indoor Hotspot scenario, the system capacity without any power saving scheme is 6 UEs per cell for XR service with 60 FPS frame rate and 30Mbps bit rate.

Observation 2: For CG service in Indoor Hotspot scenario, the system capacity without any power saving scheme is at least 12 UEs per cell for CG service with 60FPS and 8Mbps bit rate.

Observation 3: For XR service at Indoor Hotspot scenario, the system capacity of XR with the DRX configuration could be reduced by 18.1%~75% and even up to 100% compared to the system capacity of XR without any power saving scheme.

Observation 4: Compare to common DRX configuration parameter for R15 UE power saving, the DRX configuration matched the inter-arrival of XR traffic has less impact on capacity.

Observation 5: The performance of DRX for 30Mbps XR service in Indoor Hotspot and detailed DRX configurations are shown as follows,

Table 4: Detailed DRX configuration

| DRX configuration | DRX Cycle | OnDurationTimer | InactivityTimer |
| --- | --- | --- | --- |
| C-DRX 1 | 160ms | 100ms | 8ms |
| C-DRX 2 | 17ms | 10ms | 2ms |

Table 5: Evaluation of UE power saving schemes for Indoor Hotspot

| Power Saving Scheme | Power Saving Gain (PSG) compared to Case 1 |  |  |  | #satisfied UEs per cell / #UEs per cell |
| --- | --- | --- | --- | --- | --- |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |
| Case 1 | - | - | - | - | 5 / 6 |
| C-DRX 1 | 7.3 % | 7.13% | 4.68% | 20.5% | 0 / 6 |
| C-DRX 2 | 10.37% | 4.9 % | 9.87 % | 17.28% | 2 / 6 |

Observation 6: For XR service in Dense Urban scenario, the system capacity without any power saving scheme is 10 UEs per cell with XR service of 60 FPS frame rate and 30Mbps bit rate.

Observation 7: For XR service in Dense Urban scenario, the system capacity with DRX configuration is reduced by 15.9%~52.4% compare to the system capacity without any power saving scheme.

Observation 8: The performance of DRX for 30Mbps XR service in Dense Urban and detailed DRX configurations are shown as follows,

Table 4: Detailed DRX configuration

| DRX configuration | DRX Cycle | OnDurationTimer | InactivityTimer |
| --- | --- | --- | --- |
| C-DRX 2 | 17ms | 10ms | 2ms |

Table 6: Evaluation of UE power saving schemes for Dense Urban

| Power Saving Scheme | Power Saving Gain (PSG) compared to baseline |  |  |  | #satisfied UEs per cell / #UEs per cell |
| --- | --- | --- | --- | --- | --- |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |
| baseline | - | - | - | - | 9 / 10 |
| C-DRX 2 | 10.3 % | 4.7% | 8.9 % | 19% | 7 / 10 |

References

- RAN1#104-e Chairman Notes.