3GPP TSG RAN WG1 #104b-e                                         R1-2102617

e-Meeting, April 12th – April 23th, 2021

Agenda Item:	8.15.1

Source:	CATT

Title:	        Applicable scenario and initial evaluation result to NB-IoT/eMTC

Document for:	Discussion and Decision

Introduction

In the RAN #104 e-meeting，the following agreements on scenarios and configuration have been reached[1]:

Agreement:

The following assumptions are agreed for a common set of link budget parameters:

- UE power class (PC5=20 dBm)

- UE Noise Figure (NF=9 dB)

- Channel Bandwidth for NB-IoT and eMTC as was included in IoT NTN reference scenario parameters agreed in RAN1#103e

  - NB-IoT 180 kHz (DL), Up to 180 kHz with all permissible smaller resource allocations 12*15 kHz, 6*15 kHz, 3*15 kHz, 1*15 kHz, 1*3.75 kHz

  - eMTC: 1080 kHz (DL), Up to 1080 kHz with all permissible smaller resource allocations, including 2*180 kHz, 180 kHz, 2*15 kHz or 3*15 kHz or 6*15 kHz (UL)

- Other losses

| Other Losses | GEO (35786 km) | LEO (1200 km) | LEO (600 km) |
| --- | --- | --- | --- |
| Scintillation losses | 2.2 | 2.2 | 2.2 |
| Atmospheric losses | 0.2 | 0.1 | 0.1 |
| Polarization loss | 3 | 3 | 3 |
| Shadow margin | 3 | 3 | 3 |

NOTE 1: With PC3 (23 dBm) there is a 3dB gain compared to the PC5 (20 dBm) assumption on UL.

NOTE 2: With NF=7 dB, there is a 2 dB improvement compare to NF=9 dB on DL.

NOTE 3: Link budgets with other link budget parameters are not excluded from being captured in the TR.

NOTE 4: These parameters are only for the purpose of link budget calculations.

NOTE 5: Atmospheric losses are a function of elevation angle.

Agreement:

Link budget analysis assumes 3 dB polarization loss for DL and 3 dB polarization loss on UL for satellite parameters Set 1, Set 2, Set 3, and Set 4

Agreement:

Include in TR 36.763, the 3 dB beam width (HPBW), central beam center elevation and central beam edge elevation in the satellite parameter set(s) to be used in link budget calculations – (Corresponding satellite parameter Set 3 and Set 4 are given in Section 9.4)

| SET 3 | GEO 35786 km | LEO-600 km | LEO-1200 km |
| --- | --- | --- | --- |
| 3 dB Beam width (HPBW) | 0.735 degree | 22.0631 degree | 22.0631 degree |
| Central beam center elevation | 20.88 degree | 43.78 degree | 46.05 degree |
| Central beam edge elevation | 12.5 degree | 30 degree | 30 degree |
| Central beam edge satellite-UE distance | 40316 km | 1074 km | 1998 km |

| SET 4 | LEO-600 km |
| --- | --- |
| 3 dB Beam width (HPBW) | 104.7 degree |
| Central beam center elevation | 90 degree |
| Central beam edge elevation | 30 degree |
| Central beam edge satellite-UE distance | 1076 km |

NOTE 1: The 3 dB beam width (HPBW)  is already included in satellite parameter set 1 and Set 2 in TR 38.821 Table 6.1.1.1-1 and Table 6.1.1.1-2  respectively. The central beam center elevation  for Set-1 and Set-2 is defined as the target elevation angle that is included in in TR 38.821 Table 6.1.3.2-1.   The central beam edge satellite-UE distance can be derived from the central beam edge elevation and does not need to be included.

NOTE 2: Central beam center elevation is the beam center elevation of the central beam in the beam layout.

NOTE 3: Central beam edge elevation is the minimum beam edge elevation of the central beam in the beam layout.

NOTE 4: In SLS evaluation with a multiple beam layout, the central beam is the serving beam for UEs. The outer beams have beam center elevation that is different from the central beam center elevation.  For the interference modelling, the interference due to the outer beams is determined by using their respective beam center elevations.

NOTE 5: For the multiple-beam satellite cell, the longest beam edge distance will correspond to the minimum beam edge elevation of the most outer beam as illustrated in figure below.

Agreement:

Include the following tables in TR 36.763:

- Set 1 satellite parameters (based on TR 38.821, Table 6.1.1.1-1)

- Set 2 satellite parameters (based on TR 38.821, Table 6.1.1.1-2)

- Set 3 satellite parameters (Eutelsat R1-2101146 with central beam edge elevation 12.5 degree for GEO, and 30 degree for LEO-600 km and 1200 km)

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Central beam edge elevation |  | 12.5 degree | 30 degree | 30 degree |
| Central beam center elevation |  | 20.9 degree | 46.05 degree | 43.8 degree |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (NOTE 1) | S-band (i.e. 2 GHz) | 12 m | 0.4m | 0.4 m |
| Satellite EIRP density |  | 59.8 dBW/MHz | 33.7 dBW/MHz | 28.3 dBW/MHz |
| Satellite Tx max Gain |  | 45.7 dBi | 16.2 dBi | 16.2 dBi |
| 3dB beam width (HPBW) |  | 0.7353 degree | 22.1 degree | 22.1 degree |
| Satellite beam diameter (NOTE 2) |  | 459km | 470 km | 234 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (NOTE 1) | S-band (i.e. 2 GHz) | 12 m | 0.4 m | 0.4 m |
| G/T |  | 16.7dB K-1 | -12.8 dB K-1 | -12.8 dB K-1 |
| Satellite Rx max Gain |  | 45.7 dBi | 16.2 dBi | 16.2 dBi |

NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of TR 38.811

NOTE 2: Satellite beam diameter is at Nadir point

NOTE 3: Central beam center elevation is referred to as central beam elevation in TR 38.821

NOTE 4: Central beam edge elevation is the minimum beam edge elevation of the central beam in the beam layout.

- Set 4 satellite parameters (Thales, Sateliot, Gatehouse R1-2101019)

| Satellite orbit |  | LEO-600 |
| --- | --- | --- |
| Satellite altitude |  | 600 km |
| Central beam edge elevation |  | 30 degree |
| Central beam center elevation |  | 90 degree |
| Payload characteristics for DL transmissions |  |  |
| Equivalent satellite antenna aperture (NOTE 1) | S-band (i.e. 2 GHz) | 0.097 m |
| Satellite EIRP density |  | 21.45 dBW/MHz |
| Satellite Tx max Gain |  | 11 dBi |
| 3dB beam width (HPBW) |  | 104.7 degree |
| Satellite beam diameter (Note 2) |  | 1700 km |
| Payload characteristics for UL transmissions |  |  |
| Equivalent satellite antenna aperture (Note1) | S-band (i.e. 2 GHz) | 0.097 m |
| G/T |  | - 18.6 dB·K-1 |
| Satellite Rx max Gain |  | 11 dBi |

NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of TR 38.811

NOTE 2: Satellite beam diameter is at Nadir point

NOTE 3: Central beam center elevation is referred to as central beam elevation in TR 38.821

NOTE 4: Central beam edge elevation is the minimum beam edge elevation of the central beam in the beam layout.

In this contribution, firstly we discussed the possible scenario prioritization based on RAN plenary discussion, and  provided our initial results following simulation assumptions agreed in last meeting for IoT over NTN.

Scenario prioritization consideration

In RAN#91 meeting, several companies proposed to speed up the IoT NTN SI, and to start a follow-up work item in Rel-17. Based on the discussion summary[2], we present our views for some basic points as follows.

- Support of GEO/LEO?

Today, some GEO satellites are already in the orbit with stable and very wide coverage. Considering the very wide coverage and limited network capacity, it’s naturally suitable for IoT services.

LEOs could provide shorter latency and more resources, which could be a good supplement to extend the capacity of the NTN system. Therefore, we see both GEO and LEO should be considered for IoT NTN in Rel-17.

Proposal 1: Both GEO and LEO should be considered for IoT NTN in Rel-17.

- Prioritize LEO-600km or consider the other orbits for LEO?

For LEO deployment, considering that different operators may have different orbit resources, only support 600km altitude may limit the development of satellite industry for IoT NTN, and may cause the confliction/interference between different operators’ NTN networks if deployed in the same altitude. So we assume the orbits other than 600km may also need to be considered for LEO in Rel-17, e.g. 1200km.

Proposal 2: LEO-600km could be prioritized. However, the other orbits for LEO may also need to be considered for IoT NTN in Rel-17.

- Prioritize earth moving cell vs. earth fixed cell for LEO?

During the email discussion in [1], some companies provided their view to prioritize the earth moving cell scenario for LEO. To our understanding, LEO with earth moving cell is easier for LEO satellites. But in this case we should further investigate the potential mobility enhancement required to adapt the quick change of the coverage caused by the fast moving of the LEO satellites.

Even if we assume the UE is static, the serving cell of a UE may change in every few seconds. Thus, the solutions for idle mode and connected mode mobility used in LTE-NB/eMTC may need to be further enhanced or optimized to adapt such situation if we prioritize the earth moving cell for LEO in IoT NTN Rel-17.

Observation 1: If LEO with earth moving cell is prioritized, we should further consider the solutions for idle/connected mode mobility, to adapt the frequent change of the cell coverage caused by the movement of the LEO satellites.

Earth fixed beam is also a typical deployment, which could provide more stable coverage. In this case, the mobility solutions for NB-IoT/eMTC could be greatly reused without any further enhancement.  Above all, we see both earth moving cell and earth fixed cell should be considered for LEO in Rel-17, which allows the flexibility of network deployment.

Proposal 3: Both earth moving cell and earth fixed cell should be considered for LEO in Rel-17 to allow the flexibility of network deployment.

- Prioritize NB-IoT vs. eMTC?

For the priority between NB-IoT and eMTC, we don’t have strong preference on prioritisation between each other. To support different commercial requirements, it seems both of them should be covered in Rel-17.

Proposal 4: Both NB-IoT and eMTC should be supported in Rel-17 to support different commercial requirements.

Link budget

In this section, link budget under different scenarios are evaluated. According to agreed link budget parameters, we calculate the link budget for the scenarios following the agreements in last meeting. Set-1 and Set-2 satellite sets reuse the configurations in TR38.321 [3]. Set-3 and Set-4 satellite sets were agreed in the RAN #104 e-meeting. Results are shown in the following.

  - Link budget results for Set-1

Table 1 Link budget result for eMTC NTN with Set-1

| Satellite orbit | GEO |  |  |  |  |  |  |  | LEO1200 |  |  |  |  |  |  |  |  | LEO600 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B(KHZ) | DL |  | UL |  |  |  |  |  | DL |  | UL |  |  |  |  |  |  | DL |  | UL |  |  |  |  |  |  |
|  | 1080 |  | 360 |  | 180 | 90 | 45 | 30 | 1080 |  | 360 |  | 180 |  | 90 | 45 | 30 | 1080 |  | 360 |  | 180 |  | 90 | 45 | 30 |
| Frequency (GHz) | 2 |  | 2 |  |  |  |  |  | 2 |  | 2 |  |  |  |  |  |  | 2 |  | 2 |  |  |  |  |  |  |
| TX: EIRP [DL:dBW/MHz UL;dBW] | 59 |  | -10 |  |  |  |  |  | 40 |  | -10 |  |  |  |  |  |  | 34 |  | -10 |  |  |  |  |  |  |
| RX: G/T [dB/K] | -33.62 |  | 19 |  |  |  |  |  | -33.62 |  | 1.1 |  |  |  |  |  |  | -33.62 |  | 1.1 |  |  |  |  |  |  |
| Additional losses [dB] | 0 |  | 0 |  |  |  |  |  | 0 |  | 0 |  |  |  |  |  |  | 0 |  | 0 |  |  |  |  |  |  |
| Central beam edge elevation [degrees] & Free space path loss [dB] | Central beam edge elevation: 2.3 FSPL: 190.81 |  |  |  |  |  |  |  | Central beam edge elevation: 26.3 FSPL: 165.11 |  |  |  |  |  |  |  |  | Central beam edge elevation: 27.0 FSPL: 159.71 |  |  |  |  |  |  |  |  |
| CNR [dB] (with different bandwidth) | -5.2 | -17.1 |  | -14.1 |  | -11.1 | -8.1 | -6.3 | 1.5 | -9.2 |  | -6.2 |  | -3.2 |  | -0.2 | 1.5 | 0.9 | -3.8 |  | -0.8 |  | 2.1 |  | 5.1 | 6.9 |
| Central beam centre elevation [degrees] & Free space path loss [dB] | Central beam centre elevation: 12.5 FSPL: 190.58 |  |  |  |  |  |  |  | Central beam centre elevation: 30 FSPL: 164.49 |  |  |  |  |  |  |  |  | Central beam centre elevation: 30 FSPL: 159.10 |  |  |  |  |  |  |  |  |
| CNR [dB] (with different bandwidth) | -5.0 | -16.9 |  | -13.9 |  | -10.9 | -7.9 | -6.1 | 2.1 | -8.6 |  | -5.6 |  | -2.6 |  | 0.3 | 2.1 | 1.5 | -3.2 |  | -0.2 |  | 2.7 |  | 5.7 | 7.5 |

Table 2 Link budget result for NB-IoT NTN with Set-1

| Satellite orbit | GEO |  |  |  |  |  |  |  |  |  | LEO1200 |  |  |  |  |  |  | LEO600 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B(KHZ) | DL |  |  | UL |  |  |  |  |  |  | DL |  | UL |  |  |  |  | DL |  | UL |  |  |  |  |
|  | 180 |  |  | 180 |  | 90 |  | 45 | 15 | 3.75 | 180 |  | 180 | 90 | 45 | 15 | 3.75 | 180 |  | 180 | 90 | 45 | 15 | 3.75 |
| Frequency (GHz) | 2 |  |  | 2 |  |  |  |  |  |  | 2 |  | 2 |  |  |  |  | 2 |  | 2 |  |  |  |  |
| TX: EIRP [DL:dBW/MHz UL;dBW] | 59 |  |  | -10 |  |  |  |  |  |  | 40 |  | -10 |  |  |  |  | 34 |  | -10 |  |  |  |  |
| RX: G/T [dB/K] | -33.62 |  |  | 19 |  |  |  |  |  |  | -33.62 |  | 1.1 |  |  |  |  | -33.62 |  | 1.1 |  |  |  |  |
| Additional losses [dB] | 0 |  |  | 0 |  |  |  |  |  |  | 0 |  | 0 |  |  |  |  | 0 |  | 0 |  |  |  |  |
| Central beam edge elevation [degrees] & Free space path loss [dB] | Central beam edge elevation: 2.3 FSPL: 190.81 |  |  |  |  |  |  |  |  |  | Central beam edge elevation: 26.3 FSPL: 165.11 |  |  |  |  |  |  | Central beam edge elevation: 27.0 FSPL: 159.71 |  |  |  |  |  |  |
| CNR [dB] (with different bandwidth) | -5.2 |  | -14.1 |  | -11.1 |  | -8.1 |  | -3.3 | 2.6 | 1.5 | -6.2 |  | -3.2 | -0.2 | 4.5 | 10.5 | 0.9 | -0.8 |  | 2.1 | 5.1 | 9.9 | 15.9 |
| Central beam centre elevation [degrees] & Free space path loss [dB] | Central beam centre elevation: 12.5 FSPL: 190.58 |  |  |  |  |  |  |  |  |  | Central beam centre elevation: 30 FSPL: 164.49 |  |  |  |  |  |  | Central beam centre elevation: 30 FSPL: 159.10 |  |  |  |  |  |  |
| CNR [dB] (with different bandwidth) | -5.0 | -13.9 |  |  | -10.9 |  | -7.9 |  | -3.1 | 2.8 | 2.1 | -5.6 |  | -2.6 | 0.3 | 5.1 | 11.1 | 1.5 | -0.2 |  | 2.7 | 5.7 | 10.5 | 16.5 |

  - Link budget results for Set-2

Table 3 Link budget result for eMTC NTN with Set-2

| Satellite orbit | GEO |  |  |  |  |  |  |  |  | LEO1200 |  |  |  |  |  |  |  | LEO600 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B(KHZ) | DL |  | UL |  |  |  |  |  |  | DL |  | UL |  |  |  |  |  | DL |  | UL |  |  |  |  |  |  |
|  | 1080 |  | 360 |  | 180 |  | 90 | 45 | 30 | 1080 |  | 360 |  | 180 | 90 | 45 | 30 | 1080 |  | 360 |  | 180 |  | 90 | 45 | 30 |
| Frequency (GHz) | 2 |  | 2 |  |  |  |  |  |  | 2 |  | 2 |  |  |  |  |  | 2 |  | 2 |  |  |  |  |  |  |
| TX: EIRP [DL:dBW/MHz UL;dBW] | 53.5 |  | -10 |  |  |  |  |  |  | 34 |  | -10 |  |  |  |  |  | 28 |  | -10 |  |  |  |  |  |  |
| RX: G/T [dB/K] | -33.62 |  | 14 |  |  |  |  |  |  | -33.62 |  | -4.9 |  |  |  |  |  | -33.62 |  | -4.9 |  |  |  |  |  |  |
| Additional losses [dB] | 0 |  | 0 |  |  |  |  |  |  | 0 |  | 0 |  |  |  |  |  | 0 |  | 0 |  |  |  |  |  |  |
| Central beam edge elevation [degrees] & Free space path loss [dB] | Central beam edge elevation: 11 FSPL: 190.61 |  |  |  |  |  |  |  |  | Central beam edge elevation: 22.2 FSPL: 165.85 |  |  |  |  |  |  |  | Central beam edge elevation: 23.8 FSPL: 160.42 |  |  |  |  |  |  |  |  |
| CNR [dB] (with different bandwidth) | -10.5 | -21.9 |  | -18.9 |  | -15.9 |  | -12.9 | -11.1 | -5.1 | -16.0 |  | -13.0 |  | -9.9 | -6.9 | -5.2 | -5.7 | -10.5 |  | -7.5 |  | -4.5 |  | -1.5 | 0.2 |
| Central beam centre elevation [degrees] & Free space path loss [dB] | Central beam centre elevation: 20 FSPL: 190.41 |  |  |  |  |  |  |  |  | Central beam centre elevation: 30 FSPL: 164.49 |  |  |  |  |  |  |  | Central beam centre elevation: 30 FSPL: 159.10 |  |  |  |  |  |  |  |  |
| CNR [dB] (with different bandwidth) | -10.3 | -21.7 |  | -18.7 |  | -15.7 |  | -12.7 | -10.9 | -3.8 | -14.6 |  | -11.6 |  | -8.6 | -5.6 | -3.8 | -4.4 | -9.2 |  | -6.2 |  | -3.2 |  | -0.2 | 1.5 |

Table 4 Link budget result for NB-IoT NTN with Set-2

| Satellite orbit | GEO |  |  |  |  |  | LEO1200 |  |  |  |  |  | LEO600 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B(KHZ) | DL | UL |  |  |  |  | DL | UL |  |  |  |  | DL | UL |  |  |  |  |
|  | 180 | 180 | 90 | 45 | 15 | 3.75 | 180 | 180 | 90 | 45 | 15 | 3.75 | 180 | 180 | 90 | 45 | 15 | 3.75 |
| Frequency (GHz) | 2 | 2 |  |  |  |  | 2 | 2 |  |  |  |  | 2 | 2 |  |  |  |  |
| TX: EIRP [DL:dBW/MHz UL;dBW] | 53.5 | -10 |  |  |  |  | 34 | -10 |  |  |  |  | 28 | -10 |  |  |  |  |
| RX: G/T [dB/K] | -33.62 | 14 |  |  |  |  | -33.62 | -4.9 |  |  |  |  | -33.62 | -4.9 |  |  |  |  |
| Additional losses [dB] | 0 | 0 |  |  |  |  | 0 | 0 |  |  |  |  | 0 | 0 |  |  |  |  |
| Central beam edge elevation [degrees] & Free space path loss [dB] | Central beam edge elevation: 11 FSPL: 190.61 |  |  |  |  |  | Central beam edge elevation: 22.2 FSPL: 165.85 |  |  |  |  |  | Central beam edge elevation: 23.8 FSPL: 160.42 |  |  |  |  |  |
| CNR [dB] (with different bandwidth) | -10.5 | -18.9 | -15.9 | -12.9 | -8.1 | -2.1 | -5.1 | -13.0 | -9.9 | -6.9 | -2.2 | 3.8 | -5.7 | -7.5 | -4.5 | -1.5 | 3.2 | 9.2 |
| Central beam centre elevation [degrees] & Free space path loss [dB] | Central beam centre elevation: 20 FSPL: 190.41 |  |  |  |  |  | Central beam centre elevation: 30 FSPL: 164.49 |  |  |  |  |  | Central beam centre elevation: 30 FSPL: 159.10 |  |  |  |  |  |
| CNR [dB] (with different bandwidth) | -10.3 | -18.7 | -15.7 | -12.7 | -7.9 | -1.9 | -3.8 | -11.6 | -8.6 | -5.6 | -0.8 | 5.1 | -4.4 | -6.2 | -3.2 | -0.2 | 4.5 | 10.5 |

  - Link budget results for Set-3

Table 5 Link budget result for eMTC NTN with Set-3

| Satellite orbit | GEO |  |  |  |  |  |  |  | LEO1200 |  |  |  |  |  |  | LEO600 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B(KHZ) | DL | UL |  |  |  |  |  |  | DL |  | UL |  |  |  |  | DL | UL |  |  |  |  |
|  | 1080 | 360 | 180 |  | 90 |  | 45 | 30 | 1080 |  | 360 | 180 | 90 | 45 | 30 | 1080 | 360 | 180 | 90 | 45 | 30 |
| Frequency (GHz) | 2 | 2 |  |  |  |  |  |  | 2 |  | 2 |  |  |  |  | 2 | 2 |  |  |  |  |
| TX: EIRP [DL:dBW/MHz UL;dBW] | 59.8 | -10 |  |  |  |  |  |  | 33.7 |  | -10 |  |  |  |  | 28.3 | -10 |  |  |  |  |
| RX: G/T [dB/K] | -33.62 | 16.7 |  |  |  |  |  |  | -33.62 |  | -12.8 |  |  |  |  | -33.62 | -12.8 |  |  |  |  |
| Additional losses [dB] | 0 | 0 |  |  |  |  |  |  | 0 |  | 0 |  |  |  |  | 0 | 0 |  |  |  |  |
| Central beam edge elevation [degrees] & Free space path loss [dB] | Central beam edge elevation: 12.5 FSPL: 190.58 |  |  |  |  |  |  |  | Central beam edge elevation: 30 FSPL: 164.49 |  |  |  |  |  |  | Central beam edge elevation: 30 FSPL: 159.10 |  |  |  |  |  |
| CNR [dB] (with different bandwidth) | -4.2 | -19.2 |  | -16.2 |  | -13.2 | -10.2 | -8.4 | -4.1 |  | -22.5 | -19.5 | -16.5 | -13.5 | -11.7 | -4.1 | -17.1 | -14.1 | -11.1 | -8.1 | -6.3 |
| Central beam centre elevation [degrees] & Free space path loss [dB] | Central beam centre elevation: 20.9 FSPL: 190.39 |  |  |  |  |  |  |  | Central beam centre elevation: 46.05 FSPL: 162.33 |  |  |  |  |  |  | Central beam centre elevation: 43.78 FSPL: 156.85 |  |  |  |  |  |
| CNR [dB] (with different bandwidth) | -4.0 | -19.0 | -16.0 |  | -13.0 |  | -10.0 | -8.2 | -1.9 | -20.3 |  | -17.3 | -14.3 | -11.3 | -9.6 | -1.8 | -14.9 | -11.9 | -8.8 | -5.8 | -4.1 |

Table 6 Link budget result for NB-IoT NTN with Set-3

| Satellite orbit | GEO |  |  |  |  |  |  |  |  |  | LEO1200 |  |  |  |  |  |  |  |  |  | LEO600 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B(KHZ) | DL |  | UL |  |  |  |  |  |  |  | DL |  | UL |  |  |  |  |  |  |  | DL | UL |  |  |  |  |  |  |
|  | 180 |  | 180 |  | 90 |  | 45 | 15 | 3.75 |  | 180 |  | 180 |  | 90 | 45 | 15 |  | 3.75 |  | 180 | 180 |  | 90 | 45 | 15 | 3.75 |  |
| Frequency (GHz) | 2 |  | 2 |  |  |  |  |  |  |  | 2 |  | 2 |  |  |  |  |  |  |  | 2 | 2 |  |  |  |  |  |  |
| TX: EIRP [DL:dBW/MHz UL;dBW] | 59.8 |  | -10 |  |  |  |  |  |  |  | 33.7 |  | -10 |  |  |  |  |  |  |  | 28.3 | -10 |  |  |  |  |  |  |
| RX: G/T [dB/K] | -33.62 |  | 16.7 |  |  |  |  |  |  |  | -33.62 |  | -12.8 |  |  |  |  |  |  |  | -33.62 | -12.8 |  |  |  |  |  |  |
| Additional losses [dB] | 0 |  | 0 |  |  |  |  |  |  |  | 0 |  | 0 |  |  |  |  |  |  |  | 0 | 0 |  |  |  |  |  |  |
| Central beam edge elevation [degrees] & Free space path loss [dB] | Central beam edge elevation: 12.5 FSPL: 190.58 |  |  |  |  |  |  |  |  |  | Central beam edge elevation: 30 FSPL: 164.49 |  |  |  |  |  |  |  |  |  | Central beam edge elevation: 30 FSPL: 159.10 |  |  |  |  |  |  |  |
| CNR [dB] (with different bandwidth) | -4.2 | -16.2 |  | -13.2 |  | -10.2 |  | -5.4 |  | 0.5 | -4.1 | -19.5 |  | -16.5 |  | -13.5 |  | -8.7 |  | -2.7 | -4.1 |  | -14.1 | -11.1 | -8.1 | -3.3 |  | 2.6 |
| Central beam centre elevation [degrees] & Free space path loss [dB] | Central beam centre elevation: 20.9 FSPL: 190.39 |  |  |  |  |  |  |  |  |  | Central beam centre elevation: 46.05 FSPL: 162.33 |  |  |  |  |  |  |  |  |  | Central beam centre elevation: 43.78 FSPL: 156.85 |  |  |  |  |  |  |  |
| CNR [dB] (with different bandwidth) | -4.0 | -16.0 |  |  | -13.0 |  | -10.0 | -5.2 |  | 0.7 | -1.9 | -17.3 |  | -14.3 |  | -11.3 |  | -6.5 |  | -0.5 | -1.8 |  | -11.9 | -8.8 | -5.8 | -1.1 |  | 4.9 |

  - Link budget results for Set-4

Table 7 Link budget result with Set-4

| Satellite orbit | LEO600-eMTC |  |  |  |  |  | LEO600-NB-IoT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B(KHZ) | DL | UL |  |  |  |  | DL | UL |  |  |  |  |
|  | 1080 | 360 | 180 | 90 | 45 | 30 | 180 | 180 | 90 | 45 | 15 | 3.75 |
| Frequency (GHz) | 2 | 2 |  |  |  |  | 2 | 2 |  |  |  |  |
| TX: EIRP [DL:dBW/MHz UL;dBW] | 21.45 | -10 |  |  |  |  | 21.45 | -10 |  |  |  |  |
| RX: G/T [dB/K] | -33.62 | -18.6 |  |  |  |  | -33.62 | -18.6 |  |  |  |  |
| Additional losses [dB] | 0 | 0 |  |  |  |  | 0 | 0 |  |  |  |  |
| Central beam edge elevation [degrees] & Free space path loss [dB] | Central beam edge elevation: 30 FSPL: 159.10 |  |  |  |  |  | Central beam edge elevation: 30 FSPL: 159.10 |  |  |  |  |  |
| CNR [dB] (with different bandwidth) | -10.9 | -22.9 | -19.9 | -16.9 | -13.9 | -12.1 | -10.9 | -19.9 | -16.9 | -13.9 | -9.1 | -3.1 |
| Central beam centre elevation [degrees] & Free space path loss [dB] | Central beam centre elevation: 90 FSPL: 154.03 |  |  |  |  |  | Central beam centre elevation: 90 FSPL: 154.03 |  |  |  |  |  |
| CNR [dB] (with different bandwidth) | -5.9 | -17.9 | -14.8 | -11.8 | -8.8 | -7.1 | -5.9 | -14.8 | -11.8 | -8.8 | -4.0 | 1.9 |

Proposal 5：Capture Table 1-Table 7 results into TR 36.763.

Analysis for the results

Based on above link evaluation results, we can get some observations:

Observation 2：In eMTC system, the UL CNR’s difference in uplink bandwidth between 360khz and 30khz is about 10dB.

Observation 3：In NB-IoT system, the UL CNR’s difference in uplink bandwidth between 180khz and 3.75khz is about 15dB.

Observation 4：For Set-1, the worst UL CNR for the GEO system reaches -17dB, and the worst UL CNR for the LEO system is about -9dB.

Observation 5：For Set-2, the worst UL CNR for the GEO system reaches -22dB, and the worst UL CNR for the LEO system is about -16dB.

Observation 6：For Set-3, the worst UL CNR for the GEO system reaches -19dB, and the worst UL CNR for the LEO system is about -22dB.

Observation 7：For Set-4, the worst UL CNR for the LEO600-eMTC is close to -23dB, and the worst UL CNR for the LEO600-NB-IoT is close to -20dB.

Observation 8：For Set-4, the cell radius is 1700km, the UL CNR gap between the cell edge user and the center user is close to 5dB, and the UL CNR of the cell center user for the LEO600-eMTC is close to -7dB although the bandwidth decreases to 30khz.

Based on above link budget results, it can be found that when the uplink transmission bandwidth is relatively small, the corresponding UL CNR will be larger. Therefore, it is recommended to consider using a small bandwidth when the channel condition is poor.

In the evluated results, the CNR in some cases reached below -20dB. For the IoT NTN scenario, we need to further consider whether we need to support the case with -20 dB CNR.

In addition, Set-4 parameter considers the scenario with the large cell radius. Based on the results of the link budget, it can be observed that when the cell radius is large, reducing the bandwidth cannot perfectly solve the problem of poor coverage of edge users. Besides, based on the existing IoT coverage enhancement scheme, large number of repetitions should be used, but considering the Doppler shift in NTN, it is necessary to doubly check if repetition gain is relied. Therefore, it is necessary to further consider if we should support the case that the radius is large.

Proposal 6：Based on evaluated results, the use case with below -10dB is not recommanded to support in IoT over NTN.

Conclusion

In this contribution, we analzyed scenarion prioritization for NB-IoT and eMTC over satellite and provided the initial evaluation results for link budget.

Regarding the scenario priortization, observations and proposals are as follows:

Observation 1: If LEO with earth moving cell is prioritized, we should further consider the solutions for idle/connected mode mobility, to adapt the frequent change of the cell coverage caused by the movement of the LEO satellites.

Proposal 1: Both GEO and LEO should be considered for IoT NTN in Rel-17.

Proposal 2: LEO-600km could be prioritized. However, the other orbits for LEO may also need to be considered for IoT NTN in Rel-17.

Proposal 3: Both earth moving cell and earth fixed cell should be considered for LEO in Rel-17 to allow the flexibility of network deployment.

Proposal 4: Both NB-IoT and eMTC should be supported in Rel-17 to support different commercial requirements.

Regarding the evaluation result for link budget, observations and proposals are as follows:

Observation 2：In eMTC system, the UL CNR’s difference in uplink bandwidth between 360khz and 30khz is about 10dB.

Observation 3：In NB-IoT system, the UL CNR’s difference in uplink bandwidth between 180khz and 3.75khz is about 15dB.

Observation 4：For Set-1, the worst UL CNR for the GEO system reaches -17dB, and the worst UL CNR for the LEO system is about -9dB.

Observation 5：For Set-2, the worst UL CNR for the GEO system reaches -22dB, and the worst UL CNR for the LEO system is about -16dB.

Observation 6：For Set-3, the worst UL CNR for the GEO system reaches -19dB, and the worst UL CNR for the LEO system is about -22dB.

Observation 7：For Set-4, the worst UL CNR for the LEO600-eMTC is close to -23dB, and the worst UL CNR for the LEO600-NB-IoT is close to -20dB.

Observation 8：For Set-4, the cell radius is 1700km, the UL CNR gap between the cell edge user and the center user is close to 5dB, and the UL CNR of the cell center user for the LEO600-eMTC is close to -7dB although the bandwidth decreases to 30khz.

Proposal 5：Capture Table 1-Table 7 results into TR 36.763.

Proposal 6：Based on evaluated results, the use case with below -10dB is not recommanded to support in IoT over NTN.

References

- RAN1 #104e Chairman notes

- RP-210915 Moderator's summary for email discussion [91E][42][NTN_IoT_roadmap]

- TR 38.821, Solutions for NR to support non-terrestrial networks (NTN).