3GPP TSG RAN WG1 #104b-e                                             R1-2102659

e-Meeting, April 12th – 20th, 2021

Source:         ZTE

Title:            Maintenance of NR positioning support

Agenda item:    7.2.8

Document for:	Discussion and Decision

Introduction

In this contribution, we propose text changes for UE Rx-Tx time difference measurements per TRP in the current 38.214 [1].

TP on UE Rx-Tx time difference measurements per TRP

The following UE capabilities for Multi-RTT positioning method were defined in UE feature session [2],

| 13. NR Positioning | 13-11a | Association between SRS for positioning and DL PRS for Multi-RTT | - Support of measurements derived on one or more DL PRS resource/resource sets which may be in different positioning frequency layers for SRS transmitted in a single CC. Note: PRS and SRS may be in a different band |
| --- | --- | --- | --- |
| 13. NR Positioning | 13-11 | UE Rx-Tx Measurement Report for Multi-RTT | - Max number of UE Rx–Tx time difference measurements corresponding to a single SRS resource/resource set for positioning with each measurement corresponding to a single DL PRS resource/resource set. Value for component 1: {1,2,3,4} Note: DL PRS resource/sets are on the same frequency layer Note: the number of UE Rx – Tx time difference measurements refers to the measurements for a single TRP - Support RSRP measurements. Values = {0, 1} Note: If the UE reports value 1 for component 2, same number of RSRP measurements supported as UE Rx-Tx measurements for component 1 |

However, the agreed UE capabilities are not correctly captured in current specification 38.214 [2] at least in the following aspects.

- For FG13-11a, it should be UE’s capability to support measurements derived on one or more DL PRS resource/resource sets which may be in different positioning frequency layers for SRS transmitted in a single CC. However, the current specification in TS 38.214 doesn’t mention that UE should report this capability, and all the measurements should correspond to SRS transmitted in a single CC.

- For FG 13-11, the following aspects are not captured in the current specification, which may cause ambiguity to understand this FG.

- Different UE Rx–Tx time difference measurements are based on different DL PRS resources or DL PRS resource sets.

- Up to 4 UE Rx–Tx time difference measurements are based on DL PRS resources associated with the same TRP.

- Up to 4 UE Rx–Tx time difference measurements are based on DL PRS resources associated with the same positioning frequency layer.

Proposal 1: Adopt the following text changes in clause 5.1.6.5 of TS 38.214,

5.1.6.5	PRS reception procedure

==================================Unchanged parts omitted=============================

The UE may be configured to measure and report, subject to UE capability, up to 4 DL RSTD measurements per pair of dl-PRS-ID with each measurement between a different pair of DL PRS resources or DL PRS resource sets within the DL PRS configured for those dl-PRS-ID. The up to 4 measurements being performed on the same pair of dl-PRS-ID and all DL RSTD measurements in the same report use a single reference timing.

The UE may be configured to measure and report, subject to UE capability, up to 8 DL PRS-RSRP measurements on different DL PRS resources associated with the same dl-PRS-ID. When the UE reports DL PRS-RSRP measurements from one DL PRS resource set, the UE may indicate which DL PRS-RSRP measurements associated with the same higher layer parameter nr-DL-PRS-RxBeamIndex  [17, TS 37.355] have been performed using the same spatial domain filter for reception if for each nr-DL-PRS-RxBeamIndex reported there are at least 2 DL PRS-RSRP measurements associated with it within the DL PRS resource set.

The UE may be configured to measure and report, subject to UE capability, up to 4 UE Rx-Tx time difference measurements based on DL PRS resources associated with the same dl-PRS-ID and the same positioning frequency layer, and corresponding to a single configured SRS resource or resource set for positioning. Different measurements correspond to different received DL PRS resources or resource sets, which can be in different positioning frequency layers corresponding to SRS transmitted in a single carrier, subject to UE capability.

==================================Unchanged parts omitted=============================

Conclusion

In this contribution, we propose the following TP for UE Rx-Tx time difference measurements per TRP in the current 38.214.

Proposal 1: Adopt the following text changes in clause 5.1.6.5 of TS 38.214,

5.1.6.5	PRS reception procedure

==================================Unchanged parts omitted=============================

The UE may be configured to measure and report, subject to UE capability, up to 4 DL RSTD measurements per pair of dl-PRS-ID with each measurement between a different pair of DL PRS resources or DL PRS resource sets within the DL PRS configured for those dl-PRS-ID. The up to 4 measurements being performed on the same pair of dl-PRS-ID and all DL RSTD measurements in the same report use a single reference timing.

The UE may be configured to measure and report, subject to UE capability, up to 8 DL PRS-RSRP measurements on different DL PRS resources associated with the same dl-PRS-ID. When the UE reports DL PRS-RSRP measurements from one DL PRS resource set, the UE may indicate which DL PRS-RSRP measurements associated with the same higher layer parameter nr-DL-PRS-RxBeamIndex  [17, TS 37.355] have been performed using the same spatial domain filter for reception if for each nr-DL-PRS-RxBeamIndex reported there are at least 2 DL PRS-RSRP measurements associated with it within the DL PRS resource set.

The UE may be configured to measure and report, subject to UE capability, up to 4 UE Rx-Tx time difference measurements based on DL PRS resources associated with the same dl-PRS-ID and the same positioning frequency layer, and corresponding to a single configured SRS resource or resource set for positioning. Different measurements correspond to different received DL PRS resources or resource sets, which can be in different positioning frequency layers corresponding to SRS transmitted in a single carrier, subject to UE capability.

==================================Unchanged parts omitted=============================

References

- 3GPP TS 38.214 V16.5.0 (2021-03), Physical layer procedures for data (Release 16)

- R1-2102006 Updated RAN1 UE features list for Rel-16 NR after RAN1#104-e Moderators (AT&T, NTT DOCOMO, INC.)