3GPP TSG RAN WG1 #104b-e                                             R1-2102666

e-Meeting, April 12th – April 20th, 2021

Source:         ZTE

Title:            CSI enhancements for Multi-TRP and FR1 FDD reciprocity

Agenda item:    8.1.4

Document for:	Discussion and Decision

Introduction

In RAN#86, the Rel-17 WID for further enhancements on NR MIMO was approved, in which one potential enhancement is for CSI measurement and reporting as shown as follows

Enhancement on CSI measurement and reporting: - Evaluate and, if needed, specify CSI reporting for DL multi-TRP and/or multi-panel transmission to enable more dynamic channel/interference hypotheses for NCJT, targeting both FR1 and FR2 - Evaluate and, if needed, specify Type II port selection codebook enhancement (based on Rel.15/16 Type II port selection) where information related to angle(s) and delay(s) are estimated at the gNB based on SRS by utilizing DL/UL reciprocity of angle and delay, and the remaining DL CSI is reported by the UE, mainly targeting FDD FR1 to achieve better trade-off among UE complexity, performance and reporting overhead

After RAN1#104e, the basic framework has been established for both MTRP and FDD reciprocity based CSI. Based on the achieved agreement so far, we provide our views and simulation results for further details on these enhancement aspects in this contribution.

Multi-TRP CSI enhancement

During the WG1 meeting #104e, the following agreements were achieved by RAN1 for MTRP CSI [1]. The remaining details of MTRP CSI enhancements include aspects like the reuse of CMRs, CMR pair configuration, CPU and active resource occupation, etc.. In this section, we discuss and present our views for these aspects.

Agreement#1 For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, the UE can be configured with Ks ≥ 2 NZP CSI-RS resources in a CSI-RS resource set for CMR and N ≥ 1 NZP CSI-RS resource pairs whereas each pair is used for a NCJT measurement hypothesis - Configure UE with two CMR groups with Ks=K1+K2 CMRs. CMR pairs are determined from two CMR groups by following method(s). - K1 and K2 are the number of CMRs in two groups respectively. FFS K1=K2 or different K1/K2. - Note that CMRs in each CMR group can be used for both NCJT and Single-TRP measurement hypotheses - N CMR pairs are higher-layer configured by selecting from all possible pairs - signalling mechanism can be discussed further, e.g. using a bitmap - FFS: Whether MAC-CE or RRC+MAC CE indication is needed - FFS: how to support NCJT measurement hypotheses in FR2 - Support N=1 and Ks =2, FFS other maximal values of N>1 and Ks>2 - Note: for CPU/resource/port occupation, NCJT hypothesis is considered separately from single TRP hypothesis Agreement#2 - Strive to agree at most one of the following options, if needed - Option 1: Confirm the Working Assumption from RAN1 103e. - Option 2: The UE can be expected to report one RI, one PMI, one LI and one CQI per TRP, up to 2 TRPs, for Multi-DCI based NCJT - The time of decision is RAN1#105e (May 2021) Agreement#3 For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, support following two options: - Option 1: the UE can be configured to report X CSIs associated with single-TRP measurement hypotheses and one CSI associated with NCJT measurement hypothesis - X = 0, 1, 2 - If X=2, two CSIs are associated with two different single-TRP measurement hypotheses with CMRs from different CMR groups - Support of X=1,2 is UE optional for the UE supporting option 1 - FFS omission of CSI associated with NCJT measurement hypothesis - Option 2: the UE can be configured to report one CSI associated with the best one among NCJT and single-TRP measurement hypotheses - FFS how to report recommended measurement hypothesis associated with that CSI report

2.1 The reuse of CMRs in FR2

As shown in the above Agreement#1, UE can be configured with Ks ≥ 2 NZP CSI-RS resources in a CSI-RS resource set for CMR and N ≥ 1 NZP CSI-RS resource pairs, whereas each pair is used for a MTRP measurement hypothesis. CMRs in each CMR group can be used for both MTRP and Single-TRP measurement hypotheses. However, this agreement mainly focuses on FR1. For FR2, whether the CMRs used for a MTRP CMR pair can also be reused for another MTRP CMR pair or for STRP CSI led to intensive discussion. In summary, the two questions in observation 1 should be clarified for FR2.

Observation 1: Two questions should be clarified for MTRP CSI in FR2

- Question 1: whether a CMR configured for a MTRP CMR pairt can also be used for another MTRP CMR pair in FR2

- Question 2: whether a CMR configured for a MTRP CSI can also be used for a STRP CSI in FR2

Fig. 2-1 elaborates a typical scenario for MTRP CSI-RS resource configuration to make the questions clearer, where CSI-RS resources 1-4 are from TRP1, and CSI-RS resource 5-8 are from TRP2. Further, two receive beams are assumed at UE side.

Fig. 2-1 MTRP CSI-RS resource configuration

- Question 1: whether a CMR configured for a MTRP CMR pair can also be used for another MTRP CMR pair in FR2.

Question 1 can be exemplified as, whether CMR 1 can be configured in both CMR pair (1,5) and CMR pair (1,6). For CMR pair (1,5), since joint CSI measurement for RI, PMI and CQI should be implemented on CMR 1 and 5 together, UE needs to use corresponding receive beam r1 and r5 to receive both CMR1 and CMR 5 for inter-beam interference measurement. That is, for CMR1 measurement, UE needs to use both receive beam r1 and r5. For CMR5 measurement, UE also needs to use both receive beam r1 and r5.

Likewise, for CMR pair (1,6), UE needs to use corresponding receive beam r1 and r6 for receiving both CMR1 and CMR 6 for inter-beam interference measurement. That is, for CMR1 measurement, UE needs to use both receive beam r1 and r6. For CMR6 measurement, UE also needs to use both receive beam r1 and r6.

However, if both CMR pair (1,5) and (1,6) are configured to the UE, UE may not able to receive CMR 1 with all of receive beam r1, r5 and r6 since UE only has two panels. There are two options to address this issue.

- Option 1: a CMR configured for a MTRP CSI cannot be used for another MTRP CSI in FR2. That is, CMR1 is not allowed to be configured in two or more pairs.

- Option 2: if a CMR x paired with a CMR m, and also paired with another CMR n, CMR m and CMR n should be quasi co-located in terms of QCL-TypeD. In the example, CMR 5 and CMR 6 should have the same QCL-TypeD. Then, UE will be able to receive both CMR pair (1,5) and (1,6) via two receive beams.

- Question 2: whether a CMR configured for a MTRP CSI can also be used for a STRP CSI in FR2

Question 2 can be exemplified as, whether CMR 1 can be used for STRP CSI and also be configured in CMR pair (1,5) for MTRP CSI in FR2. In our view, for MTRP CSI, UE needs to use receive beam r1 and r5 corresponding to two panels respectively for receiving CMR1 and CMR5. The motivation is to perform inter-beam interference measurement. However, for STRP CSI, there is no inter-beam interference, UE can use receive beam r1 via both panels for receiving CMR1. Hence the UE implementation can be different on the same CMR for MTRP and STRP CSI measurement, which may cause issues for UE implementation or unintentional CSI from gNB perspective. To avoid these issues, we propose that a CMR configured for a MTRP CSI should not be used for a STRP CSI in FR2

Proposal 1: In FR2, one of the following two options should be adopted

- Option 1: a CMR configured for a MTRP CMR pair cannot be used for another MTRP CMR pair in FR2.

- Option 2: if a CMR x is paired with a CMR m, and also paired with another CMR n, CMR m and CMR n should be quasi co-located with QCL-TypeD.

Proposal 2: In FR2, a CMR configured for a MTRP CSI should not be used for a STRP CSI.

2.2 CMRs pair configuration

For CMR configurations as shown in above Agreement#1, CMR pairs are determined from two CMR groups, where K1 and K2 are the numbers of CMRs in two groups respectively. To select N CMR pairs from all possible pairs, bitmap signaling can be used in RRC. For example, a 6-bit bitmap can be used to select N CMR pairs as shown in Table 2-1, where group 0 includes CMR1 and CMR2, and group 1 includes CMR3, CMR4 and CMR5. One bit value in the table corresponds to one CMR pair.

Table 2-1 bitmap indication associated with MTRP

|  |  | Group 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | CMR3 | CMR4 | CMR5 |
| Group 0 | CMR1 | 1 | 1 | 0 |
|  | CMR2 | 0 | 1 | 1 |

If the bitmap is set/configured to “1” (e.g. CMR3 and CMR1), the corresponding CMR pair can be used for MTRP measurements. If the CMR is set/configured to “0” (e.g. CMR3 and CMR2), the CMR pair cannot be used for MTRP measurements. In this case, all resources or the resources not indicated for MTRP measurements in group 0 and group1 may be used for STRP measurements.

However, in some case, not all resources in group 0 and group1 always should be used for STRP measurements, especially when some transmit beams may only be suitable for MTRP transmission according to group based beam reporting. In such case, two bitmaps can be used for MTRP CSI indication and STRP CSI indication respectively. For example, bitmap in Table 2-1 is for MTRP. In addition, bitmap in Table 2-2 is introduced for STRP, only if the bit value in the bitmap is set/configured to “1”, the corresponding CMR can be used for STRP measurements.

Table 2-2 bitmap indication associated with STRP

| Group 0 |  | Group 1 |  |  |
| --- | --- | --- | --- | --- |
| CMR1 | CMR2 | CMR3 | CMR4 | CMR5 |
| 1 | 0 | 1 | 1 | 0 |

Proposal 3: Two bitmaps should be configured by RRC signaling for MTRP CMR pair selection and STRP CMR selection respectively.

2.3 CRI mapping

In Option 1 of Agreement#3, X = 0, 1, 2 best STRP CSI and 1 best MTRP CSI will be reported, where X should be configured by RRC signaling. Thus, UE needs to report X+1 CRIs where X CRIs are for STRP and 1 CRI is for MTRP. Each CRI bit size depends on the number of value 1 of configured bitmaps for either STRP or MTRP as described in our proposal 3.

Fig. 2-2 CMR configuration

For example, in Fig. 2-2, CMR 1-4 are from TRP0, CMR 5-8 are from TRP1. Two bitmaps configured by RRC signaling for MTRP and STRP are shown as Table 2-3 and Table 2-4 respectively. As a result, CRI for MTRP occupies 1 bit since only two pairs are selected by RRC signaling. CRI for TRP 0 occupies 2 bits since four CMRs are selected, and CRI for TRP 1 costs 1 bit.

Table 2-3 bitmap indication associated with MTRP

|  |  | Group 1 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | CMR5 | CMR6 | CMR7 | CMR8 |
| Group 0 | CMR1 | 1 | 0 | 0 | 0 |
|  | CMR2 | 0 | 1 | 0 | 0 |
|  | CMR3 | 0 | 0 | 0 | 0 |
|  | CMR4 | 0 | 0 | 0 | 0 |

Table 2-4 bitmap indication associated with STRP

| Group 0 |  |  |  | Group 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CMR1 | CMR2 | CMR3 | CMR4 | CMR5 | CMR6 | CMR7 | CMR8 |
| 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 |

For CSI report option 2, one best CSI is selected between STRP and MTRP CSI. Thus, UE only needs to report one CRI. The CRI bit size should depend on the total number of 1s in the configured bitmaps for both STRP and MTRP. For example, likewise in Table 2-3 and 2-4, 2 valid CMR pairs for MTRP, 6 valid CMRs for STRP, so the total number of valid CMR pairs for MTRP and CMRs for STRP is 8. Thus, CRI bit size should be 3.

Proposal 4: For CSI report option 1, X+1 CRI should be reported, where X CRIs are for STRP and 1 CRI is for MTRP CSI, each CRI bit size depends on the corresponding number of valid CMR pairs for MTRP or valid CMRs for STRP.

Proposal 5: For CSI report option 2, 1 CRI should be reported, where the CRI bit size depends on the total number of the valid CMR pairs for MTRP and valid CMRs for STRP.

2.4 CPU and active resource

In Rel-15 and Rel-16, the number of occupied CPUs OCPU = KsOCPU=KS for a CSI reporting where Ks Ks is the number of CMRs in the CSI-RS resource set.

For MTRP CSI calculation in Rel-17, it is straight-forward to follow the assumption that the number of CPUs depends on the number of valid CMR pairs for MTRP and the number of valid CMRs for STRP. Further, the number of occupied CPUs at UE side for each valid CMR pair should be more than 1, saying 2, since UE needs to perform more processing to calculate and feed back two sets of CSI for paired CMRs. Hence the total number of occupied CPUs can be OCPU=2N+T.

For calculation of active resources or ports, it is natural to follow Rel-15/16 rule. That is, if a CSI-RS resource appears twice no matter in the same or different report settings, no matter for MTRP or STRP, it should be calculated twice for counting active resources and active ports.

Proposal 6: The number of occupied CPU should be OCPU=2N+T, where N is the number of valid CMR pairs for MTRP and T is the number of valid CMRs for STRP.

Proposal 7: If a CMR appears twice no matter in the same or different report settings, for MTRP or STRP, it should be calculated twice to count active resources and active ports.

2.6 Mapping order of CSI field

In NR Rel-15 and Rel-16, the CSI content in a single CSI report is divided into part 1 and part 2 CSI when subband format is configured. Part 2 CSI is further divided into part 2 wideband and part 2 subband CSI. Part 1 CSI includes CRI, RI and CQI of the first transport block. Part 2 CSI includes LI, PMI and CQI of the second transport block when rank is more than 4.

In Rel-17, in a single CSI report, UE can be configured with X=0,1,2 best CSI for STRP and one best CSI for MTRP for CSI report option 1 as shown in above Agreement#3. In other words, UE needs to report X+1 CSI where each CSI still includes CRI, RI, LI, PMI and CQI. Straightforwardly, the legacy CSI partition rule can be reused in Rel-17. That is, each of X+1 CSI is divided into two parts as Rel-15/16, so UE needs to report X+1 CSI part 1 and X+1 CSI part 2 in a single CSI report.

Proposal 8: For CSI report option 1, each of X+1 CSI should be divided into CSI part 1 and CSI part 2 following Rel-15/16 mechanism.

Based on the above proposal, the mapping order of CSI field should be specified accordingly. For example, assuming that the UE is configured to report X=2 CSIs associated with STRP measurement hypotheses and one CSI associated with MTRP measurement hypothesis, the mapping order of legacy CSI fields for CSI part 1, wideband CSI part 2 and subband CSI part 2 can be naturally extended as given in Table 2-5, 2-6 and 2-7 respectively for a CSI report #n. It is noted that two RIs and PMIs are needed for the MTRP CSI report.

Table 2-5: Mapping order of CSI fields of one CSI report #n, CSI part 1

| For MTRP | CRI |
| --- | --- |
|  | RI |
|  | Wideband CQI for the first TB |
|  | Subband differential CQI for the first TB |
| For 1st TRP | CRI |
|  | RI |
|  | Wideband CQI for the first TB |
|  | Subband differential CQI for the first TB |
| For 2nd TRP | CRI |
|  | RI |
|  | Wideband CQI for the first TB |
|  | Subband differential CQI for the first TB |

Table 2-6: Mapping order of CSI fields of one CSI report #n, CSI part 2 wideband

| For MTRP | Wideband CQI for the second TB |
| --- | --- |
|  | LI |
|  | Wideband PMI fileds X1 |
|  | Wideband PMI fileds X2 |
| For 1st TRP | Wideband CQI for the second TB |
|  | LI |
|  | Wideband PMI fileds X1 |
|  | Wideband PMI fileds X2 |
| For 2nd TRP | Wideband CQI for the second TB |
|  | LI |
|  | Wideband PMI fileds X1 |
|  | Wideband PMI fileds X2 |

Table 2-7: Mapping order of CSI fields of one CSI report #n, CSI part 2 subband

| For MTRP | Subband differential CQI for the second TB of all even subbands |
| --- | --- |
|  | Subband PMI fields X2 of all even subbands |
|  | Subband differential CQI for the second TB of all odd subbands |
|  | Subband PMI fields X2 of all odd subbands |
| For 1st TRP | Subband differential CQI for the second TB of all even subbands |
|  | Subband PMI fields X2 of all even subbands |
|  | Subband differential CQI for the second TB of all odd subbands |
|  | Subband PMI fields X2 of all odd subbands |
| For 2nd TRP | Subband differential CQI for the second TB of all even subbands |
|  | Subband PMI fields X2 all even subbands |
|  | Subband differential CQI for the second TB of all odd subbands |
|  | Subband PMI fields X2 of all odd subbands |

2.6 CSI omission for CSI part 2

In Rel-15 and Rel-16, when CSI reporting on PUSCH or PUCCH comprises two parts, the UE may omit a portion of the Part 2 CSI. Omission of part 2 CSI is according to the priority order shown in Table 5.2.3-1 in 38.214. In short, for each CSI report, the priority order is part 2 wideband CSI with priority level 0, part 2 subband CSI of even subbands with priority level 1, and part 2 subband CSI of odd subbands with priority level 2, where smaller number for priority level has higher priority.

In our view, for Rel-17 MTRP CSI, a same priority rule can be reused as shown in Figure 2-3, where wideband CSI with priority level 0 includes all part 2 wideband CSI for X STRP and one MTRP, part 2 subband CSI of even subbands with priority level 1 also includes all part 2 subband CSI of even subbands for X STRP and one MTRP. Likewise, part 2 subband CSI of odd subbands has priority level 2.

Fig. 2-3 Priority levels for part2 CSI

Proposal 9: Reuse Rel-15/16 CSI omission rule for CSI part 2 as follows, where smaller number for priority level has higher priority.

- Priority level 0 includes all part 2 wideband CSI for X STRP and one MTRP;

- Priority level 1 includes all part 2 subband CSI of even subbands for X STRP and one MTRP;

- Priority level 2 includes all part 2 subband CSI of odd subbands for X STRP and one MTRP.

CSI enhancement based on FDD reciprocity

The following three agreements were made in RAN1#104e for CSI enhancements based on FDD partial reciprocity.

Agreement For PS codebook enhancements utilization DL/UL reciprocity of angle and/or delay, support codebook structure W=W1W2 WfH where - W1 is a free selection matrix, with identity matrix as special configuration - FFS polarization-common/specific selection - Wf is a DFT based compression matrix in which N3 = NCQISubband*R and Mv>=1 - At least one value of Mv>1 is supported - Decide on the value(s) of Mv, e.g. Mv=2, in RAN1# 104bis-e - Working assumption: Support of Mv>1 is a UE optional feature if the UE supports Rel-17 PS codebook enhancement, taking into account UE complexity related to codebook parameters - FFS candidate value(s) of R, mechanism for configuring/indicating to the UE and/or mechanism for selecting/reporting by UE for Wf - Wf can be turned off by gNB. When turned off, Wf is an all-one vector (FFS; the length of all-one vector) - FFS other signaling/CSI reporting mechanism for trade-off among signaling overhead, UE complexity and UPT gain Agreement For PS codebook enhancements utilization DL/UL reciprocity of angle and/or delay, - W1 ∈ N^{P×K1} (K1≤P) is a port selection matrix in order to freely select K1 ports out of P CSI-RS ports or K1/2 ports out of P /2 CSI-RS ports - Note that P is the number of CSI-RS ports for port selection (whose value depends on the outcome of the CSI-RS related study). Agreement For PS codebook enhancements utilization DL/UL reciprocity of angle and/or delay, study following options (or combinations) for CSI-RS configurations associated with Rel-17 PS codebook for supporting low CSI-RS overhead and/or CSI-RS processing complexity considering the impact on UPT performance under realistic CSI-RS measurement: - Option 0: No further CSI-RS enhancement as the baseline - Option 1: Support configuring a lower CSI-RS density per CSI-RS resource, e.g. 0.25 - Option 2: Support configuring one or multiple CSI-RS patterns per CSI-RS resource associated with Rel-17 PS codebook - Option 3: Support configuring multiple CSI-RS resources per CSI reporting configuration associated with Rel-17 PS codebook

In this section, we discuss more details about the design of CSI enhancement for FDD reciprocity.

3.1 General procedure

This item assumes partial reciprocity for FDD scenario. gNB can derive angular and delay for multiple paths and use it to assist CSI reporting. In general, the procedure of this FDD reciprocity based CSI reporting is depicted in Fig. 3-1.

Fig. 3-1 FDD reciprocity based CSI acquisition

Consider DL is operated in frequency A, and UL is operated in frequency B, three step are considered in this procedure.

- Step 1: gNB estimates angles and delays in frequency A by SRS in frequency B;

- Step 2: gNB uses the angles and delays to precode CSI-RS in frequency A;

- Step 3: UE measures the CSI-RS and report CSI (e.g., selected “ports” and coefficients) for frequency A.

Specifically, in Step 2, each CSI-RS port is precoded by one pair of SD basis and FD basis. For one CSI-RS tone of one CSI-RS port, the transmitted signal is v⋅fs, where v is an SD basis, f is an element of the FD basis, and s is one element in the CSI-RS sequence. In Step 3, for each CSI-RS port, i.e., each pair of SD basis and FD basis, what UE can observe is the beamformed channel by setting the delay to FD basis 0. UE can do average in frequency domain to decompress the noise, interference and residual delay vectors. Then UE can select the best ports and perform a wideband SVD to derive the coefficients.

3.2 Codebook structure and reporting details

In RAN1#104e, a Rel-16-analogous codebook structure W=W1W2 WfH is agreed. The remaining design issues include the details on W1, W2 and Wf, and further enhancements on CSI-RS overhead reduction.

Details on W1

It has been agreed that W1 performs port selection, where the selected ports are from all the CSI-RS resources associated with the CSI reporting. The following two options were proposed for the detailed structure of W1.

- Alt 1: Polarization-common W1

- Alt 2: Polarization-specific W1

Alt 1 is widely used in LTE and NR codebooks. It can save overhead without impacting performance too much, due to the relatively isomorphic character in terms of channel delay and angle for two polarizations.

- In angular domain, both Rel-15 Type II PS and Rel-16 eType II PS codebooks employs polarization common W1. These codebooks assume angular domain vectors, e.g., beams, are utilized to precode CSI-RS.

- In delay domain, previous study [2] on Rel-16 eType II codebook shows that using same frequency domain base vectors for two polarizations barely reduces performance.

Therefore, we support to adopt polarization-common W1 in Rel-17 PS codebook to reduce CSI overhead.

Proposal 10: Support polarization-common W1 in Rel-17 PS codebook.

Details on W2

The main issue for W2 design is the quantization for phases and amplitudes of non-zero coefficients. Since the Rel-17 PS codebook has the same structure as Rel-16 eType II and eType II PS codebooks, the W2 design for Rel-16 can be reused for Rel-17. That is, Rel-17 PS codebook can reuse the quantization approach adopted in Rel-16 for the non-zero coefficients, which includes the following aspects.

- Differential quantization for amplitudes per polarization: 4 bits for reference amplitude in the weaker polarization, 3 bits for differential amplitude.

- Direct quantization for phases: 4 bits for each phase.

In addition, to indicate the set of non-zero coefficients in W2, a bitmap can be used, which is also same as Rel-16.

Proposal 11: Reuse the Rel-16 non-zero coefficient quantization approach for the amplitudes and phases in W2 of Rel-17 PS codebook.

Details on Wf

The reason to include Wf in the Rel-17 codebook structure is to reduce the CSI-RS overhead. gNB can choose a subset of detected delays to precode CSI-RS, and UE can report the detected offset on the delays precoded in CSI-RS in Wf. Hence in theory there should not be many candidate values for DFT vectors in Wf, and most of the Wf vectors should be around DFT vector 0. However, due to issues like reciprocity impair, noise/interference and so on, gNB may not be able to acquire an accurate set of channel delays from measuring SRS. In this case, gNB needs the flexibility to include more report of DFT vectors in Wf, and gNB can rely more on UE reporting to get accurate CSI.

Based on the above reasoning, we think to let gNB configure candidate vectors in Wf is a good design. Specifically, gNB can configure a window including the candidate vectors in Wf, the number of vectors contained in this window can be determined by gNB flexibly. To parameterize such window, gNB can configure the window size and the first vector M_initial in this window as shown in Fig. 3-2. The configuration of M_initial can also accommodate the possibility of multiplexing more than 1 delays in one CSI-RS ports.

Fig. 3-2 An example of gNB configuring candidate vectors for Wf

UE can report the selected vectors in Wf from the window configured by gNB. The number of reported vectors can be determined by gNB’s configuration as well. For example, a fixed number like 2, 3, or a fraction of the window size.

On the number of selected Wf vectors (Mv) and the number of configured candidate vectors (N), it makes more sense to support N>Mv as we discussed above.

- Otherwise, if N=Mv, all the Wf vectors are configured by gNB, UE cannot report Wf vectors based on measuring DL channel. Then there is no need to introduce Wf as all the SD-FD pairs are given by gNB. It is equivalent to have more CSI-RS ports configured.

- Further considering rank>1 case, different layers may have different best Wf vectors, as FD basis vectors in Rel-16 is layer specific. To support N>Mv is better for higher ranks as different layers can report different Wf vectors in a common configured length-N window.

Further, to limit Mv<=2 is also restrictive from performance perspective. It’s better to support Mv>=3 and let gNB configure the detailed value of Mv.

Proposal 12:

- The set of N candidate vectors of Wf is a window configured by gNB, where both the window size and M_initial are configured.

- UE selects and reports Mv Wf vectors within the window configured by gNB.

- Support at least Mv >= 3 and N > Mv.

CSI-RS overhead reduction

One critical issue of the Rel-17 PS codebook is the large CSI-RS overhead caused by UE-specific CSI-RS. In RAN1#104e, a number of candidate solutions to reduce CSI-RS overhead is agreed to be investigated.

- Option 0: No further CSI-RS enhancement as the baseline

- Option 1: Support configuring a lower CSI-RS density per CSI-RS resource, e.g. 0.25

- Option 2: Support configuring one or multiple CSI-RS patterns per CSI-RS resource associated with Rel-17 PS codebook

- Option 3: Support configuring multiple CSI-RS resources per CSI reporting configuration associated with Rel-17 PS codebook

In the above options, Option 2 and Option 3 can achieve same performance and same flexibility for multiplexing multiple CSI-RS resources from gNB perspective. One example of Option 3 is shown in Fig. 3-3, where two CSI-RS resources with density 0.5 are configured. CSI-RS resource 1 is configured in even RBs, and CSI-RS resource 2 is configured in odd RBs. CSI-RS ports reported in W1 is selected from all the ports in the two CSI-RS resources.

Compared with Option 0,

- If the total number of ports to be selected are same, saying N, Option 2/3 can reduce CSI-RS overhead, since Option 2/3 only needs to configure two N/2-port CSI-RS resources with density 0.5, and Option 0 needs to configure an N-port CSI-RS resource.

- If the CSI-RS overhead is same, e.g., Option 0 configures an N-port resource with density 1, and Option 2/3 configures two N-port CSI-RS resources with density 0.5, Option 2/3 can use more angle-delay pairs to be selected.

Compared with Option 1, Option 2 or 3 achieves more flexibility for gNB to multiplex different UEs’ CSI-RS resources. In Option 1, one CSI-RS resource, i.e., all the angle-delay pairs, can only be put in one RB. This limits the flexibility for gNB to multiplex multiple CSI-RS resources in one slot. Option 2 or 3 can use more patterns in one slot to multiplex CSI-RS resources, so higher multiplex capacity can be achieved.

Fig. 3-3 An example of using multiple CSI-RS resources to reduce CSI-RS overhead for Rel-17 PS codebook

Between Option 2 and Option 3, we prefer Option 3 as it has smaller specification impact. Option 3 does not introduce any change from CSI-RS perspective. Further, multiple CSI-RS patterns are used to generate one CSI reporting in Option 2 and Option 3. Hence from UE processing perspective, such multiple patterns need to be counted as multiple active CSI-RS resources. Option 3 complies with such requirement for UE processing without defining any new rule, but Option 2 needs to define new rule to count active CSI-RS resources.

Observation 2: For the options on reducing CSI-RS overhead for Rel-17 PS codebook,

- Option 1 limits the flexibility for gNB to multiplex multiple CSI-RS resources in one slot compared with Option 2 or 3, and thus limits the multiplex capacity of CSI-RS.

- Option 3 has lower specification impact compared with Option 2.

We compare Option 3 and the baseline Option 0 in system level simulations. For N SD-FD pairs selected by gNB, the following detailed schemes with different R values are simulated.

- 1 resource, total N CSI-RS ports, density = 1: This is the baseline Option 0.

- 2 resources, total N CSI-RS ports, density = 0.5: This is Option 3.

To make a fair comparison among all these alternatives, we assume a same number of non-zero coefficients are selected for all of them.

Simulation results are given in Fig. 3-4.

- In each of the Rel-17 curve, N values include 16, 32, 48 and 64, which are similar as 2K0 in Rel-16.

- UE selects maximum N/2 non-zero coefficients per layer, and maximum N non-zero coefficients across all layers.

- The maximum rank is 4 per UE.

- FD vectors in CSI-RS precoding are 16x-oversampling DFT vectors, and SD vectors are CSI-RS precoding is 4x-4x-oversampling 2D-DFT vectors.

- UE-specific CSI-RS overhead is taken into account when calculating throughput. CSI-RS is assumed in a semi-persistent way. When data arrives for a UE, CSI-RS starts to be transmitted every 5 ms, and the CSI-RS transmission terminates when all the data transmission (including initial transmission and retransmission) of one UE and one packet completes.

- The other simulation assumptions are given in Table 1 of Appendix.

Fig. 3-4 Simulation results for different options on CSI-RS overhead reduction

The following observations can be drawn from the simulation results.

Observation 3: The saved overhead from Option 3 can bring significant performance gain compared with baseline.

Based on the above discussion, we have the following proposal.

Proposal 13: Support configuring multiple CSI-RS resources per CSI reporting configuration associated with Rel-17 PS codebook.

Conclusions

In this contribution, we provide our views to enhance CSI measurement and reporting for MTRP and FDD reciprocity.

For MTRP CSI enhancement:

Observation 1: Two questions should be clarified for MTRP CSI in FR2

- Question 1: whether a CMR configured for a MTRP CMR pairt can also be used for another MTRP CMR pair in FR2

- Question 2: whether a CMR configured for a MTRP CSI can also be used for a STRP CSI in FR2

Proposal 1: In FR2, one of the following two options should be adopted

- Option 1: a CMR configured for a MTRP CMR pair cannot be used for another MTRP CMR pair in FR2.

- Option 2: if a CMR x is paired with a CMR m, and also paired with another CMR n, CMR m and CMR n should be quasi co-located with QCL-TypeD.

Proposal 2: In FR2, a CMR configured for a MTRP CSI should not be used for a STRP CSI.

Proposal 3: Two bitmaps should be configured by RRC signaling for MTRP CMR pair selection and STRP CMR selection respectively.

Proposal 4: For CSI report option 1, X+1 CRI should be reported, where X CRIs are for STRP and 1 CRI is for MTRP CSI, each CRI bit size depends on the corresponding number of valid CMR pairs for MTRP or valid CMRs for STRP.

Proposal 5: For CSI report option 2, 1 CRI should be reported, where the CRI bit size depends on the total number of the valid CMR pairs for MTRP and valid CMRs for STRP.

Proposal 6: The number of occupied CPU should be OCPU=2N+T, where N is the number of valid CMR pairs for MTRP and T is the number of valid CMRs for STRP.

Proposal 7: If a CMR appears twice no matter in the same or different report settings, for MTRP or STRP, it should be calculated twice to count active resources and active ports.

Proposal 8: For CSI report option 1, each of X+1 CSI should be divided into CSI part 1 and CSI part 2 following Rel-15/16 mechanism. Based on the above proposal, the mapping order of CSI field should be specified accordingly.

Proposal 9: Reuse Rel-15/16 CSI omission rule for CSI part 2 as follows, where smaller number for priority level has higher priority.

- Priority level 0 includes all part 2 wideband CSI for X STRP and one MTRP;

- Priority level 1 includes all part 2 subband CSI of even subbands for X STRP and one MTRP;

- Priority level 2 includes all part 2 subband CSI of odd subbands for X STRP and one MTRP.

For CSI enhancement based on FDD reciprocity:

Proposal 10: Support polarization-common W1 in Rel-17 PS codebook.

Proposal 11: Reuse the Rel-16 non-zero coefficient quantization approach for the amplitudes and phases in W2 of Rel-17 PS codebook.

Proposal 12:

- The set of candidate vectors of Wf is a window configured by gNB, where both the window size and M_initial are configured.

- UE selects and reports the Wf vectors within the window configured by gNB.

Observation 2: For the options on reducing CSI-RS overhead for Rel-17 PS codebook,

- Option 1 limits the flexibility for gNB to multiplex multiple CSI-RS resources in one slot compared with Option 2 or 3, and thus limits the multiplex capacity of CSI-RS.

- Option 3 has lower specification impact compared with Option 2.

Observation 3: The saved overhead from Option 3 can bring significant performance gain compared with baseline.

Proposal 13: Support configuring multiple CSI-RS resources per CSI reporting configuration associated with Rel-17 PS codebook.

References

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #104-e, February 2020

- R1-1903343, CSI Enhancement for MU-MIMO Support, ZTE

Appendix

Table 1 SLS assumptions for CSI based on FDD reciprocity

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban |
| Frequency Range |  | 2GHz with duplexing gap of 200MHz between DL and UL, |
| Inter-BS distance |  | 200m |
| Channel model |  | The reciprocity model of DL/UL channel is based on Section 5.3 of TR 36.897 |
| Antenna setup and port layouts at gNB |  | (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| BS Tx power |  | 44 dBm for 20 MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15 kHz |
| Simulation bandwidth |  | 20 MHz (104 RBs) for 15 kHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | For 70% RU, SU/MU-MIMO with rank adaptation is assumed. For 20% RU, SU-MIMO with rank adaptation is assumed. |
| MIMO layers |  | The maximum number of MU layers is 12. The maximum number of SU layers is 4. |
| CSI feedback |  | CSI feedback periodicity: 5 ms, Scheduling delay: 4 ms |
| Overhead |  | 2 OFDM symbols for PDCCH，type 2 for DMRS(24 REs/PRB/slot) CSI-RS overhead is taken into account as specified in the main content |
| Traffic model |  | FTP 3 with packet size 0.5 Mbytes |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput and CSI feedback overhead |
| SRS modeling for UL channel estimation |  | 5ms SRS periodicity SRS error model in Table A.1-2 in 36.897 with Δ=9 dB. |
| FDD DL/UL calibration error model at gNB |  | With amplitude error (expressed in decibel of x=20log10a) and phase error are normal distribution with 0.7dB and 5 degrees standard deviation, respectively. |