3GPP TSG RAN WG1 #104b-e			R1-2102708

e-Meeting, April 12th – 20th, 2021

Agenda item:	8.11.1.1

Title:	Discussion on sidelink power saving

Source:	MediaTek Inc.

Document for:	Discussion and Decision

# Introduction

In RAN1 104-e [1], the following agreements were made for sidelink power saving:

Agreements: - Random resource selection is applicable to both periodic and aperiodic transmissions - FFS conditions for random resource selection Conclusion: - PSFCH reception is not included for Type A UE - S-SSB reception is not included for Type A UE - SL reception Type B is additionally added - Type B: Same as Type A with an exception of performing PSFCH and S-SSB reception - Note: the same conditions as in RAN1#103-e regarding the context of the discussion of Type A and Type D still apply (also applicable to type B) Agreements: In a resource pool (pre-)configured with at least partial sensing, if UE performs periodic-based partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool and resource selection/reselection is triggered at slot n, it is up to UE implementation to determine a set of Y candidate slots within a resource selection window, where - FFS condition(s) and timing(s) for which periodic-based partial sensing is performed by UE - The resource selection window is [n+T1, n+T2] - As a baseline, T1 and T2 are defined in the same way as in R16 NR-V2X according to step 1 [TS 38.214 Sec. 8.1.4] - Further discuss whether or not to introduce a threshold to re-define T1 and T2 such that - T1 ≥ 0 (subject to processing time constraint Tproc, 1), and T2 ≤ remaining PDB - T2-T1 ≤ (pre-)configured threshold - A minimum value for Y is (pre-)configured from a range of values, FFS details - FFS any restriction to determine Y candidate slots (including its relationship with SL-DRX) - FFS whether the resource selection window [n+T1, n+T2] should be confined within a set of periodic set of resources and its relationship with SL-DRX - Note: The terminology “periodic-based partial sensing” is based on the “partial sensing” used in LTE-V and it is intended to be used for the design and discussion of partial sensing in Rel-17. Agreements: In a resource pool (pre-)configured with at least partial sensing, if UE performs periodic-based partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool and resource selection/reselection is triggered at slot n, the UE monitors slots of at least one a set of periodic sensing occasions, where a periodic sensing occasion is a set of slots according to if tvSL is included in the set of Y candidate slots. - Preserve is a periodicity value from the configured set of possible resource reservation periods allowed in the resource pool (sl-ResourceReservePeriodList). Down select to one: - Option 1: Preserve corresponds to all values from the configured set sl-ResourceReservePeriodList - Option 2: Preserve Preserve corresponds to a subset of values from the configured set sl-ResourceReservePeriodList - FFS how to determine the subset (e.g., by (pre-)configuration, UE determination) - Option 3: Preserve Preserve is a common divisor among values in the configured set sl-ResourceReservePeriodList - Option 4: FFS others - k equals tois selected according to (down select to one) - Option 1: Only the most recent sensing occasion within sensing window for a given reservation periodicity before the resource (re)selection trigger or the set of Y candidate slots subject to processing time restriction - Option 2: The two most recent sensing occasions within sensing window for a given reservation periodicity before the resource (re)selection trigger or the set of Y candidate slots subject to processing time restriction - Option 3: All possible sensing occasions after n –T0 - Option 4: Only one periodic sensing occasion for one reservation period. The k value is up to UE implementation. Max value for k is (pre-)configured. - Option 5: k is (pre-)configured, including multiple values - Option 6: (pre-)configuration of a bitmap, same as in LTE-V - Option 7: FFS others - FFS relationship between periodic sensing occasions and SL-DRX - FFS condition(s) and timing(s) for which periodic-based partial sensing is performed by UE - Note: companies are encouraged to show performance data for the down selections Agreements: - In a resource pool (pre-)configured with at least partial sensing, if UE performs contiguous partial sensing and resource (re-)selection is triggered in slot n, support the following option: - Option 1: For the purpose of resource (re-)selection, the UE monitors slots between [n+TA, n+TB] and performs identification of candidate resources, in or after slot n+TB, based on all available sensing results, including periodic-based partial sensing results (if applicable). - FFS TA, TB (including the possibility of equal to zero, positive or negative) and remaining details (in particular, whether there should be exclusion of slots, changes in TA/TB values for different purposes, etc.) - FFS whether n can be replaced by e.g., index of some of Y candidate slots - FFS condition(s) in which contiguous partial sensing is performed by UE - FFS interaction with SL-DRX, if any - FFS interaction with periodic-based partial sensing, if any - Other options are not precluded - Note: This option is not to replace random resource selection only without sensing or re-evaluation and pre-emption checking

In this contribution we discuss the details of periodic and contiguous partial sensing. We also discuss random selection, resource pool sharing among UEs with different sensing capabilities, and SL-DRX aspects. We then talk about the need for wake-up signal for sidelink power saving.

# Discussion

Both periodic partial sensing and contiguous partial sensing were agreed in RAN1-104-e [1]. Periodic partial sensing is based on LTE-V partial sensing which is suitable for detecting periodic reservations in the pool when transmitting periodic traffic. But NR sidelink also supports aperiodic traffic in addition to periodic traffic. Hence, contiguous sensing can be used to detect aperiodic reservations in a resource pool when transmitting either periodic or aperiodic traffic. In Rel-16 NR, aperiodic transmissions are allowed in all sidelink resource pools. A UE performing resource selection for periodic transmission should be required to detect aperiodic reservations made by other UEs. In our view, to ensure system performance and reduce collisions, all UEs that performing periodic partial sensing should be expected to perform contiguous partial sensing to detect aperiodic reservations.

We propose the following:

Observation 1: Rel-16 NR allows aperiodic reservations in all sidelink resource pools.

Proposal 1: UEs performing periodic partial sensing are also expected to perform contiguous sensing simultaneously to detect aperiodic reservations made by other UEs in the same pool.

## Periodic partial sensing

UE selects a set of Y candidate slots from a resource selection window and the set of sensing slots are determined based on ty-k×PreserveSL for different values of k and Preserve, as illustrated in Figure 1. In LTE-V, Preserve is set to 100ms as the common denominator for periodic reservation values in {100ms, 200ms, ..., 1000ms} while reservation periodicities of 20ms and 50ms are omitted. The k parameter is pre-configured by network using a 10-bit bitmap.

Figure 1 Periodic partial sensing in NR sidelink

NR sidelink supports periodic reservations in the range of {1ms, 2ms, ..., 99ms} and {100ms, 200ms, ..., 1000ms}. Since there are more reservation periodicity values available in NR than LTE-V, it may be difficult to configure UE to perform partial sensing for all of the possible reservation periodicities in a resource pool. In addition, UEs performing contiguous sensing will be performing sensing on every slot continuously starting from ‘n+A’ to detect aperiodic reservations when transmitting periodic traffic. If ‘A’ is set to a negative value, some of the available reservation periodicities that are small (e.g., from 1ms, ..., 31ms) can be monitored via contiguous sensing. In that case, it may be unnecessary to include those periodicity values in periodic partial sensing configuration. For these reasons, it may be preferred to select a subset of available reservation periodicities for Preserve configuration in periodic partial sensing.

Observation 2: It may be unnecessary to configure Preserve with smaller reservation periodicity values (e.g., 1ms, ..., 31ms) as the corresponding slots may already be monitored via contiguous sensing.

The choice of this sub-selection for the configured Preserve values may have impact on two following aspects in sidelink performance.

- Firstly, including all reservation periodicities may lead to smaller power saving gains as UE will need to perform sensing on each corresponding slot. This is especially the case with small periodicity values below 100ms.

- Secondly, omitting some of the periodicity values from the partial sensing configuration may cause more frequent packet collisions with other UEs. Consider that another UE reserves periodic resources in the same resource pool with a specific periodicity. Omitting that periodicity from the partial sensing configuration would not enable the UE to detect the resource reservation. Such more frequent collisions would require more HARQ re-transmissions, leading to more power consumption and worse system performance.

Observation 3: Configuring Preserve with all possible reservation periodicities may lead to higher power consumption while omitting some of the reservation periodicities used by other UEs may cause more packet collisions.

Hence, determining a subset of Preserve values should not be left to UE decision. Network can configure partial sensing Preserve values according to the configured set available periodic reservation periodicities for all UEs in the same resource pool. Based on these points, we propose the following:

Proposal 2: Network configures UE with a list of Preserve values per resource pool as a subset of all possible reservation periodicities from {1ms, 2ms, ..., 99ms} and {100ms, 200ms, ..., 1000ms}.

In relation to each configured Preserve value, the corresponding k parameter needs to be configured by taking into account the power saving gain and packet collision probability. Although the parameter k is configured by a bitmap in LTE-V, following a similar approach may not be necessary in NR.

In LTE-V, the parameter Preserve is fixed to 100ms as a common denominator. To cover all potential reservation periodicity values from 100ms to 1000ms, all k occasions may need to be configured. However, if Preserve in NR sidelink should be configured for each Preserve value separately as a common denominator (i.e., greater than 1ms) does not exist. Hence, using a similar bitmap configuration is not a suitable approach in NR sidelink. Instead, similar configuration flexibility as in LTE-V should be targeted for NR sidelink. This could be achieved by configuring k to cover at least the most recent one or the most recent two sensing occasions prior to ‘n’ for each Preserve value. Performing sensing on additional occasions of k are up to UE implementation. We propose the following:

Proposal 3: UE is expected to take sensing measurement on at least one or two most recent ‘k’ occasions prior to ‘n’ for each configured Preserve value. Taking sensing measurement on additional ‘k’ occasions are up to UE implementation.

## Contiguous partial sensing

Contiguous partial sensing is agreed in the last RAN1 meeting based on performing sensing continuously from ‘n+A’ to ‘n+B’ where A and B can have negative or positive values [1]. Two separate use cases can be considered for contiguous partial sensing.

- Firstly, a power-limited UE transmitting periodic traffic can use contiguous sensing to detect aperiodic reservations made by other UEs for re-transmission resources in the same pool. In NR Rel-16 sidelink, re-transmission resources for aperiodic traffic can be reserved for the next 32 slots. For example, a UE that performs periodic partial sensing may monitor slots continuously starting from ‘tySL-32’. The contiguous sensing may continue until moment ‘n+T2-T3’ to facilitate re-evaluation and pre-emption, if enabled. See Figure 2.

- Secondly, a power-limited UE transmitting aperiodic traffic can use contiguous sensing to detect aperiodic reservations for re-transmission resources made by other UEs. Since an aperiodic packet transmission timing cannot be known beforehand, the contiguous sensing can only be triggered after ‘n’. See Figure 3.

Both uses cases above should be supported in NR sidelink for power-limited UEs. As the point A may be defined differently for both cases, the parameter A can be defined based on the value of  Prsvp_tx . For example, A can be set to ‘tySL-32’ when Prsvp_tx>0; and A can be set to ‘n+1’ when Prsvp_tx=0. On the other hand, the parameter B can be set to 'n-T2-T3' in both cases. In addition, it should be allowed to perform contiguous sensing on more slots, depending on UE implementation. We propose the following:

Proposal 4: In contiguous partial sensing:

- A is no later than 'tySL-32' when Prsvp_tx>0; or A is no later than 'n+1' when Prsvp_tx=0.

- B is no earlier than 'n+T2-T3'.

Figure 2 Contiguous partial sensing when Prsvp_tx>0

Figure 3 Contiguous partial sensing when Prsvp_tx=0

## Detecting periodic reservations when Prsvp_tx=0

Periodic partial sensing is based on identifying a set of Y candidate slots first, and then determining the set of sensing slots according to the selected set of candidates. As the packet transmission timing is known beforehand for periodic traffic, periodic partial sensing is applicable for the case when UE is transmitting periodic traffic.

Contiguous partial sensing is suitable to detect aperiodic reservations made by other UEs for re-transmission resources. Contiguous sensing can be used in both cases where UE transmits periodic traffic or aperiodic traffic. For aperiodic transmissions, UE can only start the sensing measurements after the resource (re)-selection is triggered at moment 'n' as UE cannot know the arrival time of its own aperiodic traffic.

In addition UE can also perform periodic and contiguous partial sensing operations simultaneously. However, detection of periodic reservations made by other UEs may be hard to realize when the UE itself is transmitting aperiodic traffic. This is mainly due to the fact that UE cannot know the transmission time of its aperiodic traffic, hence the sensing measurements for periodic partial sensing may not be fully aligned with the candidate resources from the selection window.

Figure 4 UE can perform periodic and contiguous sensing simultaneously w.r.t. different 'n1' and 'n2' moments

As illustrated in Figure 4, aperiodic traffic arrives and triggers resource (re)-selection at 'n2' and contiguous sensing at 'n2+1'. Let us also consider that a periodic traffic is expected to be transmitted after 'n1'. Since the set of Y candidate slots are selected according to the timeline set by 'n1', the corresponding periodic sensing occasions are also determined according to the moment 'n1' by means of selecting a set of Y candidate slots starting with tySL after n1. As these two resource selection trigger points n1and n2 are not necessarily aligned in time, a periodic reservation made by another UE in the pool may not be overlapping with the periodic sensing measurement occasions. As the time gap between 'n1' and 'n2' is random (i.e., not an integer multiple of any Preserve), the periodic sensing measurements taken with respect to 'n1' may not be enough to prevent collisions for UE’s own aperiodic transmission which occurs after n2.

To resolve such miss-detection problem with periodic reservations (when Prsvp_tx=0), we can consider predefined windowing that are placed periodically for aperiodic traffic transmission. As shown in Figure-5, UE defines a list of periodic window in time for aperiodic packet arrivals, for which resources can be selected from the subsequent set of Z candidate slots. Any aperiodic traffic that arrives at higher layers between [nk, nk+1] can be transmitted on a subset of Z candidate slots in Wk+1 For example, assume that an aperiodic packet-1 arrives at higher layer at point t1 in time, and another packet-2 arrives at t2. As these packets become available for transmission during the time internal [n2, n3], resource selection can be performed in W3=[n3+T1, n3+T2]. Another aperiodic packet-3 that becomes available for transmission at time t3 can be transmitted on resources from the subset of candidate slots from W4=[n4+T1, n4+T2].

Figure 5 Aperiodic traffic that arrives in [nk, nk+1] is transmitted in Wk+1 using a subset of Z candidate slots

In the following analysis, we study the probability of detecting a periodic reservation in a resource pool when a UE is transmitting aperiodic traffic. For simplicity, we assume that UE transmits one aperiodic TB in a resource pool shared by one more UE. This other UE has a single periodic resource reservation with a periodicity of 50ms, 100ms, 200ms, 500ms, or 1sec. We consider packet arrival with uniform distribution for simpler calculation. Analysis results are shown in Table I below. As the reservation periodicity is increased, the approach with predefined windowing achieves 100% detection as the sensing occasions are always aligned with the candidate slots from the selection window. The percentage of slots on which sensing is performed is lower when the other UE’s reservation periodicity (Prsvp_tx) and thus UE’s own sensing periodicity (Preserve) is increased from 50ms to 1sec. As for contiguous sensing, sensing is performed continuously during the interval n+A, n+B on 30+32=62 slots. Thus, UE can detect periodic reservations with Prsvp_tx=50ms. However, the detection probability is lower for greater Prsvp_tx values as UE cannot detect all periodic reservations. Next, we also analyze the case where UE performs periodic sensing along with contiguous sensing. As the periodic partial sensing occasions are determined according to the expected transmission time of another periodic traffic, the alignment between the transmission time of the aperiodic traffic and the periodic partial sensing occasions may not be integer multiples of Prsvp_tx. In those cases, detection probability cannot be improved. As we assume uniform packet arrival timing in our analysis, the time gap between the periodic and aperiodic traffic transmission times are also uniformly distributed. Hence, the detection probability can only be improved when the said time gap is an integer multiple of Prsvp_tx. Overall, the use of periodic partial sensing together with contiguous partial sensing is not sufficient to overcome the issue of periodic reservation miss-detection when UE is transmitting aperiodic traffic.

Table I   Analysis of detection probability for periodic reservation when UE is transmitting aperiodic traffic

| Periodic reservation by another UE | Predefined windowing N0 = 200ms Z = 20 slots [T1,T2] = [10, 30] | Contiguous sensing A = -32 slots B = 30 slots [T1,T2] = [10, 30] | Periodic sensing + Contiguous sensing A = -32 slots B = 30 slots Y = 20 slots [T1,T2] = [10, 30] |
| --- | --- | --- | --- |
| N = 1 Prsvp_tx = 50ms | Sensing slot rate: 40% Detection prob.: 100% | Sensing slot rate: 6.2% Detection prob.: 100% | Sensing slot rate: 46.2% Detection prob.: 100% |
| N = 1 Prsvp_tx = 100ms | Sensing slot rate: 20% Detection prob.: 100% | Sensing slot rate: 6.2% Detection prob.: 62% | Sensing slot rate: 26.2% Detection prob.: 62.4% |
| N = 1 Prsvp_tx = 200ms | Sensing slot rate: 10% Detection prob.: 100% | Sensing slot rate: 6.2% Detection prob.: 31% | Sensing slot rate: 16.2% Detection prob.: 31.3% |
| N = 1 Prsvp_tx = 500ms | Sensing slot rate: 4% Detection prob.: 100% | Sensing slot rate: 6.2% Detection prob.: 12.4% | Sensing slot rate: 10.2% Detection prob.: 12.6% |
| N = 1 Prsvp_tx = 1sec | Sensing slot rate: 2% Detection prob.: 100% | Sensing slot rate: 6.2% Detection prob.: 6.2% | Sensing slot rate: 8.2% Detection prob.: 6.3% |

As discussed above, simultaneous use of periodic and contiguous sensing is not enough to avoid the miss-detection problem with periodic reservations when UE is transmitting aperiodic traffic. Therefore, a better approach (as described in Figure 5) should be adopted to always align the periodic sensing occasions with the candidate set of slots intended for the transmission of aperiodic traffic. We propose the following.

Proposal 5: Adopt predefined windowing with sensing occasions to detect periodic reservations in the pool when UE is transmitting aperiodic traffic.

## Random selection

For the coexistence of UEs with different sensing capabilities including full sensing, partial sensing, and no sensing (random selection) in a shared resource pool, the resource reservation can be prioritized for random selection UEs. Since UEs operating in random selection may not be able to perform re-evaluation or pre-emption (e.g., Type-A or Type-B UEs), the other UEs in the pool that perform full or partial sensing should avoid packet collisions once a resource reservation from a random selection UE is detected.

We propose the following:

Proposal 6: UEs that perform random selection with no sensing capability should be assigned higher resource reservation priority to minimize collisions in resource pools shared by other UEs with different sensing capabilities.

## Resource pool sharing among UEs

In regards to Type-D UE’s configuration to use either partial sensing or random selection, if UE is pre-configured with only partial sensing or only random selection, UE should use the configured method only. However, when UE is pre-configured with both partial sensing and random selection for the same pool, two different approaches can be considered.

In one approach, when UE is pre-configured with both selection methods in a pool, it can be left to UE implementation to choose and apply one of these pre-configured methods. In another approach, UE can be pre-configured with a condition to automatically switch to one of these resource selection methods. For example, a condition can be defined as a threshold based on pool congestion, or a latency/reliability criterion to determine whether UE needs to switch to using partial sensing to mitigate its transmission collisions. Since the support for pool sharing between UEs performing random selection, partial sensing, and full sensing is already agreed in RAN1-103-e meeting, the second approach of conditional switching between different (re)-selection methods (e.g., random selection, partial sensing, full sensing) can be preferable as such dynamic approach can help mitigate collisions.

Proposal 7: UEs that are capable of performing sensing (i.e., Type-D UEs) can be pre-configured with a condition to dynamically switch between partial sensing and random resource selection. Such condition may depend on sidelink congestion, transmission packet reliability, etc.

## SL-DRX and Wake-up signalling

Baseline power saving solutions from Rel-14 LTE sidelink help minimize Tx-UE’s workload on sensing operation as both monitoring SCI for sensing measurements and UE’s sensing computations consume power. Rel-14 LTE sidelink focuses on power saving at TX-UE as the transmission of sidelink BSM messages from the pedestrian VRU UEs to vehicles was an important use case. Rel-17 NR sidelink supports more diverse use cases including VRU, public safety, and some commercial cases (e.g., smart home, smart factory, etc.) where power saving enhancements are needed over Rel-16 NR sidelink design. Power saving solutions should be introduced separately from both TX-UE’s perspective and RX-UE’s perspective.

Observation 4: Power saving solutions should be studied separately from TX-UE’s perspective and RX-UE’s perspective.

One of the objectives of Rel-17 WI on sidelink enhancements is the introduction of SL-DRX. The motivation for SL-DRX is to make UE fall into the deep sleep by switching off RF frontend for maximized power saving when there is no traffic for a while. Power consumption at RX-UE is dominated by SCI monitoring and by the decoding of the received packets. Rel-16 NR sidelink design requires UE to monitor every sidelink slot in a resource pool. SL-DRX helps RX-UE to enter into a sleep state during which no SCI monitoring may be needed. Going into longer sleep durations may firther help minimize power consumption especially at low traffic scenarios where frequent SCI monitoring is not needed. However, such long sleep states may also cause the loss of incoming packets. Packet losses would cause disruptions to both system reliability and increase sidelink congestion as the lost packets may need to be re-transmitted in the system. Therefore, further enhancements should be considered for power saving from RX-UE’s perspective to improve UE power effiency without impacting sidelink system reliability/latency.

To further reduce power consumption, the concept of wake-up signal in Uu may be considered in SL. For example, if a TX UE has packet to transmit, the TX UE sends a wake-up signal before the start of SL on duration of the RX UE. If the RX UE receives the wake-up signal, RX UE should monitor PSCCH during the SL on duration. Otherwise, RX UE can skip PSCCH monitoring during SL on duration of this SL DRX cycle.

Although wake-up signal requires additional signaling overhead and configuration, the power saving gain of the Rx UE may be significant when packet arrival rate is low or when on duration is relatively long comparing to SL DRX cycle. For example, consider a power-limited UE with high-reliability/low-latency traffic performing aperiodic packet reception. It would not be desirable to configure such a UE with a long DRX cycle and short on duration as system performance would be adversely impacted. On the other hand, configuring a short SL-DRX cycle with long SL-DRX-on-time duration would severely reduce power saving gains.

Proposal 8: Study wake-up signal in sidelink to enhance power saving from Rx-UE’s perspective.

# Conclusions

We have the following observations:

Observation 1: Rel-16 NR allows aperiodic reservations in all sidelink resource pools.

Observation 2: It may be unnecessary to configure Preserve with smaller reservation periodicity values (e.g., 1ms, ..., 31ms) as the corresponding slots may already be monitored via contiguous sensing.

Observation 3: Configuring Preserve with all possible reservation periodicities may lead to higher power consumption while omitting some of the reservation periodicities used by other UEs may cause more packet collisions.

Observation 4: Power saving solutions should be studied separately from TX-UE’s perspective and RX-UE’s perspective.

We have the following proposals:

Proposal 1: UEs performing periodic partial sensing are also expected to perform contiguous sensing simultaneously to detect aperiodic reservations made by other UEs in the same pool.

Proposal 2: Network configures UE with a list of Preserve values per resource pool as a subset of all possible reservation periodicities from {1ms, 2ms, ..., 99ms} and {100ms, 200ms, ..., 1000ms}.

Proposal 3: UE is expected to take sensing measurement on at least one or two most recent ‘k’ occasions prior to ‘n’ for each configured Preserve value. Taking sensing measurement on additional ‘k’ occasions are up to UE implementation.

Proposal 4: In contiguous partial sensing:

- A is no later than 'tySL-32' when Prsvp_tx>0; or A is no later than 'n+1' when Prsvp_tx=0.

- B is no earlier than 'n+T2-T3'.

Proposal 5: Adopt predefined windowing with sensing occasions to detect periodic reservations in the pool when UE is transmitting aperiodic traffic.

Proposal 6: UEs that perform random selection with no sensing capability should be assigned higher resource reservation priority to minimize collisions in resource pools shared by other UEs with different sensing capabilities.

Proposal 7: UEs that are capable of performing sensing (i.e., Type-D UEs) can be pre-configured with a condition to dynamically switch between partial sensing and random resource selection. Such condition may depend on sidelink congestion, transmission packet reliability, etc.

Proposal 8: Study wake-up signal in sidelink to enhance power saving from Rx-UE’s perspective.

# References

- Chairman’s notes, RAN1 104-e, February 2021.