3GPP TSG RAN WG1 #104b-e	R1-2102712

e-Meeting, April 12th – 20th, 2021

Agenda Item:	8.1.1

Source: 	Fujitsu

Title: 	Enhancements on Multi-beam Operation

Document for:	Discussion

Introduction

In RAN#86, further enhancements on MIMO for NR have been approved [1]. The objective includes:

- Enhancement on multi-beam operation, mainly targeting FR2 while also applicable to FR1:

  - Identify and specify features to facilitate more efficient (lower latency and overhead) DL/UL beam management to support higher intra- and L1/L2-centric inter-cell mobility and/or a larger number of configured TCI states:

    - Common beam for data and control transmission/reception for DL and UL, especially for intra-band CA

    - Unified TCI framework for DL and UL beam indication

    - Enhancement on signaling mechanisms for the above features to improve latency and efficiency with more usage of dynamic control signaling (as opposed to RRC)

  - Identify and specify features to facilitate UL beam selection for UEs equipped with multiple panels, considering UL coverage loss mitigation due to MPE, based on UL beam indication with the unified TCI framework for UL fast panel selection

In this contribution, we will discuss the enhancement on multi-beam operation including L1-based beam indication and UL multi-panel selection.

L1-based beam indication

In RAN1#104_e, the following agreements related to L1-based beam indication were reached [2].

Agreement On the beam application time for Rel.17 DCI-based beam indication, the beam application time can be configured by the gNB based on UE capability - Support a UE capability for the minimum value of beam application time - FFS: the exact minimum values of beam application time supported by UE - FFS: whether existing UE capability can be reused as this UE capability. - FFS: whether different beam application time values are supported for uplink and downlink - FFS: whether UE capability needs to be introduced for the maximum value of beam application time - FFS: the reference for defining the UE capability (e.g. from DCI reception or ACK transmission) - FFS: whether a UE is allowed to report more than 1 values in case of MPUE - FFS: the application time when DCI and applied channel(s) are on different CCs with same/different SCS(s)s Agreement On the Rel.17 DCI -based beam indication, in RAN1#104bis-e, down-select at least one of the following alternatives regarding the support of DCI format(s) for beam indication in addition to the agreed DCI formats 1_1/1_2 with DL assignment (in RAN1#103-e): - Alt0: No additional DCI format is supported - Alt1: DCI formats 1_1 and 1_2 without DL assignment, applicable for joint TCI as well as separate DL /UL TCI - Support DCI acknowledgment mechanism, e.g. based on SPS PDSCH release, based on triggered SRS, based on DCI indicating SCell dormancy - FFS: How to identify DCI formats 1_1/1_2 used for beam indication only (not for scheduling a PDSCH reception, not indicating a SPS PDSCH release, or not indicating SCell dormancy), considering impacts on PDCCH coverage and scheduling mechanism - FFS: Whether the UE can/shall assume the gNB configured application time is after ACK transmission - Alt2: Dedicated DCI format other than 1_1/1_2 without DL assignment, applicable for joint TCI as well as separate DL /UL TCI - Support DCI acknowledgment mechanism, e.g. based on SPS PDSCH release, based on triggered SRS, based on DCI indicating SCell dormancy - FFS: If the format is based on an existing DCI format, how to identify the DCI format used for beam indication only - FFS: Whether the UE can/shall assume the gNB configured application time is after ACK transmission - Alt3: UL-related DCI formats 0_1/0_2 with UL grant, applicable only for UL-only TCI of separate DL /UL TCI Agreement On Rel.17 DCI-based beam indication, regarding application time of the beam indication: if beam indication is successfully received and the newly indicated beam in the beam indication is different from the previously indicated beam, down-select (no later than RAN1#105-e) one from the following. No other alternatives will be considered: - Alt1: the first slot that is at least X ms or Y symbols after the [first/last] symbol of the DCI with the joint or separate DL/UL beam indication - Alt2A: the first slot that is at least X ms or Y symbols after the [first/last] symbol of the acknowledgment of the joint or separate DL/UL beam indication - Alt 2B: the first slot that is at least X ms or Y symbols after the [first/last] symbol of the acknowledgment of the joint or separate DL/UL beam indication, except that the (new) TCI state update can be applied to the PDSCH, if it exists, (scheduled by the beam indication DCI) and corresponding ACK transmission (provided that the time offset between the DCI and the scheduled PDSCH exceed the threshold, analogous to Rel.15/16) - Alt2C: Support both Alt1 and Alt2A, and introduce a UE capability that indicates the support of Alt1 or Alt2A - Alt3: the first slot that is at least X1 ms or Y1 symbols after the [first/last] symbol of the DCI with beam indication and X2 ms or Y2 symbols after the [first/last] symbol of the acknowledgment of the beam indication - FFS: whether any existing timing defined for DCI based TCI/spatial relation update can be used for X/Y

Beam application time

It was agreed in RAN1#103_e that a DCI format 1_1/1_2 can be used to support joint or separate DL/UL beam indication. For the purpose of robustness, the ACK/NACK of the PDSCH scheduled by the DCI can be used as an acknowledgement to the beam indication. A leftover issue is to determine when to apply the beam indication.

In RAN1#104_e, 6 options are provided for the application time of the beam indication. Among them, Alt2A is preferred, which is the first slot that is at least X ms or Y symbols after the [first/last] symbol of the acknowledgment of the joint or separate DL/UL beam indication. This is because Alt2A is consistent with the design principle of a beam indication that corresponding to an acknowledgement in Rel-15 (e.g. MAC-CE based beam indication).

Proposal 1: For the down-selection of beam application time, the Alt2A is preferred:

- if beam indication is received, the beam indication is applied from the first slot that is at least X ms or Y symbols after the [first/last] symbol of the acknowledgment of the joint or separate DL/UL beam indication.

DCI formats for beam indication

Another leftover issue is whether to support for L1-beam indication in addition to the agreed DCI formats 1_1/1_2 with DL assignment. In our view, this is unnecessary. Several alternatives are proposed including the introduction of a DL-related DCI formats without DL assignment or a UL-related DCI format with UL grant for the beam indication. The proposed methods are aiming for the case that there is no DL assignment available. However, to our understanding, this may be a corner case and the corresponding issue can be solved by Rel-15/16 beam indication mechanism. Hence, we have the the following proposal.

Proposal 2: For the down-selection of alternatives regarding the support of DCI format(s) for beam indication in addition to the agreed DCI formats 1_1/1_2 with DL assignment (in RAN1#103-e), Alt0 is preferred,

- Alt0: No additional DCI format is supported.

Conclusions

In this contribution, we discussed the enhancements on multi-beam operation and the corresponding proposals are listed as follows.

Proposal 1: For the down-selection of beam application time, the Alt2A is preferred:

- if beam indication is received, the beam indication is applied from the first slot that is at least X ms or Y symbols after the [first/last] symbol of the acknowledgment of the joint or separate DL/UL beam indication.

Proposal 2: For the down-selection of alternatives regarding the support of DCI format(s) for beam indication in addition to the agreed DCI formats 1_1/1_2 with DL assignment (in RAN1#103-e), Alt0 is preferred,

- Alt0: No additional DCI format is supported.

- References

[1] RP-193133, “Further enhancements on MIMO for NR”, December 2019.

[2] 3GPP RAN1#104_e Chairman’s Notes, February 2021.