3GPP TSG RAN WG1 #104b-e		                                    R1-2102739

e-Meeting, April 12th – 20th, 2021

Agenda Item:	8.3.1.2

Source:	InterDigital Inc.

Title:	CSI feedback enhancements

Document for:	Discussion and Decision

# Introduction

The work item on Enhanced IIoT and URLLC support for NR [1] has an objective on studying, identifying and specifying CSI feedback enhancements to allow for more accurate MCS selection. In RAN1#104-e, a detailed set of Case 1 and Case 2 schemes was identified for continued evaluation ([5], Appendix B) and additional discussions took place after RAN1#104-e to better understand each scheme and associated aspects such as implementation complexity, specification impact and testability [6].

This contribution provides an analysis and recommendation for each Case 1 and Case 2 scheme identified in Appendix B of [5] as well as performance evaluation for some of the schemes.

# Evaluation results

In this section, evaluation results are presented for a set of Case 1 and Case 2 schemes and report periodicities of 2 ms or 20 ms. Compared to the results presented in [7], the following assumptions were modified. First, the OLLA ACK step size (when enabled) is decreased to 0.0005 dB. This change improved the baseline results. Second, the minimum RB allocation was changed from 2 RBs to 1 RB.

## Case 1 schemes

Scheme-specific assumptions for Case 1 are summarized in following Table:

Table 1. Simulation assumptions for evaluated Case 1 schemes

| Scheme | CSI report quantity | Scheduling |
| --- | --- | --- |
| Baseline | 2-bits differential subband CQI and subband PMI | In best reported subband |
| Case 1-1 | Mean and standard deviation of subband SINR | In random subband gNB schedules assuming a worst-case SINR (SINRwc) SINRwc=SINRmean- K x SINRstd K=4.1 x sqrt(subband_size/allocation_size) |
| Case 1-5 | 2-bits differential subband CQI and subband PMI Measurement taken on worst IMR since last report IMR periodicity = 2 ms | In best reported subband |
| Case 1-6 | Worst-M CQI (M=1) | In random subband |
| Case 1-8 | 3-bits differential subband CQI and subband PMI 4-bits full CQI and subband PMI | In best reported subband |
| Case 1-9 | 2-bits or 3 bits differential subband CQI and subband PMI WB-CQI used as reference excludes 5 worst subbands | In best reported subband |
| Case 1-11 | Full CSI every 20 ms 2-bits differential subband CQI every 2 ms (only interference is updated) IMR periodicity = 2 ms | In best reported subband |

The percentage of satisfied UEs and the PDSCH resource utilization for selected Case 1 schemes are shown in Table 2 and Table 3 respectively (results for Factory automation are not yet available). The simulations were run with OLLA disabled or enabled, to better isolate the effect of the different functionalities. Distributions of MCS errors are available in Appendix.

Table 2. % of satisfied UEs for Case 1 schemes

| Scheme/Scenario | Report periodicity (ms) | Rel-15 enabled AR/VR (OLLA OFF) | Rel-15 enabled AR/VR (OLLA ON) |
| --- | --- | --- | --- |
| Baseline | 2 | 98.6 | 100 |
| Baseline | 20 | 91.4 | 99.5 |
| Case 1-1 | 2 | 100 | 100 |
| Case 1-5 | 20 | 85.7 | 99.5 |
| Case 1-6 | 2 | 98.6 | 100 |
| Case 1-8 (3-bits) | 2 | 97.1 | 100 |
| Case 1-8 (4-bits) | 2 | 97.1 | 100 |
| Case 1-9 | 2 | 97.1 | 100 |
| Case 1-8 (3-bits) +Case 1-9 | 2 | 97.1 | 100 |
| Case 1-11 Int. update only | 2 (CQI) 20 (Full) | 90.5 | 99.0 |

Table 3. Average PDSCH resource usage for Case 1 schemes

| Scheme/Scenario | Report periodicity (ms) | Rel-15 enabled AR/VR (OLLA OFF) | Rel-15 enabled AR/VR (OLLA ON) |
| --- | --- | --- | --- |
| Baseline | 2 | 6.16 | 6.47 |
| Baseline | 20 | 6.23 | 6.59 |
| Case 1-1 | 2 | 6.64 | 6.43 |
| Case 1-5 | 20 | 6.28 | 6.65 |
| Case 1-6 | 2 | 6.43 | 6.78 |
| Case 1-8 (3-bits) | 2 | 6.16 | 6.48 |
| Case 1-8 (4-bits) | 2 | 6.16 | 6.48 |
| Case 1-9 | 2 | 6.16 | 6.48 |
| Case 1-8 (3-bits) +Case 1-9 | 2 | 6.16 | 6.48 |
| Case 1-11 Int. update only | 2 (CQI) 20 (Full) | 6.22 | 6.60 |

It can be seen that the “baseline” scheme of reporting sub-band PMI/CQI already has good performance in the R15-enabled scenario, even with no OLLA.

Observation 1: For R15-enabled scenario, reporting sub-band PMI/CQI with low periodicity (2 ms) can achieve 100% UE satisfaction.

However, the cost of this baseline reporting in terms of uplink overhead is arguably quite high. Schemes that can achieve similar performance but with reduced uplink overhead (and/or reduced PDSCH resource utilization) would still prove beneficial. Case 1-1 and 1-6 appear to meet this requirement.

Observation 2: For R15-enabled scenario, providing statistical or worst-case information (Case 1-1 or Case 1-6) still achieves 100% UE satisfaction and similar PDSCH resource utilization with reduced overhead .

On the other hand, Case 1-8/Case1-9 do not seem to improve performance while having even higher overhead than the baseline. This could be due to the fact that the most dominant source of MCS error is the difference in interference level between IMR and actual PDSCH transmission, and not the finite granularity of the subband reporting.

Observation 3: For R15-enabled scenario, increasing granularity of subband reporting (Case 1-8) increases overhead without gain in PDSCH resource utilization.

## Case 2 schemes

Scheme-specific assumptions for Case 2 are summarized in following Table:

Table 4. Simulation assumptions for evaluated Case 2 schemes

| Scheme | Report quantity | Scheduling |
| --- | --- | --- |
| Baseline | CSI as in baseline for Case 1 schemes No PDSCH-based report | OLLA with ACK step size of 0.0005 dB |
| Case 2-1 (LDPC iters) | Binary indication of whether or not more than 5 LDPC iterations are required | Apply NACK step in case of indication of LDPC iterations above 5 |
| Case 2-1 (dB margin) | Binary indication of whether SINR is within 0.5 dB margin of SINR meeting BLER target for the MCS. Accumulated and reported every N PDSCH | Apply NACK step in case indication that SINR is below margin |
| Case 2-2 | BLEP exponent | Apply NACK step in case BLEP exponent is above BLER target |
| Case 2-3 (NACK-only) | In case of NACK, report MCS that would achieve BLER target for retransmission | Schedule retransmission using reported MCS |

The percentage of satisfied UEs and the PDSCH resource utilization for selected Case 2 schemes are shown in Table 5 and Table 6 respectively (results for Factory automation are not yet available).

Table 5. % of satisfied UEs

| Scheme/Scenario | Report periodicity (ms) | Rel-15 enabled AR/VR (OLLA ON) |
| --- | --- | --- |
| Baseline | 2 | 100 |
| Baseline | 20 | 99.5 |
| Case 2-1 (LDPC iters) | 20 | 99.5 |
| Case 2-1 (slow) | 20 | 100 |
| Case 2-2 | 20 | 100 |
| Case 2-3 (NACK) | 20 | 99.5 |

Table 6. Average PDSCH resource usage

| Scheme/Scenario | Report periodicity (ms) | Rel-15 enabled AR/VR (OLLA ON) |
| --- | --- | --- |
| Baseline | 2 | 6.47 |
| Baseline | 20 | 6.59 |
| Case 2-1 (LDPC iters) | 20 | 6.59 |
| Case 2-1 (slow) | 20 | 6.75 |
| Case 2-2 | 20 | 6.80 |
| Case 2-3 (NACK) | 20 | 6.59 |

It can be observed that at least for the R15-enabled scenario, none of the enhancements overperform the baseline OLLA algorithm (now using a 0.0005 dB step size).

Observation 4: For R15-enabled scenario, none of the proposed Case 2 schemes improve resource utilization.

# Analysis

The following Tables summarize Pros/Cons on the proposed Case 1 and Case 2 schemes, taking into account the discussions and available evaluation results so far.

## Case 1 schemes

| Scheme | Pros | Cons |
| --- | --- | --- |
| Case 1-1 Statistical CSI (CQI) | +Enables less frequent reporting (reduces UE computation burden and overhead) +Reduces overhead compared to subband reporting +Avoids issue of subband report not providing worst-case CQI (due to quantization) +Some evaluation results show gain | -Statistics (e.g. mean, stddev) may not accurately reflect true CQI distribution. -Medium impact on implementation/specification/testing |
| Case 1-1 Statistical CSI (SINR) | +May enable more accurate scheduling if TBS/BLER target is different than what is assumed by UE +In this evaluation, some performance benefits shown over baseline, on top of reduced overhead. | -Sensitive to UE performance variability for given SINR -May increase testability effort |
| Case 1-2 CSI prediction | +UE knows interference better than gNB | -UE does not know when gNB schedules -Unclear if it would work for bursty interference -No benefits shown -Unclear how to specify or test |
| Case 1-3 Interference statistics | +May help scheduler determine worst-case interference level | -Inefficient way of getting worst-case CQI information – need both average-level and variance of interference, on top of channel part -No benefits shown -High impact on specification and testing |
| Case 1-4 Interference covariance matrix | +Scheduler can select MCS matching SINR with MMSE-IRC receiver | -MU operation not typical of URLLC -Not appropriate for sparse traffic -Not appropriate if more than one single dominant interferer -Large overhead -Large implementation and specification impact -No benefits shown in agreed scenarios |
| Case 1-5 CSI based on worst IMR occasion | +Enables less frequent reporting (reduces UE computation burden and overhead) +Low implementation, specification, testing impact +Some evaluation results show gain | -In this evaluation, no gain is found so far. |
| Case 1-6 Worst-M CQI | +Scheduler gets worst-case CQI level +Low implementation, specification, testing impact +In this evaluation, some performance benefits shown over baseline (2 ms) |  |
| Case 1-7 Worst-best criteria for subband CQI | +Scheduler gets worst-case CQI level +Low implementation, specification, testing impact | -Unclear why subband-specific CRI needs to be reported, unless for multi-TRP case |
| Case 1-8 3-bits differential subband CQI or 4-bit full subband CQI | +More accurate subband information compared to R16 +Low implementation, specification, testing impact | -Increased overhead compared to R16 -Does not address problem of bursty interference -In this evaluation, no system-level gain is found so far (satisfied UEs or RU) |
| Case 1-9 Reference wideband CQI excludes worst subbands | +Report WB-CQI that better represent CSI for subbands to be allocated to the UE. +Low implementation, specification, testing impact | -Does not address problem of bursty interference -In this evaluation, no system-level gain is found so far (satisfied UEs or RU) |
| Case 1-10 CSI expiration time | +May help gNB determine proper P-CSI periodicity | -Unclear how this is motivated by URLLC. Existing network implementation seem fine without it. -No benefits shown. -Unclear testability and specificiability |
| Case 1-11 Partial information update | +Reduces CSI processing time and overhead +Some evaluations show gain compared to sparser periodic CSI | -In this evaluation, no gain compared to 20 ms periodic CSI only so far (for interference-only updates) |

Based on the above analysis, the following Case 1 schemes: Case 1-1 (Statistical CSI), Case 1-5 (CSI based on worst IMR occasion), Case 1-6 (Worst-M CQI), Case 1-11 (Partial CQI update) appear the most promising for providing improvement in performance, overhead and/or UE complexity in URLLC scenario. In addition, the implementation/specification complexity of most of these schemes is very reasonable. Case 1-1 may have somewhat higher complexity compared to other schemes.

On the other hand, remaining Case 1 schemes appear to fall short in one or more respects: High complexity (Case 1-4), unclear specifyability (Case 1-2, Case 1-10), unclear suitability to URLLC scenarios (Case 1-4, Case 1-7, Case 1-8, Case 1-9) or lack of demonstrated system-level benefit (all remaining Case 1 schemes) in URLLC scenarios.

For these reasons, this contribution proposes the following:

Proposal 1: For Case 1, RAN1 focuses on the following schemes:

- Case 1-1 (Statistical CSI/SINR)

- Case 1-5 (CSI based on worst IMR occasion)

- Case 1-6 (Worst-M CQI)

- Case 1-11 (Partial information update)

## Case 2 schemes

| Scheme | Pros | Cons |
| --- | --- | --- |
| Case 2-1 Soft-ACK / Slow Soft-ACK | +May increase effectiveness of OLLA by triggering step up before actual NACK occurs +Some evaluations show gain Note: may be equivalent to “binary” Case 2-3 if low margin is defined in terms of CQI/MCS/SINR | -In this evaluation, no gain compared to baseline OLLA with small step size -May be difficult to specify/test (unless based on CQI/MCS/SINR as in Case 2-3) |
| Case 2-2 BLEP | +May allow OLLA to converge toward a wanted EP level in absence of NACKs. | -In this evaluation, no gain compared to baseline OLLA with small step size -May be difficult to test -Possibly large overhead |
| Case 2-3 (Delta) CQI/MCS/SINR | +Some simulation results show benefit +Easier to specify/test | -In this evaluation, no gain compared to baseline OLLA with small step size |
| Case 2-4 HARQ RV sequence |  | -No evaluation result -Impact of RV may be negligible -Unclear how to test |
| Case 2-5 Reason for NACK |  | -No evaluation result -Unclear how to specify or test |
| Case 2-6 Number of NACK values | +Allows the network to detect missed PDCCH | -No evaluation result |

Based on the above analysis, the most attractive Case 2 scheme appears to be Case 2-3 as it may provide the desired benefit on improving OLLA and/or optimizing retransmission. Case 2-3 may also provide similar benefit as Case 2-1 if  binary indication is adopted (coarse 2-levels granularity for CQI/MSC/SINR), some evaluation results have shown benefits. Remaining schemes have unclear specifyability (Case 2-4, Case 2-5) or benefits (Case 2-6).

Proposal 2: For Case 2, RAN1 focuses on Case 2-3: (Delta) CQI/MCS/SINR.

# Conclusion.

This contribution provided evaluation results and analysis for Case 1 and Case 2 schemes identified at RAN1#104. Based on this analysis and results, it proposed the following:

Proposal 1: For Case 1, RAN1 focuses on the following schemes:

- Case 1-1 (Statistical CSI/SINR)

- Case 1-5 (CSI based on worst IMR occasion)

- Case 1-6 (Worst-M CQI)

- Case 1-11 (Partial information update)

Proposal 2: For Case 2, RAN1 focuses on Case 2-3: (Delta) CQI/MCS/SINR.

# References

RP-201310, “Revised WID: Enhanced IIoT and URLLC support for NR”, Nokia, Nokia Shanghai Bell.

3GPP TR 38.824, “Study on physical layer enhancements for NR URLLC”, v16.0.0.

3GPP TR 38.802, “Study on New Radio Access Technology; Physical layer aspects”, v14.2.0.

R1-2009775,	“Feature lead summary#4 on CSI feedback enhancements for Rel-17 URLLC/IIoT”,	Moderator (InterDigital).

R1-2102131, “Feature lead summary#4 on CSI feedback enhancements for enhanced URLLC/IIoT”, Moderator (InterDigital).

R1-2102749, “Summary of additional discussions on CSI feedback enhancements for enhanced URLLC/IIoT after RAN1#104-e”, Moderator (InterDigital).

R1-2100830, “CSI feedback enhancements”, InterDigital.

# Appendix A	MCS error distribution

- The following shows histograms of MCS error for various evaluated Case 1 schemes. The MCS error is the difference between the MCS that would have achieved the BLER target and the MCS that was actually used.

# Appendix B

Agreements from RAN1#104-e

R1-2101811

Conclusion: Continue evaluation of new reporting Case 1 and Case 2 for the schemes identified in Appendix B of R1-2102131.

- Companies are encouraged to provide their views on each scheme against each criterion in respective Tables in Appendix B.

- Companies are encouraged to provide additional evaluation results for as many schemes as possible, based on assumptions agreed in RAN1#102-e.

- Aim for down-selection at RAN1#104-b-e by taking into account evaluation results and assessment against criteria from Appendix B.

Agreements from RAN1#103-e

Agreements

- No change of CSI processing time relative to Rel-16 CSI in this WI

- CSI processing time specific to a new CSI reporting quantity/type (if supported) can be studied

Agreement:

- For Case-2 new reporting, continue studying with focus on the new reporting type based on PDSCH decoding for OLLA performance enhancement for initial and re-transmissions of PDSCH.

Agreements:

For Case-1 New reporting, the following candidate schemes have been identified to address the fast interference change over time. Continue studying with focus on the identified schemes below for further study and evaluation.

- Scheme 1a: New reporting quantity based on CQI/SINR statistics, e.g.,

  - CQI/SINR statistics (e.g., mean, variance, etc.)

  - CSI prediction

- Scheme 1b: New reporting quantity of interference statistics (e.g., mean, variance, interference covariance matrix, etc.)

- Scheme 1c: New reporting quantity based on modifying existing reporting format, e.g.,

  - CQI reporting considering the worst subbands

  - Subband CQI granularity enhancement

- Scheme 1d: New reporting quantity related to CSI expiration time

- Scheme 1e: New reporting quantity with partial information update, e.g.,

  - CSI reporting with interference update only

Companies are encouraged to investigate the above schemes, aiming for down-selection in RAN1#104-e

Email summary in R1-2009775

Agreements from RAN1#102-e

Agreement:

- CSI feedback enhancement for Multi-TRP transmission is not to be discussed further under IIoT/URLLC enhancement WI

Agreements:

- Baseline assumptions are used as the required minimum to be simulated for the evaluation of candidate CSI enhancement schemes

  - Reuse the assumptions in TR 38.824 and TR 38.901 as a starting point

  - Companies shall report additional parameters (e.g., CSI measurement settings, CSI reporting schemes) used in their evaluation

  - FFS details of baseline assumptions

- Companies can bring additional simulation results with other set(s) of assumptions

Agreements:

- Study/evaluate further on following CSI enhancement schemes in terms of technical benefit, specification and implementation impacts.

  - New triggering methods for A-CSI and/or SRS

  - New reporting based on one or more of the following:

    - Case 1: channel/interference measurement for new CSI reporting, considering aspects such as one or more of the following:

      - Reporting more accurate interference characteristics

      - Reduced CSI feedback overhead (e.g., reporting interference measurement only)

      - Enhanced CSI reporting such as WB/SB CQI

    - Case 2: other measurement (other than channel/interference) for additional information

      - E.g., PDCCH/PDSCH decoding, recommended HARQ RV sequence, etc.

    - It targets to help gNB scheduler for better link adaptation of (re)transmission

  - [Reduced CSI computation time/complexity]

  - [CSI feedback for PDCCH]

  - Other CSI enhancement schemes that enable accurate MCS selection are not precluded

- Detailed assumptions of the proposed CSI enhancement schemes should be provided by the proponent, such as

  - Reporting values

  - Triggering conditions for the reporting

  - Associated measurement resource

  - Uplink resource to be used for the reporting

  - How to use the reported information at the gNB scheduler

  - CSI-RS overhead and CSI reporting frequency

  - CSI reporting latency/timeline

  - Etc.

Agreements:

- Consider Table 1 as baseline assumption for system level simulation for evaluating CSI enhancement schemes

  - The uses cases in Table 1 is for simulation purposes and it does not preclude a CSI enhancement scheme which is beneficial for the other URLLC use cases

- No baseline assumption is used for link level simulation

  - Companies are encouraged to use one of LLS assumption tables in Section A.3 in TR38.824 for any link level simulation

Table 1. Baseline SLS assumption for CSI enhancement schemes in URLLC/IIoT

| Parameters | Values |
| --- | --- |
| Performance metric | Option-1 (section 5.1 of TR 38.824) Additional metrics (it is up to company to bring results with additional metric): - MCS prediction error (e.g., difference of a scheduled MCS and an ideal MCS) - DL/UL signaling overhead - CCDF of latency samples from all UEs - BLER of 1st transmission - Resource utilization - Spectral efficiency |
| Use cases | Following two use cases can be considered for new triggering method and new reporting. Companies are encouraged to evaluate the following cases in descending priority: - Rel-15 enabled use case (e.g. AR/VR) in TR 38.824 - Reliability: 99.999 - Latency: 4ms (200bytes) - Traffic mode: FTP model 3 (100p/s) - Factory automation in TR 38.824 - Reliability: 99.9999 - Latency: 1ms (32bytes) - Traffic mode: Periodic deterministic traffic model with arrival interval 2ms - Rel-15 enabled use case (e.g. AR/VR) in TR 38.824 - Reliability: 99.999 - Latency: 1ms (32bytes) - Traffic mode: FTP model 3 (100p/s) - Assumptions for eMBB and URLLC UEs sharing the same carrier is used (as in A2.5 of TR 38.824) |
| Simulation assumptions | Following simulation assumption is used based on the use case selected: - Rel-15 enabled use case with UMa (Table A.2.4-1 in TR 38.824) - Factory automation at 4GHz (Table A.2.2-1 in TR38.824) with following update: - Channel model is replaced with InF (InF-DH) in TR 38.901 - Companies can bring results with other InF scenarios additionally - Layout is replaced with BS deployment in Table 7.8-7 in TR 38.901 |
| Transmission scheme | Multiple antenna ports Tx scheme - Companies report the details of Tx scheme used |

Additional simulation assumptions for this contribution

| - Parameters | - Value |
| --- | --- |
| - Layout | - R15 AR/VR use case- Single layer (Macro) - Factory automation - Indoor Factory (InF-DH) |
| - Carrier frequency | - 4 GHz |
| - System bandwidth | - 20 MHz |
| - Channel model | - 3D Uma and InF |
| - Bs Tx power | - 49 dBm for Outdoor UMa - 24 dBm for Indoor factory |
| - Subcarrier spacing | - 30 kHz |
| - Antenna configuration | - 4 x 4 - 32 antenna elements at the gNB - 4 antenna elements at the UE |
| - User distribution | - R15 AR/VR use case - 80% indoor, 20% outdoor, 10 UEs per cell for UMa - Factory automation - 15 UEs per cell for Indoor factory |
| - Scheduler | - Time-domain PF SU-MIMO |
| - Traffic model | - R15 AR/VR use case - FTP Model 3 (Poisson arrival with packet arrival of 100p/s) and packet size of 200 bytes. - Factory automation – Periodic deterministic with 2ms interarrival (500p/s) and packet size of 32 bytes |
| - HARQ/repetition | - Adaptive HARQ retransmissions. - Maximum 2 HARQ transmissions including retransmission. |
| - Channel Estimation | - Realistic |
| - UE receiver | - MMSE-IRC |
| - CQI Table | - CQI Table 3 |