3GPP TSG RAN WG1 #104-bis-e	Tdoc R1-2102747

e-Meeting, April 12th – 20th, 2021

Agenda Item:	8.3.3

Source:	Ericsson

Title:	Intra-UE Multiplexing/Prioritization Enhancements for IIoT/URLLC

Document for:	Discussion, Decision

# 1	Introduction

In RAN#86, the work item on Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR was approved [1]. Further, the WID was revised in RAN#88e, where the updated WID [2] includes the following objective:

Intra-UE multiplexing and prioritization of traffic with different priority based on work done in Rel.16 [RAN1]:

- Specify multiplexing behavior among HARQ-ACK/SR/CSI and PUSCH for traffic with different priorities, including the cases with UCI on PUCCH and UCI on PUSCH.

- Specify PHY prioritization of overlapping dynamic grant PUSCH and configured grant PUSCH of different PHY priorities on a BWP of a serving cell including the related cancelation behavior for the PUSCH of lower PHY priority, taking the solution developed during Rel.16 as the baseline

This topic was discussed during the last meetings and the corresponding discussions resulted in a set of agreements that are listed in Appendix.

In the following, we discuss our view on the Rel-17 solutions for intra-UE multiplexing and prioritizations.

# 2	Discussion

## 2.1 	General overview and design methodology

Before discussing the details for supporting intra-UE multiplexing in Rel-17, we believe that it is important to understand the direction that different solutions in Rel-17 for intra-UE multiplexing would lead to.

As we progress, it is important to establish early on a common understanding on the overall framework of the expected procedures. The understanding is crucial to deliver a proper design that would be useful for industry. Given that the overlapping resolution procedures are well known for complexity since Rel-15, with additional growth in complexity in Rel-16 due to introduction of two-level priority, further increasing the complexity in Rel-17 would jeopardize the usage of the developed features in real deployments. That means that the delivered solutions should provide gain at a reasonable complexity to be used.

Therefore, a meaningful design methodology for accomplish the task at hand would be to understand first the structure of the candidate frameworks (HOW), and to perform gain and complexity assessments (WHY) to identify the most suitable framework.

The overview provided in this section is intended to initiate and facilitate discussions to accomplish a common understanding on the general framework.

### 2.1.1	Overview of candidate frameworks (HOW)

In the following, we try to categorize the candidate frameworks based on our understanding of the status of the discussions so far.

On a high level, Rel-17 currently supports two alternatives for intra-UE multiplexing with respect to overlapping PUCCH/PUSCH resources as follows:

- Alternative A) PUCCH/PUSCH multiplexing with different priority a.k.a. Intra-UE multiplexing

- Alternative B) Simultaneous PUCCH and PUSCH transmission

While both alternatives are supported, it is also discussed and proposed to consider enabling/disabling either alternative independently.

Our understanding from the discussions so far is that two general approaches are proposed for Alternative A as the following:

A1) PUCCH/PUSCH multiplexing between priorities (i.e. framework A1 in

- Figure 1 )

A2) PUCCH/PUSCH multiplexing per priority first and then between priorities (i.e. framework A2 in

- Figure 1)

Figure 1: Rel-17 A1 and A2 candidate frameworks for Intra-UE multiplexing of overlapping PUCCHs/PUSCHs

With respect to Alternative B, the feature is currently supported in case of inter-band CA but different priorities, however it should fundamentally be applicable for the same priority. Moreover, we can assume that on the PUCCH cell, the UCI multiplexing on PUSCH can occur in case of overlapping PUCCH/PUSCH on that cell. Since the feature supports UCI multiplexing on PUSCH in the same cell as PUCCH, one can consider the following two options for this alternative w.r.t the timing of UCI multiplexing in PUSCH.

- B1) Simultaneous PUCCH/PUSCH before UCI multiplexing on PUSCH (i.e. framework B1 in Figure 2)

- B2) Simultaneous PUCCH/PUSCH before UCI multiplexing on PUSCH (i.e. framework B2 in Figure 2)

Note that we assume that simultaneous PUCCH/PUSCH transmission is applied for the same and different priority. Moreover, alternatives B1 and B2 are realized by applying simultaneous PUCCH/PUSCH transmission on Rel-16 procedures as shown in Figure 2. One can alternatively considers a more simplified framework as shown there, as well.   using the Rel-16 framework.

Figure 2: Rel-17 B1 and B2 candidate frameworks for simultaneous PUCCH/PUSCH transmission of overlapping PUCCHs/PUSCHs. In case of B1, B2 is discarded and vice-versa.

It is interesting to consider a framework for the combined features by Alternatives A and B where simultaneous PUCCH/PUSCH transmission is enabled with intra-UE multiplexing. The possible options are as the following:

- Alternative A1 for PUCCH/PUSCH multiplexing with enabling simultaneous PUCCH/PUSCH prior to UCI multiplexing on PUSCH (i.e. framework A1-B1 in Figure 3)

- Alternative A2 for PUCCH/PUSCH multiplexing with enabling simultaneous PUCCH/PUSCH prior to UCI multiplexing per priority (i.e. framework A2-B1 in Figure 3)

- Alternative A2 for PUCCH/PUSCH multiplexing with enabling simultaneous PUCCH/PUSCH prior to UCI multiplexing between different priorities (i.e. framework A2-B2 in Figure 3)

Figure 3: Rel-17 A1-B1, A2-B1 and A2-B2 candidate frameworks for intra-UE multiplexing of overlapping PUCCH/PUSCH enabled with simultaneous PUCCH/PUSCH. In case of B1, B2 is discarded and vice-versa.

### 2.1.2	Overview of gain and complexity assessments (WHY)

Next, we present few examples to show how assess different framework with respect to complexity and gain can be assessed on a high-level without focusing on the details. If a framework appears to be beneficial from high-level point of view, it is worthwhile to be considered for further study. Otherwise, unnecessary efforts would be spent on a complicated design that would likely be abandoned in practice.  For sake of discussion, we present two sets of examples for single carrier and inter-band carriers in a PUCCH group in Figure 4 and Figure 5, respectively.

Figure 4: Examples of overlapping resolution of PUCCHs/PUSCHs in a PUCCH group with single carrier based on different candidate frameworks

Figure 5: Examples of overlapping resolution of PUCCHs/PUSCHs in a PUCCH group with inter-band carriers based on different candidate frameworks

The overall behavior for single carrier for different framework is as expected. One could observe that if recovering a LP UCI is crucial, the framework should support multiplexing of both HP and LP UCI on PUSCH, as well as PUSCH.

However, in case of inter-band CA, few interesting observations can be made:

- Comparing B1/B2 with A1/A2 suggests that in many cases intra-UE multiplexing (i.e. A1/A2) is not necessary.

- When intra-UE multiplexing is enabled, it is reasonable to enable simultaneous PUCCH/PUSCH as well (i.e. A1-B1, A2-B1/B2). That results in avoiding multiplexing of HP and LP UCI on a PUCCH or PUSCH resource.

- Comparing A1-B1 with A2-B2 suggests that A2-B2 is less complex since LP UCI is multiplexed on LP PUCCH or PUSCH. That means there is no need to multiplex the LP transmission on a high priority.

- Comparing B2 and A2-B2 suggests that intra-UE multiplexing can be disabled for some cases and still the same results can be achieved.

These observations suggest that a framework based on B2 and A2-B2 provides reasonable complexity and meets the expectation in case of inter-band carriers.

On the other hand, based on the discussions so far, it seems that the operation of multiplexing HP and LP UCI on a PUCCH or a PUSCH is a controversial and complicated issue. In some cases, multiple options are proposed for UCI multiplexing on PUCCH, and the topic of joint or separate coding has raised strong concerns among companies. Therefore, from complexity point of views, frameworks that avoid multiplexing HP and LP UCI but can deliver LP UCI are preferred.

Considering these facts, one can argue whether it is worth the effort to design complicated schemes for HP and LP UCI multiplexing on PUCCH/PUSCH. Maybe the effort can be invested on other means to retrieve canceled LP transmission if needed.

Also, the analysis shows the importance of enabling/disabling of intra-UE multiplexing and simultaneous PUCCH/PUSCH transmission dynamically.

Based on the above discussion, we recommend the group to consider the following suggestions:

Prioritize work to stablish early on a common understanding on the overall framework of the expected procedures than detailed solutions.

Investigate the candidate frameworks A1, A2, B1, B2 and their combinations for gain and complexity analysis, starting with B2 and A2-B2.

Consider investigating the complexity of potential features for multiplexing UCI with different priority in PUCCH/PUSCH.

## 2.3	On Simultaneous PUCCH/PUSCH transmission

From our perspective, this feature can be enabled by configuration of an RRC parameter. However, when it is enabled, similarly to PUCCH/PUSCH multiplexing, we would prefer to dynamically enable or disable this feature to provide better flexibility for the network to operate such that the requirements for URLLC are met. The illustrated examples in previous sections clearly shows the benefit of enabling/disabling of intra-UE multiplexing and simultaneous PUCCH/PUSCH transmission dynamically.

It is also important to note that since URLLC traffic usually has a sporadic or periodic pattern, overlapping cases occur either occasionally or predictably. Therefore, the gNB can plan accordingly and decide for the proper actions. For example, whether it is better to apply multiplexing with low priority transmission in case of overlapping with a high priority transmission or to drop the low priority transmission. In that sense, the gNB can ensure that multiplexing is avoided when it would affect the required delay or reliability requirements. Therefore, the gNB should be able to dynamically enable or disable multiplexing.

Moreover, it is not clear why the feature should be limited to be used for PUCCH/PUSCH with different priorities. As discussed in the previous section, benefits are evident if used for the same priority.

Finally, with respect to support of the feature for intra-band CA, we are open to understand better the concerns as well as motivations from the UE vendors. Therefore, more discussions are needed.

Based on the above discussion, we propose the following:

Support simultaneous PUCCH/PUSCH transmission of same PHY priorities over different cells can be RRC configured within the same PUCCH group

When simultaneous PUCCH/PUSCH transmissions is enabled by RRC configuration, simultaneous PUCCH/PUSCH transmissions can be dynamically disabled.

In case of overlapping between PUCCH and/or PUSCH resources in a slot with different priorities, dynamically enabling or disabling UCI multiplexing on PUCCH or PUSCH is supported.

## 2.2 	On Intra-UE multiplexing

### 2.2.1	Multiplexing UCIs of different priorities in a PUCCH

In the following we discuss our view for resolving the overlapping between PUCCH resources irrespective of the associated priorities.

Before doing the exercise, we note that based on the agreement made in previous meeting, UCI multiplexing with different priorities is supported only for the following combinations of UCI:

- Case 1: HP SR and LP HARQ-ACK

- Case 2: HP HARQ-ACK/SR and LP HARQ-ACK

In the following we discuss our view on how to determine a PUCCH resource to carry the UCI corresponding to Case 1 and Case 2 above.

#### 2.1.2.1	PUCCH resource with HP SR and LP HARQ-ACK

In overlapping resolution procedure for Case 1, based on the Rel-15 procedures, the SR in general is multiplexed on HARQ-ACK PUCCH resource. However, the case when PUCCH format 1 is used for both SR and HARQ-ACK, is not supported. If PUCCH format 0 is used for SR and PUCCH format 1 for HARQ-ACK, in fact SR is dropped. In our view, the cleanest approach is to adopt the general principle of Rel-15 and that is to multiplex SR on LP HARQ-ACK PUCCH resource. With respect to reliability of SR, we suggest using a different PRB when SR is triggered. The offset value can be a fixed value of configured. In case of LP HARQ-ACK with PUCCH format 0, 1 or 4, detection of PUCCH on two different PRBs determines whether SR is triggered or not. In case of PUCCH format 2 and 3, even partially overlapping PRBs provide good reliability for SR detection. Figure 6 illustrates a few examples.

When PUCCH with HP SR overlaps with PUCCH with LP HARQ-ACK:

    - For 1-2 LP HARQ-ACK bits: The PUCCH resource for HARQ-ACK is used for multiplexing of the HP SR and LP HARQ-ACK. If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource (i.e. Opt. 2a).

    - For more than 2 LP HARQ-ACK bits: Rel-15 rules are used for multiplexing HARQ-ACK and SR in a PUCCH resource. If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the PUCCH resource (i.e. Opt. 2a).

Figure 6: Multiplexing HP SR on LP HARQ-ACK PUCCH resource. With positive SR, an offset PRB is applied to HARQ-ACK PUCCH resource.

#### 2.1.2.2	PUCCH resource for HP HARQ-ACK/SR and LP HARQ-ACK

In overlapping resolution procedure for Case 2, Rel-15 procedure can be reused by determining a PUCCH resource that would carry the total UCI, i.e. HP HARQ-ACK/SR and LP HARQ-ACK. Since these two PUCCH resources have different priority, they correspond to different PUCCH-Config and associated PUCCH resource sets, accordingly. In order to ensure reliability, we suggest using the PUCCH resource sets of the high priority PUCCH-Config. In that case, based on the total UCI size, i.e. the total number of HP HARQ-ACK/SR and LP HARQ-ACK, a PUCCH resource set among those configured in the second PUCCH-Config, is determined. A resource in this set based on the last DCI corresponding to the HP HARQ-ACK, is identified. This resource is the single resource that would carry HP HARQ-ACK/SR and LP HARQ-ACK.

When PUCCH with HP HARQ-ACK/SR overlaps with PUCCH with LP HARQ-ACK:

      - First, a PUCCH resource set associated to HP HARQ-ACK based on the total number of HP HARQ-ACK/SR and LP HARQ-ACK is determined. Then, a PUCCH resource in the PUCCH resource set to carry both HP and LP HARQ-ACK based on the last DCI corresponding to the HP HARQ-ACK is determined.

#### 2.1.2.3	 Joint or separate coding

If the single PUCCH resource is of format 2,3,4, our view on separate or joint encoding, as well as target code rate, is as follows:

From a complexity point of view, joint encoding of HARQ feedback does not require separate decoding attempts and is simpler at the receiver. Furthermore, separate block coding of 1-2 bits is only supported on PUSCH in Rel. 16.

In Figure 7 - Figure 10 below we show performance results for joint vs separate encoding of URLLC and eMBB HARQ feedback. We assume a BLER target of 1e-2 for eMBB HARQ feedback and 1e-5 for URLLC HARQ feedback.

Figure 7: Performance of joint vs separate coding for PUCCH Format 3 for 12 eMBB and 8 URLLC bits.

Figure 8: Performance of joint vs separate coding for PUCCH Format 3 for 12 eMBB and 12 URLLC bits.

Figure 9: Performance of joint vs separate coding for PUCCH Format 3 for 24 eMBB and 8 URLLC bits.

Figure 10: Performance of joint vs separate coding for PUCCH Format 3 for 48 eMBB and 8 URLLC bits.

In the simulated cases with a small number of URLLC HARQ bits, the observed gain from separate coding is between 2.5 – 5 dB. The gain is sensitive to a proper split of radio resources between the two traffic types. In the simulated cases, we see splits ranging between 50% - 90% of the resources used for URLLC.

Separate coding shows a gain over joint coding when the number of URLLC bits is small and the total number of bits is larger than 2. A proper split of radio resources is needed to maximize gain.14 OS PUCCH formats, but joint coding

Therefore, we propose the following:

Support separate encoding of high and low priority HARQ feedback in a PUCCH resource.

For very few bits of UCI the situation is not as clear. In Figure 11Figure 12 we show the performance of joint vs separate coding for a single URLLC bit and a single eMBB bit for long PUCCH formats with 14 OFDM symbols and short PUCCH formats with 2 OFDM symbols. For PUCCH formats 2 and 3 the single bits are encoded using the channel codes for short block lengths in 38.212 Section 5.3.3. In the single bit case for QPSK this is a repetition code. The total number of REs are split between the URLLC and eMBB bit to obtain the best performance.

Figure 11: Performance of joint vs separate coding for 1 eMBB bit and 1 URLLC bit for 14 OS.

Figure 12: Performance of joint vs separate coding for 1 eMBB bit and 1 URLLC bit for 2 OS.

For the long PUCCH format with 14 OFDM symbols, separate coding performs better than joint coding by 4 dB, but for the short PUCCH format with 2 OFDM symbols joint coding performs better by at least 4 dB.

For single bits of eMBB and URLLC UCI, separate coding performs better for 14 OS PUCCH formats, but joint coding performs better for 2 OS PUCCH formats.

Therefore, we propose the following:

Study joint vs separate coding further for very small payloads.

#### 2.1.2.4	Procedure to resolve overlapping between PUCCH resources

For alternatives A1 and A2 discussed in section 2.1.1, when overlapping resolved between PUCCHs with different priorities , the Rel-15 procedures can be reused by taking into account the applicable combinations of UCI based on the previous agreements and adopt the suitable adjustments as the following:

- As in Rel-15, the first set of mutually overlapping PUCCH resources in a slot are identified and sorted in the set Q.

- A single PUCCH resource is determined for the PUCCH resources in the set Q as the following:

  - If all resources have the same priority, follow the already existing procedures.

  - Otherwise

    - Drop SR and CSI of low priority, if any.

    - If one of resources is associated to a HARQ-ACK with high priority, apply the associated PUCCH resource sets and last DCI to determine a single PUCCH resource (i.e. Proposal 6).

      - If the high priority HARQ-ACK PUCCH resource is associated to sub-slot, start from the earlier and smallest sub-slot.

    - Otherwise, if PUCCH resources for high priority SR and low priority HARQ-ACK overlap, determine a single PUCCH resource based on HARQ-ACK PUCCH resource as described in Proposal 5.

- As in Rel-15, the above procedure is continued until there is no overlapping PUCCH resources in the slot.

As it is shown in Figure 13, this approach effectively results in running the procedures sub-slot based if there are overlapping sub-slot based PUCCH resources.

Figure 13: Determining a single PUCCH resource for mutually overlapping PUCCH resource in set Q

We propose to reuse the Rel-15 procedures by taking into account the applicable combinations of UCI based on the previous agreements and adopt the suitable adjustments.

Resolve overlapping between PUCCH resources based on Rel-15 procedures where the overlapping is resolved starting from the first set of mutually overlapping PUCCH resources in a slot (a.k.a. set Q) until there are no overlapping PUCCH resources in the slot.

To determine a single PUCCH resource for a set of mutually overlapping PUCCH resources with different priority, drop SR and CSI of low priority, if any. Then, use sub-slot PUCCH resources if there is a sub-slot HARQ-ACK PUCCH in the set, starting from the earlier and smaller sub-slot.

### 2.2.2	Multiplexing	UCIs of different priorities in a PUSCH

As discussed previously, when resolving overlapping PUCCH resources the outcome would be non-overlapping PUCCH resources where the UCI in each PUCCH resource can be one of the following:

- Case A: LP UCI (HARQ-ACK, SR, CSI or any combination of them)

- Case B: HP UCI (HARQ-ACK, SR or HARQ-ACK/SR)

- Case C: Mix of HP and LP UCI (HP HARQ-ACK and LP HARQ-ACK, HP HARQ-ACK and HP SR and LP HARQ-ACK, HP SR and LP HARQ-ACK)

In case of PUCCH/PUSCH overlapping for any of the above cases, our view on multiplexing/dropping rules is summarized in table below.

Table 1: Expected behavior to resolve overlapping between PUCCH and PUSCH resource

|  | Case A (LP UCI) | Case B (HP UCI) | Case C (Mix LP/HP UCI) | Summary of cases when UCI is multiplexed on PUSCH |
| --- | --- | --- | --- | --- |
| LP PUSCH | Mux/drop as in Rel-15 (SR dropped) | Drop PUSCH if UCI includes HP SR Otherwise, multiplex UCI on LP PUSCH | Drop PUSCH if UCI includes HP SR Otherwise, multiplex UCI on LP PUSCH | Case A: LP HARQ-ACK, LP CSI Case B: HP HARQ-ACK Case C: LP HARQ-ACK, HP HARQ-ACK |
| HP PUSCH | Mux/drop as in Rel-15 (SR dropped) | Drop HP SR if UCI includes HP SR (as in Rel-15/16) Otherwise, multiplex UCI on HP PUSCH | Drop HP SR if UCI includes SR (as in Rel-15/16) Otherwise, multiplex UCI on HP PUSCH | Case A: LP HARQ-ACK, LP CSI Case B: HP HARQ-ACK Case C: LP HARQ-ACK, HP HARQ-ACK |

When we study the combinations that UCI is multiplexed on a PUSCH, according to the table above the UCI may include the following:

- LP CSI

- LP HARQ-ACK

- HP HARQ-ACK

For the purpose of multiplexing, as we discussed in Section 2.1.1, we would like to use the already existing procedures. That implies that for multiplexing, the corresponding timeline requirements should be met. Moreover, for multiplexing, we can reuse the existing framework for HARQ-ACK and CSI, and extend it one level higher from LP HARQ-ACK to HP HARQ-ACK. In that sense, more than one overlapping PUCCHs can overlap with a PUSCH. In case the total UCI includes UCI with different priorities, as agreed different beta-offset values for multiplexing UCI with different priority is used. As we discussed earlier, if multiplexing causes delay issues harmful for URLLC, the gNB should be able to dynamically disable it. Finally, It is not necessary to configure different alpha values since the same goal on controlling number of REs can be achieved with combination of alpha and different beta values.

Therefore, we propose the following:

For UCI multiplexing on PUSCH, one or more PUCCH can overlap with PUSCH where the corresponding UCI can be multiplexed in the PUSCH.

For UCI multiplexing on PUSCH, a different target code rate and beta factor is considered for high priority HARQ-ACK.

Support dynamically enable/disable multiplexing by beta factor (e.g. beta=0 to disable mux)

### 2.1.2	Other enhancements

Methods such as puncturing, resuming or bundling are proposed for enabling HARQ-ACK multiplexing of different priorities in PUCCH or PUSCH. From our perspective, dropping and rescheduling is preferred over puncturing/resuming or bundling since these methods are either too complicated to be implemented or too simplified to achieve the goal of multiplexing.

In case of overlapping between PUCCH and/or PUSCH resources in a slot with different priorities, methods based on partial puncturing with or without resuming and HARQ-ACK bundling as part of overlapping resolution procedures are not supported.

In case of overlapping between PUCCH and/or PUSCH resources in a slot with different priorities, only UCI multiplexing methods on PUCCH or PUSCH resources that are extension of already existing UCI multiplexing methods are supported.

## 2.3	Prioritizing DG/CG-PUSCH with different priorities

### 2.3.1 	Prioritization of HP DG and LP CG PUSCHs

An agreement was made in the 3GPP meeting RAN1#103e on the support of PHY layer prioritization between high priority DG (HP DG) and low priority CG (LP CG) PUSCHs in R17.  However, the agreement has the limited scope of “on a BWP of a serving cell”. That is, the support in CA case is still missing.

In our view, the intra-UE prioritization of HP DG and LP DG PUSCHs may occur in both CA and non-CA cases. It is too restrictive to allow non-CA case only in Rel-17. Indeed, the uplink skipping issue mentioned in the agreement is handled for both CA and non-CA cases. It is understood that the UE capability for this feature should be further discussed in the UE feature session after the Rel-17 normative texts are in place.

For CA case, support PHY prioritization of overlapping high-priority dynamic grant PUSCH and low-priority configured grant PUSCH on different serving cells in R17.

For the issues related to UCI overlapping and uplink skipping, currently there is ongoing discussion in AI 7.1 (Maintenance of Release 15 NR) and AI 7.2.5 (Maintenance of Physical Layer Enhancements for NR URLLC). We recommend that detailed Rel-17 discussion starts after the issues are fully resolved under AI 7.1 and 7.2.5.

For the moment, it is sufficient to align on the principle below so that hypothesis testing can be avoided in both gNB implementation and UE implementation, in Rel-17 as well as in Rel-15 and Rel-16.

For cases where a UCI overlaps with multiple PUSCHs, the PUSCH to be multiplexed with the UCI is determined based on signaling known to both gNB and UE.

It is noted that the procedure to determine the PUSCH to be multiplexed with the UCI needs to be revised in Rel-17, as compared to Rel-15 and Rel-16. This is due to the support of multiplexing UCI (e.g., HARQ-ACK) and PUSCH of different physical layer priorities. The exact details are pending further discussion.

For cases where a UCI overlaps with multiple PUSCHs, RAN1 discuss the procedure for determining the PUSCH to be multiplexed with the UCI, taking into account the Rel-17 support of multiplexing UCI and PUSCH of different priorities.

In case of difficulties to agree on multiple PUSCH overlapping scenarios, one can focus on simple cases when only two physical channels overlap. We think that this can be reasonable restriction since the whole feature is getting too complicated.

### 2.3.2 	Prioritization of LP DG and HP CG PUSCHs

It was agreed in RAN1#102e that the alternative prioritization scenario should be also supported, when low priority DG and high priority CG overlap. This enables the more efficient methods for scheduling, when data traffic dedicated for CG is sporadic, relative to the CG occasions configured.

Specifically, it is meaningful to consider low-priority DG (LP DG) being scheduled to overlap with high priority CG (HP CG), because the HP CG PUSCH may or may not exist in every transmission occasion. The HP CG PUSCH disappears if MAC has no buffer data for it (i.e., MAC does not generate a TB for the HP CG PUSCH). In this case, LP DG PUSCH survives, and is transmitted by the UE. Absence of TB for HP CG PUSCH can occur very often if the CG PUSCH configurations are over provisioned for the purpose of latency reduction. One example where this is intentionally designed in is to resolve the issue of periodicity misalignment between the TSN periodicity and CG/SPS periodicity. In RAN2#105bis, RAN2 agreed that a solution is to use short CG periodicities and/or multiple CG configurations and/or combinations thereof. This implies that many CG occasions are provided for one potential data transmission, with most CG occasions having no data.

Thus, without considering complications due to UCI overlapping, UL skipping enabled or not, the following basic procedure should be supported.

- If MAC generates a TB for the HP CG PUSCH, the HP CG PUSCH is prioritized over the LP DG PUSCH.

- Otherwise, HP CG PUSCH is considered non-existent for the intra-UE multiplexing/prioritization procedure. LP DG PUSCH is transmitted.

Overall, the Rel-16 principle of overlapping CG and DG with same/different PHY priorities still applies, both for HP DG vs LP CG and LP DG vs HP CG. Thus, we propose the following.

Maintain the same understanding as in Rel-16, i.e., in the collision scenario between CG and DG with same/different PHY-priority index, and only one transport block is delivered to PHY, PHY transmit on the grant for which a transport block is delivered and skip the transmission on the other grant.

Regarding UE capability issues in Rel-17, it is acknowledged that various degree of UE complexity is involved. There is also the complication of differentiating Rel-17 behavior from existing Rel-15 and Rel16 behavior. Thus, UE capability should be discussed later, in the UE feature sessions.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Separate coding shows a gain over joint coding when the number of URLLC bits is small and the total number of bits is larger than 2. A proper split of radio resources is needed to maximize gain.14 OS PUCCH formats, but joint coding

Observation 2	For single bits of eMBB and URLLC UCI, separate coding performs better for 14 OS PUCCH formats, but joint coding performs better for 2 OS PUCCH formats.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Prioritize work to stablish early on a common understanding on the overall framework of the expected procedures than detailed solutions.

Proposal 2	Investigate the candidate frameworks A1, A2, B1, B2 and their combinations for gain and complexity analysis, starting with B2 and A2-B2.

Proposal 3	Consider investigating the complexity of potential features for multiplexing UCI with different priority in PUCCH/PUSCH.

Proposal 4	Support simultaneous PUCCH/PUSCH transmission of same PHY priorities over different cells can be RRC configured within the same PUCCH group

Proposal 5	When simultaneous PUCCH/PUSCH transmissions is enabled by RRC configuration, simultaneous PUCCH/PUSCH transmissions can be dynamically disabled.

Proposal 6	In case of overlapping between PUCCH and/or PUSCH resources in a slot with different priorities, dynamically enabling or disabling UCI multiplexing on PUCCH or PUSCH is supported.

Proposal 7	When PUCCH with HP SR overlaps with PUCCH with LP HARQ-ACK:

	For 1-2 LP HARQ-ACK bits: The PUCCH resource for HARQ-ACK is used for multiplexing of the HP SR and LP HARQ-ACK. If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource (i.e. Opt. 2a).

	For more than 2 LP HARQ-ACK bits: Rel-15 rules are used for multiplexing HARQ-ACK and SR in a PUCCH resource. If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the PUCCH resource (i.e. Opt. 2a).

Proposal 8	When PUCCH with HP HARQ-ACK/SR overlaps with PUCCH with LP HARQ-ACK:

	First, a PUCCH resource set associated to HP HARQ-ACK based on the total number of HP HARQ-ACK/SR and LP HARQ-ACK is determined. Then, a PUCCH resource in the PUCCH resource set to carry both HP and LP HARQ-ACK based on the last DCI corresponding to the HP HARQ-ACK is determined.

Proposal 9	Support separate encoding of high and low priority HARQ feedback in a PUCCH resource.

Proposal 10	Study joint vs separate coding further for very small payloads.

Proposal 11	Resolve overlapping between PUCCH resources based on Rel-15 procedures where the overlapping is resolved starting from the first set of mutually overlapping PUCCH resources in a slot (a.k.a. set Q) until there are no overlapping PUCCH resources in the slot.

Proposal 12	To determine a single PUCCH resource for a set of mutually overlapping PUCCH resources with different priority, drop SR and CSI of low priority, if any. Then, use sub-slot PUCCH resources if there is a sub-slot HARQ-ACK PUCCH in the set, starting from the earlier and smaller sub-slot.

Proposal 13	For UCI multiplexing on PUSCH, one or more PUCCH can overlap with PUSCH where the corresponding UCI can be multiplexed in the PUSCH.

Proposal 14	For UCI multiplexing on PUSCH, a different target code rate and beta factor is considered for high priority HARQ-ACK.

Proposal 15	Support dynamically enable/disable multiplexing by beta factor (e.g. beta=0 to disable mux)

Proposal 16	In case of overlapping between PUCCH and/or PUSCH resources in a slot with different priorities, methods based on partial puncturing with or without resuming and HARQ-ACK bundling as part of overlapping resolution procedures are not supported.

Proposal 17	In case of overlapping between PUCCH and/or PUSCH resources in a slot with different priorities, only UCI multiplexing methods on PUCCH or PUSCH resources that are extension of already existing UCI multiplexing methods are supported.

Proposal 18	For CA case, support PHY prioritization of overlapping high-priority dynamic grant PUSCH and low-priority configured grant PUSCH on different serving cells in R17.

Proposal 19	For cases where a UCI overlaps with multiple PUSCHs, the PUSCH to be multiplexed with the UCI is determined based on signaling known to both gNB and UE.

Proposal 20	For cases where a UCI overlaps with multiple PUSCHs, RAN1 discuss the procedure for determining the PUSCH to be multiplexed with the UCI, taking into account the Rel-17 support of multiplexing UCI and PUSCH of different priorities.

Proposal 21	Maintain the same understanding as in Rel-16, i.e., in the collision scenario between CG and DG with same/different PHY-priority index, and only one transport block is delivered to PHY, PHY transmit on the grant for which a transport block is delivered and skip the transmission on the other grant.

# 4	Appendix

## 4.1 Additional illustrations

Figure 14: Rel-15 framework for PUCCHs/PUSCHs overlapping resolution

Figure 15: Rel-16 framework for PUCCHs/PUSCHs overlapping resolution

Figure 16: List of Rel-17 candidate frameworks for PUCCH/PUSCHs overlapping resolution

## 4.2	List of agreements

### 4.2.1	List of agreements in RAN1#102-e

Agreements:

Support multiplexing for following scenarios in R17:

- Multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH in R17.

- Multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH for some HARQ-ACK/SR PF combinations (FFS applicable combinations).

- Multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH.

For the above multiplexing scenarios,

- FFS conditions, if needed, for the multiplexing, e.g

  - Whether to support multiplexing between different resources not confined within a sub-slot.

  - Whether to support multiplexing in case a PUCCH overlaps with more than one PUCCH.

  - Timeline requirements.

- FFS: details, if needed, of the multiplexing scheme, e.g.

  - How to minimize impact on the latency for high-priority HARQ-ACK.

  - How to determine the PUCCH resource used for multiplexing (e.g. HP or LP PUCCH resource, or a dedicated PUCCH resource for the multiplexing).

  - How to multiplex the HARQ-ACK bits (e.g. multiplexing, bundling).

  - How to encode the UCIs with different priorities (e.g. separate coding vs. joint coding)

  - How to guarantee the target code rate (e.g. payload control, multiplexing priority, LP HARQ-ACK compression/compaction).

  - Explicit indication for enabling multiplexing.

  - Multiplexing rule and order (e.g. HP/LP multiplexing is after resolving collision within the same priority).

Agreements:

Support multiplexing for following scenarios in R17:

- Multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only).

- Multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only)

- Multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI.

- Multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI.

For the above multiplexing scenarios,

- Support separate configurations of at least beta-offset values (FFS for alpha) for multiplexing with different priority combinations.

  - FFS for other separate configurations.

  - FFS: value range of beta-offset (e.g. <1).

- FFS the conditions, if needed, for multiplexing, e.g.

  - FFS: Whether to support multiplexing in case a PUCCH/PUSCH overlaps with more than one PUCCH/PUSCH.

  - Timeline requirements.

- FFS: details, if needed, of the multiplexing scheme, e.g.

  - How to minimize impact on the latency for high-priority HARQ-ACK.

  - How to multiplex the HARQ-ACK bits (e.g. multiplexing, bundling)?

  - How to encode the UCIs with different priorities (e.g. separate coding vs. joint coding).

  - How to guarantee the target code rate (e.g. payload control, multiplexing priority, LP HARQ-ACK compression/compaction).

  - Explicit indication for multiplexing.

  - Multiplexing rule and order (e.g. HP/LP multiplexing is after resolving collision within the same priority).

  - How to handle multiplexing of UCI of different priorities and CG-UCI in a CG-PUSCH

Agreements:

Support simultaneous PUCCH/PUSCH transmissions on different cells at least for inter-band CA.

- FFS how to trigger this function.

- FFS for intra-band CA.

Agreements:

Support PHY prioritization for the case where low-priority DG-PUSCH collides with high-priority CG-PUSCH in R17.

- FFS details

- Clarify R16 baseline if needed.

### 4.2.2	List of agreements in RAN1#103-e

Agreements:

For multiplexing UCIs of different priorities in a PUCCH in R17,

- Support of multiplexing between different resources not confined within a sub-slot if conditions are met

  - FFS: Details

- Support multiplexing in case a PUCCH overlaps with more than one PUCCH if conditions are met

  - FFS details

Agreements:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits are more than 2 bits, down-select from the following options in RAN1#104-e:

l Option 1: Support joint coding.

l Option 2: Support separate coding.

l Option 3: Combination of Option1 and 2.

l FFS the details

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is 2 bits, provide design details for decision for the following cases in RAN1#104-e:

·        Multiplexing on a PUCCH format 0

·        Multiplexing on a PUCCH format 1

Agreements:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, support a mechanism for gNB to enable/disable the multiplexing.

- FFS the type of the mechanism, e.g. DCI indication and/or RRC configuration

- FFS: Interaction between the enable/disable mechanism and other multiplexing conditions

- FFS for other types of UCI.

Agreements:

For HARQ-ACK multiplexing on PUSCH of different priority in R17, support a mechanism for gNB to enable/disable the multiplexing.

- FFS the type of the mechanism, e.g. DCI indication and/or RRC configuration, beta_offset=0

- FFS: Interaction between the enable/disable mechanism and other multiplexing conditions

- FFS for other types of UCI.

Agreements:

Support PHY prioritization of overlapping high-priority dynamic grant PUSCH and low-priority configured grant PUSCH on a BWP of a serving cell in R17.

- FFS the related cancelation behavior for the PUSCH of lower PHY priority and other details.

  - First clarify what is the scope of this feature, e.g. if overlapping between more than 2 channels is considered.

- FFS the timeline requirements.

  - First clarify what is the behavior of Rel-16 UE in case of DG/CG/UCI overlapping, with and without uplink skipping enabled.

- FFS UE capability for this feature.

- Note: The main bullet has been agreed in the WID by RAN Plenary.

### 4.2.3	List of agreements in RAN1#104-e

Agreements:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17,

- Use a PUCCH resource in the second PUCCH-Config (the PUCCH-config containing the PUCCH resource of the HP HARQ-ACK) at least in case the total number of LP and HP HARQ-ACK bits is more than 2.

- FFS: The PUCCH resource is configured dedicated for multiplexing of HP HARQ-ACK and LP HARQ-ACK.

- FFS in case the total number of LP and HP HARQ-ACK bits is 2.

- FFS details

Working assumption:

Reuse Rel-15 intra-UE PUCCH/PUSCH multiplexing timeline requirements for Rel-17 intra-UE PUCCH/PUSCH multiplexing with different priorities

- FFS whether or not to specify a different behavior than Rel-15 when the timeline requirements are not met

Agreements:

For multiplexing LP HARQ-ACK in a HP PUSCH, support 0< beta-offset <1.

- FFS value(s)

- FFS to additionally support beta-offset =0 or a value disabling the multiplexing

- Aim to NOT increase the corresponding bitwidth in the DCI (compared to Rel-16)

Agreements:

Per UE with the capability of inter-band CA, simultaneous PUCCH/PUSCH transmission of different PHY priorities over different cells can be RRC configured within the same PUCCH group

- FFS: dynamic indication

Agreements:

When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, further study the following options (proponents are encouraged to provide more details and analysis):

l  Opt.1: The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource.

l  Opt.1a: The UE does not transmit negative SR.

l  Opt.1b: For negative SR, the UE transmit only HARQ-ACK on the HARQ-ACK resource.

l  Opt.1c: For negative SR, the UE transmits SR and HARQ-ACK on the SR resource

l  FFS: whether with power boost to transmit multiplexed payload or not.

l  Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource.

l  Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource.

l  Opt.2b: Using 4 CS values as for SR+1-bit HARQ-ACK in Rel-15/16. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit.

l  Opt.2c: If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.

l  Opt.3: No enhancement over Rel-16.

l  Other options not excluded.

l  FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK?

Agreements:

When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF1, further study the following options (proponents are encouraged to provide more details and analysis):

l  Opt.1: The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource.

l  Opt.1a: The UE does not transmit negative SR.

l  Opt.1b: For negative SR, the UE transmit only HARQ-ACK on the HARQ-ACK resource.

l  Opt.1c: For negative SR, the UE transmits SR and HARQ-ACK on the SR resource

l  FFS: whether with power boost to transmit multiplexed payload or not.

l  Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource.

l  Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource.

l  Opt.2b: Applying QPSK for SR+1-bit HARQ-ACK. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit.

l  FFS on conditions of multiplexing.

l  Opt.3: For positive SR, transmit HARQ-ACK on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource.

l  Opt.4: For positive SR, transmit SR on the SR resource and drop HARQ-ACK. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource.

l  Opt.5: No enhancement over Rel-16.

l  Other options not excluded.

l  FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK?

Agreements:

When a PUCCH carrying HP SR with PF1 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, further study the following options (proponents are encouraged to provide more details and analysis):

l  Opt.1: The SR and HARQ-ACK are multiplexed and transmitted on the SR resource.

l  Opt.1a: For positive SR, the UE transmits the PUCCH in the resource using PUCCH format 1 for SR. The value of cyclic shift of sequence, i.e., , of this PUCCH format 1 is determined by HARQ-ACK, and the bit, i.e., b(0), of this PUCCH format 1 is determined by SR. For negative SR, the UE transmits only a PUCCH with HARQ-ACK information and drops the PUCCH with negative SR.

l  Opt.1b: SR and HARQ-ACK are multiplexed and modulated to be transmitted on the SR resource

l  Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource.

l  Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource.

l  Opt.2b: Using 4 CS values as for SR+1-bit HARQ-ACK in Rel-15/16. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit.

l  Opt.2c: If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.

l  Opt.2d: HP SR and LP HARQ-ACK are multiplexed by the Rel-15 cyclic shift only if latency requirement for HP SR is met. Otherwise, drop the LP HARQ-ACK and only transmit the HP SR on its resource.

l  Opt.3: For positive SR, transmit HARQ-ACK on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource.

l  Opt.4: No enhancement over Rel-16.

l  Other options not excluded.

l  FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK?

# References

RP-193233 - New WID on enhanced Industrial Internet of Things (IoT) and URLLC support, Nokia, Nokia Shanghai Bell

RP-201310 - Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR, Nokia, Nokia Shanghai Bell

3GPP TS38.214 v 16.1.0, Physical layer procedures for data, March 2020.

R1-2100265, Intra-UE Multiplexing and Prioritization for Rel-16 URLLC, Ericsson, January 2021.