3GPP TSG RAN WG1 #104bis-e	                                           	      R1-2102813

e-Meeting, Apr 12th – 20th, 2021

Agenda Item:	8.3.2

Source:	NEC

Title:	Enhancements for unlicensed band URLLC/IIoT

Document for:	Discussion

# Introduction

In RAN#104e, the following agreements have been made on enhancements for unlicensed band URLLC/IIoT [1]:

Agreement:

- PUSCH repetition Type B is supported for unlicensed band operation when using NR IIoT Rel-16 based CG

  - FFS whether/how to enhance

Agreement:

- In semi-static channel access mode, UE FFP periodicity is chosen from the following set of values in ms: {1, 2, 2.5, 4, 5,10}.

  - FFS on other values

Agreement:

- In semi-static channel access mode:

- An FFP period for UE-initiated COT is configured as the same, integer multiple of, or inter-factor of the FFP period configured for gNB-initiated COT

- FFP period for UE-initiated COT can be configured independently from FFP period of gNB-initiated COT, if the UE indicates the corresponding capability

- FFP offset for UE-initiated COT is the starting point of first UE FFP relative to the radio frame X boundary.

  - The offset value range is 0 ≤ offset ＜FFP period of UE-initiated COT

    - FFS on X (e.g. X=0, or X= even index number)

Agreement:

In semi-static channel access mode when a UE can operate as initiating device,

- Select one of the following alternatives to determine whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT:

- Alt-a: Determination based on the content in the scheduling DCI

  - FFS on whether the corresponding field(s) can be absent in DCI

    - If absent, determination based on the rules applied for configured UL transmissions is applied

  - FFS whether/how to handle the case when the gNB schedules an UL transmission in the next gNB’s FFP period

  - Alt-b: Determination based on the rules applied for a configured UL transmission

Agreement:

In semi-static channel access mode when a UE can operate as UE-initiated COT,

- Select one of the following alternatives to determine whether a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, is based on UE-initiated COT or sharing a gNB-initiated COT:

  - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT

  - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT.

Agreement:

- In semi-static channel access mode, sharing a UE initiated COT through the gNB to other intra-cell UEs for UL transmissions, is not supported.

In this contribution, we discuss issues related to URLLC transmissions for support of UE-initiated COT for FBE, Release of UE initiated COT and harmonization of UL configured grant.

# Support of UE-initiated COT for FBE

In semi-static channel access mode, when a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE, we propose to support the following:

•	Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB has initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT.

UE initiated COT is supported as a supplement for gNB initiated COT, to reduce signalling overhead and to improve the system performance in particular for URLLC use cases. It is not additionally beneficial for gNB to dynamically signal if a UE should initiate a COT for each FFP. If the gNB has initiated a COT and sends an UL Grant to the UE scheduling a PUSCH, the gNB would have obtained the COT. The gNB would not schedule a transmission onto another UE’s COT since that would cause a collision. The UE would not have already initiated a CO in the UE FFP in which the UL transmission is scheduled since gNB has the COT. Hence the only period which is outside gNB’s COT is gNB’s own idle period. The UE does not need to initiate a COT if the PUSCH still falls within the gNB’s COT since the UE can be a responding device.

Whenever feasible, it is desirable for UE to transmit using a gNB initiated FFP for URLLC when gNB schedules a UE transmission since the gNB would have already obtained the COT and it can determine the UE’s transmission priority. However, if the transmission is not confined within a gNB FFP before the idle period of that gNB’s FFP, it is beneficial for the UE to transmit the configured UL transmission according to the UE’s initiated COT.

Similarly, a UE is able to determine whether a scheduled UL transmission should be transmitted according to shared gNB-initiated COT or UE-initiated COT. DL signaling would be increased if the gNB has to always transmit PDCCH for each UE to enable a UE initiated FFP.

When a scheduled UL transmission starts after a gNB FFP boundary and ends before the idle period of that gNB FFP:

- If the UE has already determined that gNB has initiated that gNB FFP, then UE assumes that the scheduled UL transmission corresponds to gNB-initiated COT.	Otherwise, the UE assumes that the scheduled UL transmission corresponds to UE-initiated COT.

When a scheduled UL transmission starts after a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE:

•	If the UE has already initiated the UE FFP, then UE assumes that the scheduled UL transmission corresponds to UE-initiated COT

•	Otherwise, if the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and if the UE has already determined that gNB has initiated that gNB FFP, then UE assumes that the scheduled UL transmission corresponds to gNB-initiated COT.

The basic operations can be achieved via gNB initiated COT. Within each COT, there can be multiple transmissions in both DL and UL. Subject to CCA, UEs served by the gNB can transmit within the gNB’s shared COT. With the introduction of UE initiated FFP, collision handling can become more complex. For example, UE1 may have a transmission via UL configured grant using UE1 initiated COT based on UE’s FFP, another high priority UE2 may also want to transmit, the high priority UE2 may not be able to transmit even at the next FFP if the start of FFP is aligned with UE1’s configured grant.

It is agreed in RAN1#104e that in semi-static channel access mode, sharing a UE initiated COT through the gNB to other intra-cell UEs for UL transmissions, is not supported. If gNB shares UE1’s COT, it cannot transmit unicast DL signal to serve another UE, e.g. UE2.

However, if gNB shares UE1’s COT, gNB should still be allowed to transmit the broadcast or group common signalling to UE2, since UE2 does not need to identify if the received DL signalling is from gNB initiated COT or gNB sharing other UE’s COT, and UE2 does not need to know what to assume for the idle periods as it has already detected the DL signal.

Proposal 1: Predetermined rules are supported for both configured transmission and scheduled transmission.

# Release of UE Initiated COT

UE initiated COT is introduced to improve the support of URLLC UL configured grant transmission. We propose that gNB can explicitly release a UE initiated COT in order to support high priority URLLC transmissions. As shown in Figure 1, UE#1 is sending UL transmissions using FFP-ue1, while UE#2 would need to send a high priority UL transmissions via configure grant. Even though UE#2 has a higher priority UL transmission, it can be blocked due to LBT failure, also the gNB is not aware that UE#2 has data to transmit. UE#1 may initiate a COT successfully in every FFP if its configured grant transmission is aligned with the FFP boundary. To overcome this, a higher energy detection threshold level may be configured for the URLLC UE, and UE with URLLC data may be allowed to transmit immediately, even if the channel is considered occupied otherwise. gNB can then detect this UL transmission and identify the UE.

Figure 1 Release of UE Initiated COT

Upon detecting the high priority transmission, gNB may send a PDCCH to cancel a low priority UE’s transmission and/or release the corresponding UE initiated COT (if it is not a scheduled transmission). To avoid the case in which the gNB may need to repeatedly cancel the UE initiated COT, as the UE with configured grant transmission may successfully obtain the channel again at its next FFP, it is proposed that once a UE initiated COT is released by gNB, the low priority UE may not initiate another COT for the same transmission/service until gNB reschedules its UL transmission.

Proposal 2: gNB may send a PDCCH to cancel a low priority UE’s transmission and/or release the corresponding UE initiated COT in order to support another high priority URLLC transmission.

Proposal 3: Once a UE initiated COT is released by gNB, the UE may not initiate another COT for the same transmission/service until gNB reschedules its UL transmission.

We share the view that for URLLC in unlicensed band, the controlled environment may only contain devices operating on the unlicensed band installed by the facility owner. There may be little need for other RAT from the perspective of coexistence. Even when the coordination among networks is needed, gNB can support this by releasing the UE initiated COT for configured grant and by not configuring/scheduling the UL transmission, right before the next gNB FFP. Therefore, we think the UE initiated COT should be allowed to transmit during the idle period of FFP associated with the serving gNB.

Proposal 4: Support dynamic UE COT Release to stop UE initiated UL transmissions during the idle period of gNB FFP if needed.

# 4 Harmonization of UL Configured Grant

For harmonization of UL configured grant, we propose that the operation is based on combined URLLC and NR-U CG features without new RRC parameters:

•	Option 3: CG-UCI based procedures are supported for unlicensed. CG-DFI based procedures are enabled or disabled for unlicensed using existing RRC parameter i.e. cg-RetransmissionTimer-r16

Procedures based on CG-UCI can rely on UE including CG-UCI in CG PUSCH at least as in Rel-16 where the values of the respective fields of CG-UCI are decided by UE. Procedures based on CG-DFI can rely on automatic re-transmission on CG configuration and reception of CG downlink feedback information (DFI) in DCI for re-transmissions.

Proposal 5: Support the operation based on combined URLLC and NR-U CG features without new RRC parameters.

# 5 Conclusion

In this contribution, we discussed issues related to UE initiated COT for FBE, Release of UE initiated COT and harmonization for UL configured grant, and proposed the following:

Proposal 1: Predetermined rules are supported for both configured transmission and scheduled transmission.

Proposal 2: gNB may send a PDCCH to cancel a low priority UE’s transmission and release the corresponding UE initiated COT in order to support high priority URLLC transmission of another UE.

Proposal 3: Once a UE initiated COT is released by gNB, the UE may not initiate another COT for the same transmission/service until gNB reschedules its UL transmission.

Proposal 4: Support dynamic UE COT Release to stop UE initiated UL transmissions during the idle period of gNB FFP if needed.

Proposal 5: Support the operation based on combined URLLC and NR-U CG features without new RRC parameters.

# References

[1] Chairman’s notes for RAN1#104e