3GPP TSG RAN WG1 #104b-e			R1-2102826

e-Meeting, April 12th – 20th, 2021

Source:	Mitsubishi Electric

Title:	Inter-UE coordination for enhanced resource allocation

Agenda Item:	8.11.1.2 - Feasibility and benefits for mode 2 enhancements

Document for: 	Discussion/Decision

# Introduction

The Rel.17 work item on NR sidelink enhancements, approved in RAN#86 meeting, was updated in RAN#90 [1]. One of the objectives of the updated work item is resource allocation enhancements, and more specifically

- Study the feasibility and benefit of solution(s) on the enhancement(s) in mode 2 for enhanced reliability and reduced latency in consideration of both PRR and PIR defined in TR37.885 (by RAN#91), and specify the identified solution(s) if deemed feasible and beneficial [RAN1, RAN2]

  - Inter-UE coordination with the following.

    - A set of resources is determined at UE-A. This set is sent to UE-B in mode 2, and UE-B takes this into account in the resource selection for its own transmission.

  - Note: The solution should be able to operate in-coverage, partial coverage, and out-of-coverage and to address consecutive packet loss in all coverage scenarios.

In RAN1#102-e meeting [2], high level concepts for mode 2 reliability enhancements were discussed by email in thread [102-e-NR-SL_enh-02], without reaching any agreements. In RAN1#103-e [3], it was concluded to classify inter-UE coordination schemes into three types, depending on the type of resources sent by UE-A to UE-B: preferred by UE-A for UE-B’s transmission, not preferred by UE-A for UE-B’s transmission, or where resource conflict is detected. Moreover, it was concluded that for the schemes of inter-UE coordination identified as feasible/beneficial, at least the following aspects are further discussed.

    - How/when UE-A determines the contents of ”A set of resources”, including consideration of UL scheduling

    - When UE-A sends ”A set of resources” to UE-B, including which UE(s) sends it

    - How UE-A and UE-B are determined

    - How UE-A sends ”A set of resources” to UE-B, including container used for carrying it, implicitly or explicitly or both

    - How/when/whether UE-B receives “A set of resources” and takes it into account in the resource selection for its own transmission

    - How/whether to define the relationship between support/signaling of inter-UE coordination and cast type

In the latest RAN1#104-e meeting [4], RAN1 evaluated different flavors of the three types of sets of resources and concluded that inter-UE coordination in Mode 2 is feasible, and is beneficial (e.g.,  reliability, etc.) compared to Rel-16 Mode 2 RA, and thus recommends specification of the feature.

RAN1’s conclusion together with observations from evaluation results were sent to RAN#91e plenary, who concluded that no WID update is necessary. WGs continue specifying inter-UE coordination. Note that enhancements other than inter-UE coordination is NOT pursued in the scope of the objective “Study the feasibility and benefit of solution(s) on the enhancement(s) in mode 2 for enhanced reliability and reduced latency in consideration of both PRR and PIR defined in TR37.885 (by RAN#91), and specify the identified solution(s) if deemed feasible and beneficial [RAN1, RAN2].

In this contribution, we give our view on inter-UE coordination, focusing on inter-UE coordination for mode 2 reliability enhancements.

# Discussion

In a V2X sidelink communication scenario, inter-UE coordination may be beneficial in several scenarios.

Sending sensing related assistance from the RX to the TX can help to mitigate the hidden node and the half duplex problems and may be used to achieve power saving.

Figure 1 - General principle of RX assistance, exemplified for the hidden node problem

## On different categories of “a set of resources”

The work item description generally mandates that a “A set of resources is determined at UE-A, […] sent to UE-B in mode 2, and UE-B takes this into account in the resource selection for its own transmission”, and RAN1 identified 3 types of possible sets of resources. The selection of the type of resources to be signaled has an important impact on the performance of the resource allocation scheme.

Let us suppose that only preferred resources (type A) are signaled, that is that UE-A sends to UE-B the set of resources decided as being unoccupied from UE-A perspective based e.g. on its own sensing. Limiting the number of resources to be signaled is an obvious need in order to keep related signaling under a reasonable limit. UE-B might perform its own sensing, or take into account several reports from different UEs (for example in a multicast scenario the intersection between the preferred lists of the transmitter (UE-B) and of the receiver (UE-A) often falls below 20% of the selection window resources. This leads to applying the RSRP thresholding step in NR V2X mode 2 resource allocation at the transmitter side (UE-B). This RSRP thresholding step often overwrites the receiver assistance report since the RSRP thresholding is done at the transmitter side only (UE-B side). Moreover, for aperiodic traffic, the reevaluation step together with the RSRP thresholding step can almost overwrite all the receiver assistance reports, rendering the assistance information useless and reducing the potential gain coming from inter-UE coordination.

Observation 1: Inter-UE coordination based on type-A “preferred list only” is inefficient, because coordination information is systematically overwritten whenever RSRP thresholding /re-evaluation steps in V2X mode 2 resource allocation are necessary.

From that perspective, using a list of non-preferred resources alleviates the issue. Another reason for which non-preferred list reporting is interesting is because half duplex issues at the receiver side (resources onto which UE-A is not able to receive because it already decided to use them for its own transmission) can simply be reported as “non-preferred” resources. UE-B doesn’t need to know the reason for which a resource is non-preferred (half duplex issue, detected interference, past or future resource conflict, etc). A unified reporting procedure can be used regardless of the target that wants to be achieved (hidden node, half duplex at the receiver side, etc…).

Observation 2: Reporting the list of non-preferred resources allows unified reporting procedure regardless of the reason why the resource is not preferred.

Observation 3: Past and/or future resource conflict is a particular case of non-preferred resources.

If UE-B is informed about non-preferred resources only, and if after the non-preferred resource exclusion step the pool of candidates is still fairly large, UE-B cannot identify which are the best resources for transmission and may choose resources which are fairly interfered from UE-A point of view even when non-interfered resources from UE-A point of view are available. In this sense, in some scenarios there is some merit in also signaling preferred resources, especially in unicast (or groupcast with small number of users).

Either “preferred” or “non-preferred” resources are to be interpreted by UE-B as a recommendation; UE-B will still have the possibility to override such recommendation in order to avoid blocking situations.

Concerning type-C technologies, they were defined by the group as a category of technologies where “UE-A sends to UE-B the set of resource where the resource conflict is detected”. While the generic principle is already covered by the definition of type B, some proposed methods (such as sending some collision flag without identifying the collided resource) cannot really qualify as “sending a set of resources” as mandated by the WID. The usefulness of having such type-C schemes when compared with what is already covered by type-B schemes is uncertain. These approaches do not give any information on the number of conflicted resources, their identity, or the number of UEs impacted by the potential conflict. The potential gain is limited to the very short communication range in which a very limited number of UEs lie. When considering less limited ranges, the probability of having at least one UE detecting a potential conflict on any selected resource is highly increasing, which strongly limits the interest of such methods.

Proposal 1: “A set of resources” signaled by UE-A as assistance information consists of either non-preferred or a mix of preferred and non-preferred resources from UE-A perspective, possibly with ordered prioritization, that UE-B is recommended to avoid/use.

## On assistance for unicast transmission

Let us take the example of the hidden node problem. It is intuitive to say that the number of nodes hidden to the TX that a certain RX experiences collision from is dependent on the TX-RX distance. Indeed, when TX and RX are close together, they have similar sensing information. When the RX is in the outer part of the communication range, the sensing information at the receiver side can be significantly different from the one at the transmitter side, and in this case it is expected that receiver assistance information is useful to the transmitter in order to reduce the collision probability. When the TX-RX distance approaches or surpasses the communication range, the interference becomes a secondary issue since the link budget vanishes anyhow, so trying to solve the hidden node problem would probably not bring too much benefit at system level.

Our unicast evaluations in Figure 2 show the potential PRR gains that can be achieved by using RX assisted scheduling instead of TX scheduling for unicast transmissions with periodic traffic and when the TX has full knowledge of the RX sensing.

Figure 2 – PRR in function of TX-RX distance with and without assistance information for unicast with periodic traffic, full duplex

The simulation assumptions are summarized in the appendix. The corresponding communication range is around 420m. UE-A sends to UE-B the ordered list of non-preferred resources based on RSRP criterion. UE-B excludes them from its candidate list. In the case of blocking situation (not enough remaining resources), RSRP-based thresholding at UE-B may re-integrate some of the excluded resources in the inverse order from the ordered list provided by UE-A. A gain of up to ≈10% can be achieved in the simulated scenario, different results can be obtained with other parameters. It is clear that the PRR gain of the RX assisted scheme increases with the communication distance, until a certain limit where the communication becomes noise limited rather than interference limited; when the distance approaches the communication range, both schemes suffer mainly from the pathloss effect and exhibit similarly bad PRR. The gain of RX assisted scheduling over TX scheduling mainly comes from overcoming the hidden node effect loses in the interference-limited scenarios, where the advantage lies in predicting and eliminating hidden nodes.

Observation 4: Receiver-assisted TX scheduling can achieve 10% PRR gain with respect to TX autonomous scheduling in unicast scenarios.

## On assistance for groupcast/broadcast transmission

As a first aspect, assistance should not be restricted to a cast type, since all cast types can achieve benefits with inter-UE coordination methods.

Proposal 2: Support inter-UE coordination for all cast types.

### On overhead control for inter-UE coordination for multicast/broadcast

Systematically sending assistance info from all users on a regular basis is counterproductive and overhead consuming.

Figure 3– Illustration figure of assistance information provided by a set of selected UEs from a multicast group with different selection ranges

Reasonable criteria of selecting which UEs are fit to act as UE-A and of the events which trigger the sending of the assistance information are needed. It is intuitive to deduce in Figure 3 that, with a full-duplex assumption, orange ”UE-A”-type UEs close to the blue UE-B Tx would provide redundant assistance with respect to what UE-B is already sensing. Groups of UEs (e.g. groups of green UEs) in the outer communication detect similar hidden nodes and need not simultaneously report assistance information. A selection criterion is needed.

Reasonable selection criteria can include, for example, distance-based criteria, since UEs close to each other will produce similar sensing results, at least for the full duplex case. For example, only users in zones situated at a distance between configurable dmin and dmax distances from UE-B provide assistance, possibly with further down-selection to one user per zone. This is particularly beneficial to eliminate hidden node problems all in keeping the associated overhead low.

Time interval restrictions may be applied on top of that to address half duplex issues. For example, a first UE selection is performed based on distance as here-above. Among the users in zones closer than dmin from UE-B, only UEs able to sense during blind intervals of UE-B are allowed to send assistance information. Such a strategy mitigates both half duplex issue at the transmitter and hidden nodes problem.

Observation 5: In groupcast/broadcast inter-UE coordination, a selection mechanism needs to be introduced to select few assisting UEs (UE-As) to avoid excessive overhead and signaling loads.

Proposal 3: Investigate methods achieving candidate selection of UEs to transmit assistance information (e.g. based on UE-B request, on distance between UE-A and UE-B, on timing restrictions).

### Performance evaluation of inter-UE coordination in multicast

As previously discussed, and as it can also be seen from Figure 4 and Figure 6, few users reasonably selected can provide the most important part of the significant assistance information.

One strategy of selecting users is to pick them in function of the distance from the transmitter UE, in a similar manner as for Rel.16 distance based HARQ. In the following dmin and dmax are understood as normalized with respect to the communication range. In our proposed simulation scenario, the communication range is ≈420 m. For example, with dmin=0.5 and dmax=1.1, UEs in the outer ring of the communication range at least 0.5*communication range away from the transmit UE are selected to provide assistance. We use dmax=1.1*communication range to account for the vehicles’ displacement in the 1 second between each groupcast formation, this displacement in 1s is ≈10%  of the communication range. In the chosen highway scenario, and since dmin is usually higher than the lane width, this is equivalent to choosing some front vehicles and some rear vehicles at least dmin and at most dmax away from the transmitter.

For example, in Figure 4, 5 vehicles are selected (the figure takes into account the fact that the communication range is significantly higher than the road width). With the numerical values simulated in Figure 7 and Figure 8, among 168UEs in the drop ≈35 UEs are part of the multicast group. On average, ≈35 are in the communication range at a given time. The strategy (dmin,dmax)=(0.25,1.1) selects on average 27.5UEs to send assistance info, and (0.5, 1.1) selects in average 18.5UEs . The strategy (0.75,1.1) selects on average 7.6UEs to send assistance info, and (0.9, 1.1) selects in average 3.5UEs. In order to have a better UE distribution, we also simulated the following cases:

- “2 UEs (1 front – 1 back”): (dmin, dmax) = (0.5,1.1) for a first selection step, and we further randomly select one front UE and one rear UE in a second selection step, as in Figure 5;

- “4 UEs (2 front – 2 back)”: 4 vehicles selection by picking one front and one rear vehicle in each range from (dmin, dmax) = (0.5, 0.75) and (0.75, 1.1), as in Figure 6.

Figure 4 – Illustration figure of assistance information provided by a set of selected UEs from a multicast group with different selection ranges

Figure 5 – Illustration figure of assistance information provided by a set of selected UEs from a multicast group with different selection ranges – 2 UEs (1 front – 1 back)

Figure 6 – Illustration figure of assistance information provided by a set of selected UEs from a multicast group with different selection ranges – 4 UEs (2 front – 2 back)

We evaluated the different strategies of user selection to provide assistance. The extreme cases where TX UE has either no assistance or has full assistance information from all UEs in the multicast group are presented as reference.

Figure 7 – PRR in function of TX-RX distance with assistance information provided by a set of selected UEs from a multicast group with different selection strategies, full duplex

Figure 8 – PRR in function of TX-RX distance with assistance information provided by a set of selected UEs from a multicast group with different selection strategies over different delays, full duplex

Our multicast evaluations in Figure 7 show the potential PRR gains that can be achieved by using RX assisted scheduling instead of TX scheduling for groupcast/multicast transmissions with periodic traffic and when the TX has full knowledge of the RX sensing. In our simulation, the selected UE A candidates share an assistance report encompassing a prioritized list of non-preferred resources to the TX (UE B). This list of non-preferred resources is determined based on a given (predefined/pre-configured) RSRP threshold. Then, the ordering of prioritization of such non-preferred resources list is done based on the received RSRP. When the TX (UE B) receives these assistance reports from the candidate UE As, it carries out a second round of prioritization of the non-preferred resources based on the number of occurrences of such resource in the received assistance reports. In such a way, a resource which is declared as non-preferred resource in multiple assistance reports is to be excluded with a higher probability than a resource declared as not preferred in one report.

The list of receiver UEs from the multicast group is updated every 1 second to keep only UEs within the communication range as RXs UEs. The simulation assumptions are summarized in the appendix. Only a part of the users in the multicast group provide assistance information to UE-B. UE-A candidate selection is performed on distance-based criteria: only UEs lying in zones at a distance between dmin and dmax from the transmitter (UE-B) are selected as UE-A candidates.

Results show that RX assistance is also beneficial to the multicast case. Although the attainable gain is a bit inferior to the unicast case, assistance can still provide around ≈5-7% PRR gain. Having assistance information from a large number of users is overhead consuming. An overhead analysis was performed in our previous contribution [6]. Distance based selection of UE candidates to act as UE-A allows to get significant gain with reduced overhead. Selecting only 4 UEs in a well distributed manner procures almost the full amount of the achievable gain as shown in the results in Figure 7. This is intuitively explained in the setting in Figure 6, where it can be seen that 4 selected UEs can in some cases provide the full amount of significant assistance information for e.g. hidden node discovery.

Observation 6: Receiver-assisted TX scheduling can achieve 5-7% PRR gain with respect to TX autonomous scheduling in multicast scenarios.

Observation 7: Distance based selection of UE candidates to act as UE-A allows to get significant gain with reduced overhead. Selecting 4 UEs over different directions and distances out of all the users procures almost the full amount of the achievable gain.

Proposal 4: Support distance/zone-based candidate selection of UEs to transmit assistance information in a multicast/broadcast scenario.

In Figure 8, we present the results of the assistance-based multicast scenario when the latency in delivering the assistance is simulated in a realistic manner. We assume that a possible delay for assistance report can be N=1,2, or 4 slots, in a similar approach to Rel.16 based HARQ feedback, in order to account for possible delays that may arise due to assistance container limitations and RXs (UE As) processing. Also, the assistance report is decoded at the TX (UE B) side using a given MCS as shown in the simulation assumptions in the appendix. It is shown in Figure 8 that, in case only 4 UEs are selected as UE A to send assistance reports to the TX (UE B), although performance is declining with the latency increase (because the assistance information becomes outdated), a delay of up to 4 slots still shows gain over the case where no assistance is used.

Observation 8: The gain achieved by assistance in multicast scenarios depends on the delay of sending such assistance reports. A latency of up to 4 slots still provides gain over the case where no assistance is used.

# Conclusions

The contribution investigates inter-UE coordination, focusing on inter-UE coordination for mode 2 reliability enhancements. Based on our analysis and simulation results, we arrived at the following conclusions:

Observation 1: Inter-UE coordination based on type-A “preferred list only” is inefficient, because coordination information is systematically overwritten whenever RSRP thresholding /re-evaluation steps in V2X mode 2 resource allocation are necessary.

Observation 2: Reporting the list of non-preferred resources allows unified reporting procedure regardless of the reason why the resource is not preferred.

Observation 3: Past and/or future resource conflict is a particular case of non-preferred resources.

Observation 4: Receiver-assisted TX scheduling can achieve 10% PRR gain with respect to TX autonomous scheduling in unicast scenarios.

Observation 5: In groupcast/broadcast inter-UE coordination, a selection mechanism needs to be introduced to select few assisting UEs (UE-As) to avoid excessive overhead and signaling loads.

Observation 6: Receiver-assisted TX scheduling can achieve 5-7% PRR gain with respect to TX autonomous scheduling in multicast scenarios.

Observation 7: Distance based selection of UE candidates to act as UE-A allows to get significant gain with reduced overhead. Selecting 4 UEs over different directions and distances out of all the users procures almost the full amount of the achievable gain.

Observation 8: The gain achieved by assistance in multicast scenarios depends on the delay of sending such assistance reports. A latency of up to 4 slots still provides gain over the case where no assistance is used.

Therefore, we propose the following:

Proposal 1: “A set of resources” signaled by UE-A as assistance information consists of either non-preferred or a mix of preferred and non-preferred resources from UE-A perspective, possibly with ordered prioritization, that UE-B is recommended to avoid/use.

Proposal 2: Support inter-UE coordination for all cast types.

Proposal 3: Investigate methods achieving candidate selection of UEs to transmit assistance information (e.g. based on UE-B request, on distance between UE-A and UE-B, on timing restrictions).

Proposal 4: Support distance/zone-based candidate selection of UEs to transmit assistance information in a multicast/broadcast scenario.

# References

### RP-202846, WID revision: NR sidelink enhancement, RAN#90, December 2020.

### RAN1#102-e Chairman’s notes, August 2020.

### RAN1#103-e Chairman’s notes, October/November 2020.

### RAN1#104-e Chairman’s notes, January/February 2021.

### RAN1#91b Chairman’s notes, October 2018.

### R1-2005903, Inter-UE coordination for enhanced resource allocation, Mitsubishi Electric, RAN1#102-e, August 2020.

# Annex: Simulation assumptions

| Parameter | Value |
| --- | --- |
| Deployment scenario | - Highway Option A scenario [TR 37.885] - Vehicle speed= 140 km/hr |
| Channel model | NR V2X channel model (LoS and NLoSv components) [TR 37.885] |
| Vehicle density | 7 vehicles/km/lane |
| Spectrum allocation | - Carrier frequency: 6 GHz - Simulated bandwidth: 20 MHz |
| Vehicle type | Type 2 [TR 37.885] (omnidirectional antenna) |
| Subcarrier spacing | 15 KHz |
| Communication mode | Unicast, Groupcast |
| UE-to-UE association (Unicast) | - UEs are randomly paired, within the transmitter communication range initially - Each UE may participate only in a single pair - Only one UE in each pair transmit data |
| UE-to-UE association (Groupcast) | - A given Tx UE selects all UEs, dynamically within its communication range, to be receivers for its groupcast transmissions |
| Traffic model | Periodic variable packet size traffic [TR 37.885]: - Packet size: [800,1200] Byte with probabilities [0.8,0.2] respectively - Inter-packet arrival time: 20 ms - Latency requirement: 20 ms |
| TTI structure | NR slot: 10 symbols for data, 4 symbols total overhead (DMRS, SCI, etc.) |
| Sidelink control TX parameters | - 64 bits - QPSK modulation, polar coding |
| SCI/Data frequency resource allocation | - PSCCH: 5 PRB - PSSCH: 25 PRB (per sub-channel) |
| SCI/Data time resource allocation | - PSCCH: 2 symbols - PSSCH: 10 symbols |
| Data packet TX parameters | - 800 Byte packet: 16-QAM, LDPC (CR=0.5) - 2 sub-channels - 1200 Byte packet: 16-QAM, LDPC (CR=0.5) - 2 sub-channels |
| Assistance Parameters | - 16-QAM, LDPC (CR=0.5) |
| HARQ feedback | - Disabled |
| Resource selection | - Semi-persistent resource allocation (Similar to LTE mode 4 sensing and selection, with considering the NR additional procedures) |