3GPP TSG RAN WG1 #104-bis-e	R1- 2102830

e-Meeting, April 12th – April 20th, 2021

Agenda item:      8.14.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Potential enhancements for better XR support over NR

Document for:     Discussion

# Introduction

RAN#91 approved a revised SID on XR Evaluations for NR [1]. The XR SI is now ongoing with focus on related system-level performance evaluation to assess how well the currently standardized NR system can support such services. In addition to those ongoing performance evaluations, we want to also slowly start discussing potential areas of improvement to have better XR support over NR. The purpose of this contribution is therefore to raise potential areas of XR improvements for NR that may be further discussed as the SI progresses.

# Semi-persistent scheduler enhancements

SPS offers the advantage that it reduces the overhead as compared to sending DL dynamic scheduling​ grants (i.e. reduced PDCCH overhead). In addition to the lower PDCCH overhead, use of SPS also helps offload the computational burden from the busy dynamic gNB MAC scheduler. Several DL SPS enhancements where introduced in NR Rel-16, mainly with the purpose of supporting URLLC and TSC use cases with transmission of small payloads. But, it is clear that SPS in its current form is not well suited for XR use cases. In short, the main characteristics of NR Rel-16 DL SPS are:

- SPS is a method where DL radio resources for sending one transport block with a regular time-periodicity is configured for a UE.

- Up to 8 simultaneous active SPS configurations can be configured for a UE (configured through RRC signaling) ​

- Periodicity of any integer of a slot (N*14). ​Minimum periodicity in Rel-15 is 10 ms​.

- Separate configuration (RRC-based) and activation/deactivation (PDCCH addressed to CS-RNTI can either signal and activate the configured downlink assignment, or deactivate it)​.

As reported by several companies in RAN1#104-e, it is evident that current SPS (with integer periodicity) does not support configurations that match the considered values of 60 fps, 90 fps, and 120 fps. Secondly, it is evident that the payloads coming with XR services are rather large; if the average source data rate e.g. equals 45 Mbps with 60 fps, the average payload size per frame equals 750 kbit. This is so large that in many cases will need to be transmitted to the UE in multiple transport blocks. It is therefore desirable to consider SPS enhancements, where each SPS periodicity include resources for transmission of an integer number of transport blocks. Those may be time-domain consecutive resources. Or, if a UE is in Carrier Aggregation (CA) mode, consider enhanced SPS configurations where a single SPS config include transmissions on multiple Component Carriers (CC). Furthermore, it should be recognized that in reality, XR applications may use adaptive encoding schemes where the source data rate (and potentially the number of frames per second) may be occasionally changed. For addressing such cases, it may be worth investigating solutions, where an existing SPS configuration could be (semi-)dynamically reconfigured on the fly, without having to fully release an existing one and setup of a new SPS configuration.

Observation 1: It is recommended to further study potential SPS enhancements to better accommodate XR services. Possible enhancements to consider may include support of broader set of periodicities, support for more than one transport block transmission per periodicity, and dynamic change of an existing SPS configuration. Other SPS enhancements may also be considered.

# Enhancements related to FR2 operation

In line with the current Rel-17 SI XR agreements, the XR performance will also be evaluated for FR2 at the 28 GHz band. Notice that this include cases where UEs are equipped with multiple antenna panels, and assumptions where the UE may only be able to measure (and Tx/Rx) on a single panel at a time. The UE selects it best antenna panel for Tx/Rx with its serving cell based on local RRM (RSRP) measurements. However, for FR2 operation, it is important to notice that there are additional scheduling restrictions (due to RRM measurements) that may challenge the XR performance unless some enhancements are introduced. As per the current NR specifications, the network configures the UE when to measure RSRP from e.g. SSBs by means of RRC signalling of the so-called SMTC (see section 5.5.2.10 in 38.331). The time-resolution of SMTC is on subframe level, corresponding to 1 ms intervals, while scheduling of latency sensitive (e.g. XR and URLLC/TCS) payloads typically happen on TTI resolutions of 0.1-0.5ms. It should be noted that the SMTC only instruct the UE when (in time domain) it could/should measure RSRP, while it is left completely open for UE implementation exactly when to measure, and which antenna panel to be used for conducting such measurement during those “SMTC measurement windows”. Scheduling restrictions apply for the UEs during time-intervals where it may be performing RSRP measurements as per the SMTC configuration appears in 38.133, Section 9.5.6.3. In particular for FR2 and L1-RSRP on SSB, “The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS…..”. Secondly, scheduling restrictions / RSRP measurements does not mention anything related to how/when the UE shall/may measure on different antenna panels, and when to only measure on its currently selected best panel. Scheduling of users during time-periods of SSB transmissions (and UE measurements) can likely not be avoided without violating the QoS constraints for latency critical services such as XR (and URLLC/TSC). It shall therefore be considered to introduce options where the gNB can configure a UE to prioritize decoding of critical PDCCH/PDSCH transmissions from its serving cell, even if colliding with SMTC windows where the UE may perform RSRP measurements. This kind of configuration for the UE also serves as instructing to the UE to prioritize listening to its current best antenna panel for reception from its serving cell during such time-instances, so the UE does not switch panels at those times e.g. for RRM measurements.

Observation 2: Further consider the impact on XR performance in FR2 from SMTC scheduling restrictions, incl. options where the gNB may configure a UE to prioritize decoding of critical PDCCH/PDSCH transmissions from its serving cell, even if colliding with SMTC windows where the UE may perform RSRP measurements. Instructing the UE to prioritize listening to its current best antenna panel for Rx from its serving cell during such time-instances.

Moreover (although not currently part of the first phase of the XR simulation agreements), it would be beneficial to also validate that proper XR performance can be guaranteed for use cases where the CA between FR1 and FR2 is applied. Considering cases where the most critical RAN control messages (e.g. RRC and MAC-CE messages for beam management, etc.) is conveyed via FR1, and also the XR application data are divided between FR1 and FR2; considering cases where e.g. different payloads are directed to different CCs (e.g. in accordance with experienced quality) to most efficiently protect the most vulnerable packets. The starting point may be to assume PCell at FR1 (i.e. mobility anchor) and the SCell on FR2, considering potential differences in gNB GoB configuration for the two CCs and UE antenna pattern(s). As an example, enhancements that may involve improved time-synchronicity actions such that e.g. beam-switching is conducted at different times for two CCs as there are also some uncertainties and risks involved in gNB beam-switches.

Observation 3: Identify how to most efficiently accommodate XR for UEs with CA between FR1 and FR2, including aspects of most efficient resource allocation and transmission of RAN user- and control-plane messages, as well as policies for handling different payloads, on the different CCs. Including aspect of dynamic MAC-based beam management and RRC-based mobility and SCell configuration/release actions, involving synchronicity between actions for the CC.

# QoS and Related Layer-2 Enhancements

## QoS Enhancements

Several XR services such as Virtual Reality (VR), Augmented Reality (AR), and Mixed Realty (MR) use pose information (also known as 6DoF information) to minimize data rate by sending only the visible field of view. Pose information is composed of position (x, y, z) and rotation (yaw, pitch, roll) information. Data rate is adapted according to the accuracy of the pose information. Higher 3D information is transmitted to compensate the pose/6DoF error. Therefore, the lower the pose/6DoF accuracy is, the higher the data rate generated by the XR application is. See illustration in Fig. 1.

Figure 1 – Simple illustration of impact of 6DoF error and impact on quality and data rates.

In [2], it was illustrated the relationship between the 6DoF error and bitrate by evaluating the ratio between the information needed to cover all possible displacements and rotations of the Field of View (FOV) due to the 6DoF error and the information covered by the volume of the actual FOV. We call this metric Information Ratio, since it approximates the amount of extra information needed to encode the non-visible 3D space due to the 6DoF error. Results of the information ratio as a function of error on positioning and the angle of rotation are illustrated in Fig. 2. The figure shows that the Information Ratio varies between 2 to 12 for relative positioning error between 5% to 30% of the horizontal FoV and error of the angle of ration between 5 to 30 degrees. For example, with positioning and rotation errors as low as 5% and 5°, the network should expect a bit rate for the XR connection twice as large as the bitrate of the connection without 6DoF error. The bitrate can increase up to a factor 12 in the case of positioning error and rotation errors as large as 30% and 30°. For the RAN to be able to conduct efficient Radio Resource Management (RRM), including scheduling, it is therefore considered useful to have some a priori information on the 6DoF error. This new 6DoF error information is provided to the network by the UE or the Application Function (AF) during the Session Initiation Procedure or Session Modification Procedure. It can be observed that there exists an interplay between the 6DoF error and other QoS characteristics like PER, PDB, and guaranteed bitrate. As an example, a low PER limits the maximum 6DoF error since the additional bitrate needed to transmit the extra-information to compensate the 6DoF error may not be achievable in certain radio conditions.

Figure 2 – Information Ratio for different values of position error (x-axis) and angle or rotation (curves). The information ratio represents the multiplier of the bitrate due to the 6DoF Error.

Observation 4: 6DoF error and QoS characteristics like Packet Delay Budget (PDB) and Packet Error Rate (PER) depend on each other. For example, a low PER limits the maximum 6DoF error since the additional bitrate needed to transmit the extra-information to compensate the 6DoF error may not be achievable in certain radio conditions.

Observation 5:  Consider including the 6DoF error as a new QoS characteristic for QoS Flows and 5QIs defined for interactive XR and gaming services as this is useful information for the gNB RRM decisions, including scheduling decisions.

## Flow Synchronization in Carrier Aggregation and Multi-Connectivity

XR applications generate multiple types of frames and multiple data streams (such as data and control streams) within the same flow. Different frames and streams may have different QoS requirements. For example, the control stream may typically have much lower data rate, but tighter latency and Packet Error Rate (PER) requirements. Similarly, different type of foveal image tiles or directional sound streams may have different requirements in term of PER and bitrate. For example, tiles in the central/near, mid, and far peripheral FoV of the user have different bitrate and packet error rate requirements given their importance for the final rendered image. Different frames and data streams generated by the same XR application need to be received approximately at the same time, since they need to be processed together to give the user the impression of seamless experience when the 3D media content is displayed to the final user. In contrast, if some frame or packet is delayed, the entire 3D media processing is delayed, and the final user may experience glitches in the multimedia stream.

Multi Connectivity (MC) can be used to bundle radio resources by splitting the data radio bearer (DRB), thus increasing the 5G system capacity. However, possible delay differences between multiple legs affects the end-to-end latency. For example, the path with higher delay may become the bottleneck due to PDCP reordering. Hence, delivery time of different frames and/or data streams in Multi Connectivity (MC) should be synchronized to a certain extent in order to ensure acceptable Quality of Experience (QoE) for XR, such as e.g. online game streaming applications over 5G-NR. For example, packets of video and audio streams need to be delivered to the application within the same deadline to avoid jitter between video and audio which affects the user QoE.

Observation 6: XR applications may generate multiple types of frames and/or multiple types of data streams within the same flow with different characteristics and QoS requirements. Different frames and/or streams of the same XR flow may be transmitted on different carriers (in case of CA) and/or different paths (in case of MC) and experience different delay. Therefore, proper synchronization of multiple frames and/or multiple data streams generated by the same XR application should be ensured.

# Summary

In this contribution we have discussed various directions of possible enhancements that may further help improve the XR performance of 5G NR. Those naturally require further studies and discussions. In summary, those are captured by the following observations:

- Observation 1: It is recommended to further study potential SPS enhancements to better accommodate XR services. Possible enhancements to consider may include support of broader set of periodicities, support for more than one transport block transmission per periodicity, and dynamic change of an existing SPS configuration. Other SPS enhancements may also be considered.

- Observation 2: Further consider the impact on XR performance in FR2 from SMTC scheduling restrictions, incl. options where the gNB may configure a UE to prioritize decoding of critical PDCCH/PDSCH transmissions from its serving cell, even if colliding with SMTC windows where the UE may perform RSRP measurements. Instructing the UE to prioritize listening to its current best antenna panel for Rx from its serving cell during such time-instances.

- Observation 3: Identify how to most efficiently accommodate XR for UEs with CA between FR1 and FR2, including aspects of most efficient resource allocation and transmission of RAN user- and control-plane messages, as well as policies for handling different payloads, on the different CCs. Including aspect of dynamic MAC-based beam management and RRC-based mobility and SCell configuration/release actions, involving synchronicity between actions for the CC.

- Observation 4: 6DoF error and QoS characteristics like Packet Delay Budget (PDB) and Packet Error Rate (PER) depend on each other. For example, a low PER limits the maximum 6DoF error since the additional bitrate needed to transmit the extra-information to compensate the 6DoF error may not be achievable in certain radio conditions.

- Observation 5:  Consider including the 6DoF error as a new QoS characteristic for QoS Flows and 5QIs defined for interactive XR and gaming services as this is useful information for the gNB RRM decisions, including scheduling decisions.

- Observation 6: XR applications may generate multiple types of frames and/or multiple types of data streams within the same flow with different characteristics and QoS requirements. Different frames and/or streams of the same XR flow may be transmitted on different carriers (in case of CA) and/or different paths (in case of MC) and experience different delay. Therefore, proper synchronization of multiple frames and/or multiple data streams generated by the same XR application should be ensured.

# References

[1] RP-210460 Revised SID on XR Evaluations for NR, RAN#91

[2] S2-2100665 Positioning Error impact on 5QI