3GPP TSG RAN WG1 Meeting #104b-e                       R1-2102859

e-Meeting, April 12th – 20th, 2021

Source:         ZTE

Title:            Remaining issues on DL TBS increase for eMTC

Agenda Item:    8.9.3

Document for:   Discussion

- Introduction

In RAN1#104 e-meeting, the maximum DL TBS of 1736 bits for CE mode A was discussed. And the following agreements were reached [1]:

- The number of soft channel bits is calculated based on the equation:

- Working Assumption: N=8

- The 1736 bits DL TBS feature is enabled by unicast RRC configuration.

- For a UE configured with “1736 bits DL TBS” and 64-QAM:

- If the UE is signaled with a TBS of up to and including 1736 bits, the UE shall apply the signaled TBS.

- If the UE is signaled with a TBS of greater than 1736 bits, the UE shall apply a TBS of 1736 bits.

In this contribution, we provide our considerations on the remaining issues.

- Discussion

According to the formula of the soft buffer size, the number of HARQ processes and the maximum transport block size impact DL soft buffer size. On number of HARQ processes, three UE capabilities of 8, 10 and 14 HARQ processes are involved for category M1 UEs. As a result, following three options need to be down-selected to calculate soft buffer size for DL TBS of 1736 bits.

- Option 1: Based on 8 HARQ processes and 1736 bits

- Option 2: Based on 10 HARQ processes and 1736 bits

- Option 3: Based on 14 HARQ processes and 1736 bits

The existing DL soft buffer size is calculated based on the maximum TBS of 1000 bits and 8 HARQ processes. Though 10 HARQ processes are introduced in Rel-14, the soft buffer size is not increased to avoid hardware updates. The soft buffer size based on 8 HARQ processes can also be applicable for the UE with 10 or 14 HARQ processes since not all the HARQ processes are used for re-transmission in most cases. Meanwhile, the soft buffer size based on 10 or 14 HARQ processes will cause a waste of complexity and cost for low capability UEs. Thus, we recommend defining new soft buffer size according to low UE capability requirement for category M1 UEs. That is, the soft buffer size is calculated based on 8 HARQ processes and 1736 bits. The working assumption of N=8 can be confirmed. And the number of soft channel bits is calculated as below.

8∙96∙1736+2832=43008 soft channel bits

Proposal: The working assumption of N=8 can be confirmed and 43008 is specified for soft buffer size for DL TBS of 1736 bits.

- Conclusion

In this contribution, we discuss the remaining issues on DL TBS of 1736 bits for Cat. M1 UEs in CE mode A. And the proposal is given as below:

Proposal: The working assumption of N=8 can be confirmed and 43008 is specified for soft buffer size for DL TBS of 1736 bits.

- Reference

- 3GPP, Draft Report of 3GPP TSG RAN WG1 #104-e, v0.1.0