3GPP TSG RAN WG1 #104b-e																																																	R1-2102911

e-Meeting, April 12th – 20th, 2021

Source:	CMCC

Title:	Discussion on CSI feedback enhancements for URLLC

Agenda Item:	8.3.1.2

Document for:	Discussion and Decision

# Introduction

In previous RAN1 meetings[1][2][3], the following agreements of CSI feedback for URLLC were made.

Agreement: - CSI feedback enhancement for Multi-TRP transmission is not to be discussed further under IIoT/URLLC enhancement WI Agreements: - Baseline assumptions are used as the required minimum to be simulated for the evaluation of candidate CSI enhancement schemes - Reuse the assumptions in TR 38.824 and TR 38.901 as a starting point - Companies shall report additional parameters (e.g., CSI measurement settings, CSI reporting schemes) used in their evaluation - FFS details of baseline assumptions - Companies can bring additional simulation results with other set(s) of assumptions Agreements: - Study/evaluate further on following CSI enhancement schemes in terms of technical benefit, specification and implementation impacts. - New triggering methods for A-CSI and/or SRS - New reporting based on one or more of the following: - Case 1: channel/interference measurement for new CSI reporting, considering aspects such as one or more of the following: - Reporting more accurate interference characteristics - Reduced CSI feedback overhead (e.g., reporting interference measurement only) - Enhanced CSI reporting such as WB/SB CQI - Case 2: other measurement (other than channel/interference) for additional information - E.g., PDCCH/PDSCH decoding, recommended HARQ RV sequence, etc. - It targets to help gNB scheduler for better link adaptation of (re)transmission - [Reduced CSI computation time/complexity] - [CSI feedback for PDCCH] - Other CSI enhancement schemes that enable accurate MCS selection are not precluded - Detailed assumptions of the proposed CSI enhancement schemes should be provided by the proponent, such as - Reporting values - Triggering conditions for the reporting - Associated measurement resource - Uplink resource to be used for the reporting - How to use the reported information at the gNB scheduler - CSI-RS overhead and CSI reporting frequency - CSI reporting latency/timeline - Etc. Agreements - No change of CSI processing time relative to Rel-16 CSI in this WI - CSI processing time specific to a new CSI reporting quantity/type (if supported) can be studied Agreement: - For Case-2 new reporting, continue studying with focus on the new reporting type based on PDSCH decoding for OLLA performance enhancement for initial and re-transmissions of PDSCH. Agreements: For Case-1 New reporting, the following candidate schemes have been identified to address the fast interference change over time. Continue studying with focus on the identified schemes below for further study and evaluation. - Scheme 1a: New reporting quantity based on CQI/SINR statistics, e.g., - CQI/SINR statistics (e.g., mean, variance, etc.) - CSI prediction - Scheme 1b: New reporting quantity of interference statistics (e.g., mean, variance, interference covariance matrix, etc.) - Scheme 1c: New reporting quantity based on modifying existing reporting format, e.g., - CQI reporting considering the worst subbands - Subband CQI granularity enhancement - Scheme 1d: New reporting quantity related to CSI expiration time - Scheme 1e: New reporting quantity with partial information update, e.g., - CSI reporting with interference update only Companies are encouraged to investigate the above schemes, aiming for down-selection in RAN1#104-e Conclusion: Continue evaluation of new reporting Case 1 and Case 2 for the schemes identified in Appendix B of R1-2102131. - Companies are encouraged to provide their views on each scheme against each criterion in respective Tables in Appendix B. - Companies are encouraged to provide additional evaluation results for as many schemes as possible, based on assumptions agreed in RAN1#102-e. - Aim for down-selection at RAN1#104-b-e by taking into account evaluation results and assessment against criteria from Appendix B.

In this contribution, we provide some discussions on enhancements on new triggering methods for A-CSI.

# Enhancement on New triggering methods for A-CSI

The necessity of CSI feedback enhancement has been discussed in Rel-16 among companies. From our perspective, on one hand, the aperiodic CSI report could be useful for bursty traffic, when traffic arrives between two occasions of periodic CSI report. The periodic occasions may be too early or too late for accurate CSI acquisition. One the other hand, the aperiodic CSI report could be useful for retransmission. The periodic CSI feedback may be more likely to be wideband since the sub-band feedback may require longer time to obtain the CSI of the whole wideband. In this regard, the CSI result may not be accurate enough for the bursty and sub-band scheduled URLLC traffic which then limits the spectral efficiency of the traffic if the reported CSI is too conservative and leads to transmission error if the reported CSI is too aggressive.

In Rel-15 and Rel-16, the aperiodic CSI can only be reported in PUSCH and triggered by UL grant. In order to reduce PDCCH overhead and facilitate efficient retransmission, it has been proposed by some companies to introduce DL grant triggering A-CSI report on PUCCH.

For DL dominant URLLC traffic, trigger A-CSI report by DL grant could save PDCCH overhead and benefit link adaptation. For example, A-CSI report can be triggered by the DL grant scheduling a PDSCH transmission. If retransmission for the PDSCH is needed, the reported A-CSI could be used to determine the MCS parameter for retransmission. If A-CSI can be triggered by DL grant, whether multiplex A-CSI report and HARQ-ACK in the same PUCCH resource or map to different resources should be studied. According to Table 1 and Table 2 (given in TS 38.214), UE CSI computation time is longer than PDSCH processing time especially for sub-band CSI report. If CSI computation time can be reduced to the same level with PDSCH processing time, A-CSI and HARQ-ACK might be multiplexed in the same PUCCH resource. The CSI computation time or complexity is related to the report contents. New CSI reporting content can be considered to reduce the complexity.

Table1-1 PDSCH processing time for PDSCH processing capability 1

Table1-2 PDSCH processing time for PDSCH processing capability 2

Table2-1 CSI computation delay requirement 1

Table2-2 CSI computation delay requirement 2

Proposal 1: DL grant triggering aperiodic CSI feedback should be supported.

- Whether multiplex A-CSI report and HARQ-ACK in the same PUCCH resource or map to different resources should be studied.

- New CSI reporting content can be considered to reduce the complexity.

One important issue of DL grant triggering A-CSI is that the UCI payload size is unknown to UE if the DCI is missed. As shown in Fig.1, the HARQ-ACK for PDSCH transmissions scheduled by DL slot#1~#4 is multiplexed in the same PUCCH resource. The A-CSI reporting is triggered by DL DCI in slot#2, so the payload size for the UCI is the sum of HARQ-ACK for 4 PDSCH transmissions and an A-CSI reporting. If the DCI in slot#2 is missed, the UE would assume the UCI payload size as the HARQ-ACK for 4 PDSCH transmissions. For example, if the DCI in slot#2 is successfully decoded by UE, the PUCCH resource #1 will be used for UCI transmission. But if the DCI in slot#2 is missed, UE may use PUCCH resource to report UCI.

Fig.1 HARQ-ACK and A-CSI multiplexed in the same PUCCH

To solve this issue, one way is to let the gNB do blind detection to obtain the HARQ-ACK information. However, it would result in much of waste of PUCCH resource. Like in Fig.1, both PUCCH resource #1 and PUCCH resource#2 will be reserved. Another way is to add a new field in DCI to represent the numbers of times the A-CSI reporting is triggered. For example, the new field values of DL DCI in slot#1~slot#4 are respectively 0,1,1,1. If the DCI in slot#2 is missed, the UE could know that A-CSI reporting is triggered from the new field value of DL DCI in slot#3 and slot#4. The A-CSI reporting content can be specified so that the payload size of A-CSI reporting is also fixed. In this way, UE would properly determine the PUCCH resource and report HAQR-ACK information.

Proposal 2: To solve the DCI missing issue, support adding a new field in DCI to represent the numbers of times the A-CSI reporting is triggered, and the fixed payload size of A-CSI reporting can be considered.

# Conclusions

In this contribution, we share our views on enhancements for CSI feedback for URLLC. The proposals are summarised as follows:

Proposal 1: DL grant triggering aperiodic CSI feedback should be supported.

- Whether multiplex A-CSI report and HARQ-ACK in the same PUCCH resource or map to different resources should be studied.

- New CSI reporting content can be considered to reduce the complexity.

Proposal 2: To solve the DCI missing issue, support adding a new field in DCI to represent the numbers of times the A-CSI reporting is triggered, and the fixed payload size of A-CSI reporting can be considered.

# References

- 3GPP RAN1#102-e, RAN1 Chairman’s Notes, Aug 2020.

- 3GPP RAN1#103-e, RAN1 Chairman’s Notes, Oct 2020.

- 3GPP RAN1#104-e, RAN1 Chairman’s Notes, Jan 2021.