3GPP TSG RAN WG1 #104-bis-e		R1-2102936

e-Meeting, April 12th – April 20th, 2021

Source:	vivo

Title:	Discussion on CSI-RS Resource counting for UE feature 2-24 and 2-33

Agenda Item:	7.1

Document for:	Discussion and Decision

- Introduction

The following understanding was clarified in RAN1 #104e:

Agreement: The following changes highlighted in red are adopted

| 16-1a-1 | SSB/CSI-RS for L1-SINR measurement | Per slot limitations: - The max number of SSB/CSI-RS (1Tx) for CMR - The max number of CSI-IM/NZP-IMR resources - The max number of CSI-RS (2Tx) resources for CMR - The max number of aperiodic CSI-RS resources across all CCs configured to measure L1-SINR (including CMR and IMR) shall not exceed MD_1 | Note4: For components 1, 2, 3, 7, a SSB/CSI-RS resource is counted within the duration of a reference slot in which the corresponding reference signals are transmitted |
| --- | --- | --- | --- |

Agreement: The following changes highlighted in red are adopted

| 16-1g | Resources for beam management, pathloss measurement, BFD, RLM and new beam identification | 1. The maximum total number of SSB/CSI-RS/CSI-IM resources configured to measure within a slot across all CCs in one frequency range for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, RLM and new beam identification | Note: The “configured to measure” RS is counted within the duration of a reference slot in which the corresponding reference signals are transmitted |
| --- | --- | --- | --- |

Agreement: The following changes highlighted in red are adopted

| 16-1g-1 | Resources for beam management, pathloss measurement, BFD, RLM and new beam identification across frequency ranges | The maximum total number of SSB/CSI-RS/CSI-IM resources configured to measure within a slot across all CCs for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, RLM and new beam identification | Note: The “configured to measure” RS is counted within the duration of a reference slot in which the corresponding reference signals are transmitted |
| --- | --- | --- | --- |

In this paper, we try to further clarify whether the newly defined counting is applicable to Rel-15 UE feature 2-24 and 2-33.

- Discussion

Similar to UE feature 16-1a, 16-1g and 16-1g-1, UE feature 2-24 and UE feature 2-33 also define the RS measurement limitations as following:

| 2-24: SSB/CSI-RS for beam measurement | 1) The max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP within a slot shall not exceed MB_1 2) The max number of CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP shall not exceed MC_1 3) The max number of CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs to measure L1-RSRP within a slot shall not exceed MB_2 4) Supported density of CSI-RS 5) The max number of aperiodic CSI-RS resources across all CCs configured to measure L1-RSRP shall not exceed MD_1 |
| --- | --- |
| 2-33 CSI-RS and CSI-IM reception for CSI feedback | 1) Supported max # of configured NZP-CSI-RS resources per CC, 2) Supported max # of ports across all configured NZP-CSI-RS resources per CC 3) Supported max # of configured CSI-IM resources per CC 4) Supported max # simultaneous NZP-CSI-RS resources in active BWPs across all CCs 5) Supported max # simultaneous NZP-CSI-RS resources per CC 6) Supported max total # of CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7) Supported max total # of CSI-RS ports in simultaneous NZP-CSI-RS resources per CC |

However, the agreed counting for 16-1a, 16-1g and 16-1g-1 is different from the counting behavior defined in 38.214. In Rel-15 NR, counting of CSI-RS ports and resource occupation [1] is defined in 38.214 as following. The measured CSI-RS resources are not only counted in the slot that the RS is transmitted.

In any slot, the UE is not expected to have more active CSI-RS ports or active CSI-RS resources in active BWPs than reported as capability. NZP CSI-RS resource is active in a duration of time. For aperiodic CSI-RS, starting from the end of the PDCCH containing the request and ending at the end of the PUSCH containing the report associated with this aperiodic CSI-RS. For semi-persistent CSI-RS, starting from the end of when the activation command is applied, and ending at the end of when the deactivation command is applied. For periodic CSI-RS, starting when the periodic CSI-RS is configured by higher layer signalling, and ending when the periodic CSI-RS configuration is released. If a CSI-RS resource is referred N times by one or more CSI Reporting Settings, the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted N times.

The agreement leading to above description is as following:

Agreement

- …….Previous parts omitted………

- Simultaneous resources in FG 2-36, 2-40, 2-41, 2-43, 2-33

  - In any slot, the number of “active” CSI-RS resources/ports should not exceed the numbers reported in FG2-36, 2-40, 2-41, 2-43, 2-33

  - An NZP CSI-RS resource is active in a duration of time defined as follows:

    - For A-CSI-RS, starting from the end of the PDCCH containing the request, and ending at the end of the PUSCH containing the report associated with this A-CSI-RS.

    - For SP-CSI-RS, starting from the end of the activation command is applied, and ending at the end of the deactivation command is applied.

    - For P-CSI-RS, the activate duration ends when the P-CSI-RS configuration is released.

There was discussion on this issue back in RAN1 #101e. Based on above agreement, the majority understanding is that the counting of active CSI-RS resources defined in 38.214 section 5.2.2.5 is not applicable for CSI-RS for L1-RSRP.

With latest understanding on UE feature 16-1a/16-1g/16-1g-1, above understanding is confirmed again that UE feature 2-24 should be counted differently as UE feature 2-33/2-36/2-40/2-40/2-41/2-43.

With such understanding, the description in 38.214 should also be clarified. An LS can be sent to RAN2 to capture similar wording in UE feature 2-24. The following proposal can be adopted.

In any slot, the UE is not expected to have more active CSI-RS ports or active CSI-RS resources in active BWPs than reported as capability. The number of CSI-RS resources reported in beamManagementSSB-CSI-RS is determined as in [13]. NZP CSI-RS resource is active in a duration of time. For aperiodic CSI-RS, starting from the end of the PDCCH containing the request and ending at the end of the PUSCH containing the report associated with this aperiodic CSI-RS. For semi-persistent CSI-RS, starting from the end of when the activation command is applied, and ending at the end of when the deactivation command is applied. For periodic CSI-RS, starting when the periodic CSI-RS is configured by higher layer signalling, and ending when the periodic CSI-RS configuration is released. If a CSI-RS resource is referred N times by one or more CSI Reporting Settings, the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted N times.

Proposal 1: Clarify counting of CSI-RS resourced defined in UE feature 2-24 is similar as in UE feature 16-1a/16-1g/16-1g-1.

- Send LS to RAN2 to capture the clarification.

- Update 38.214 to remove potential ambiguity on CSI-RS counting for UE feature 2-24.

- Conclusion

In this contribution, the following is proposed to clarify counting of CSI-RS for UE feature 2-24.

Proposal 1: Clarify counting of CSI-RS resourced defined in UE feature 2-24 is similar as in UE feature 16-1a/16-1g/16-1g-1.

- Send LS to RAN2 to capture the clarification.

- Update 38.214 to remove potential ambiguity on CSI-RS counting for UE feature 2-24.

In any slot, the UE is not expected to have more active CSI-RS ports or active CSI-RS resources in active BWPs than reported as capability. The number of CSI-RS resources reported in beamManagementSSB-CSI-RS is determined as in [13]. NZP CSI-RS resource is active in a duration of time. For aperiodic CSI-RS, starting from the end of the PDCCH containing the request and ending at the end of the PUSCH containing the report associated with this aperiodic CSI-RS. For semi-persistent CSI-RS, starting from the end of when the activation command is applied, and ending at the end of when the deactivation command is applied. For periodic CSI-RS, starting when the periodic CSI-RS is configured by higher layer signalling, and ending when the periodic CSI-RS configuration is released. If a CSI-RS resource is referred N times by one or more CSI Reporting Settings, the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted N times.

- References