3GPP TSG RAN WG1 #104b-e		R1-2102943

e-Meeting, April 12th – 20th, 2021

Source:	vivo

Title:	Maintenance on UCI enhancements

Agenda Item:	7.2.5

Document for:  Discussion and Decision

Introduction

In this contribution, we share our views on remaining issues for UCI enhancements for URLLC.

Discussion

Corrections for sub-slot PUCCH

In the current specification, for sub-slot PUCCH, it is captured that in the remaining of this Clause, if a UE is provided subslotLengthForPUCCH-r16, a slot for an associated PUCCH transmission includes a number of symbols indicated by subslotLengthForPUCCH-r16.

However, not all the slots in section 9 should be replaced by sub-slot when a UE is provided subslotLength-ForPUCCH. The following aspects are found:

- For the timing of applying spatial setting for a PUCCH transmission defined in section 9.2.2, slot should be used.

- For SR reporting in section 9.2.4, slot should not be replaced by sub-slot since slot is used as the unit for SR periodicity and offset determination;

- For handling multiple PUCCH resources in a slot to transmit CSI reports in the beginning of section 9.2.5, where a UE determines at most two PUCCHs in a slot to transmit CSI reports or multiplexes all CSI reports in a slot on one PUCCH if a UE is configured with multiple PUCCH resources in a slot to transmit CSI reports. Here, slot should not be replaced by sub-slot since sub-slot based PUCCH was introduced for HARQ-ACK only.

- For PUCCH repetition in section 9.2.6, slot should not be replaced by sub-slot according to the conclusion of not supporting sub-slot based PUCCH repetition in Rel-16 in the last meeting, which is captured in 9.2.6.

Proposal 1: Adopt the following text proposal for sub-slot based PUCCH transmission in 38.213.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

9	UE procedure for reporting control information

*** Unchanged text is omitted ***

In the remaining of this Clause except for Clauses 9.2.2, 9.2.4 and 9.2.5 for multiple PUCCH resources in a slot to transmit PUCCH with CSI reports, if a UE is provided subslotLength-ForPUCCH, a slot for an associated PUCCH transmission includes a number of symbols indicated by subslotLength-ForPUCCH.

*** Unchanged text is omitted ***

----------------------------------------------------- End of text proposal ------------------------------------------------------

Conclusion

In this contribution, we discuss the remaining issues about enhancements to UCI, and the following proposals are made.

Proposal 1: Adopt the following text proposal for sub-slot based PUCCH transmission in 38.213.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

9	UE procedure for reporting control information

*** Unchanged text is omitted ***

In the remaining of this Clause except for Clauses 9.2.2, 9.2.4 and 9.2.5 for multiple PUCCH resources in a slot to transmit PUCCH with CSI reports, if a UE is provided subslotLength-ForPUCCH, a slot for an associated PUCCH transmission includes a number of symbols indicated by subslotLength-ForPUCCH.

*** Unchanged text is omitted ***

----------------------------------------------------- End of text proposal ------------------------------------------------------

References

TS 38.213, Physical layer procedures for control, v16.4.0, 2020-12