3GPP TSG RAN WG1 #104b-e		  R1-2102944

e-Meeting, April 12th – 20th, 2021

Source:	vivo

Title:	Corrections on parameter of MCS table set to qam256

Agenda Item:	7.2.5

Document for:	Discussion and Decision

- Introduction

In this contribution, we share our view on remaining issue about parameter of MCS table set to qam256.

- Corrections on parameter of MCS table set to qam256

In Rel-16 URLLC, DCI format 0-2 and 1-2 with the configurable fields are introduced. For DCI format 0-2 and 1-2, it can also be configured to use 256QAM with new RRC parameters. However, for some clauses, the new description on MCS table are missed, which leads to the lack of availability on the MCS table set to qam256.

For DCI format 1-2, it can be configured with 256QAM MCS table by parameter mcs-TableDCI-1-2-r16, while for DCI format 0-2, 256QAM MCS table can be configured by parameter mcs-TableDCI-0-2-r16 or mcs-TableTransformPrecoderDCI-0-2-r16. In 38.212-5.4.2.1 section, these configurations are missed.

---------------------------------Start of Text Proposal on TS 38.212 v16.5.0----------------------- 5.4.2.1 Bit selection <Unchanged parts are omitted> - if the higher layer parameter mcs-Table or mcs-TableDCI-1-2-r16 given by a pdsch-Config for at least one DL BWP of the serving cell is set to 'qam256', maximum modulation order is assumed for DL-SCH; otherwise a maximum modulation order is assumed for DL-SCH; - if the higher layer parameter mcs-Table or mcs-TableTransformPrecoder or mcs-TableDCI-0-2-r16 or mcs-TableTransformPrecoderDCI-0-2-r16 given by a pusch-Config or the higher layer parameter mcs-Table or mcs-TableTransformPrecoder given by a configuredGrantConfig for at least one UL BWP of the serving cell is set to 'qam256', maximum modulation order is assumed for UL-SCH; otherwise a maximum modulation order is assumed for UL-SCH ------------------------------------------End of Text Proposal -----------------------------------

Proposal 1: To correct the parameter of MCS table set to qam256 with the text proposal.

- Conclusion

In the contribution, we propose that,

Proposal 1: To correct the parameter of MCS table set to qam256 with the text proposal.

---------------------------------Start of Text Proposal on TS 38.212 v16.5.0----------------------- 5.4.2.1 Bit selection <Unchanged parts are omitted> - if the higher layer parameter mcs-Table or mcs-TableDCI-1-2-r16 given by a pdsch-Config for at least one DL BWP of the serving cell is set to 'qam256', maximum modulation order is assumed for DL-SCH; otherwise a maximum modulation order is assumed for DL-SCH; - if the higher layer parameter mcs-Table or mcs-TableTransformPrecoder or mcs-TableDCI-0-2-r16 or mcs-TableTransformPrecoderDCI-0-2-r16 given by a pusch-Config or the higher layer parameter mcs-Table or mcs-TableTransformPrecoder given by a configuredGrantConfig for at least one UL BWP of the serving cell is set to 'qam256', maximum modulation order is assumed for UL-SCH; otherwise a maximum modulation order is assumed for UL-SCH ------------------------------------------End of Text Proposal -----------------------------------