3GPP TSG RAN WG1 #104b-e	                                                                   R1-2102951

e-Meeting, April 12th – 20th, 2021

Source:	vivo

Title:	Discussion on Configuration for CA Half duplex operation

Agenda Item:	7.2.11

Document for:	Discussion and Decision

- Introduction

In RAN1#104e, the configuration for CA half duplex operation was discussed in email thread [104-e-AI5-LS-02] and there was a consensus to the following moderator proposals (R1-2102208)

- Agree that 38.213 should utilize the per-serving-cell configuration of directionalCollisionHandling as currently implemented by RRC, and the configuration is evaluated on a cell pair basis.

  - The TS38.213 is updated to reflect this

- Send an LS to RAN2 confirming that the directionalCollisionHandling can be configured on a cell basis and no further action is needed from RAN2

After that, the corresponding 38.213 CR was discussed in email thread [104-e-NR-TEI-01], however, the discussion was not concluded eventually.

In this contribution we share our view on the controversial points during the discussion happened in last meeting.

- Discussion

As captured in the latest summary document of email thread [104-e-NR-TEI-01]( draft_R1-2102005 Summary on 104-e-NR-TEI-01_v017_ZTE_Moderator.zip), there are two major open issues.

- UE capability dependency

The controversial point was whether the UE capability “Not supporting simultaneousRxTxInterBandCA” should be kept as the prerequisite of half-DuplexTDD-CA-SameSCS-r16, with following alternatives.

  - Alt.1: Directional collision handling is also applicable to CCs within a frequency band

    - Alt.1-1: “Not supporting simultaneousRxTxInterBandCA” should not be prerequisite of half-DuplexTDD-CA-SameSCS-r16

    - Alt.1-2: “Not supporting simultaneousRxTxInterBandCA” should be kept as prerequisite of half-DuplexTDD-CA-SameSCS-r16 only for the case with inter-band pair of cells

  - Alt.2: Directional collision handling is not applicable to CCs within a frequency band

In general, we are supportive of alt.1-2 above, which means for a given BC with inter-band carriers, the UE can indicate half-DuplexTDD-CA-SameSCS-r16 only when the UE does not support simultaneousRxTxInterBandCA for the BC. And for a BC with intra-band carriers only, UE will not indicate simultaneousRxTxInterBandCA therefore half-DuplexTDD-CA-SameSCS-r16 can still be indicated which means the half-duplex collision handling can be applied to such BC with intra-band carriers. There is no need to update the UE capability definition and the dependency.

- The exact CR text

We think the latest CR text shared in the summary document (as copied below) is in general fine, with some minor update for better clarity shown in yellow highlight as below. And it is important to keep the last paragraph of the CR so that after resolving the duplex conflict among the serving cells enabled with directionalCollisionHandling-r16, UE is not expected to handle any additional duplex conflict between any pair of serving cells including a serving cell not enabled with directionalCollisionHandling-r16.

## 11.1 Slot configuration A slot format includes downlink symbols, uplink symbols, and flexible symbols. The following are applicable for each serving cell. If a UE is provided tdd-UL-DL-ConfigurationCommon, the UE sets the slot format per slot over a number of slots as indicated by tdd-UL-DL-ConfigurationCommon. The tdd-UL-DL-ConfigurationCommon provides - a reference SCS configuration by referenceSubcarrierSpacing - a pattern1. The pattern1 provides - a slot configuration period of msec by dl-UL-TransmissionPeriodicity - a number of slots with only downlink symbols by nrofDownlinkSlots - a number of downlink symbols by nrofDownlinkSymbols - a number of slots with only uplink symbols by nrofUplinkSlots - a number of uplink symbols by nrofUplinkSymbols A value msec is valid only for . A value msec is valid only for or . A value msec is valid only for , or , or . A slot configuration period of msec includes slots with SCS configuration . From the slots, a first slots include only downlink symbols and a last slots include only uplink symbols. The symbols after the first slots are downlink symbols. The symbols before the last slots are uplink symbols. The remaining are flexible symbols. The first symbol every periods is a first symbol in an even frame. If tdd-UL-DL-ConfigurationCommon provides both pattern1 and pattern2, the UE sets the slot format per slot over a first number of slots as indicated by pattern1 and the UE sets the slot format per slot over a second number of slots as indicated by pattern2. The pattern2 provides - a slot configuration period of msec by dl-UL-TransmissionPeriodicity - a number of slots with only downlink symbols by nrofDownlinkSlots - a number of downlink symbols by nrofDownlinkSymbols - a number of slots with only uplink symbols by nrofUplinkSlots - a number of uplink symbols by nrofUplinkSymbols The applicable values of are same as the applicable values for . A slot configuration period of msec includes first slots and second slots. From the slots, a first slots include only downlink symbols and a last include only uplink symbols. The symbols after the first slots are downlink symbols. The symbols before the last slots are uplink symbols. The remaining are flexible symbols. A UE expects that divides 20 msec. The first symbol every periods is a first symbol in an even frame. A UE expects that the reference SCS configuration is smaller than or equal to a SCS configuration for any configured DL BWP or UL BWP. Each slot provided by pattern1 or pattern2 is applicable to consecutive slots in the active DL BWP or the active UL BWP where the first slot starts at a same time as a first slot for the reference SCS configuration and each downlink or flexible or uplink symbol for the reference SCS configuration corresponds to consecutive downlink or flexible or uplink symbols for the SCS configuration . If the UE is additionally provided tdd-UL-DL-ConfigurationDedicated, the parameter tdd-UL-DL-ConfigurationDedicated overrides only flexible symbols per slot over the number of slots as provided by tdd-UL-DL-ConfigurationCommon. The tdd-UL-DL-ConfigurationDedicated provides - a set of slot configurations by slotSpecificConfigurationsToAddModList - for each slot configuration from the set of slot configurations - a slot index for a slot provided by slotIndex - a set of symbols for a slot by symbols where - if symbols = allDownlink, all symbols in the slot are downlink - if symbols = allUplink, all symbols in the slot are uplink - if symbols = explicit, nrofDownlinkSymbols provides a number of downlink first symbols in the slot and nrofUplinkSymbols provides a number of uplink last symbols in the slot. If nrofDownlinkSymbols is not provided, there are no downlink first symbols in the slot and if nrofUplinkSymbols is not provided, there are no uplink last symbols in the slot. The remaining symbols in the slot are flexible For each slot having a corresponding index provided by slotIndex, the UE applies a format provided by a corresponding symbols. The UE does not expect tdd-UL-DL-ConfigurationDedicated to indicate as uplink or as downlink a symbol that tdd-UL-DL-ConfigurationCommon indicates as a downlink or as an uplink symbol, respectively. For each slot configuration provided by tdd-UL-DL-ConfigurationDedicated, a reference SCS configuration is the reference SCS configuration provided by tdd-UL-DL-ConfigurationCommon. A slot configuration period and a number of downlink symbols, uplink symbols, and flexible symbols in each slot of the slot configuration period are determined from tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated and are common to each configured BWP. A UE considers symbols in a slot indicated as downlink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated to be available for receptions and considers symbols in a slot indicated as uplink by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated to be available for transmissions. If a UE is not configured to monitor PDCCH for DCI format 2_0, for a set of symbols of a slot that are indicated as flexible by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated if provided, or when tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated are not provided to the UE - the UE receives PDSCH or CSI-RS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format - the UE transmits PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format, a RAR UL grant, fallbackRAR UL grant, or successRAR For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to receive a PDCCH, or a PDSCH, or a CSI-RS, or a DL PRS in a set of symbols of a slot, the UE receives the PDCCH, the PDSCH, the CSI-RS, or the DL PRS if the UE does not detect a DCI format that indicates to the UE to transmit a PUSCH, a PUCCH, a PRACH, or a SRS in at least one symbol of the set of symbols of the slot; otherwise, the UE does not receive the PDCCH, or the PDSCH, or the CSI-RS, or the DL PRS in the set of symbols of the slot. For operation with shared spectrum channel access, if a UE is provided csi-RS-ValidationWith-DCI, is not provided CO-DurationsPerCell, and is not provided SlotFormatCombinationsPerCell, and if the UE is configured by higher layers to receive a CSI-RS in a set of symbols of a slot, the UE cancels the CSI-RS reception in the set of symbols of the slot if the UE does not detect a DCI format indicating an aperiodic CSI-RS reception or scheduling a PDSCH reception in the set of symbols of the slot. If a UE is provided channelAccessMode ='dynamic' and is provided availableRB-SetsToAddModList and availableRB-SetsToRelease, the UE expects to be provided co-DurationsPerCellToAddModList and co-DurationsPerCellToReleaseList and/or slotFormatCombToAddModList and slotFormatCombToReleaseList. For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in a set of symbols of a slot and the UE detects a DCI format indicating to the UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then - the UE does not expect to cancel the transmission in symbols from the set of symbols that occur, relative to a last symbol of a CORESET where the UE detects the DCI format, after a number of symbols that is smaller than the PUSCH preparation time Tproc,2 for the corresponding UE processing capability [6, TS 38.214] assuming d2,1=1 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format and the SCS configuration of the SRS, PUCCH, PUSCH or μr, where μr corresponds to the SCS configuration of the PRACH if it is 15kHz or higher; otherwise μr=0 - the UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS 38.214], determined from Clauses 9 and 9.2.5 or Clause 6.1 of [6. TS 38.214], or the PRACH transmission in remaining symbols from the set of symbols and cancels the SRS transmission in remaining symbols from the subset of symbols For a set of symbols of a slot that are indicated to a UE as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, the UE does not receive PDCCH, PDSCH, or CSI-RS when the PDCCH, PDSCH, or CSI-RS overlaps, even partially, with the set of symbols of the slot. For a set of symbols of a slot that are indicated to a UE as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, the UE does not receive DL PRS in the set of symbols of the slot, if the UE is not provided with a measurement gap. For a set of symbols of a slot that are indicated to a UE as downlink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, the UE does not transmit PUSCH, PUCCH, PRACH, or SRS when the PUSCH, PUCCH, PRACH, or SRS overlaps, even partially, with the set of symbols of the slot. For a set of symbols of a slot that are indicated to a UE as flexible by tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated if provided, the UE does not expect to receive both dedicated higher layer parameters configuring transmission from the UE in the set of symbols of the slot and dedicated higher layer parameters configuring reception by the UE in the set of symbols of the slot. For operation on a single carrier in unpaired spectrum, for a set of symbols of a slot indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon, for reception of SS/PBCH blocks, the UE does not transmit PUSCH, PUCCH, PRACH in the slot if a transmission would overlap with any symbol from the set of symbols and the UE does not transmit SRS in the set of symbols of the slot. The UE does not expect the set of symbols of the slot to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, when provided to the UE. If a UE - is configured with multiple serving cells and is provided with directionalCollisionHandling-r16 = 'enabled' for at least two serving cells, and - is not capable of simultaneous transmission and reception on any of the multiple serving cells as indicated by the absence of simultaneousRxTxInterBandCA capability, and - indicates support of capability for half-duplex operation in CA with unpaired spectrum with half-DuplexTDD-CA-SameSCS-r16 capability, and - is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells, for a set of symbols of a slot that are indicated to the UE for reception of SS/PBCH blocks in any of multiple serving cells by ssb-PositionsInBurst in SystemInformationBlockType1 or by ssb-PositionsInBurst in ServingCellConfigCommon, when provided to the UE, the UE does not transmit PUSCH, PUCCH, or PRACH in the slot if a transmission would overlap with any symbol from the set of symbols, and the UE does not transmit SRS in the set of symbols of the slot in any of multiple serving cells. For a set of symbols of a slot corresponding to a valid PRACH occasion and symbols before the valid PRACH occasion, as described in Clause 8.1, the UE does not receive PDCCH, PDSCH, or CSI-RS in the slot if a reception would overlap with any symbol from the set of symbols. The UE does not expect the set of symbols of the slot to be indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated. For a set of symbols of a slot indicated to a UE by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, the UE does not expect the set of symbols to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated. If a UE is scheduled by a DCI format to receive PDSCH over multiple slots, and if tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, indicate that, for a slot from the multiple slots, at least one symbol from a set of symbols where the UE is scheduled PDSCH reception in the slot is an uplink symbol, the UE does not receive the PDSCH in the slot. If a UE is scheduled by a DCI format to transmit PUSCH over multiple slots, and if tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, indicates that, for a slot from the multiple slots, at least one symbol from a set of symbols where the UE is scheduled PUSCH transmission in the slot is a downlink symbol, the UE does not transmit the PUSCH in the slot. If a UE - is configured with multiple serving cells and is provided with directionalCollisionHandling-r16 = 'enabled' for at least two serving cells, and - is not capable of simultaneous transmission and reception on any of the multiple serving cells as indicated by the absence of simultaneousRxTxInterBandCA capability, and - indicates support of capability for half-duplex operation in CA with unpaired spectrum with half-DuplexTDD-CA-SameSCS-r16 capability, and - is not configured to monitor PDCCH for detection of DCI format 2-0 on any of the multiple serving cells, the UE determines a reference cell for a symbol as an active cell with the smallest cell index among serving cells that are configured with directionalCollisionHandling-r16 where the symbol is configured as - downlink, or uplink, as indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated - uplink, if the symbol is flexible and the UE is configured to transmit SRS, PUCCH, PUSCH, or PRACH on the symbol - downlink, if the symbol is flexible and the UE is configured to receive PDCCH, PDSCH or CSI-RS on the symbol If a UE - is configured with multiple serving cells in a frequency band and is provided with directionalCollisionHandling-r16 = 'enabled' for at least two serving cells, and - - indicates support of capability for half-duplex operation in CA with unpaired spectrum with half-DuplexTDD-CA-SameSCS-r16 capability, and - is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells, the UE does not expect - a symbol to be indicated as downlink or uplink on the reference cell and as uplink or downlink on another cell, respectively, by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated, - tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated to indicate a symbol as downlink on the reference cell and to detect a DCI format scheduling a transmission on the symbol on another cell, and - to be configured by higher layers to receive PDCCH, PDSCH, or CSI-RS on a flexible symbol on the reference cell and to detect a DCI format scheduling a transmission on the symbol on another cell. If the reference cell and another cell for a UE operate in different frequency bands and if the UE - is configured with multiple serving cells and is provided with directionalCollisionHandling-r16 = 'enabled' for the other cell, and - is not capable of simultaneous transmission and reception on any of the multiple serving cells as indicated by the absence of simultaneousRxTxInterBandCA capability, and - indicates support of capability for half-duplex operation in CA with unpaired spectrum with half-DuplexTDD-CA-SameSCS-r16 capability, and - is not configured to monitor PDCCH for detection of DCI format 2-0 on any of the multiple serving cells, the UE - UE assumes symbol as flexible, is not required to receive higher layer configured PDCCH, PDSCH, or CSI-RS and not expected to transmit higher layers configured SRS, PUCCH, PUSCH, or PRACH, when tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated indicates symbol as downlink or uplink on the other cell and as uplink or downlink for the reference cell, respectively, - transmits a signal/channel scheduled by a DCI format on a symbol of the other cell when the symbol is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated for the reference cell, - is not required to receive a higher layer configured PDCCH, PDSCH, or CSI-RS on flexible symbols on the reference cell in a set of symbols, if the UE detects a DCI format scheduling a transmission on one or more symbols in the set of symbols on the other cell. If a UE - is configured with multiple serving cells and is provided with directionalCollisionHandling-r16 = 'enabled' for at least two serving cells, and - is not capable of simultaneous transmission and reception on any cell from the multiple serving cells as indicated by the absence of simultaneousRxTxInterBandCA capability, and - indicates support of capability for half-duplex operation in CA with unpaired spectrum with half-DuplexTDD-CA-SameSCS-r16 capability, and - is not configured to monitor PDCCH for detection of DCI format 2-0 on any of the multiple serving cells, the UE - does not expect tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated for the reference cell to indicate a symbol as uplink and to detect a DCI format scheduling a reception on the symbol on another cell - does not expect to be configured by higher layers to transmit SRS, PUCCH, PUSCH, or PRACH on a flexible symbol on the reference cell and to detect a DCI format scheduling a reception on the symbol on another cell - does not transmit a PUCCH, PUSCH or PRACH that is configured by higher layers on a set of symbols on another cell among the cells configured with directionalCollisionHandling if at least one symbol from the set of symbols is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or is a symbol corresponding to a PDCCH, PDSCH, or CSI-RS reception that is configured by higher layers on the reference cell - does not transmit a SRS that is configured by higher layers on a set of symbols on another cell among the cells configured with directionalCollisionHandling if the set of symbols is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or corresponds to a PDCCH, PDSCH or CSI-RS reception that is configured by higher layers on the reference cell - does not receive a PDCCH, PDSCH or CSI-RS that is configured by higher layers on a set of symbols on another cell among the cells configured with directionalCollisionHandling if at least one symbol from the set of symbols is indicated as uplink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or is a symbol corresponding to a SRS, PUCCH, PUSCH, or PRACH transmission that is configured by higher layers on the reference cell - assumes a symbol indicated as downlink or uplink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated on another cell among the cells configured with directionalCollisionHandling to be flexible, if the UE is respectively configured by higher layers to transmit SRS, PUCCH, PUSCH, or PRACH or to receive PDCCH, PDSCH, or CSI-RS on the reference cell - does not expect to detect a first DCI format scheduling a transmission or reception on a symbol on a first cell and a second DCI format scheduling a reception or transmission on the symbol on a second cell, respectively After the UE applies the procedures described above for directional collision handling within the set of cells that have been configured with directionalCollisionHandling-r16, the UE does not expect any directional collision (with including the effects of timing advance) across any pair of cells, irrespective of whether either or both of those cells were configured with directionalCollisionHandling-r16 if the UE indicated no simultaneousRxTxInterBandCA for the pair of cells.

- Conclusion

In the contribution, we discussed the configuration for CA Half duplex operation and propose the following

- Proposal 1: Send an LS to RAN2 confirming that the directionalCollisionHandling can be configured on a cell basis and no further action is needed from RAN2

- Proposal 2: No need to update the UE capability definition and dependency.

- Proposal 3: Adopt the CR text as shown in section 2, which is a minor update compared to the latest CR text.

Reference

- 3GPP TS 38.213, “NR; Physical layer procedures for control”, v16.4.0 (2020-12).

- 3GPP TS 38.321, “NR; Medium Access Control (MAC) protocol specification”, v16.3.0 (2020-12).