3GPP TSG-RAN WG1 Meeting #104bis-e	Tdoc R1-2102956

Online, April 12th – 20th, 2021

Agenda Item:	8.14.2

Source:	Ericsson

Title:	Evaluation methodology for XR

Document for:	Discussion

# 1	Introduction

A new SI on XR evaluations for NR was approved at RAN#86 [1]. The objectives of this study item are as follows.

- Confirm XR and Cloud Gaming applications of interest

- Identify the traffic model for each application of interest taking outcome of SA WG4 work as input, including considering different upper layer assumptions, e.g. rendering latency, codec compression capability etc.

- Identify evaluation methodology to assess XR and CG performance along with identification of KPIs of interest for relevant deployment scenarios

- Once traffic model and evaluation methodologies are agreed, carry out performance evaluations towards characterization of identified KPIs

In this contribution, we provide our view on the open issues of the XR evaluation methodology. Since XR evaluations are intended mainly for simulation studies, it is important to develop appropriate methodology to well reflect realistic XR user experience and RAN characteristics in a simple manner.

# 2	Discussion

The evaluation methodology was discussed extensively during RAN1#104-e, and many agreements were reached. One important thing to notice is that there was a split in optional and mandatory simulation conditions. However, the amount of mandatory simulation cases is prohibitively large, especially since the parameters are defined independently, and all combinations essentially become mandatory. Since the total amount of (mandatory) simulation scenarios are huge, each company will have to down select which scenarios to simulate. This means that no scenario is in reality mandatory – instead all the simulation scenarios are optional:

Due to the huge number of scenarios, each company will have to down select which scenarios to simulate, and all scenarios become optional.

This is merely a remark: we realize that it is impossible to agree on a small-enough set of simulation cases. Most likely, based on the input, we will see what scenarios are de facto mandatory. To provide at least some guidance, we propose

In the XR evaluation SI, RAN1 should treat cloud gaming with first priority, AR use cases with second priority, and VR use cases with third priority.

Compared to CG and AR use cases, it is expected that VR use case is primarily targeting stationary users in a limited area, potentially with no or very low mobility. It will match well with indoor hotspot scenario. A channel model can be InH either with open office or mixed office LoS probability. Both FR1 and FR2 spectrum for InH is possible for the VR use case. Nevertheless, since VR is expected to have much higher data rate than CG and AR, FR2 would be more relevant for Rel-17 studies.

Based on this, we propose to prioritize the following relevant deployment scenarios for five XR use cases to minimize evaluation sets of each use case study.

For CG and AR, wide-area deployments such as dense urban macro, or urban macro are prioritized, whereas indoor hotspot scenarios are prioritized for VR.

## 2.1	Methodology for capacity evaluations

During RAN1#104-e, there was significant progress in developing the evaluation methodology for the capacity evaluations.  However, as already mentioned, this has resulted in a very large number of “mandatory” simulation cases. The explicit outstanding FFSs are related to if simulation cases are baseline, mandatory or optional. In our understanding, such discussions are irrelevant: due to the huge number of simulation cases, all simulation cases are optional. Hence, we propose:

Do not discuss optional vs mandatory parameters settings any further for the capacity evaluations.

Our understanding is that the currently agreed simulation cases cover the relevant use cases and challenge the system so that potential bottlenecks are exposed. There is thus no reason to add new simulation scenarios:

No more simulation cases are added.

## 2.2	Methodology for power consumption evaluations

In RAN1#104-e, it was agreed that the “UE always on” (Case 1) will be evaluated as the baseline case for UE power consumption evaluations. On whether Case 2 (i.e., power consumption assuming Rel-15/16 CDRX configuration) should be optional or baseline, our preference is to avoid having multiple baseline cases and keep only Case 1 as baseline. There are many possible Rel15/16 DRX configurations and many XR traffic models/deployment scenarios. Given this, it is preferable to let companies pick a suitable DRX pattern to evaluate power savings gain vs. latency impact and provide some explanation on why a particular pattern is chosen for a given XR traffic and deployment. This avoids the extra work in RAN1 attempting to converge on ‘baseline DRX pattern(s)’ for each XR traffic models/deployment scenario.

Only Case 1 (assuming UE is always ON) is retained as baseline scenario. 
Case 2 (assuming Rel-15/16 CDRX configuration) is not added as one more baseline but can be optionally evaluated

Companies evaluating power savings vs. latency impact with Rel15/16 DRX should include explanation on why a particular DRX pattern is chosen for a given XR scenario.

Another aspect is whether all UEs are considered in power savings gain computation, or if only satisfied UEs are considered. For this we see benefit in reporting power savings separately for only the satisfied UEs.

At least power savings gain considering only satisfied UEs should be reported as part of evaluation results. Power savings gain considering all UEs (including satisfied and not satisfied UEs) can be additionally reported.

For UL UE power consumption evaluations, two options were agreed in RAN1#104-e. Our preference is to pick one of the two Options as baseline (preferably Option 2 with linear interpolation). Also we do not see the need to limit minimum UL transmit power to 0dBm.

For UL power consumption evaluations, Option 2 (linear interpolation) should be used as baseline

## 2.3		Methodology for mobility evaluations

As XR and Cloud Gaming see consumer adoption, the services are expected to be consumed by users on the move. Minimizing user experience degradation through mobility events is a key consideration in enabling mass adoption of such services. Evaluating and understanding the XR performance during mobility is thus vital.

Mobility can be categorized in intra-cell and inter-cell mobility. For intra-cell mobility, there is typically no need for any RRC reconfiguration, and there are no interrupts in the communication. Still, the movement has impact on the large-scale propagation properties, which may lead to reduced performance. For inter-cell mobility, the UE is reconfigured using RRCReconfigurationWithSync: a handover is performed. The handover makes it possible to transfer the connection from one physical transmission point (TRP) to another. During the handover, there is an interrupt: a period when no communication is performed. The interrupt is (at least) a few tens of ms. Directly after the handover, it may take some time before the communication is again running at full speed.

MBB services are relatively resilient to interrupts since such applications can buffer data. Interrupts have no or little impact on the quality of experience.

Most probably, XR will be more sensitive to interrupts, since there is no possibility to maintain a playout buffer in the client, and the relatively strict latency bound will lead to that a late packet must be discarded. Even intra-cell mobility will lead to varying communication quality, so that occasional packets are lost. In other words, there are no users that have constantly good quality. Moreover, quality of experience may be degraded only when a small number of frames are lost. This would motivate that performance during mobility is evaluated:

Compared to eMBB, XR applications will be more impaired by intra-cell and inter-cell mobility.

Performance during inter-cell mobility is rarely evaluated by simulations in 3GPP. There are several reasons for that:

- Mobility events are rare for realistic UE speeds and cell sizes, making it challenging to collect sufficient statistics, and to highlight the impact of the mobility events.

- There is no preferred KPI to compare the performance of handover algorithms: there are several KPIs, and they all have pros and cons.

- The channel models have traditionally not supported mobility.

These issues remain for XR and solving them during the XR SI would seem unrealistic.

Based on the above discussion, we propose to evaluate the current mobility procedures analytically. The properties of the currently specified mobility procedures are summarized, and their impact on XR services are highlighted. The XR services would be described using the agreed traffic models, as well as the criteria for user satisfaction:

Inter-cell mobility is evaluated analytically by describing the currently specified mobility procedures from an XR service point of view, relying on the agreed traffic models and user satisfaction criteria.

Focus should be on three issues:

- Interrupt: how many packets/frames are lost due to the interrupt?

- Functional limitations: are there any functionality that becomes unavailable during a handover? For example, can all SCells be maintained during the handover?

- Other quality degradations: Are there any other quality degradations resulting from the interrupt? For instance, are all relevant measurements available in the new cell directly after the reconfiguration?

An embryo to such an analytic evaluation can be found in [2].

If such an analytic approach is deemed insufficient, a restricted simulation setup can be considered:

Further consider if some restricted simulation setup could complement the analytical evaluation.

## 2.4	Methodology for coverage evaluations

Coverage evaluations are also mentioned in the WID [1]. Clearly, coverage is of critical importance for any service, and XR is no exception.

Coverage can be broadly categorized into basic coverage and service coverage. Basic coverage includes setting up a connection but does not have any relation to a particular service. Service coverage on the other hand takes the properties of a certain service into account. The coverage is then related to certain quality-of-experience characteristics of a service, for example 10Mbps DL data rate.

Since the XR SI focus on a particular class of services, service coverage should be studied. The quality-of-experience criteria for the various types of services have been agreed during the discussion about capacity evaluations and can readily reused also for coverage evaluations.

Reuse the end-user satisfaction criteria agreed for the capacity evaluations also for the coverage evaluations.

Since coverage is defined as performance when there is no interference, much of the evaluation methodology derived for the capacity evaluations can be reused also for the coverage evaluations. The only difference is that performance is evaluated for low load, e.g., one user in the system. Thus, we propose

Coverage is defined as the probability that a user is satisfied when the number of users in the system is very low.

This means that we can essentially derive the XR service coverage from the system simulations for capacity.

It should be noted that the agreed simulation scenarios are not so challenging from a coverage point of view. It may be desirable to also investigate performance in a less dense deployment.

# 4	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	In the XR evaluation SI, RAN1 should treat cloud gaming with first priority, AR use cases with second priority, and VR use cases with third priority.

Proposal 2	For CG and AR, wide-area deployments such as dense urban macro, or urban macro are prioritized, whereas indoor hotspot scenarios are prioritized for VR.

Proposal 3	Do not discuss optional vs mandatory parameters settings any further for the capacity evaluations.

Proposal 4	No more simulation cases are added.

Proposal 5	Only Case 1 (assuming UE is always ON) is retained as baseline scenario.  Case 2 (assuming Rel-15/16 CDRX configuration) is not added as one more baseline but can be optionally evaluated

Proposal 6	Companies evaluating power savings vs. latency impact with Rel15/16 DRX should include explanation on why a particular DRX pattern is chosen for a given XR scenario.

Proposal 7	At least power savings gain considering only satisfied UEs should be reported as part of evaluation results. Power savings gain considering all UEs (including satisfied and not satisfied UEs) can be additionally reported.

Proposal 8	For UL power consumption evaluations, Option 2 (linear interpolation) should be used as baseline

Proposal 9	Inter-cell mobility is evaluated analytically by describing the currently specified mobility procedures from an XR service point of view, relying on the agreed traffic models and user satisfaction criteria.

Proposal 10	Further consider if some restricted simulation setup could complement the analytical evaluation.

Proposal 11	Reuse the end-user satisfaction criteria agreed for the capacity evaluations also for the coverage evaluations.

Proposal 12	Coverage is defined as the probability that a user is satisfied when the number of users in the system is very low.

# References

RP-193241, “New SID on XR Evaluations for NR”, Qualcomm, 3GPP TSG RAN #86, Dec 2019.

R1-2102958, Mobility and XR applications, Ericsson, RAN1#104bis-e, April 2021