3GPP TSG RAN WG1 #104b-e			R1-2102979

e-Meeting, April 12th – April 20th, 2021

Agenda item:	8.2.4

Source:	      Xiaomi

Title:	Discussion on beam management in NR from 52.6 GHz to 71GHz

Document for:	Discussion

Introduction

In RAN #90, the work item about beam management for NR in 52.6-71GHz are agreed as follows:

- Specify timing associated with beam-based operation to new SCS (i.e., 480kHz and/or 960kHz), study, and specify if needed, potential enhancement for shared spectrum operation

  - Study which beam management will be used as a basis: R15/16 or R17 in RAN #91-e

During RAN1 #104-e meeting, the beam management for new SCSs has been discussed based on the above work item, and some agreements are achieved as shown in [1]. In this contribution, we further discuss some potential enhancements on beam management to support NR from 52.6 GHz to 71 GHz.

Multiple beams for multiple PDSCHs/PUSHCs scheduled by a single DCI

The QCL assumption for each PDSCH

Agreements about the QCL assumption(s) the UE should apply for each PDSCH are shown as follows:

Agreement:

Further study the following:

- For multi-PDSCH scheduling with a single DCI, study the QCL assumption(s) the UE should apply for each PDSCH for the case when some of the scheduled PDSCHs have scheduling offset less than timeDurationForQCL while some have scheduling offset equal to or greater than timeDurationForQCL.

- For multi-PDSCH scheduling with a single DCI, study the QCL assumption(s) the UE should apply for each PDSCH for the case when all of the scheduled PDSCHs have scheduling offset less than timeDurationForQCL

- Note: If the current Rel-16 behavior would be extended to multiple-PDSCH scheduling, it could result in a different QCL assumption for each PDSCH due to the fact the that the CORESET with the lowest ID can be different for different slots, resulting in a potentially different TCI state for each slot

- Note: Applicability to multi-TRP can be discussed further

For discussion on QCL assumptions for the cases when some of the scheduled PDSCHs have scheduling offset less than timeDurationForQCL while some have scheduling offset equal to or greater than timeDurationForQCL, or all of the scheduled PDSCHs have scheduling offset less than timeDurationForQCL, several alternatives can be considered.

Alt 1, all the scheduled PDSCHs QCL assumption is the same as the PDSCH in the first TTI. And the QCL assumption for the first PDSCH can using the R16 behavior, that is UE may assume that the DM-RS ports of the first PDSCH are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs are monitored by the UE. By Alt 1, each PDSCH in the multiple TTI would have the same QCL assumption.

Alt 2, for the scheduled PDSCHs have scheduling offset less than timeDurationForQCL, the QCL assumption is the same as the PDSCH in the first TTI, which is determined by R16 behavior. And for the scheduled PDSCHs have scheduling offset equal to or greater than timeDurationForQCL, the QCL assumption is the same as the PDCCH scheduling the PDSCHs when there is no TCI indication field in the scheduling DCI, or the QCL assumption is indicated by the TCI indication field, if it exists, in the scheduling DCI. In Alt 2, the PDSCHs have scheduling offset less than timeDurationForQCL have the same QCL assumption, while PDSCHs have scheduling offset equal to or greater than timeDurationForQCL have the same or different QCL assumptions based on the TCI indication field in the scheduling DCI.

Alt 3, for each of the scheduled PDSCHs have scheduling offset less than timeDurationForQCL, the QCL assumption is determined by Rel-16 behavior, And for the scheduled PDSCHs have scheduling offset equal to or greater than timeDurationForQCL, the QCL assumption is the same as the PDCCH scheduling the PDSCHs when there is no TCI indication field in the scheduling DCI, or the QCL assumption is indicated by the TCI indication field, if it exists, in the scheduling DCI. In Alt 3, the PDSCHs have scheduling offset less than timeDurationForQCL may have different QCL assumptions and the PDSCHs have scheduling offset equal to or greater than timeDurationForQCL have the same or different QCL assumptions based on the TCI indication field in the scheduling DCI.

From our point of view, compared to Alt 1/3, Alt 2 can avoid frequent QCL assumption change, but also can offer some flexibility for the PDSCHs have scheduling offset equal to or greater than timeDurationForQCL by TCI indication in scheduling DCI. If gNB wants to keep the QCL assumption of all the PDSCHs the same, then gNB can indicate a TCI that is the same as the first scheduled PDSCH. And if gNB wants to set different QCL assumptions, one or more different TCIs in scheduling DCI can be indicated, which corresponds to one or more PDSCHs have scheduling offset equal to or greater than timeDurationForQCL.

Proposal 1: For the scheduled PDSCHs have scheduling offset less than timeDurationForQCL, the QCL assumption is the same as the PDSCH in the first TTI, which is determined by R16 behavior. And for the scheduled PDSCHs have scheduling offset equal to or greater than timeDurationForQCL, the QCL assumption is the same as the PDCCH scheduling the PDSCHs when there is no TCI indication field in the scheduling DCI, or the QCL assumption is indicated by the TCI indication field, if it exists, in the scheduling DCI.

Single beam or multiple beam for multi-PDSCH/PUSCH

Agreements about the supporting of multiple beams for multiple PDSCHs/PUSCHs are shown as follows:

Agreement:

Further study the following:

- For multi-PDSCH scheduling with a single DCI, study whether or not it is needed to indicate a separate TCI state for each scheduled PDSCH

- For multi-PUSCH scheduling with a single DCI, study whether or not it is needed to indicate a separate SRI (indication of TCI can be further discussed) for each scheduled PUSCH

- Note: the study should take into account DCI overhead aspects

- Note: Applicability to multi-TRP can be discussed further

Figure 1: Single TRP and Multi-TRP

In the WID [2], scheduling multiple PDSCHs/PUSCHs over multiple slots by single DCI is supported to reduce the overhead of PDCCH monitoring. Accordingly, it should be discussed whether or not it is needed to indicate a separate beam for each scheduled PDSCH/PUSCH. If we consider single TRP or multi-TRP with multi-DCI shown in Figure 1, there is no need to indicate a separate TCI state for each scheduled PDSCH/PUSCH, because from our understanding the beam measurement results can be assumed to be stable enough before performing next beam measurement based on periodic CSI-RS resources and the best beam selected based on the beam measurement results is suitable for all these PDSCHs/PUSCHs. In addition, the DCI overhead in this case is more accepted. While for multi-TRP with single DCI, if the multiple PDSCHs/PUSCHs scheduled by a single DCI is allowed to belong to different TRPs, the beams for the PDSCHs/PUSCHs of different TRPs need to be indicated separately.

Proposal 2: For single TRP and multi-TRP with multi-DCI, there is no need to indicate a separate TCI state for each scheduled PDSCH/PUSCH. For multi-TRP with single DCI, the TCI states for the PDSCHs/PUSCHs belonging to different TRPs should be indicated separately.

Beam measurement for NR from 52.6 to 71GHz

Enhancements on periodic CSI-RS transmission to deal with LBT failure

For the unlicensed frequency bands in 52.6-71GHz, because of the co-existence with other technologies occupying the same unlicensed spectrum, the periodic CSI-RS cannot be transmitted if LBT failure. From the UE side, if periodic CSI-RS are configured, it will assume that the reference signals are always there and may report the measurement results to gNB after performing the measurement on these signal, unless there is a way for UE to detect whether the transmission bursts exist. Observed from the NR-U in Rel16, there are several methods for UE to determine that when the transmission bursts starts and ends. If a UE is configured with two groups of search space sets for PDCCH, the monitoring group flag in DCI format 2_0 and a timer configured as a number of slots can be used for UE to determine whether the transmission bursts exist. In addition, a COT (Channel Occupied Time) duration bit-field per serving cell in GC-PDCCH, i.e., DCI format 2_0, indicates UE the end of COT. It can be concluded based on the description above that it is feasible to configure periodic CSI-RS via RRC for beam measurement if reusing the same methods in NR-U in Rel16 for UE to confirm whether the transmission bursts exist.

Proposal 3: Beam measurement based on periodic CSI-RS can be still supported in NR for 52.6 to 71 GHz.

However, owing to the missing of periodic CSI-RS caused by LBT failure, gNB cannot get the latest beam measurement results from UE, which may lead to huge performance loss because of the movements of UE or other objects in the environment. One possible way to deal with this is to trigger an aperiodic CSI reports, a beam measurement and report based on aperiodic CSI-RS, to patch a non-transmitted periodic CSI-RS as shown in Figure 2.

Proposal 4: Aperiodic CSI reports can be triggered to patch a non-transmitted periodic CSI-RS.

Figure 2: Aperiodic CSI reports to patch a non-transmitted periodic CSI-RS

Enhancements on aperiodic CSI reports

As discussed in section 4.1, triggering aperiodic CSI reports can help the gNB to get the latest beam measurement results and update the TCI state of UE when the periodic CSI-RS resources cannot be transmitted due to the LBT failure. Observed from Rel15/16, the aperiodic CSI-RS can be triggered by DCI format 0_1/0_2 together with aperiodic CSI reports. Each aperiodic trigger state can be associated with several report configures and the corresponding resource sets. However, only one aperiodic CSI-RS resource set can be used for beam measurement and one aperiodic CSI-RS resource set contains maxNrofNZP-CSI-RS-ResourcesPerSet (64) aperiodic CSI-RS resources at most. In 52.6-71GHz, narrower beamwidth may be needed to compensate for larger propagation loss due to high atmospheric absorption, which means a large number of beams are expected to be used. To support more beams, some enhancements on aperiodic CSI reports may needed. The most direct way is to increase the maximal number of reference signals in each CSI-RS resource set. In addition, another feasible method is that multiple aperiodic CSI-RS resource sets associated with one aperiodic trigger state are allowed to be used for beam measurement as shown in Figure 3.

Figure 3: Multiple aperiodic CSI-RS resource sets triggered by single DCI

Proposal 5: To support more beams, the maximal number of reference singles in one CSI-RS resource set should be increased. Or, multiple aperiodic CSI-RS resource sets associated with one aperiodic trigger state should be allowed to be used for beam measurement.

Nevertheless, in Rel15/16, if we setup multiple aperiodic CSI-RS resource sets in one aperiodic trigger state, which can be seen as multiple CSI reports, the beam measurement results of each resource set will report separately. For beam measurement and report, it is expected that the measurement results of all these aperiodic CSI-RS resource sets can be process jointly to report N best beams. In consequence, an implicit or explicit way to indicate UE the report method, which refers to reporting the results separately or jointly, may needed.

Proposal 6: An implicit or explicit way to indicate UE the report method, which refers to reporting the measurement results separately or jointly, is needed when multiple aperiodic CSI-RS resource sets are triggered by single DCI for beam measurement.

In NR, a CSI-RS is always configured on a per-device basis. But, in practice, the CSI-RS resource(s) can be shared among multiple UEs. Accordingly, several UEs, like these UEs in group1 or group2 in Figure 4, may perform beam measurement based on the same periodic CSI-RS resources if they have similar speed, or more precisely, beam changing rate. As we can see from Figure 4, all these UEs in group 1 cannot get the latest beam measurement results due to the loss of periodic CSI-RS resources. Therefore, aperiodic CSI reports need to be triggered for all these UEs in group 1. In Rel15/16, aperiodic CSI reports are triggered separately for each UE by DCI format 0_1/0_2 with CSI request field. However, as discussed above, it is beneficial to allow the aperiodic CSI reports for multiple UEs, like the UEs in group 1, to be triggered together to patch the non-transmitted periodic CSI-RS resources caused by LBT failure.

Proposal 7: It is beneficial to support group-based triggering of aperiodic CSI reports for UEs configured with same periodic CSI-RS resources used for beam measurement.

Figure 4: Group-based triggering of aperiodic CSI reports

The limitation on the number of periodic CSI-RS resource used for beam measurement in Rel15/16

As mentioned in section 4.2, there is a limitation on the number of aperiodic CSI-RS resource used for beam measurement in Rel15/16. Actually, there is also a limitation on the number of periodic CSI-RS resource used for beam measurement. Observed from TS 38.331, the CSI resource setting CSI-ResourceConfig contains a configuration of a list of S≥1 CSI resource sets, given by the parameter nzp-CSI-RS-ResourceSetList. This parameter is a list of references to NZP CSI-RS resources used for beam measurement and reporting. It contains up to maxNrofNZP-CSI-RS-ResourceSetsPerConfig resource sets if resourceType is 'aperiodic' and 1 otherwise, which means S= maxNrofNZP-CSI-RS-ResourceSetsPerConfig for aperiodic CSI-RS resource setting and S=1 for semi-persistent and periodic CSI-RS resource setting. In sum, for beam measurement, the periodic CSI-RS resource setting CSI-ResourceConfig associated with a reporting setting CSI-ReportConfig contains one CSI resource set and one CSI resource set contains maxNrofNZP-CSI-RS-ResourcesPerSet (64) reference signals at most. In other word, there is a limitation on the number of periodic CSI-RS resource used for beam measurement in Rel15/16. Accordingly, some enhancements are needed to deal with this limitation if the number of beams more than maxNrofNZP-CSI-RS-ResourcesPerSet are expected to be used in 52.6-71GHz.

Observation 1: There is a limitation on the number of periodic CSI-RS resource used for beam measurement in Rel15/16.

Proposal 8: Some enhancements are needed to deal with this limitation if the number of beams more than maxNrofNZP-CSI-RS-ResourcesPerSet are expected to be used in 52.6-71GHz.

Conclusions

In this contribution, we share our views on the potential enhancements on beam management to support NR from 52.6 GHz to 71 GHz.

Proposal 1: For the scheduled PDSCHs have scheduling offset less than timeDurationForQCL, the QCL assumption is the same as the PDSCH in the first TTI, which is determined by R16 behavior. And for the scheduled PDSCHs have scheduling offset equal to or greater than timeDurationForQCL, the QCL assumption is the same as the PDCCH scheduling the PDSCHs when there is no TCI indication field in the scheduling DCI, or the QCL assumption is indicated by the TCI indication field, if it exists, in the scheduling DCI.

Proposal 2: For single TRP and multi-TRP with multi-DCI, there is no need to indicate a separate TCI state for each scheduled PDSCH/PUSCH. For multi-TRP with single DCI, the TCI states for the PDSCHs/PUSCHs belonging to different TRPs should be indicated separately.

Proposal 3: Beam measurement based on periodic CSI-RS can be still supported in NR for 52.6 to 71 GHz.

Proposal 4: Aperiodic CSI reports can be triggered to patch a non-transmitted periodic CSI-RS.

Proposal 5: To support more beams, the maximal number of reference singles in one CSI-RS resource set should be increased. Or, multiple aperiodic CSI-RS resource sets associated with one aperiodic trigger state should be allowed to be used for beam measurement.

Proposal 6: An implicit or explicit way to indicate UE the report method, which refers to reporting the measurement results separately or jointly, is needed when multiple aperiodic CSI-RS resource sets are triggered by single DCI for beam measurement.

Proposal 7: It is beneficial to support group-based triggering of aperiodic CSI reports for UEs configured with same periodic CSI-RS resources used for beam measurement.

Observation 1: There is a limitation on the number of periodic CSI-RS resource used for beam measurement in Rel15/16.

Proposal 8: Some enhancements are needed to deal with this limitation if the number of beams more than maxNrofNZP-CSI-RS-ResourcesPerSet are expected to be used in 52.6-71GHz.

- References

- RAN1-104-e meeting Chairman notes.

- RP-202925, “Revised WID:  Extending current NR operation to 71 GHz”, CMCC