3GPP TSG RAN WG1 #104-bis-e						    R1-2103028

e-Meeting, April 12th – 20th, 2021

Source:	Intel Corporation

Title:	Analysis of enhanced CSI feedback schemes

Agenda item:	8.3.1.2

Document for:	Discussion and Decision

Introduction

A new Rel.17 work item on URLLC/IIoT enhancements was approved in [1]. One of the potential enhancements to Rel.16 operation is the physical layer feedback enhancement, including CSI feedback enhancement:

- Study, identify and specify if needed, required Physical Layer feedback enhancements for meeting URLLC requirements covering - UE feedback enhancements for HARQ-ACK [RAN1] - CSI feedback enhancements to allow for more accurate MCS selection [RAN1] - Note: DMRS-based CSI feedback is not in scope of this WI

In RAN1#102-e and RAN1#103-e meeting a progress was made in identifying a set of potential enhancements to be further discussed along this WI. In RAN#90-e, the progress on CSI was discussed, and the following recommendation was endorsed in [2]:

Proposed RAN conclusion on IIoT scope: … - For the UE CSI/HARQ-ACK feedback enhancements in the IIoT/URLLC WI, RAN1 work to continue the discussions. Status to be checked in March if any RAN level guidance needed. - RAN1 to continue discussion on A-CSI on PUCCH, whether to specify or not.

Further in RAN1#104-e, the following conclusion was made:

Conclusion: Continue evaluation of new reporting Case 1 and Case 2 for the schemes identified in Appendix B of R1-2102131. - Companies are encouraged to provide their views on each scheme against each criterion in respective Tables in Appendix B. - Companies are encouraged to provide additional evaluation results for as many schemes as possible, based on assumptions agreed in RAN1#102-e. - Aim for down-selection at RAN1#104-b-e by taking into account evaluation results and assessment against criteria from Appendix B.

After RAN1#104-e, additional email discussion was organized collecting detailed views on the schemes [104-e-NR-R17-IIoT_URLLC-02-AddDisc].

In this contribution, further discussion on CSI feedback enhancements is provided. Our views on other URLLC-related aspect could be found in [5]-[8].

Performance Comparison of Techniques

In this section, we analyse high-level classification of techniques by system-level evaluation motivated by the agreed assumptions.

In this meeting, we focus on the Scenario 2 for evaluation, which is the Indoor Factory scenario. Note, that Scenario 1 (Rel-15 enabled use case UMa), and Scenario 3 (eMBB-URLLC mix) could be found in our previous tdocs [3] and [4] respectively.

The Scenario 2 (Indoor Factory) was chosen due to tight latency constraints, which in FR1 do not allow to perform HARQ retransmissions, thus the accuracy of the CSI for the initial transmission becomes very important. It could also be observed that due to possibility of HARQ retransmissions in Scenario 1, there is no noticeable performance difference among CSI schemes since HARQ retransmission restores any inaccuracies very efficiently.

Observation 1

- Scenarios with single-shot PDSCH transmission are expected to show more noticeable impact from application of different CSI enhancement schemes

The following evaluation cases are considered:

- Baseline / R16. SP-CSI with 5 slots period.

- Enhanced SB/WB CSI. This evaluation case represents the proposal of enhanced signalling for SB CQI which is currently only possible by 2-bit offset from the WB CQI. The modelled enhancement simply removes the signalling restriction allowing full SB CQI reporting.

- Statistical CSI. In this evaluation case, a UE reports mean and standard deviation effective SINR to gNB measured over 40 occasions, and gNB selects the MCS using target BLER, mean and a backoff to the mean as a scale of standard deviation.

- Maximum interference. In this evaluation case, the minimum measurement on a sub-band over the measurements in a 40 measurement occasions window is applied.

- BLEP reporting. The scheme is modelled according to explanations and parameters from [Nokia] for “EP-OLLA parameters for “hard” BLERtarget”.

  - Mean EP estimation: envelope tracking ForgetFactorDown=0.001

  - Mean EP estimation: envelope tracking ForgetFactorUp=1.0

  - OLLA StepUp = 0.5 [dB]

  - OLLA StepDown = 0.001 [dB]

Other assumptions are summarized in Table 1 of the appendix section.

In Figure 1, the baseline CSI is compared to other schemes with single-shot scheduling targeting 1e-5 BLER.

Figure 1. CDF of packet error rate per UE.

|  | Baseline | Enhanced SB CQI | Maximum interference | BLEP report | Statistical CSI |
| --- | --- | --- | --- | --- | --- |
| Resource utilization | 23.64 % | 23.94 % | 35.95 % | 27.22 % | 6.29 % |

Observation 2

- In Scenario 2 (Indoor Factory), at 1e-5 packet error rate target, the following schemes outperform the baseline

  - Statistical CSI by ~5% more satisfied UEs

  - Maximum interference CSI by ~20% more satisfied UEs

- In Scenario 2 (Indoor Factory), at 1e-4 packet error rate target, the following schemes outperform the baseline

  - BLEP reporting by ~10% more satisfied UEs

  - Maximum interference CSI by ~12 more satisfied UEs

General Discussion on the Identified List of Enhancements

In RAN1#102-e and #103-e, a list of enhancements to be further discussed along this WI was identified. In this section, we go over them and provide views on their potential for URLLC/IIoT scenarios.

## A-CSI on PUCCH

Although the A-CSI on PUCCH was discussed in Rel.16, its justification remains uncertain. There are a few advantages argued around A-CSI on PUCCH:

- Since it could be triggered by a DL assignment, thus it may avoid UL grant overhead. However, in this case the DL assignment needs new fields, which increase the DCI size and therefore degrade the link budget. Overall gains from such overhead savings may be hard to estimate since the UL grant may schedule data in many cases thus no real wastage of resources may be observed.

- Depending on A-CSI reported quantities / granularities, this could be made faster than the A-CSI on PUSCH. Overall, the issue of faster CSI reporting needs a separate discussion and careful considerations since it is quite a sensitive topic from UE implementation point of view. In RAN1#103-e it was additionally agreed that reduced CSI processing times are not considered unless new CSI reporting types are concerned.

- The CSI could be used for advanced retransmission of the data scheduled by this DCI, e.g. by reporting the actual channel and interference situation on the failed PDSCH. This kind of scheme may not bring noticeable benefits, since the initial transmission itself should be quite reliable, thus the gains from more accurate retransmissions may only be observed on a very small subset of cases when the initial transmission fails.

Among the disadvantages, the following was identified:

- Uncertain gains at expense of non-negligible specification efforts. The strongest argument would be that the performance is considered not limited by channel variation estimation accuracy, rather by interference estimation accuracy. The bursty interference estimation accuracy in general is not improved with faster CSI.

In Section 2 we analysed two potential effects from A-CSI on PUCCH: (1) more up to date CSI information if A-CSI on PUCCH is made faster than on PUSCH, (2) taking into account the A-CSI in retransmission scheduling. In both scenarios it has been shown that for the considered assumptions there is no gain from A-CSI on PUCCH comparing to P/SP-CSI and/or A-CSI on PUSCH.

Observation 3

- A-CSI on PUCCH should not be further considered due to uncertain performance benefits and large spec impact.

## Case-1 new reporting

The following cases were identified in RAN1#103-e meeting with respect to new CSI reporting:

Agreements: For Case-1 New reporting, the following candidate schemes have been identified to address the fast interference change over time. Continue studying with focus on the identified schemes below for further study and evaluation. - Scheme 1a: New reporting quantity based on CQI/SINR statistics, e.g., - CQI/SINR statistics (e.g., mean, variance, etc.) - CSI prediction - Scheme 1b: New reporting quantity of interference statistics (e.g., mean, variance, interference covariance matrix, etc.) - Scheme 1c: New reporting quantity based on modifying existing reporting format, e.g., - CQI reporting considering the worst subbands - Subband CQI granularity enhancement - Scheme 1d: New reporting quantity related to CSI expiration time - Scheme 1e: New reporting quantity with partial information update, e.g., - CSI reporting with interference update only Companies are encouraged to investigate the above schemes, aiming for down-selection in RAN1#104-e

Note, we don’t use the updated classification from RAN1#104-e due to overlaps in schemes in this list.

With respect to Case-1, we continue supporting this direction since we believe this may be the most promising group of enhancements directly solving the accuracy problem by taking into account interference variations. Among them, the following enhancements can be considered:

Scheme 1a: New reporting quantity based on CQI/SINR statistics

- When a UE reports CQI and precoding information, it is usually an instantaneous observation on a given channel realization. In case of bursty interference, the instantaneous measurement may not be reliable if applied without processing at gNB. To combat this, the CSI report could include statistical information parameters about channel/interference distribution, e.g. mean and standard deviation.

- One may argue that gNB can calculate the interference distribution based on multiple CSI reports. However, this does not work on the borders of the SINR range of a given CQI table. The minimum and the maximum CQI values limit the gNB assumption on how much better / worse the observed SINR is comparing to the highest / lowest CQI respectively. Usually this could be corrected by an Outer Loop Link Adaptation (OLLA), but as was discussed extensively, OLLA is unusable in situations with very tight latency & reliability since every NACK may count towards missed QoS requirements.

Scheme 1b: New reporting quantity of interference statistics

- This scheme has similar motivation as Scheme 1a, but tries to optimize overhead & complexity assuming interference distribution can be more important than the channel. Therefore, it can be discussed together with 1a.

Scheme 1c: New reporting quantity based on modifying existing reporting format

- Although the issue of inaccurate sub-band report using 2-bit differential CQI may be straightforward, we don’t observe sufficient gains in Section 2 to change current specification.

Scheme 1d: New reporting quantity related to CSI expiration time

- This scheme is interesting, but CSI relevance could be derived by gNB from observations on different periodic or aperiodic CSI occasions, thus CSI expiration time could be classified as an optimization.

Scheme 1e: New reporting quantity with partial information update

- Overall, the concept of trigger / filter-based reporting can be generalized in the way that a UE is given decision power to provide gNB with measurements which have largest impact on performance in the same time saving the CSI report payload by filtering the less important measurements. A simplest example is if a measurement of a periodic CSI is not changed in some margin, the report may not be multiplexed. Of course, the CSI report content should be modified to indicate which CSI reports are actually transmitted.

- Similar mechanism can be applied for CSI part 1 and CSI part 2, where the content of CSI part 2 may be referred by CSI part 1. The actual content of the CSI part 2 may be a function of measurements, e.g. measurements with large observed interference or least observed interference can be prioritized for multiplexing.

Proposal 1

- RAN1 to prioritize the following schemes for further specification:

  - New reporting quantity based on CQI/SINR statistics;

  - New reporting quantity of interference statistics;

  - New reporting quantity with partial information update.

## Case-2 new reporting

In the last RAN1#103-e meeting, the following agreement was made to focus the discussion with respect to Case-2 new reporting schemes:

Agreement: - For Case-2 new reporting, continue studying with focus on the new reporting type based on PDSCH decoding for OLLA performance enhancement for initial and re-transmissions of PDSCH.

We further consider two different applications, as mentioned in the agreement:

- Optimization of OLLA performance for re-transmission of PDSCH

  - The fundamental problem is that URLLC should have quite robust initial transmission, e.g. with BLER target in the range of ~0.001 – 1%. That means the proposed techniques optimize the resource allocation for very small percentage of cases, which obviously can bring small overall gains.

- Optimization of OLLA performance for initial transmission of PDSCH

  - We are not sure if PDSCH decoding margin can be applied for another initial transmission directly and efficiently. It can be used same way as ACK or NACK information to calculate a delta effective SINR for OLLA settings, but in the same time the advanced CSI reporting of Case-1 could also provide more precise effective SINR.

  - As for the BLEP reporting, the results in section 2 show trade-off between performance on different BLER targets, but operate worse than the baseline at the highest error requirement. The associated spec change & implementation complexity seem not justified.

Observation 4

- Case-2 PDSCH decoding margin reporting targets improved re-transmission resource allocation, which could not provide justifying gains due to low probability of re-transmissions in URLLC.

- Case-2 BLEP reporting gains do not justify the spec impact

## CSI feedback for PDCCH

It was also discussed in RAN1#102-e and #103-e whether some kind of a feedback for PDCCH link adaptation may be useful. During that occasion, it was argued that in URLLC scenarios PDCCH resource allocation can limit the performance.

Although the intention may be reasonable, there are several counterarguments about PDCCH CSI feedback:

- PDCCH already has several mechanisms for rough link adaptation:

  - CSI for PDSCH can be largely applicable to PDCCH

  - RSRP, L1-SINR and other auxiliary measurements can also be used to predict PDCCH performance

- PDCCH resource allocation granularity is not very suitable for accurate adaptation. Up to R16, PDCCH has only a small number of different aggregation levels: 1,2,4,8,16. With such granularity of changing PDCCH spectrum efficiency, the already available less accurate mechanisms of link adaptation should work fine (as outlined above).

Observation 5

- Current design of PDCCH does not require further optimization in acquiring channel quality for PDCCH transmission.

## Other enhancements - CSI multiplexing & prioritization

Rel.15 and Rel.16 specifications usually treat a P/SP-CSI on PUCCH with the least priority when it collides with other channels. However, this could be revised assuming the CSI report for URLLC traffic also has high importance, especially if it gets enhancements in Rel-17.

To resolve that, the P/SP-CSI first needs to be associated with a priority index, through semi-static configuration or activation signal. Second, the multiplexing & prioritization procedures need additional handling based on the indicated high or regular priority.

Proposal 2

- RAN1 to consider assigning a priority index to P/SP-CSI on PUCCH and handle collision & multiplexing scenarios based on the associated priority.

Conclusion

In this contribution the CSI measurement and feedback enhancements targeting URLLC/IIOT scenarios in Release 17 have been presented. Based on analysis and discussion, the following observations and proposals have been made:

Observation 1

- Scenarios with single-shot PDSCH transmission are expected to show more noticeable impact from application of different CSI enhancement schemes

Observation 2

- In Scenario 2 (Indoor Factory), at 1e-5 packet error rate target, the following schemes outperform the baseline

  - Statistical CSI by ~5% more satisfied UEs

  - Maximum interference CSI by ~20% more satisfied UEs

- In Scenario 2 (Indoor Factory), at 1e-4 packet error rate target, the following schemes outperform the baseline

  - BLEP reporting by ~10% more satisfied UEs

  - Maximum interference CSI by ~12 more satisfied UEs

Observation 3

- A-CSI on PUCCH should not be further considered due to uncertain performance benefits and large spec impact.

Observation 4

- Case-2 PDSCH decoding margin reporting targets improved re-transmission resource allocation, which could not provide justifying gains due to low probability of re-transmissions in URLLC.

- Case-2 BLEP reporting gains do not justify the spec impact

Observation 5

- Current design of PDCCH does not require further optimization in acquiring channel quality for PDCCH transmission.

Proposal 1

- RAN1 to prioritize the following schemes for further specification:

  - New reporting quantity based on CQI/SINR statistics;

  - New reporting quantity of interference statistics;

  - New reporting quantity with partial information update.

Proposal 2

- RAN1 to consider assigning a priority index to P/SP-CSI on PUCCH and handle collision & multiplexing scenarios based on the associated priority.

- References

- RP-201310, “Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR”, Nokia, Nokia Shanghai Bell, July 2020

- RP-202872, “Moderator's summary for email discussion [90E][26][IIOT_scope]”, Nokia, Nokia Shanghai Bell (Moderator), December 2020

- R1-2008985, “Discussion on prioritized CSI feedback enhancements for URLLC/IIoT”, Intel Corporation, e-Meeting, October 26 – November 13, 2020

- R1-2100650, “CSI feedback enhancements for URLLC/IIoT”, Intel Corporation, e-Meeting, January 25 – February 05, 2021

- R1-2103027, “Further details on UE HARQ feedback enhancements”, Intel Corporation, e-Meeting, April 12 – 20, 2021

- R1-2103029, “Further Details on Enabling URLLC/IIoT in Unlicensed Band”, Intel Corporation, e-Meeting, April 12 – 20, 2021

- R1-2103030, “Further analysis and details of intra-UE multiplexing and prioritization”, Intel Corporation, e-Meeting, April 12 – 20, 2021

- R1-2103031, “Further analysis and design considerations regarding propagation delay compensation”, Intel Corporation, e-Meeting, April 12 – 20, 2021

- Appendix – Evaluation Assumptions

Table 1. Detailed assumptions based on scenario 3 agreed in RAN1#102-e

| Parameters | Value |
| --- | --- |
| Layout | 120 x 60 m indoor factory 18 BS |
| Inter-BS distance | 20 m |
| Carrier frequency | 4 GHz |
| Channel model | InF-DH in TR 38.901 |
| UE Tx power | 23dBm |
| BS antenna configurations | Table A.2.2-1 in TR38.824 |
| BS antenna height | 25m |
| BS antenna element gain + connector loss | 8 dBi |
| BS receiver noise figure | 5dB |
| UE antenna configuration | Table A.2.2-1 in TR38.824 |
| UE antenna height | Follow the modelling of TR 38.901 (e.g. 1.5m) |
| UE antenna gain | 0dBi |
| UE receiver noise figure | 9 dB |
| Total transmit power per TRxP | 49 dBm |
| Number of UEs per cell | 240 URLLC UEs per factory |
| Simulation bandwidth | 40 MHz |
| SCS | 30 kHz |
| UE distribution | 100% of users are indoors 75 km/h for modeling fading channel |
| TDD configuration | 7 symbols DL: 7 symbols UL repeated |
| Traffic model | URLLC: 32 byte, 1 ms latency, 2 ms periodicity |
| Resource allocation | 2 PRB granularity |
| PDCCH | 1 symbol overhead |
| DMRS | 1 symbol overhead |
| CSI reporting | Regular delay 5 slots |
| Transmission scheme | Rank 1 restriction |