3GPP TSG RAN WG1 #104-bis-e						    R1-2103031

e-Meeting, April 12th – 20th, 2021

Source:	Intel Corporation

Title:	Further analysis and design considerations regarding propagation delay compensation

Agenda item:	8.3.4

Document for:	Discussion and Decision

Introduction

Rel. 17 URLLC/IIoT work started in RAN1#102-e based on the objectives of the agreed WID [1]. One of the objectives assumes RAN1 involvement with respect to enhanced time synchronization and propagation delay compensation, and in particular it includes the following:

- Enhancements for support of time synchronization: - RAN impacts of SA2 work on uplink time synchronization for TSN, if any. [RAN2] - Propagation delay compensation enhancements (including mobility issues, if any). [RAN2, RAN1, RAN3, RAN4]

In RAN1#103-e, moderate progress was made defining more assumptions on the sources of propagation delay estimation error. After RAN1#103-e, RAN1 received the reply LS from RAN2 [R1-2100024, R2-2010837] regarding Uu time synchronicity budget:

Scenario; Single Uu interface Budget. Control-to-Control; ±145ns to ±275ns. Smart Grid; ±795ns to ±845ns. These values are determined with assumptions such that network-side synchronization for Control-to-control is based on gPTP and Smart Grid is based on GNSS. The Uu interface time synchronization budget can be interpreted as the maximum 5GS time synchronization error between the UE and the gNB-DU. RAN2 agrees that the decision on PDC option is up to RAN1, and RAN1 should aim to meet the most stringent requirement among these scenarios, but a number within the range is also acceptable. RAN2 would like to point out that the error caused by the limited granularity of referenceTimeInfo-r16 IE (±5ns) is already included in the network part budget, and RAN1 should not include this error in Uu interface again.

In RAN1#140-e, a limited progress was made with the following agreement:

Agreements: Take ±100 ns as the assumption for downlink frame timing detection error (errorUE,DL,RX) at the UE for evaluation of the overall time synchronization error for TA based propagation delay compensation, if downlink frame timing detection error needs to be considered separately. - Send a LS to RAN4 to ask for clarification on whether downlink frame timing detection error is included in Te or not - In the LS, to include more details about option 1 (included) & option 2 (not included); also including the necessary background - FFS whether to apply the same value to RTT-based propagation delay compensation, and the corresponding condition (if any) if the same value will be applied

The LS was sent in [R1-2102245]. Since the deadlines in RAN1 and RAN4 are separated, it was possible to check the majority view on this LS in RAN4, and it seems the overwhelming majority is for Option 1, i.e. the DL frame boundary detection error is already included in Te. We further use this assumption in analysis.

Lastly, RAN1 received an LS from RAN3 [R1-2102293], indicating that:

RAN3 considers that gNB-based PDC may have RAN3 specification impacts. However, it is RAN3 understanding that support for gNB-based PDC is up to RAN1 and RAN2 decisions. Therefore, RAN3 will not further discuss gNB-based PDC unless support for the functionality is first confirmed by RAN1/RAN2.

In this contribution we provide further analysis of timing error based on different techniques for propagation delay compensation and discuss RAN1 related design aspects for the propagation delay compensation. In [2]-[5], views on other topic in R17 URLLC/IIOT are provided.

Error Model and Analysis

For TA-like propagation delay compensation at the UE, the following error model is used:

Errorclocksync=ABS,DL,tx+BUE,DL,rx+CUE,UL,tx+DBS,UL,rx+ETAG+FTAAA2

where,

- ABS,DL,tx =± 65 ns for Control-to-control, and ± 65 or ± 200 ns for Smart Grid.

  - For analysis, all values are checked.

- BUE,DL,rx

  - UE reception error. It was agreed to be ± 100 ns in the last meeting. However, due to the discussion around LS [R1-2102245] in RAN4, this component does seem to be absorbed in CUE,UL,tx.

- CUE,UL,tx=Te

  - UE uplink transmission error. Usually it is less than Te, especially considering situations not right after the DRX cycle, i.e. when a UE acquired several SSBs. However, as a worst-case assumption we use Te, Table 7.1.2-1 in TS 38.133.

- DBS,UL,rx=±100 ns

  - BS reception error as per the agreement from RAN1#102-e.

- ETAG=±8∙64∙Tc/2μ

  - Baseline TA command granularity as per agreement.

- FTAAA

  - As per agreements, this component is considered to be 0.

- UL SCS and SSB subcarrier spacing combination

  - Some parameters are dependent on SCS of both UL BWP and SSB. For simplicity, those are assumed to be equal for the targeted SCS, i.e. 15 kHz for both UL signal and SSB and 30 kHz for both UL signal and SSB.

- Asymmetry

  - The asymmetry between UL and DL propagation channels is not considered as per agreements.

Based on the above assumptions, we further identified the following cases for evaluation:

- Case 1a-1: Baseline TA-based compensation;

- Case 1a-2: Enhanced granularity TA-based compensation;

  - ETAG is reduced 4 times to show potential gains / trends.

- Case 1b-1: Tightened requirements and TA-based compensation;

  - Te is reduced 4 times to show potential gains / trends.

- Case 3: Non-RTT pre-compensation at gNB;

  - TA measurement framework is reused, but in this case it is assumed that at least TA granularity component is omitted, since gNB can avoid these errors:

    - a – no change to requirements;

    - b – tightened requirements as in 1b-1.

Table 1 and Table 2 summarize the example values of the timing synchronization error for different assumptions for Control-to-Control and Smart Grid use cases, respectively.

Table 1. Uu interface timing synchronization error after propagation delay compensation for Smart Grid. As for the color code: i) green highlights indicate below the minimum bound of the budget; ii) blue highlights indicate above the minimum, but below the maximum bound of the budget.

| Case | SCS | ABS,DL,tx = 65 ns | ABS,DL,tx = 200 ns |
| --- | --- | --- | --- |
| Case 1a-1 | 15 | 441 | 576 |
| Case 1a-2 |  | 343 | 478 |
| Case 1b-1 |  | 294 | 429 |
| Case 1a-2+1b-1 |  | 196 | 331 |
| Case 3a |  | 310 | 445 |
| Case 1a-1 | 30 | 310 | 445 |
| Case 1a-2 |  | 261 | 396 |
| Case 1b-1 |  | 213 | 348 |
| Case 1a-2+1b-1 |  | 164 | 299 |
| Case 3a |  | 245 | 380 |

Table 2. Uu interface timing synchronization error after propagation delay compensation for Control-to-Control. As for the color code: i) green highlights indicate below the minimum bound of the budget; ii) blue highlights indicate above the minimum, but below the maximum bound of the budget.

| Case | SCS | ABS,DL,tx = 65 ns |
| --- | --- | --- |
| Case 1a-1 | 15 | 441 |
| Case 1a-2 |  | 343 |
| Case 1b-1 |  | 294 |
| Case 1a-2+1b-1 |  | 196 |
| Case 3a |  | 310 |
| Case 3b |  | 164 |
| Case 1a-1 | 30 | 310 |
| Case 1a-2 |  | 261 |
| Case 1b-1 |  | 213 |
| Case 1a-2+1b-1 |  | 164 |
| Case 3a |  | 245 |
| Case 3b |  | 148 |

For the case of RTT-based propagation delay compensation, we use the following error model:

Errorclock_sync=ABS,DL,Tx+BUE,DL,rx+DBS,UL,rx+GRxTxDiff,report2

where,

- ABS,DL,tx, BUE,DL,rx, DBS,UL,rx assume similar values as in the TA-based PD compensation model, although additional usage of specific DL and UL signals can improve the numbers comparing to the ones assumed for TA-based compensation.

- GRxTxDiff,report

  - This component denotes the granularity of indication of Rx-Tx time difference 2k∙Tc, where k is from [2] to 5 for FR1. For analysis, k=2 is assumed.

Note, that here RTT-based compensation could be executed on both gNB and UE sides, but we don’t make any assumption for analysis. Table 3 shows the error for the case of RTT-based (pre-)compensation.

Table 3. Uu interface timing synchronization error after propagation delay (pre-)compensation for RTT-based method. As for the color code: i) green highlights indicate below the minimum bound of the budget; ii) blue highlights indicate above the minimum but below the maximum bound of the budget.

| Case | SCS | BUE,DL,rx = 100 ns |
| --- | --- | --- |
| RTT-based | 15, 30 | 166 |

Observation 1

- For Smart Grid, any type of propagation delay compensation can achieve the timing synchronization error lower than the most stringent requirement provided by RAN2 LS.

- For Control-to-Control

  - None of the considered propagation delay compensation types under specific assumptions can achieve the timing synchronization error lower than the most stringent requirement provided by RAN2 LS.

  - The following options can achieve timing synchronization error higher than the most stringent requirement but lower than the looser requirement provided by RAN2 LS:

    - TA-based UE compensation with tightened UE transmit timing requirement + smaller TA indication granularity in 15 kHz and 30 kHz;

    - Non RTT gNB-based pre-compensation + tightened UE transmit timing requirement in 15 kHz and 30 kHz;

    - RTT-based UE compensation or gNB pre-compensation in 15 kHz and 30 kHz;

    - TA-based UE compensation with enhanced granularity of TA command in 30 kHz;

    - TA-based UE compensation with tightened UE transmit timing requirement in 30 kHz;

    - Non RTT gNB-based pre-compensation in 30 kHz.

Design Aspects

From the analysis in Section 2, in some challenging conditions, mechanisms beyond current specification are required. In this section, we analyze the options identified in RAN1#102-e plus the RAN2-led option of gNB based pre-compensation, since in LS [R1-2100024] RAN2 requested RAN1 to lead the work on defining the propagation delay compensation scheme(s).

Option 1

Although this option can fulfil the requirements in a subset of use cases, it seems more work is required for better performance. Current requirement on Te may be hard to reconsider without changes to the legacy signals, such as SSB and PRACH.

Furthermore, we don’t see much motivation to mimic TA measurement mechanism based on new signals, since standalone RTT measurements optimized for timing estimation can provide larger potential for better accuracy.

Option 2 – RTT-based UE compensation or gNB pre-compensation

The RTT-based compensation could be realized using the existing gNB Rx-Tx time difference and UE Rx-Tx time difference measurements, or re-defined Rx-Tx time difference using other signals. In this matter, there are two possible flavors:

- Alt. 1: UE side compensation. A UE measures UE Rx-Tx time difference and receives from gNB the gNB Rx-Tx time difference, so that total PD can be calculated and compensated. The signaling in this case should be UE-specific. This introduces additional signaling overhead in DL, same way as UE-specific pre-compensation at gNB, where reference timing information is assumed to be delivered in dedicated RRC message.

  - In order to reduce the gNB Rx-Tx time difference signaling overhead towards UEs, group-common signaling options could be considered at physical or higher layer.

- Alt. 2: gNB side pre-compensation. A UE measures UE Rx-Tx time difference and reports it to gNB. gNB measures the gNB Rx-Tx time difference, receives the UE Rx-Tx time difference, and pre-compensates the reference timing information before sharing it with the UE. From perspective of the overall signaling exchange, this alternative may be a bit easier to implement if the UE Rx-Tx time difference measurement is defined as just another regular measurement as part of MeasurementReport.

Observation 2

- RTT-based propagation delay compensation requires additional UE-specific signal exchange between gNB and UE:

  - In DL direction, group-common signalling could be utilized to reduce overhead.

Option 3 – Non-RTT based gNB pre-compensation

As RAN2 already discussed, there is a leftover Rel-16 mechanism of gNB-based pre-compensation and adjustment of the reference time information. Since RAN2 LS reply [R1-2100024] expects RAN1 to define the propagation delay compensation scheme after completing the analysis, the list of options can be further updated. In our understanding, given the decision and that RAN2 considered both UE based compensation and gNB based pre-compensation up until the last meeting, it is fair to add the gNB based pre-compensation to the list of options for further consideration in RAN1:

Observation 3

- After RAN2 offloaded the decision of propagation delay compensation scheme to RAN1, the list of potential options from RAN1#102-e does not match to the latest status of discussions in RAN2 and misses the gNB based pre-compensation option.

Proposal 1

- RAN1 to expand the list of propagation delay compensation options with gNB-based pre-compensation (both RTT-based and non-RTT based) in order to match with the latest status of RAN2 discussion.

As it is shown in Section 2, the pre-compensation at gNB has a potential especially if UE transmission timing requirements are tightened. Furthermore, the RTT-based scheme can also be realized by gNB pre-compensation. Additionally, mechanisms to inform the UE about pre-compensation are required to avoid double compensation.

In summary, at this point there seems no evidence to adopt only one of the options, and we expect RAN1 to work on specification of the following features for the most flexible operation:

- Component 1: Support of gNB-based pre-compensation transparent to propagation delay calculation scheme;

  - The reference time information in this case is modified by gNB to include the necessary adjustment and to indicate to the UE that no other compensation is required.

- Component 2: Support of UE-based compensation transparent to propagation delay calculation scheme;

  - The reference time information in this case is not modified by gNB, and gNB indicates to the UE that UE-side compensation is required.

- Component 3: Measurement and indication to gNB of the UE Rx-Tx time difference based on DL signals provided outside of LPP;

- Component 4: Measurement and indication to UE of the gNB Rx-Tx time difference based on UL signals provided outside of LPP.

Once all four components are defined, then network can have freedom to implement one of the suitable options, e.g. RTT-based pre-compensation at the gNB, non RTT-based pre-compensation at the gNB, RTT-based compensation at the UE, non RTT-based compensation at the UE, etc.

Proposal 2

- RAN1 to agree to specify the following components to enable flexible propagation delay compensation scheme:

  - Component 1: Support of gNB-based pre-compensation transparent to propagation delay calculation scheme;

  - Component 2: Support of UE-based compensation transparent to propagation delay calculation scheme;

  - Component 3: Non-LPP measurement and indication to gNB of the UE Rx-Tx time difference based on DL signals;

  - Component 4: Non-LPP measurement and indication to UE of the gNB Rx-Tx time difference based on UL signals.

Conclusions

In this document we presented views on the issue of propagation delay compensation as part of enhanced accurate time synchronization. The following observations and proposals have been made:

Observation 1

- For Smart Grid, any type of propagation delay compensation can achieve the timing synchronization error lower than the most stringent requirement provided by RAN2 LS.

- For Control-to-Control

  - None of the considered propagation delay compensation types under specific assumptions can achieve the timing synchronization error lower than the most stringent requirement provided by RAN2 LS.

  - The following options can achieve timing synchronization error higher than the most stringent requirement but lower than the looser requirement provided by RAN2 LS:

    - TA-based UE compensation with tightened UE transmit timing requirement + smaller TA indication granularity in 15 kHz and 30 kHz;

    - Non RTT gNB-based pre-compensation + tightened UE transmit timing requirement in 15 kHz and 30 kHz;

    - RTT-based UE compensation or gNB pre-compensation in 15 kHz and 30 kHz;

    - TA-based UE compensation with enhanced granularity of TA command in 30 kHz;

    - TA-based UE compensation with tightened UE transmit timing requirement in 30 kHz;

    - Non RTT gNB-based pre-compensation in 30 kHz.

Observation 2

- RTT-based propagation delay compensation requires additional UE-specific signal exchange between gNB and UE:

  - In DL direction, group-common signalling could be utilized to reduce overhead.

Observation 3

- After RAN2 offloaded the decision of propagation delay compensation scheme to RAN1, the list of potential options from RAN1#102-e does not match to the latest status of discussions in RAN2 and misses the gNB based pre-compensation option.

Proposal 1

- RAN1 to expand the list of propagation delay compensation options with gNB-based pre-compensation (both RTT-based and non-RTT based) in order to match with the latest status of RAN2 discussion.

Proposal 2

- RAN1 to agree to specify the following components to enable flexible propagation delay compensation scheme:

  - Component 1: Support of gNB-based pre-compensation transparent to propagation delay calculation scheme;

  - Component 2: Support of UE-based compensation transparent to propagation delay calculation scheme;

  - Component 3: Non-LPP measurement and indication to gNB of the UE Rx-Tx time difference based on DL signals;

  - Component 4: Non-LPP measurement and indication to UE of the gNB Rx-Tx time difference based on UL signals.

- References

- RP-201310, “Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR”, Nokia, Nokia Shanghai Bell, July 2020

- R1-2103027, “Further details on UE HARQ feedback enhancements”, Intel Corporation, e-Meeting, April 12 – 20, 2021

- R1-2103028, “Analysis of enhanced CSI feedback schemes”, Intel Corporation, e-Meeting, April 12 – 20, 2021

- R1-2103029, “Further Details on Enabling URLLC/IIoT in Unlicensed Band”, Intel Corporation, e-Meeting, April 12 – 20, 2021

- R1-2103030, “Further analysis and details of intra-UE multiplexing and prioritization”, Intel Corporation, e-Meeting, April 12 – 20, 2021

- R4-2106445, “Reply to LS on UE transmit timing error”, Intel Corporation, e-Meeting, April 12 – 20, 2021