3GPP TSG RAN WG1 #104b-e	                                              R1-2103054

e-Meeting, April 12th – 20th, 2021

Agenda Item:	8.14.1

Source: 	Intel Corporation

Title:	Traffic model for XR

Document for:        Discussion/Decision

- Introduction

In RAN1#104e, RAN1 made the following agreement:

Agreements - Statistical traffic model for a single DL video stream for a single UE - The statistical traffic model for a single UE for a single DL video stream in Figure 1 is adopted, where a packet is assumed to represent multiple IP packets corresponding to a single video frame for modelling/evaluation purposes, e.g., traffic arrival, packet size, evaluation of latency and reliability.

This is also shown the following figure where a hypothetical frame is assumed in RAN1 simulations that is comprised of all IP packets corresponding to the same rendering time instant.

Figure 1: Statistical modeling assumption in RAN1 where a hypothetical frame is constructed that is comprised of all IP packets corresponding to the same rendering time instant

- SA4 trace analysis (focusing on VR2)

Overview:

In [5] SA4 provided an overview of software tools, trace formats and configurations for VR2, CG and AR2 traffic. The document also provides initial recommended configurations for each of the above scenarios and provides relevant P-Traces. An overview of the different software modules and APIs that are provided for the purposes of RAN1 simulation is shown in the following

Figure 2: Overview of RAN evaluation framework

Figure 3: Overview of RAN evaluation framework

The following describes the individual functional blocks shown above and specifically the details for VR2 use-case is indicated below:

Content Model

This block provides typical characteristics for the XR content model for video, audio and potentially other data. This content is rendered for XR/CG consumption. For VR2

  - Game output with 2 eye buffers at 2Kx2K at 60 fps, 8bit.

  - Content and Trace Preview is here: http://dash.akamaized.net/WAVE/3GPP/XRTraffic/Traces/Qualcomm-VR2

  - No audio

Encoding Model

This functional block provides the details for the content encoding in order to meet certain objectives. This includes the generation of sequences of application data units (slices, video frames, audio frames, etc.) and the incurred timestamp of each of the units is available.

- HEVC model as defined in S4aV200575

- target bitrate 30 Mbit/s, equally split across eye buffers, independently encoded.

- Pre-encoding delay: Encoder pre-delay is varying between 10 to 20ms

Encoding delay is modelled to vary with truncated Gaussian

Content Delivery Model:

This functional block provides details on content delivery, for example packetization, delay jitter, but possibly also more sophisticated models such as retransmission, TCP operations and so on. This also includes emulation of 5G Core Network. It produces packet traces (timestamp and size of each packet) based on traces of application data units. For VR2:

- Edge/Cloud to gNB bitrate is 1.5 media bitrate

The following configurations are recommended with priority according to order in [5].

| Configuration | Basic Content Parameters |
| --- | --- |
| VR2-1 | 8 slices per eye buffer, 1 slice per frame is intra coded, 30Mbit/s capped VBR with window 200ms, buffer sent at same time, 1500 byte max packet size |
| VR2-2 | 8 slices per eye buffer, 1 slice per frame is intra coded, 30Mbit/s capped VBR with window 200ms, buffer sent at same time, unlimited packet size |
| VR2-3 | 8 slices per eye buffer, 1 slice per frame is intra coded, 30Mbit/s CBR with window 1 frame, buffer sent at same time, 1500 byte max packet size packets |
| VR2-4 | 8 slices per eye buffer, 1 slice per frame is intra coded, 30Mbit/s CBR with window 1 frame, buffer sent at same time, unlimited packet size |
| VR2-5 | 1 slice per eye buffer, every 8th frame is intra coded, 30Mbit/s capped VBR with window 200ms, buffer sent at same time, 1500 byte max packet size |
| VR2-6 | 8 slices per eye buffer, 1 slice per frame is intra coded, 30Mbit/s capped VBR with window 200ms, buffers sent interleaved, 1500 byte max packet size |
| VR2-7 | 8 slices per eye buffer, 1 slice per frame is intra coded, 45Mbit/s capped VBR with window 200ms, buffer sent at same time, 1500 byte max packet size packets |
| VR2-81 | 8 slices per eye buffer, 1 slice per frame is intra coded, 45Mbit/s capped VBR with window 200ms, buffer sent at same time, unlimited packet size |

## P-Trace analysis for VR2

A summary of the P-Trace analysis for VR2 use-case based on the configurations VR2-1 to VR2-8 is shown below.

|  | VR2-1 | VR2-3 | VR2-5 | VR2-7 | VR2-2 | VR2-4 | VR2-8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| #slice per eye buffer | 8 | 8 | 1 | 8 | 8 | 8 | 8 |
|  | 1 slice per frame is intra coded | 1 slice per frame is intra coded | every 8th frame is intra coded | 1 slice per frame is intra coded | 1 slice per frame is intra coded | 1 slice per frame is intra coded | 1 slice per frame is intra coded |
| Target bitrate | 30Mbit/s capped VBR with window 200ms | 30Mbit/s CBR with window 1 frame | 30Mbit/s capped VBR with window 200ms | 45Mbit/s capped VBR with window 200ms | 30Mbit/s capped VBR with window 200ms | 30Mbit/s CBR with window 1 frame | 45Mbit/s capped VBR with window 200ms |
| Max packet size (byte) | 1500 byte max packet size | 1500 byte max packet size packets | 1500 byte max packet size | 1500 byte max packet size | unlimited packet size | unlimited packet size | unlimited packet size |
| Bitrate (Mbps) | 28.81 | 29.83 | 26.15 | 41.01 | 28.15 | 29.15 | 40.08 |
| mean. frame-size (kbytes) | 59.02 | 61.12 | 53.58 | 84.02 | 57.68 | 59.73 | 82.11 |
| max/mean. frame-size | 1.18 | 1.06 | 1.94 | 1.24 | 1.23 | 1.06 | 1.23 |
| min/mean frame-size | 0.48 | 0.93 | 0.24 | 0.34 | 0.48 | 0.94 | 0.34 |
| std/mean frame-size | 0.07 | 0.02 | 0.12 | 0.14 | 0.07 | 0.02 | 0.14 |

Observations-1:

- for CBR configurations the frame-size variations are quite small

  - the max/mean frame-size ratio is ~ 1.06

  - the min/mean frame-size ratio is ~0.93

  - the std/mean frame-size ratio is ~0.02

- for cVBR configurations the frame-size variations are larger

  - the max/mean frame-size ratio is ~ 1.18 – 1.94 with smaller ratios corresponding to 8 slice/eye buffer case while large ratio corresponding to 1 slice case

  - the min/mean frame-size ratio is ~ 0.24 – 0.48 with larger ratios corresponding to 8 slice/eye buffer case while small ratio corresponding to 1 slice case

  - the std/mean frame-size ratio is ~ 0.07 – 0.14

The detailed analysis for the above configurations are shown below:

## 1500 byte max packet size cases

| Figure 4: Bitrate variation around target bitrate | Figure 5: packet-size CDFs (max 1.5 kbyte) |
| --- | --- |

| Figure 6: frame-size probability (calculated from P-trace) | Figure 7: frame-size CDFs (calculated from P-trace) |
| --- | --- |

| Figure 8: # of packets vs frame-size scatter plot | Figure 9: burst-length (ms) vs frame-size scatter plot |
| --- | --- |

## Unlimited max packet size configurations

| Figure 10: Bitrate variation around target bitrate | Figure 11: packet-size CDFs (max unlimited) |
| --- | --- |

| Figure 12: frame-size probability (calculated from P-trace) | Figure 13: frame-size CDFs (calculated from P-trace) |
| --- | --- |

| Figure 14:# of packets vs frame-size scatter plot | Figure 15: burst-length (ms) vs frame-size scatter plot |
| --- | --- |

Observations-2:

We observe that

- there is significant variation in traffic observed at L2 (P-trace) for a given content model (V-trace) and use-case example: VR2 30 Mbps 2 eye buffers at 2Kx2K at 60 fps, 8bit

- this variation is due to encoding model (slices, encoding delay, etc.) and content delivery model (packetization etc.)

- distribution of frame-size based on P-trace is asymmetric (heavy tail below mean, light tail above mean)

## V-Trace analysis for VR2

Just as a reference we also analyze the V-trace for VR2 traffic using the following files:

filename='XRTraffic\Traces\Qualcomm-VR2\V-Trace-left.xlsx';

filename='XRTraffic\Traces\Qualcomm-VR2\V-Trace-right.xlsx';

| Figure 16: Bitrate variation with time (left eye) | Figure 17: Bitrate variation with time (right eye) |
| --- | --- |

| Figure 18: frame-size CDF (left eye) | Figure 19: frame-size CDF (right eye) |
| --- | --- |

| Figure 20: frame-size probability (calculated from V-trace) – left eye | Figure 21: frame-size probability (calculated from V-trace) – right eye |
| --- | --- |

|  | bitrate (Mbps) | mean frame size (Kbytes) | max/mean frame size | min/mean frame size | std/mean |
| --- | --- | --- | --- | --- | --- |
| V-Trace-left | 16.44 | 33.68 | 6.59 | 0.15 | 0.26 |
| V-Trace-right | 16.41 | 33.62 | 6.83 | 0.14 | 0.25 |

Observations-3:

Comparing V-trace and P-trace frame-sizes we can observe that:

- distribution of frame-size based on P-trace is asymmetric (heavy tail below mean, light tail above mean) while frame-size based on V-trace is more symmetric in shape

- the max/mean frame-size calculated based on V-trace is much larger than that calculated based on P-trace. The encoding and the content delivery model clearly affects the frame-size distribution.

Then we also observe the following:

Observations-4:

The current frame-based statistical model lacks the following:

- different distributions for IP packet sizes (limited, unlimited, etc.)

- variation in the number of packets per burst, even for a given frame-size

- variation in burst length, even for a given frame-size

- assymmetry in frame-size distribution below and above mean

- variation due to use-cases (CG video is envisioned to be more interactive than VR)

Based on the above observations we make the following proposal:

Proposal 1: Adopt trace-based traffic model leveraging the SA4 work in RAN1 XR simulations

- Conclusions

Observations-1:

- for CBR configurations the frame-size variations are quite small

  - the max/mean frame-size ratio is ~ 1.06

  - the min/mean frame-size ratio is ~0.93

  - the std/mean frame-size ratio is ~0.02

- for cVBR configurations the frame-size variations are larger

  - the max/mean frame-size ratio is ~ 1.18 – 1.94 with smaller ratios corresponding to 8 slice/eye buffer case while large ratio corresponding to 1 slice case

  - the min/mean frame-size ratio is ~ 0.24 – 0.48 with larger ratios corresponding to 8 slice/eye buffer case while small ratio corresponding to 1 slice case

  - the std/mean frame-size ratio is ~ 0.07 – 0.14

Observations-2:

We observe that

- there is significant variation in traffic observed at L2 (P-trace) for a given content model (V-trace) and use-case example: VR2 30 Mbps 2 eye buffers at 2Kx2K at 60 fps, 8bit

- this variation is due to encoding model (slices, encoding delay, etc.) and content delivery model (packetization etc.)

- distribution of frame-size based on P-trace is asymmetric (heavy tail below mean, light tail above mean)

Observations-3:

Comparing V-trace and P-trace frame-sizes we can observe that:

- distribution of frame-size based on P-trace is asymmetric (heavy tail below mean, light tail above mean) while frame-size based on V-trace is more symmetric in shape

- the max/mean frame-size calculated based on V-trace is much larger than that calculated based on P-trace. The encoding and the content delivery model clearly affects the frame-size distribution.

Observations-4:

The current frame-based statistical model lacks the following:

- different distributions for IP packet sizes (limited, unlimited, etc.)

- variation in the number of packets per burst, even for a given frame-size

- variation in burst length, even for a given frame-size

- assymmetry in frame-size distribution below and above mean

- variation due to use-cases (CG video is envisioned to be more interactive than VR)

Proposal-1: Consider more accurate trace-based traffic model leveraging the SA4 work in RAN1 XR simulations

# References

| [1] | 3GPP SA4, "TR 22.842, Study on Network Controlled Interactive Service (NCIS) in the 5G System (5GS)," 20 12 2019. [Online]. |
| --- | --- |
| [2] | 3GPP SA4, "TR 22.925, Quality of Service (QoS) and network performance 3.1.1," 23 04 1999. [Online]. |
| [3] | 3GPP SA4, "TR26.928, Extended Reality (XR) in 5G 16.1.0," 23 12 2020. [Online]. |
| [4] | Qualcomm, "S4-201399, FS_XR Traffic: Permanent document v0.3.1," [Online] |
| [5] | R1-2101765, LS to on XR-Traffic Models, SA4 |

# Appendix

The trace format for P-trace is shown below:

| Name | Type | Semantics |
| --- | --- | --- |
| number | BIGINT | Unique packet number in the delivery |
| time_stamp_in_micro_s | BIGINT | Availability time of packet for next processing step relative to start time 0 in microseconds (0 means lost). |
| size | BIGINT | packet size in bytes. |
| user_id | BIGINT | assigns an id to the user in order to differentiate |
| buffer | BIGINT | The associated eye buffer 1=left 2=right In general, differentiates application traffic for different buffers, for example audio, video, left eye, right eye. For example mapped to port or track. |
| delay | BIGINT | Delay observed of the packet in the last processing step (-1 means lost) |
| render_timing | BIGINT | the rendering generation timing associated to the media included in the packet. |
| number_in_unit | BIGINT | The number of the packet within the unit (slice), start at 1 |
| last_in_unit | BIGINT | Indicates if this is the last packet in the slice/unit 0=no, 1=yes |
| type | BIGINT | The data type of the unit 0 unknown For video 1=intra 2=inter |
| importance | BIGINT | assigned relative importance information (higher number means higher importance) |
| index | BIGINT | Unique index increased by 1 and indexing this row in the S-Trace file. |
| s_trace | STRING | Reference to s_trace file containing information for each slice |

This configuration is not mentioned in [5]