3GPP TSG RAN WG1 #104b-e                                                             R1-2103056

e-Meeting, 12th – 20th April, 2021

Source: 	Intel Corporation

Title:	On synchronization for NB-IoT and eMTC NTN

Agenda item:	8.15.2

Document for:	Discussion and Decision

# Introduction

At the 3GPP TSG RAN meeting #86 new study item (SI) on NB-IoT/eMTC support for Non-Terrestrial Network (NTN) was approved in [1]. This SI aims to evaluate and confirm solutions to address the minimum necessary specifications for NB-IoT and eMTC NTN. One of the objectives which is related to RAN1 is to study and recommend necessary changes to support NB-IoT and eMTC over satellite, reusing as much as possible the conclusions of the studies performed for NR NTN in TR38.821 [2] including the following items.

- Aspects related to random access procedure/signals

- Mechanisms for time/frequency adjustment including Timing Advance, and UL frequency compensation indication

- Timing offset related to scheduling and HARQ-ACK feedback

- Aspects related to HARQ operation

In this contribution possible enhancements on UL and DL synchronisation for NB-IoT/eMTC to support NTN are discussed.

# Discussion

As it is stated in the study item description (SID) [1] accurate UL synchronisation can be achieved by using pre-compensation of delay and Doppler for UL transmission at the UE side. UE can derive values of frequency shift and timing advance for pre-compensation based on UE location obtained from GNSS and satellite ephemeris broadcasted by the gNB. The note from the SID [1] which is related to the pre-compensation of delay and Doppler for UL transmission is captured below.

GNSS capability in the UE is taken as a working assumption in this study for both NB-IoT and eMTC devices. With this assumption, UE can estimate and pre-compensate timing and frequency offset with sufficient accuracy for UL transmission. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed.

Large time and frequency offset are observed for both feeder link (UE-satellite) and service link (satellite-gateway) due to large distance to the satellite and high velocity of the satellite. Time and frequency offset for NTN is schematically illustrated in figure 1.

Figure 1. Time and frequency offset for NTN

Time and frequency offset introduced in service link is different for different UEs in the deployment since it is a function of UE location. Thus, compensation of service link time and frequency offset for UL transmission should be done at the UE side based on UE location (from GNSS) and satellite ephemeris (broadcasted by the gNB).

Feeder link time and frequency offset is common for all the UEs in the deployment and can be post-compensated at the gNB for UL. Alternatively, feeder link time offset can be broadcasted by the gNB and pre-compensated at the UE side for UL transmission. This will allow operation of deployment with aligned UL and DL timing at the gNB. Compensation of feeder link frequency offset at the UE side may be problematic for some cases due to the fact that for transparent payload deployments different frequency bands can be used for feeder link and service link.

Proposal 1:

- Time and frequency offset introduced in service link is pre-compensated by the UE for UL transmission based on UE location (from GNSS) and satellite ephemeris (broadcasted by the gNB)

- The following options are considered for compensation of time offset introduced in feeder link for UL transmission

  - Post-compensation at the gNB side

  - Pre-compensation at the UE side

- Compensation at the gNB side should be used for frequency offset introduced in feeder link for UL and DL transmission

In order to support the pre-compensation of time offset introduced in feeder link for UL, indication of the feeder link time offset should be supported. It can be done by broadcasting common TA value and common TA drift rate. Alternatively, indication of reference point for pre-compensation can be considered. In this case UE can determine common TA based on the distance between satellite and reference point and update it autonomously.

Proposal 2:

- If pre-compensation of time offset introduced in feeder link for UL is used, at least one of the following options should be supported

  - Broadcasting of common TA and common TA drift rate

  - Broadcasting of reference point for common TA calculation

For NTN non-GEO deployments with moving beams, frequent beam change is required even if the UE is stationary. If frequency reuse is deployed in such scenario then different beams are using different frequency for transmission and reception as it is schematically illustrated in figure 2.

Figure 2. NTN deployment with frequency reuse

Since some physical layer-based solution for beam change are not supported for LTE (like beam management and BWP switching in NR), beam change should be discussed assuming existing eMTC and NB-IoT features (e.g. multi-carrier operation, mobility). For example, different beams with different carrier frequency can represent different carriers for multi-carrier operation.

Since different beams are serving separate geographical areas the synchronization signals and PRACH should be transmitted in every carrier for this case. Carrier with synchronization signals for NB-IoT is called anchor carrier. The number of anchor carrier is limited in existing NB-IoT specification. In order to support scenario with frequency reuse and non-GEO satellite deployment with moving beams increased number of anchor carriers for multi-carrier operation in NB-IoT can be considered.

Proposal 3:

- Enhancements for non-GEO satellite deployment with moving beams and frequency reuse should be discussed assuming existing features of eMTC and NB-IoT (e.g. multi-carrier operation and mobility)

  - Increased number of anchor carriers for NB-IoT multi-carrier operation can be considered

UL compensation gaps (UCG) are supported for NB-IoT in order to maintain time/frequency synchronisation for long UL transmission (longer than 256 ms). This feature was introduced mainly for correction of frequency drift of the UE oscillator caused by temperature variations. There is another issue in NTN for UL synchronisation during long UL transmission. Due to high satellite velocity delay and Doppler of service link and feeder link is variating over time which may lead to degradation of the performance for long UL transmission due to accumulation of time and frequency error.

For correction of frequency drift due to oscillator imperfections measurements of DL signals are needed. For correction of delay and Doppler variations in NTN DL measurements are not required. Since delay and Doppler is predictable, UE can autonomously update delay and Doppler values based on satellite ephemeris and UE location. So, additional gap in long UL transmission is not needed for DL measurements. However, it is not clear whether additional gap is needed in order to calculate and apply the updated TA and UL frequency.

Proposal 4:

- Additional gap for DL measurements during long UL transmission is not needed

  - It is assumed that UE can autonomously update delay and Doppler values based on satellite ephemeris and UE location

- FFS: whether additional gap during long UL transmission is needed to calculate and apply the updated TA and UL frequency at the UE

# Conclusion

In this contribution aspects related to enhancements for NB-IoT/eMTC NTN were discussed. The following proposals are made.

Proposal 1:

- Time and frequency offset introduced in service link is pre-compensated by the UE for UL transmission based on UE location (from GNSS) and satellite ephemeris (broadcasted by the gNB)

- The following options are considered for compensation of time offset introduced in feeder link for UL transmission

  - Post-compensation at the gNB side

  - Pre-compensation at the UE side

- Compensation at the gNB side should be used for frequency offset introduced in feeder link for UL and DL transmission

Proposal 2:

- If pre-compensation of time offset introduced in feeder link for UL is used, at least one of the following options should be supported

  - Broadcasting of common TA and common TA drift rate

  - Broadcasting of reference point for common TA calculation

Proposal 3:

- Enhancements for non-GEO satellite deployment with moving beams and frequency reuse should be discussed assuming existing features of eMTC and NB-IoT (e.g. multi-carrier operation and mobility)

  - Increased number of anchor carriers for NB-IoT multi-carrier operation can be considered

Proposal 4:

- Additional gap for DL measurements during long UL transmission is not needed

  - It is assumed that UE can autonomously update delay and Doppler values based on satellite ephemeris and UE location

- FFS: whether additional gap during long UL transmission is needed to calculate and apply the updated TA and UL frequency at the UE

# References

- RP-193235 Study on NB-Io/eMTC support for Non-Terrestrial Network

- 3GPP TR 38.821 V16.0.0 (2019-12); 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Solutions for NR to support non-terrestrial networks (NTN) (Release 16)