3GPP TSG-RAN WG1 Meeting #104bis-e                              R1-2103082

# e-Meeting, April 12th – 20th, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | XXXX | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to VRB-to-PRB in DCI Format 1_2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_L1enh_URLLC-Core |  |  |  |  |  | Date: |  |  | 2020-04-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The presence or absence of VRB-to-PRB mapping field in DCI Format 1_2 is determined according to: VRB-to-PRB mapping – 0 or 1 bit: - 0 bit if the higher layer parameter vrb-ToPRB-InterleaverDCI-1-2 is not configured; - 1 bit according to Table 7.3.1.2.2-5 otherwise, only applicable to resource allocation type 1, as defined in Clause 7.3.1.6 of [4, TS 38.211]. It can be seen that with the RRC configurations as follows: - Resource allocation Type: 0 - vrb-ToPRB-InterleaverDCI-1-2-r16: configured The VRB-to-PRB mapping field is present in the DCI. In contrast, the presence or absence of VRB-to-PRB mapping field in DCI Format 1_1 is determined as follows, and no un-necessary overhead is incurred: VRB-to-PRB mapping – 0 or 1 bit: - 0 bit if only resource allocation type 0 is configured or if interleaved VRB-to-PRB mapping is not configured by high layers; - 1 bit according to Table 7.3.1.2.2-5 otherwise, only applicable to resource allocation type 1, as defined in Clause 7.3.1.6 of [4, TS 38.211]. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Align the condition of VRB-to-PRB mapping’s presence in DCI Format 1_2 as that in DCI Format 1_1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DCI size reduction of DCI Format 1_2 is not achieved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Isolated Impact Analysis: This CR has isolated impact. Inter-operability analysis: - The changes have no interoperability consequences. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts are omitted>

7.3.1.2.3	Format 1_2

….

VRB-to-PRB mapping – 0 or 1 bit:

-	0 bit if only resource allocation type 0 is configured or if the higher layer parameter vrb-ToPRB-InterleaverDCI-1-2 is not configured;

-	1 bit according to Table 7.3.1.2.2-5 otherwise, only applicable to resource allocation type 1, as defined in Clause 7.3.1.6 of [4, TS 38.211].

<Unchanged parts are omitted>