3GPP TSG- RAN WG1 Meeting #104b-e                                                 R1-2103093

# e-Meeting, April 12th – 20th, 2021

Agenda Item:	8.1.3

Source:	Apple Inc.

Title:	Views on Rel-17 SRS enhancement

Document for:	Discussion/Decision

# Introduction

In RAN#86 meeting, Rel-17 FeMIMO WI was approved in RP-193133 [1]. Four main areas haven been identified for the Rel-17 FeMIMO, namely

- Enhancement on multi-beam operation, mainly targeting FR2 while also applicable to FR1

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2

- Enhancement on SRS, targeting both FR1 and FR2

- Enhancement on CSI measurement and reporting

Regarding SRS enhancement, the following objectives are approved as part of Rel-17 FeMIMO [1]

- Enhancement on SRS, targeting both FR1 and FR2: - Identify and specify enhancements on aperiodic SRS triggering to facilitate more flexible triggering and/or DCI overhead/usage reduction - Specify SRS switching for up to 8 antennas (e.g., xTyR, x = {1, 2, 4} and y = {6, 8}) - Evaluate and, if needed, specify the following mechanism(s) to enhance SRS capacity and/or coverage: SRS time bundling, increased SRS repetition, partial sounding across frequency

In the last RAN1 meeting RAN1#104e, the following agreement is reached [2]

Agreement Confirm the following working assumption with modifications An “available slot” is a slot satisfying there are UL or flexible symbol(s) for the time-domain location(s) for all the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set. - From the first symbol carrying the SRS request DCI and the last symbol of the triggered SRS resource set, UE does not expect to receive SFI indication, UL cancellation indication or dynamic scheduling of DL channel/signal(s) on flexible symbol(s) that may change the determination of “available slot”. - Note: Collision handling between the triggered SRS and any other UL channel/signal is performed after the determination of available slot. - FFS: Rules to handle the case of multiple SRS resource sets with overlapping symbols and/or triggered by a same DCI Decision: As per email posted on Jan 30th, Agreement - For aperiodic antenna switching SRS, support to configure N <=N_max resource sets, where totally K resources are distributed in the N resource sets flexibly based on RRC configuration. - For 1T6R, K=6, N_max = [4], and each resource has 1 port. - For 1T8R, K=8, N_max = [4], and each resource has 1 port. - For 2T6R, K=3, N_max = [3], and each resource has 2 ports. - For 2T8R, K=4, N_max = [4], and each resource has 2 ports. - (Working Assumption) For 4T8R, K=2, N_max = [2], and each resource has 4 ports. - FFS the number of supported candidate values of N for each xTyR. - FFS extension to increase N_max for 1T4R, 2T4R, T=R and 1T2R cases for aperiodic, periodic and semi-persistent SRS resources - FFS the number of resources and resource sets for semi-persistent and periodic antenna switching SRS - Note: SRS could be transmitted over the last 6 OFDM symbols, or over any OFDM symbols within the slot subject to UE capability. Agreement For Rel-17 SRS capacity and coverage enhancement, support the following - Increase the maximum number of repetition symbols in one slot and one SRS resource to S - Support at least one S value from {8, 10, 12, 14} - FFS other candidate values - Support to transmit SRS only in contiguous RBs in one OFDM symbol, where indicates the number of RBs configured by BSRS and CSRS - Support at least one PF value from {2, [3], 4, 8} - FFS other candidate values, e.g., non-integer values for PF - Note: SRS sequence shorter than the minimum length supported in the current specification is not pursued. - No new sequence including length is introduced - FFS it is applicable to frequency hopping and non-frequency hopping - FFS detailed signaling mechanism to determine PF and the location of the RBs - Support Comb 8 - Note: SRS sequence shorter than the minimum length supported in the current specification is not pursued. - FFS whether and if needed, how to use harmonized approach to define the three supported schemes - Note: other schemes for SRS capacity and coverage enhancements are not supported in Rel-17. Agreement Further study whether and if needed, how to achieve further enhancements on aperiodic SRS triggering and resource management based on repurposing unused fields in DCI format 0_1/0_2 without data and without CSI. Consider the following examples - CAT A: Time-domain parameters - A-1: Indication of available slot position, i.e., the t values - A-2: Indication of slot offset - A-3: Indication of SRS symbol-level offset - A-4: Indication of time-domain behaviour for SRS transmission over multiple OFDM symbols, e.g., repetition, hopping, and/or splitting - CAT B: Frequency-domain parameters - B-1: Indication of a group of CCs for SRS transmission - B-2: Indication of frequency domain resource in a BWP for SRS transmission - B-3: Indication of whether DL/UL BWP is applied for SRS transmission - CAT C: Power control parameters - C-1: Re-purpose ‘TPC command for PUSCH’ as ‘TPC command for SRS’ - FFS impact on power control, impact from triggering a group of CCs for SRS - C-2: Indication of open loop power control parameter e.g., p0. - CAT D: Spatial-domain parameters, i.e., indication of SRS port and beamforming - CAT E: Extend the number of DCI codepoints for aperiodic SRS trigger states - Other examples are not precluded Agreement A list of t values is configured in RRC for each SRS resource set. Adopt at least one of the following for DCI indication of t. - In DCI format 0_1/0_2 without data and without CSI request, - Alt 1-1: Reuse the same scheme used for DCI format 0_1/0_2/1-1/1-2 that schedules a PDSCH or PUSCH - Alt 1-2: Re-purpose unused DCI field to indicate t - Alt 1-3: t is indicated by a configurable DCI field, where the DCI field may contain bits from unused fields and additional bits configured by gNB - FFS design details with other potential field(s) - FFS: whether t can be slot offset - In DCI format 0_1/0_2/1-1/1-2 that schedules a PDSCH or PUSCH - Alt 2-1: t is indicated by adding a new configurable DCI field - Alt 2-2: t is indicated without adding DCI payload - Note: The size of DCI payload does not change dynamically - Note: RAN1 should strive for unified solution for different DCI formats. - FFS: The number of RRC configured t values per SRS resource set and DCI bit field size.

In this contribution, we provide our views on the Rel-17 FeMIMO SRS enhancement

# AP-SRS triggering flexibility enhancement

In the last meeting, we agreed to allow flexible and dynamic indication of AP-SRS triggering slot offset. For the dynamic DCI based indication of the AP-SRS slot offset, i.e., t, we have the following proposal

Proposal 1 For DCI based indication of AP-SRS slot offset, i.e., t, for both DCI format 0_1/0_2/1-1/1-2 that schedules a PDSCH or PUSCH and DCI format 0_1/0_2 without data and without CSI request

- t is indicated by adding a new configurable DCI field

We  also agreed to allow DCI format 0_1 and 0_2 to trigger AP-SRS without CSI and without data, following proposal address the remaining issue

Proposal 2 When DCI format 0_1 and 0_2 is used to trigger AP-SRS without data and without CSI, do not

re-purpose unused fields in DCI format 0_1 and 0_2

# SRS resource set with multiple usage

In Rel-15/16, SRS-ResourceSet can only be configured with 4 possible “usage” among {beamManagement, codebook, nonCodebook, antennaSwitching}, and, for each SRS-ResourceSet, only one usage can be configured.

For example, when SRS-ResourceSet is configured with “usage” of “antennaSwitching”, it is intended to be used for reciprocity-based DL channel acquisition. However, the same SRS-ResourceSet may also be allowed to be used for antenna selection to be combined with other use cases, such as, codebook based or non-codebook based PUSCH operation. Therefore, we have the following proposal

Proposal 3 In Rel-17, support to use SRS-ResourceSet configured with usage = “antennaSwitching” for codebook based PUSCH scheduling

- This feature is RRC explicitly configured

- This feature is optional with capability signalling for the UE

- FFS: when multiple SRS-ResourceSet are configured with usage = “antennaSwitching”

- FFS: when SRS-ResourceSet configured with usage = “antennaSwitching” contains more than 2 SRS-Resource

For the dynamic antenna switching SRS, we have the following proposal

Proposal 4 In Rel-17, consider to allow UE to indicate the preferred antenna switching configuration in terms of nTmR

# SRS capacity and coverage enhancement

In the last meeting, we agreed on three solutions to enhance the SRS capability and coverage, we have the following proposing to address some of the remaining issues

Proposal 5 For SRS capacity and coverage enhancement,

- For increasing the maximum number of repetition symbols in one slot and one SRS resource to S at least from {8, 10, 12, 14}

  - Consider extending the repetitionFactor beyond {n1, n2, n4}

- For configuring SRS only in 1PFmSRS,BSRS  contiguous RBs in one OFDM symbol

  - Both the PF and partial subband offset between 0 and PF-1 need to be configured

# Conclusion

In RAN#86 meeting, Rel-17 FeMIMO WI was approved in RP-193133 [1]. Four main areas haven been identified for the Rel-17 FeMIMO, namely

- Enhancement on multi-beam operation, mainly targeting FR2 while also applicable to FR1

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2

- Enhancement on SRS, targeting both FR1 and FR2

- Enhancement on CSI measurement and reporting

In this contribution, we provide our views on the Rel-17 FeMIMO SRS enhancement with the following proposals

Proposal 1 For DCI based indication of AP-SRS slot offset, i.e., t, for both DCI format 0_1/0_2/1-1/1-2 that schedules a PDSCH or PUSCH and DCI format 0_1/0_2 without data and without CSI request

- t is indicated by adding a new configurable DCI field

Proposal 2 When DCI format 0_1 and 0_2 is used to trigger AP-SRS without data and without CSI, do not

re-purpose unused fields in DCI format 0_1 and 0_2

Proposal 3 In Rel-17, support to use SRS-ResourceSet configured with usage = “antennaSwitching” for codebook based PUSCH scheduling

- This feature is RRC explicitly configured

- This feature is optional with capability signalling for the UE

- FFS: when multiple SRS-ResourceSet are configured with usage = “antennaSwitching”

- FFS: when SRS-ResourceSet configured with usage = “antennaSwitching” contains more than 2 SRS-Resource

Proposal 4 In Rel-17, consider to allow UE to indicate the preferred antenna switching configuration in terms of nTmR

Proposal 5 For SRS capacity and coverage enhancement,

- For increasing the maximum number of repetition symbols in one slot and one SRS resource to S at least from {8, 10, 12, 14}

  - Consider extending the repetitionFactor beyond {n1, n2, n4}

- For configuring SRS only in 1PFmSRS,BSRS  contiguous RBs in one OFDM symbol

  - Both the PF and partial subband offset between 0 and PF-1 need to be configured

# Reference

- RP-193133, "New WID: Further enhancements on MIMO for NR", 3GPP TSG RAN Meeting #86, Sitges, Spain, December 9-12, 2019

- Chairman Notes, 3GPP TSG RAN WG1 Meeting #104-e, e-Meeting, January 25th – February 5th, 2021