3GPP TSG-RAN WG1 #104bis-e		R1-2103099

e-Meeting, January 25th – February 5th, 2021

Source: 	Apple Inc.

Title:                     Beam Management for New SCSs

Agenda item:	8.2.4

Document for:	Discussion and Decision

# 1 Introduction

In RAN1 #104 e-meeting, the following was agreed for different initial access channels so as to extend current NR operation to 71GHz [1]:

Agreement: - For NR operation in 52.6-71GHz with new SCSs, new parameter values for at least the following timing parameters are needed: - timeDurationForQCL - beamSwitchTiming - beamReportTiming - Companies are encouraged to provide preferred values on timeDurationForQCL, beamSwitchTiming and beamReportTiming Agreement: Rel-15/16 and any Rel-17 beam management enhancements can be considered for 52.6-71 GHz. Whether particular features should be excluded for 52.6-71 GHz can be further discussed. - Note: As per usual procedure, duplication of work between work items in Rel-17 should be avoided Agreement: - Further study new parameter values for at least the following parameters: - maxNumberRxTxBeamSwitchDL - Additional beam switching time delay d for triggering AP-CSI-RS when triggering PDCCH with 120kHz or 480kHz has a smaller subcarrier spacing than AP-CSI-RS - Study whether/how to introduce a beam switching gap between signals/channels - FFS: condition to apply including potential UE capability definition - Study should account for inputs from RAN4 Agreement: Further study the following: - For multi-PDSCH scheduling with a single DCI, study the QCL assumption(s) the UE should apply for each PDSCH for the case when some of the scheduled PDSCHs have scheduling offset less than timeDurationForQCL while some have scheduling offset equal to or greater than timeDurationForQCL. - For multi-PDSCH scheduling with a single DCI, study the QCL assumption(s) the UE should apply for each PDSCH for the case when all of the scheduled PDSCHs have scheduling offset less than timeDurationForQCL - Note: If the current Rel-16 behavior would be extended to multiple-PDSCH scheduling, it could result in a different QCL assumption for each PDSCH due to the fact the that the CORESET with the lowest ID can be different for different slots, resulting in a potentially different TCI state for each slot - Note: Applicability to multi-TRP can be discussed further Agreement: Further study the following: - For multi-PDSCH scheduling with a single DCI, study whether or not it is needed to indicate a separate TCI state for each scheduled PDSCH - For multi-PUSCH scheduling with a single DCI, study whether or not it is needed to indicate a separate SRI (indication of TCI can be further discussed) for each scheduled PUSCH - Note: the study should take into account DCI overhead aspects - Note: Applicability to multi-TRP can be discussed further

In this contribution, we discuss the remaining issues of initial access channels, focusing on the perspectives below:

- Timing associated with Beam Management.

- Potential Enhancements for BM operation.

# 2. Discussion

## 2.1 Timing associated with Beam Management

In RAN1 103 e-meeting, a set of timing parameters related to BM were identified for new SCSs 480kHz and 960kHz SCS summarized in Table 1 below:

Table 1: Rel-15/16 timing related to beam management

| Index | Parameter/IE | Definition | Value ranges | Note |
| --- | --- | --- | --- | --- |
| 1 | timeDurationForQCL | minimum number of OFDM symbols required by the UE to perform PDCCH reception and applying spatial QCL information received in DCI for PDSCH processing. | 60kHz: {s7, s14, s28} 120kHz: {s14, s28} | Per SCS Per FS |
| 2 | beamSwitchTiming | The minimum number of OFDM symbols between the DCI triggering of aperiodic CSI-RS and aperiodic CSI-RS transmission | 60kHz/120kHz: {sym14, sym28, sym48, sym224, sym336} | Per SCS Per FS |
| 3 | Additional beam switching time delay ‘d’ | Additional delay for beam switching when different SCSs is used for PDCCH and A-CSI-RS. | 15kHz: {sym8} 30kHz: {sym8} 60kHz:{sym14} | Per SCS Per FS New values for PDCCH with 120kHZ and 480kHZ. |
| 4 | beamReportTiming | The number of OFDM symbols between the last symbol of SSB/CSI-RS and the first symbol of the transmission channel containing beam report. | 15kHz: {sym2, sym4, sym8} 30kHz: {sym4, sym8, sym14, sym28} 60kHz: {sym8, sym14, sym28} 120kHz: {sym14, sym28, sym56} | Per SCS Per band |
| 5 | maxNumberRxTxBeamSwitchDL | The number of Tx and Rx beam changes UE can perform on this band within a slot. | 15kHz/30khz/60kHz/120kHz: {n4, n7, n14} | Per SCS Per band |

For the first three parameters, i.e., ‘timeDurationForQCL’ and ‘beamSwitchTiming’, the processing timeline generally involves two parts, one is PDCCH processing and the other is associated beam adjustment e.g., intra-UE communication, beam selection, sub-array switching and phase-shift change, etc. Given the fact that the higher frequency band is sort of FR2 extension to beyond 52.6GHz, processing time for new SCSs is preferrable to allow reusing the existing FR2 hardware for this new frequency regardless of SCS. It is also the reason that RAN1 agreed the following in the RAN1 #104 e-meeting as one general guideline for timeline related discussions [1]:

Agreement: - RAN1 use the absolute time duration for 120 kHz SCS as the upper bound for the discussion of UE processing timelines (not related to PDCCH monitoring) for 480 kHz and 960 kHz SCS for NR operation in 52.6 to 71 GHz - RAN1 strives to reduce the absolute time durations from the upper bound if feasible

Proposal 1:

- Reuse the absolute time duration defined for 120kHz SCS for new SCSs (i.e., scaling up 4/8 times for 480kHz and 960kHz SCS respectively):

  - timeDurationForQCL

  - beamSwitchTiming

  - beamReportTiming

- Reuse the absolute time defined for 60kHz for Additional beam switching time delay ‘d’, i.e., 28 symbols for 120kHz and 112 symbols for 480kHZ.

## 2.2 Potential Enhancements for BM operation

In RAN1 104 e-meeting, support of Multiple Beams for Multiple PDSCHs was discussed. It was agreed to further study the QCL assumption(s) the UE should apply for each PDSCH in multi-PDSCH scheduled by a single DCI as shown in FIG.2:

- Case 1: When all of the scheduled PDSCHs have scheduling offset less than timeDurationForQCL.

- Case 2: When some of the scheduled PDSCHs have scheduling offset less than timeDurationForQCL while some have scheduling offset equal to or greater than timeDurationForQCL.

Figure 2: Identified use cases for potential beam management enhancement

In Rel-15/16, for Case 1 and 2, a UE assumes that the DM-RS ports of PDSCH of a serving cell are QCLed with the RS(s) in the CORESET associated with a monitored search space with the lowest CORESET ID in the latest slot within the active BWP of the serving cell are monitored by the UE. Consequently, QCL assumption for the TBs in Case 1 and TB1/TB2 in Case 2 maybe varied if the QCL assumption of CORESETs in the latest slot are different.

For 480kHz and 960kHz SCS, the slot duration becomes much shorter compared to SCSs supported for FR2 in earlier release. It motivates to introduce a new mechanism to allow a single default QCL assumption for TBs that have scheduling offset less than timeDurationForQCL. There are a set of solutions to be considered. One example is to allow gNB to configure a default QCL assumption e.g., based on the latest beam measurement report from UE and then configure the best beam through RRC signaling or scheduling DCI format as being discussed in Rel-17 FeMIMO WI.

Proposal 2: Support a mechanism to allow a single QCL assumption at least for multi-PDSCH scheduled by a single DCI that have scheduling offset less than timeDurationForQCL.

As illustrated in FIG.3, different QCL assumptions between the latest CORESETs and PDSCHs result in additional gap(s) to perform beam switching, given the fact that the UE is still required to monitor PDCCHs in the CORESET. The spectrum efficiency loss caused by the gap depends on several factors, e.g., length of switching gap, the period of search space associated with CORESET and TCI configuration of the CORESET(s). Since all of this relevant information is known at the gNB side, it may be beneficial to consider some mechanisms to allow gNB enabling or disabling the PDCCH monitoring within the scheduled multiple PDSCH duration such that the switching gap can be dynamically adapted based on the relevant factors.

Figure 3: The switching gap in case different TCIs between default beam and CORESETs

Proposal 3: Consider a mechanism to enable/disable the PDCCH monitoring within the multi-PDSCH duration to improve resource efficiency.

# 3. Conclusion

In this contribution, we have presented our views on open issues related to beam management enhancement with new SCSs on above 52.6GHz frequency bands. Based on the discussions above, the following was proposed:

Proposal 1:

- Reuse the absolute time duration defined for 120kHz SCS for new SCSs (i.e., scaling up 4/8 times for 480kHz and 960kHz SCS respectively):

  - timeDurationForQCL

  - beamSwitchTiming

  - beamReportTiming

- Reuse the absolute time defined for 60kHz for Additional beam switching time delay ‘d’, i.e., 28 symbols for 120kHz and 112 symbols for 480kHZ.

Proposal 2: Support a mechanism to allow a single QCL assumption at least for multi-PDSCH scheduled by a single DCI that have scheduling offset less than timeDurationForQCL.

# References

- RAN1 104 e-meeting Chairman notes