3GPP TSG- RAN WG1 Meeting #104b-e                    R1-2103104

e-Meeting, April 12th – 20th, 2021

Agenda Item:	8.3.1.2

Source:	Apple Inc.

Title:	Views on URLLC CSI feedback enhancements

Document for:	Discussion/Decision

# Introduction

At RAN1 #102-e, the following was agreed:

Agreements:

- Study/evaluate further on following CSI enhancement schemes in terms of technical benefit, specification and implementation impacts.

  - New triggering methods for A-CSI and/or SRS

  - New reporting based on one or more of the following:

    - Case 1: channel/interference measurement for new CSI reporting, considering aspects such as one or more of the following:

      - Reporting more accurate interference characteristics

      - Reduced CSI feedback overhead (e.g., reporting interference measurement only)

      - Enhanced CSI reporting such as WB/SB CQI

    - Case 2: other measurement (other than channel/interference) for additional information

      - E.g., PDCCH/PDSCH decoding, recommended HARQ RV sequence, etc.

    - It targets to help gNB scheduler for better link adaptation of (re)transmission

  - [Reduced CSI computation time/complexity]

  - [CSI feedback for PDCCH]

  - Other CSI enhancement schemes that enable accurate MCS selection are not precluded

- Detailed assumptions of the proposed CSI enhancement schemes should be provided by the proponent, such as

  - Reporting values

  - Triggering conditions for the reporting

  - Associated measurement resource

  - Uplink resource to be used for the reporting

  - How to use the reported information at the gNB scheduler

  - CSI-RS overhead and CSI reporting frequency

  - CSI reporting latency/timeline

  - Etc.

In this contribution we provide our initial views on those enhancements.

# Discussion on new reporting type

During the study item stage of URLLC/IIoT of Rel-16, the processing timeline for URLLC/IIoT was studied by companies and the evaluation has been captured in TR 38.824.  To support URLLC/IIloT with stringent latency requirement, in one approach, the network can choose the MCS level conservatively, and ensure there is a high probability that a UE receives a single transmission and decodes transmitted data packet successfully. With that transmission approach, the number of UEs which can be supported in a network can be rather small, as each of them consumes considerable resource accompanying with the low MCS level assignment, which translates into low system spectral efficiency of the network. Given spectrum can be rather scarce, serving a small number of UEs while meeting URLLC’s latency & reliability requirements may not be an economically viable solution.  In another approach, the network may not choose the MCS level conservatively for the first transmission. For example, targeting a block error rate at 10-1 10% or 10-2 instead of 10-6; then for most cases, with a not small probability (1-10-1) or (1-10-2), the first transmission of a transport block leads to its successful decoding at a UE; then only for the fractional cases (10-1 10% or 10-2(1%) ), HARQ retransmission is needed. For retransmission, the base station can choose a robust transmission to ensure the high reliability (e.g. 10-6 error rate after retransmission(s)) is achieved. With the second approach, a higher system spectrum efficiency can be achieved than in the case with the first approach.

From the study item phase evaluations, it is also seen that in many cases, the second transmission (or the first retransmission) is the only chance for the network to retransmit the transport block as the latency requirement can be quite stringent (e.g. 1 millisecond). From that, how to provide useful feedback information to the network by the UE becomes a critical issue.

When reviewing the current NR framework for hybrid ARQ transmission scheme and CSI feedback framework, we identify two issues:

Issue 1:

In the conventional hybrid ARQ transmission scheme, the UE feeds back HARQ-ACK with either ACK to NACK in response to a successful or failed decoding of a transport block; the network retransmits the transport block with an additional PDSCH; and the UE tests whether it can decode the transport block successfully with the newly available soft bits, and generates another HARQ-ACK feedback as a result. The hybrid feedback retransmission scheme can take multiple rounds until the UE finally receives the transport block successfully. For an URLLC application, the network and the UE may not have the luxury of multiple rounds of information exchange between them, in this contribution we propose to study several options to address this issue.

Issue 2:

Also conventionally for CSI feedback, from measurement of the desired channel (from CMR (Channel Measurement Resource)) and interference (from ZP IMR (Zero Power Interference Measurement Resource), which is also called CSI-IM in the NR specification) and potentially structured interference from NZP IMR (None Zero Power Interference Measurement Resource), the UE generates a feedback including all or some of CQI, PMI and RI for a hypothetical transmission with a targeted block error rate over a reference resource, which inherently does not have anything to do with the ongoing URLLC transmission, the gNB cannot easily deduce actionable information from the feedback. To address the current framework’s deficiency, we also propose to study options to address this issue.

Related to the discussion here, several companies provided proposals on soft NACK/ACK from  the RAN1 #103-e contributions, an incomplete list includes:

- Ericsson: 	Soft ACK, soft NACK

- IDC:		Soft ACK, slow soft-ACK, control the adjustment rate.

- Sony:     		Failure reason, soft-NACK

- ZTE:		Soft NACK

- …

Figure 1 Soft NACK/ACK schemes

In Figure 1, 3 soft NACK/ACK schemes are illustrated. it is realized for OLLA operation with soft ACK, even though the target BLER can be quite low, the target BLER cannot be directly used; a reference BLER which is much higher than the target BLER is used to generate soft ACK (in Scheme 3 a High-ACK (H-ACK) or  a Low-ACK (L-ACK) can be generated with respect to the reference BLER).

In the proposed scheme with HARQ redundancy version sequence, which can be converted as soft NACK for OLLA operation in addition to the utility as feedback for the current transmission, the target BLER for the first transmission can be set high, e.g. at 1%, so an aggressive MCS level selection can be used for the first transmission. Note for most of the time, the first transmission is successful. The reference BLER for soft NACK generation can be lower than that of the target BLER for the first transmission.

The HARQ redundancy version sequence feedback scheme to be discussed below, can be treated as a soft NACK feedback scheme.

Figure 2 HARQ feedback in Rel-15/Rel-16

## Discussion on potential enhancements

In conventional hybrid ARQ scheme, as latency requirement may be more relaxed, e.g. for eMBB, than for URLLC, the network and the UE can afford to use parsimony of feedback information: HARQ feedback consisting of a single feedback bit for each round of feedback, and potentially conducted over many rounds to explore and finally find the number of retransmissions needed for successful decoding.

In URLLC, reliability requirement can vary for specific services, e.g. the BLER can be at upper-bounded by 1%, 0.1% or 0.01%, etc. Even though the final BLER requirement may be at 10-6, by no means that can be translated into a requirement for the first transmission’s BLER requirement, i.e. the first transmission’s BLER may well above 10-6.Multiple scenarios were examined during the study item stage (Section 6.4.1.1 “latency analysis”, TR 38.824), where two transmissions were assumed for URLLC.

In URLLC, in many cases, due to stringent latency requirement, the 2nd transmission is the only opportunity for the gNB to provide more coded bits to the UE, so they can be combined with previously received coded bits (LLRs) for successful decoding. As such, if a UE does not decode PDSCH successfully for the first transmission or for a retransmission when the latency bound is in danger of being exceeded, the more relevant information for the UE to provide is not merely the fact the UE fails to decode the transport block, rather how much more redundancy is needed from the gNB to allow the UE to decode the transport block in the next attempt, which can be the only chance for the UE to receive the transport block within the latency bound. From that, it is reasonable to allow the UE to indicate how much redundancy is needed further for the UE to decode the transport block. Also the UE can consider the current status of the soft buffer in its feedback to the gNB. We suggest considering the following scheme:

A basic redundancy version sequence with [0 2 3 1] is agreed between the gNB and UE beforehand; and the redundancy version sequence of length Nrv   recommended by the UE is read out over the basic redundancy version sequence with  a starting position, and wrap-around is used if the last element of the basic redundancy version sequence is reached before the desired length is met, as shown in Figure 4, the UE can recommend a starting position at “3” with length 3 (or 3 versions), so [3 1 0] can be indicated to the gNB. Other examples include:

  - [0 2], starting version “0”, length 2;

  - [0 2 3 1], starting version “0”, length 4;

  - [2 3 1], starting version “2”, length 3.

  - [3], starting version “3”, length 1.

The benefit of indicating the starting version can be seen as follows:

A number of factors affect the status of UE’s soft bits: the redundancy versions utilized in previous transmissions, missed reception of PDCCH scheduling transmission of PDSCH, interference from other cells, aged CSI which leads to sub-optimal MCS level selection. While the UE has a clear view of the current status of its soft bits, affected by those listed factors at least, the gNB may be oblivious of them. We use two cases below to illustrate that.

- In one case, the first transmission is with redundancy version “0”, and the UE receives the PDSCH with benign channel/interference condition, and the UE determines just a little bit more redundancy information from the gNB would allow it to decode the transport block successfully, and in this case the UE can indicate redundancy version “1” for retransmission (e.g. for lowering the code rate);

- In another case, the first transmission is with redundancy version “0”, and the UE receives the PDSCH with severe interference and/or channel fading, the soft bits for systematic bits of codeblock(s) are unreliable, then the UE can ask for retransmission to start with redundancy version “0” to ensure systematic bits are retransmitted.

It can be seen that the UE may desire different starting version for the recommended redundancy version sequence, depending on the current status of its soft bits. Including Nrv  a length of the HARQ redundancy version sequence essentially provides to the network on the shortage to successful decoding. The gNB can choose to use Nrv   times of the resource allocation as the current one’s.

Figure 3 wrap-around read-out with the basic redundancy version sequence at [0, 2, 3, 1]

A number of examples are provided in Figure 3.

Figure 4 Examples of enhanced HARQ feedback schemes

To simplify implementation, the redundancy version sequence [ 0 0 0 0], can be also considered.

We have

Observation: feedback with HARQ redundancy version sequence is another form of soft NACK design.

Proposal 1: study the benefit of UE’s recommendation of a HARQ redundancy version sequence to the gNB:

If the UE receives the transport block correctly,

the UE feeds back “ACK”;

Else the UE feeds back a HARQ redundancy version sequence   which can be indicated by assuming read-out over a basic redundancy version sequence say [0 2 3 1] or [0 0 0 0] with a starting version and a length Nrv  .

# Discussion on AP CSI over PUCCH

For the discussion of AP-CSI over PUCCH, we assume the following:

For CSI measurement, the measurement resources are still as found in Rel-16, i.e. CSI-RS including CSI-RS for CMR and IMR,  CSI-IM. Whether measurement resources include CSI-RS for other use cases such as beam management and TRS can be further discussed. For measurement over P/SP CSI measurement resources, we don’t envision any change is needed in Rel-17. For AP CSI resources, which can be triggered by an uplink DCI in both Rel-15 and Rel-16, we can discuss extension of the triggering mechanism to DL DCIs, such as DCI format 1_1 and 1_2; by introducing a trigger state field in DL DCI 1_1 and 1_2 respectively. Following the design precedence established for new DCI formats under URLLC, the trigger state field should be made configurable through RRC signaling.

As for CSI report quantity, in Rel-15, 8 report quantities including “none” are supported; in Rel-16, L1-SINR is further introduced. If no new CSI report quantity is introduced, then the same trigger state list (CSI-AperiodicTriggerStateList)  can be shared between uplink DCI formats and downlink DCI formats. In the current trigger state design, one trigger state can be associated with up to 16 CSI reports (maxNrofReportConfigPerAperiodicTrigger = 16). Considering for URLLC, the number of CSI reports needed for the gNB to make scheduling decision may not be so many, e.g. only CSI reporting relevant to the ongoing transmission is needed, and for other transmissions CSI reporting can be shared between eMBB and URLLC traffic, the number of CSI reports associated with a trigger state can be much reduced, e.g.  reducing the number of CSI reports to one for AP CSI reporting triggered by a DL DCI. Considering from that angle, it seems also possible and perhaps even desirable to introduce a separate trigger state list for DL DCIs.  In any case, with trigger state lists for both DL DCIs and UL DCIs, on-chip memory is required to store the trigger state information, hence the total number of trigger states including those for DL and uplink DCIs should be limited.  For that, a UE capability can be introduced to limit the total number of activated trigger states for both DL DCIs and UL DCIs. We have

Observation 2: If separate trigger state lists are introduced for DL DCIs and UL DCIs,  the total number of activated trigger states should be limited; and the supported total number is a UE capability. Alternatively, the total number of trigger states associated with DL DCIs is limited to X (X is FFS), and X is a UE capability.

In DCI format 0_1 and 0_2, the PRI indication is used to indicate the PUCCH resource for HARQ-feedback. For DL DCI triggered AP-CSI report, if PRI is introduced in a DL DCI, the UCI for CSI feedback can be multiplexed with HARQ-feedback in the indicated PUCCH resource.

The additional information concerning PDSCH decoding can be also transmitted with AP CSI over PUCCH.

# Interference characteristics

Interference can change at a faster pace than the desired signal. The variation of the desired signal strength can be due to UE mobility and/or updated beamforming due to updated CSI feedback. The variation in interference can be caused by scheduled transmissions in adjacent cells, e.g. with mini-slot transmissions at adjacent cells for different UEs, the interference can change quickly even within a single slot. To characteristics in the fluctuation in CSI, approaches were proposed in previous meetings. In the feature lead summary from RAN1 #103-e, a summary can be found:

Issue #2-1: Support new report type based on CQI/SINR statistics (e.g. mean, variance, percentile, predicted CSI)

- Yes: Futurewei [2], Ericsson [5], Nokia [17], Intel [20], Qualcomm [25]

  - Allow network to set conservative MCS with a certain margin [2]

  - Requires less UL overhead and complexity than network estimating variance from UE CSI reports[2][5]

  - More accurate link adaptation for low target BLER and bursty interference [17]

  - Averaging/filtering can combat interference uncertainties in bursty traffic [20]

- Study: ZTE [15], Lenovo [22], Qualcomm [25]

  - Configure channel/interference filtering (number of occasions) [15]

  - Need for additional resources, time window size [22]

  - Consider case when URLLC traffic is sporadic [22]

- No: CATT [6], Samsung [10]

  - Performance gain depends on algorithm used at gNB. Should be discussed in MIMO SI/WI. [6]

  - Network can use antenna or frequency diversity to cope with short-term interference variations [10]

Several companies discuss new report types based on separately reporting interference statistics:

Issue #2-2: Support new report types based on interference statistics

- Yes: Futurewei [2], Huawei [3]

  - More accurate estimate of interference covariance matrix for MU-MIMO compared to gNB utilizing CQI for reconstructing matrix [3]

  - Less sensitive to sub-optimal UE pair selection by SRS [3]

  - Processing simpler than traditional CSI computation [3]

- Study: Qualcomm [25]

  - Mean, time-domain autocorrelation, percentile of interference

- No: CATT [6], Samsung [10], ZTE [15]

  - Difficult to perform testing for the reporting (this is explicit feedback) [6]

  - Already discussed in several releases of LTE and NR. Should be discussed in MIMO SI/WI. Not enough time in URLLC WI. [6]

  - Performance gain depends on algorithm used at gNB side [6]

  - gNB can use RSRP/RSRQ/RSSI/SINR reports [10]

  - High standardization effort to define new CSI feedback overhead and processing latency [15]

In general, generating CQI/SINR statistics can be more involved than generating statistics for interference alone. Under Issue #2-1, between CQI statistics and SINR statistics, the CQI statistics provides feedback can be directly used by gNB in for MCS selection if the CQI statistics is assumed for a distribution actually experienced by the UE, as CQI is tied to the hypothetical BLER with which the CQI is calculated. Mean and standard deviation can quantify the tail behavior of the Gaussian distribution well. It can be expected if feedback with mean and standard deviation for CQI/SINR is specified, some quantization schemes need to be specified.  Other distributions, such as Gamma distribution, truncated Gaussian distribution, or lognormal distribution, can have rather different tail behaviors, and the case is  still to be made in terms of the selection of distribution, it is not clear mean and standard deviation still are the right quantities for feedback.

Compared with CQI/SINR statistics, SINR statistics does not lead to a direct use at the gNB side, as mapping from SINR to CQI may be nontrivial, and testability issue for SINR statistics needs to be addressed. Under Issue #2-2, testability issue should be addressed also.  If SINR statistics with Issue #2-1 or interference statistics with Issue #2-2 is to be specified, then a reference CSI reporting needs to be reported by the UE:

- With SINR statistics, a reference CSI reporting with a report quantity with CQI included in its report and the associated SINR should be reported in the new report type:

  - SINR statistics for the assumed distribution

  - A reference CSI report with reportQuantity from {cri-RI-PMI-CQI, cri-RI-i1-CQI, cri-RI-CQI, cri-RI-LI-PMI-CQI}

  - The SINR associated with the reference CSI report

- With interference statistics, a reference CSI reporting with a report quantity with CQI included in its report and the associated interference should be reported in the new report type:

  - interference statistics for the assumed distribution

  - A reference CSI report with reportQuantity from {cri-RI-PMI-CQI, cri-RI-i1-CQI, cri-RI-CQI, cri-RI-LI-PMI-CQI}

  - The measured/assumed interference associated with the reference CSI report

We have

Proposal 2: Clarify how gNB uses the new feedback from #2-1 and #2-2.

Under Issue #2-4 in the RAN1 #103-e feature lead summary, several schemes were also proposed in previous meetings:

Issue #2-4: Support new report types based on modifying existing formats

- Simplify report quantity of subband CSI feedback [7]

- Worst-M CQI

  - Support: Nokia [17] (lowest measured quality is what is most important for URLLC)

  - Study: Lenovo [21] (does not rely on UL DCI triggering but additional PUCCH payload)

  - No support: Samsung [10] (Best-M CQI is optimal)

- Worst-Best CQI

  - Support: Qualcomm [25] (rely on worst-case to be conservative but not too much)

- Wideband CQI excluding lowest-Q subbands

  - Support: Mediatek [21] (smaller CQI offset range, relies on gNB avoiding worst subbands)

- 3-bits differential CQI

  - Support: Mediatek [21] (Reduce MCS prediction error)

- 4-bits subband CQI (no differential CQI)

  - Support: Huawei [3] (2-bits differential CQI has large MCS prediction errors)

  - No support: Nokia [17] (Can obtain same or better performance with worst-M at lower uplink overhead cost)

For URLLC traffic, wideband scheduling can be used to mitigate frequency selective fading and/or interference. It seems worst-M CQI, worst-Best CQI, etc can be beneficial if localized scheduling (e.g. several clusters of resources in the BWP), and interference profile over those clusters does not change from CSI feedback to PDSCH scheduling. It is not clear such stationarity for interference can be safely assumed. If some network coordination for cells can be used to make interference more predictable, e.g. with fractional frequency use following certain reuse patterns across cells, then CSI measurement with reportFreqConfiguration can be already used, assume there are 12 subbands are present in the active BWP, a bitmap with subbands12 such as [110011…1] can be used from Rel-15 to limit the subbands where the UE will report the CQI. Thus the benefits of those schemes are not clear.

As for finer differential CQI granularity, a fundamental fact is that while there are 5 bits for MCS levels and there are only 4 bits for CQI; some kind of OLLA adaption is inevitable. Also to justify the finer differential CQI granularity, the stationarity of interference/CQI is also implicitly assumed, the question raised for worst-M CQI can be asked here also. From these considerations, the benefits with such schemes are also not clear.

We have

Proposal 3: Clarify the assumption on stationarity of interference for solutions under issue #2-4.

# 3. Conclusions

In this contribution we share our views on HARQ/CSI-RS enhancements. We have

Observation 1: feedback with HARQ redundancy version sequence is another form of soft NACK design.

Observation 2: If separate trigger state lists are introduced for DL DCIs and UL DCIs,  the total number of activated trigger states should be limited; and the supported total number is a UE capability. Alternatively the total number of trigger states associated with DL DCIs is limited to X (X is FFS), and X is a UE capability.

Proposal 1: study the benefit of UE’s recommendation of a HARQ redundancy version sequence to the gNB:

If the UE receives the transport block correctly,

the UE feeds back “ACK”;

Else the UE feeds back a HARQ redundancy version sequence   which can be indicated by assuming read-out over a basic redundancy version sequence say [0 2 3 1] or [0 0 0 0] with a starting version and a length Nrv  .

Proposal 2: Clarify how gNB uses the new feedback from #2-1 and #2-2.

Proposal 3: Clarify the assumption on stationarity of interference for solutions under issue #2-4.

# References

- RP-193233, “New WID on enhanced Industrial Internet of Things (IoT) and URLLC support”, Nokia, Nokia Shanghai Bell, RAN #86, December 2019

- RP-201310, “Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR”, Nokia, Nokia Shanghai Bell, RAN #88e, June 2020

- R1-2006515, “CSI feedback enhancements for URLLC”, Apple, RAN1 #102-e, August 2020.