3GPP TSG RAN WG1 #104bis-e		R1-2103105

e-Meeting, April 12th – April 20th, 2021

Agenda Item:	8.3.2

Source:	Apple Inc.

Title:	URLLC uplink enhancements for unlicensed spectrum

Document for:	Discussion/Decision

# Introduction

A work item on enhanced IIoT and URLLC was approved in RAN#86, and the WID was further updated in RP-201310 (RAN#88, June 2020) [1]. One objective is about the uplink enhancements for URLLC in unlicensed spectrum:

- Uplink enhancements for URLLC in unlicensed controlled environments [RAN1, RAN2]:

  - Specify support for UE-initiated COT for FBE with minimum specification effort

  - Harmonizing UL configured-grant enhancements in NR-U and URLLC introduced in Rel-16 to be applicable for unlicensed spectrum

In this contribution, we discuss these aspects and share our views based on the agreements made in previous meetings (see Appendix A).

# UE-initiated COT for FBE

## Configuration parameters

The following was agreed in RAN1#104-e for the FFP periodicity and offset for UE-initiated COT:

Agreement:

- In semi-static channel access mode, UE FFP periodicity is chosen from the following set of values in ms: {1, 2, 2.5, 4, 5,10}.

  - FFS on other values

Agreement:

- In semi-static channel access mode:

- An FFP period for UE-initiated COT is configured as the same, integer multiple of, or inter-factor of the FFP period configured for gNB-initiated COT

- FFP period for UE-initiated COT can be configured independently from FFP period of gNB-initiated COT, if the UE indicates the corresponding capability

- FFP offset for UE-initiated COT is the starting point of first UE FFP relative to the radio frame X boundary.

  - The offset value range is 0 ≤ offset ＜FFP period of UE-initiated COT

    - FFS on X (e.g. X=0, or X= even index number)

Regarding the two FFS issues,

- For the FFP periodicity, gNB does not need to have it exactly match the CG PUSCH periodicity, and it can choose to over-provision to achieve the same purpose. For example, if the CG PUSCH has a periodicity of 3ms, the gNB can still configure the FFP periodicity of 1ms to allow the UE to initiate COT. In this sense, the current value set is sufficient, and there is no need to introduce additional values.

- Assuming no new values are introduced, we can reuse the way how FFP offset is defined for gNB-initiated COT. That is, FFP offset is defined relative to the even radio frame boundary.

Proposal 1-1: No additional value is introduced for UE FFP periodicity. UE FFP offset is defined relative to the even radio frame boundary.

In terms of enabling UE-initiated COT, there are a few options and some options may be combined:

- Option 1: it is enabled per UE.

  - This can be easily done by reusing the FFP periodicity and offset configuration. That is, when the FFP periodicity and offset are configured for a UE, UE-initiated COT is enabled for the UE.

- Option 2: it is enabled per type of channel (e.g. PUSCH, PUCCH, SRS)

- Option 3: it is enabled per type of signal (e.g. PUSCH/data, HARQ-ACK, SR, P-CSI, SRS)

- Option 4: it is enabled per PHY priority

- Option 5: it is enabled per configuration for configured UL transmission.

More granularity certainly means better control of which transmissions can use UE-initiated COT. But the tradeoff between overhead and flexibility should also be considered.

It makes sense to always allow HARQ-ACK and SR to initiate a COT once UE-initiated COT is configured for the UE. However, it may be desirable to provide the flexibility to disable UE-initiated COT for P-CSI and/or SRS, for UE/gNB power saving purpose.

For PUSCH transmission, from channel access perspective, if there is no overlapping in the resources allocated to UL transmissions from different UEs (i.e., there is no collision), it always provides more channel access opportunities by enabling UE-initiated COT for all the UL transmissions, without much need to differentiate different types of transmissions.

Whether there is a need to introduce per-CG configuration to enable/disable UE-initiated COT depends on whether it makes sense to configure overlapping resources for multiple UEs. Overlapping CG PUSCH resources for multiple UEs was considered a valid use case in Rel-16, because it increases the channel utilization efficiency (larger probability that at least one of the UEs can access the channel). In Rel-16, if multiple UEs are configured with overlapping resources for UL transmissions (i.e., CG PUSCH) to increase the probability of UEs using the resources, the collision issue has been handled by random offset for UEs’ LBT so that one UE’s transmission can prevent other UEs from starting their transmission. However, for UE-initiated COT with FBE, the FFP boundary is pre-defined and cannot move. The Rel-16 mechanism does not apply any more.

If for CG PUSCH, the UE always initiates its own COT instead of sharing gNB’s COT (even when it is available), it does not make much sense to configure overlapping resources for multiple UEs any more because they will most likely collide. Otherwise, it could still be useful at least when the UEs share gNB’s COT, by reusing Rel-16 mechanism to resolve the collision. But when the UEs initiate their own COT, it would be better if only one UE is allowed to initiate the COT to avoid collision. In this case, it can be useful to introduce configuration for each CG configuration to enable or disable UE-initiated COT.

With these considerations, we propose the following:

Proposal 1-2: UE-initiated COT is considered enabled once the FFP periodicity and offset are configured. Introduce a RRC parameter to disable UE-initiated COT for P-CSI and/or SRS.

- FFS whether to introduce a RRC parameter to disable UE-initiated COT for each CG configuration, which overrides the per-UE configuration for this CG if configured.

## Determination of UE-Initiated COT or Sharing gNB’s COT

### Configured UL transmission

For a configured UL transmission, the following has been agreed.

Agreements: (RAN1#103-e)

In semi-static channel access mode:

- When a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE, down-select one of the following:

  - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT

  - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT.

  - Alt-c: The UE assumption on whether the configured UL transmission is allowed to correspond to UE-initiated COT is based on gNB configuration.

- When a configured UL transmission starts after a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE:

  - If the UE has already initiated the UE FFP, then UE assumes that the configured UL transmission corresponds to UE-initiated COT

  - Otherwise, If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and if the UE has already determined that gNB has initiated that gNB FFP, then UE assumes that the configured UL transmission corresponds to gNB-initiated COT.

- FFS on other conditions for determining the corresponding UE or gNB initiated COT

- Note: A configured UL transmission cannot be transmitted according to both shared gNB COT and UE-initiated COT.

Agreement: (RAN1#104-e)

In semi-static channel access mode when a UE can operate as UE-initiated COT,

- Select one of the following alternatives to determine whether a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, is based on UE-initiated COT or sharing a gNB-initiated COT:

  - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT

  - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT.

we can separate them into a few sub-cases to consider, listed as cases A, B and C below and illustrated in Figure 1.

- Case A: When the UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE

- Case B: When the UL transmission starts after a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE

  - This has already been covered by the agreements made in RAN1#103-e, so we do not need to discuss further.

- Case C: When the UL transmission overlaps with the UE FFP’s idle period

Figure 1 Illustration of Case A, Case B and Case C

For case A, we have two alternatives, Alt-a and Alt-b, per RAN1#104-e agreements.

First of all, Alt-a and Alt-b are the same in terms of the effectiveness in acquiring the channel, because for both alternatives the UE would use Cat-2 LBT to access the channel.

Alt-a partly inherits the behavior from Rel-16, and it avoids the situation where there is UE-initiated COT coexisting with gNB’s COT at the same time. Even though this may be considered conceptually cleaner, we do not see any issues or disadvantages with Alt-b where a UE may initiate its own COT within a gNB’s COT. Even if the UE initiates a COT for the UL transmission, it does not interfere with gNB’s COT, and the gNB can continue to use its own COT for any DL transmission.

Alt-b has some benefit in terms of UE power saving, because the UE is not required to always monitor all the DL signals. Certainly the UE is still required to monitor the configured DL transmission, such as configured PDCCH, periodic CSI-RS, etc. However, Alt-b may allow the UE to skip e.g. SSB in some occasions.

Comparing the two alternatives, Alt-b provides some benefit in UE power saving, without any obvious downside.

Proposal 1-3: When a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE, adopt Alt-b, i.e., the UE assumes that the configured UL transmission corresponds to UE-initiated COT.

For case C (when the UL transmission overlaps with the UE FFP’s idle period), the most straightforward behavior is as follows:

Proposal 1-4: When a configured UL transmission overlaps with the UE FFP’s idle period,

- If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the UL transmission corresponds to gNB-initiated COT using Cat-2 LBT.

- Otherwise, it is not transmitted.

### Scheduled UL transmission

For a scheduled UL transmission, the following options have been agreed on how to determine whether the UL transmission should share gNB’s COT or use UE-initiated COT, with the latest options agreed in RAN1#104-e.

Agreements: (RAN1#103-e)

In semi-static channel access mode, a UE should be able to determine whether a scheduled UL transmission should be transmitted according to shared gNB COT or UE-initiated COT.

- UE determines the initiator of a COT based on at least one of the following alternatives:

  - Alt 1: Introduce additional bit field in the scheduling DCI

  - Alt 2: Based on ChannelAccess-CPext field in DCI

  - Alt. 3: Based on a predetermined rule(s)

  - Alt. 4: Based on RRC signalling

  - Alt. 5: Based on MAC CE

  - FFS other alternatives

- FFS on overriding possibility and/or the assumption

- Note: A scheduled UL transmission cannot be transmitted according to both shared gNB COT and UE-initiated COT.

Agreement: (RAN1#104-e)

In semi-static channel access mode when a UE can operate as initiating device,

- Select one of the following alternatives to determine whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT:

- Alt-a: Determination based on the content in the scheduling DCI

  - FFS on whether the corresponding field(s) can be absent in DCI

    - If absent, determination based on the rules applied for configured UL transmissions is applied

  - FFS whether/how to handle the case when the gNB schedules an UL transmission in the next gNB’s FFP period

  - Alt-b: Determination based on the rules applied for a configured UL transmission

Comparing Alt-a and Alt-b, Alt-a has the advantage that it allows the possibility for the gNB to indicate Cat-1 LBT (i.e. no LBT) to the UE by properly controlling the gap between DL and UL. On the other hand, with Alt-b, the UE always needs to perform Cat-2 LBT. Therefore, we think Alt-a should be supported.

Regarding whether the indication field can be absent or not in the scheduling DCI, it is straightforward to support it and use the same rules for configured UL transmissions if there is no default channel access mechanism explicitly configured.

For the case when the gNB schedules an UL transmission not in the same gNB’s FFP as the scheduling DCI, it is feasible to either (1) follow the indication in the scheduling DCI regardless (which depends on proper indication from the gNB), or (2) to follow the same rules as the configured UL transmissions. There is no fundamental difference between the two approaches. The first approach can be adopted to keep unified behavior for scheduled UL transmission.

Proposal 1-5: For a scheduled UL transmission, it is supported that the scheduling DCI indicates whether the UE initiates its own COT or shares gNB’s COT (Alt-a), and the corresponding LBT scheme (Cat-1 or Cat-2 with proper CP extension). If such indication is not included in the scheduling DCI, the UE follows the same rules as a configured UL transmission.

# Configured grant enhancements

CG enhancements have been introduced separately in Rel-16 eURLLC and NR-U WIs.

The CG enhancements in NR-U mainly addressed the channel access issue, and the design targeted to increase the chance for UE to acquire the channel. To support the design, CG-UCI was introduced for CG to provide necessary uplink control information.

The main CG enhancement in eURLLC is the support of multiple CG configurations to support multiple traffic flows within a UE. In addition, PUSCH repetition Type B has been introduced, which applies to both CG and DG.

The following has been agreed in RAN1#102-e and RAN1#103-e:

Agreements: - At least for FBE, configuration of (cg-RetransmissionTimer) should not be mandated when configured grant Type 1 or Type 2 are configured on unlicensed spectrum. Conclusion: Further study and decide how to harmonize the CG features for Rel-16 URLLC and Rel-16 NR-U. Table 1 in R1-2005376 can be used as a starting point for the corresponding discussion and decision. Agreements: Down-select one of the following options (target RAN1#104-e): - Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16. - Option 2-a: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and cg-RetransmissionTimer-r16, respectively. - Option 2-b: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and new parameter Y, respectively, where X and Y are different from cg-RetransmissionTimer-r16. - Option 3: CG-UCI based procedures are supported for unlicensed. CG-DFI based procedures are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16 - Note: Procedures based on CG-UCI rely on UE including CG-UCI in CG PUSCH at least as in Rel-16 where the values of the respective fields of CG-UCI are decided by UE. - Note: Procedures based on CG-DFI rely on automatic re-transmission on CG configuration and reception of CG downlink feedback information (DFI) in DCI for re-transmissions.

R1-2005376 provides a summary table for the comparison between URLLC CG and NR-U CG features, as copied below.

From R1-2005376 CG features; Rel.16 URLLC; Rel.16 NR-U. Multiple CG configurations; Supported; Supported. HARQ process number/ ID determination; Associated with the configured/indicated first TO, calculated based on the equation defined in TS 38.321; Decide and reported by the UE in CG-UCI. Management of HARQ process number/ ID among multiple CG configurations; Not shared between different CG configurations in the same BWP; Can be shared between different CG configurations in the same BWP. RV determination; One of the three RV sequence can be configured and associated with TO {0,0,0,0}; {0,3,0,3}; {0,2,3,1}; Decide and reported by the UE in CG-UCI. Flexible initial transmission occasion (TO); If the CG is configured with Configuredgrantconfig-StartingfromRV0 set to 'off', the initial transmission only starts at the first TO of the K repetitions; otherwise, the initial transmission TO depends on the configured RV sequence and K repetitions.; Multiple consecutive potential TOs are configured by cg-nrofPUSCH-InSlot-r16 and cg-nrofSlots-r16, can start initial transmission at any TOs depending on the LBT results.. Repetition scheme(s); PUSCH repetition Type A and PUSCH repetition Type B; Similar as PUSCH repetition Type B without supporting segmentation. (no support of cross-slot resource allocation, and if collide with invalid symbol(s), drop the repetition). CG-Downlink feedback information (DFI); No support. If Re-scheduling UL grant is not received, UE assumes ACK.; Support, If CG-DFI is not received, UE assumes NACK.. CG Re-transmission timer; No support; Support and always configured. CG Re-transmission scheme; Only support Re-transmission scheduled by UL grant; Support automatic re-Transmission on the same or different CG configuration decided by UE, and support re-Transmission scheduled by UL grant.

Among the list, we have one row for CG re-transmission timer, and another row for CG-DFI. The remaining functionality blocks marked as Orange are related to CG-UCI support.

First of all, it is clear that cg-RetransmissionTimer-r16 requires CG-UCI and CG-DFI to make the retransmission scheme work. This means that if cg-RetransmissionTimer-r16 is configured, the other functions need to be enabled as well.

Then the question is whether it could be useful to enable CG-UCI or CG-DFI by itself without CG retransmission.

CG-DFI provides HARQ-ACK for CG PUSCH, and it can address the issue of mis-detection of CG PUSCH, which could be a bigger issue on unlicensed spectrum due to uncontrolled interference. But this should work together with CG retransmission, because otherwise the UE cannot take any action even if it receives NACK for a CG transmission. Therefore, there does not seem to be any advantage or a use case to support CG-DFI without retransmission timer, and it makes sense to always configure these two together.

CG-UCI, on the other hand, is slightly different. Even though CG-UCI is required to support CG retransmission, CG-UCI by itself is also used to handle the LBT failure. Therefore, it can be an independent function.

Proposal 2-1: Adopt Option 2-a, i.e., “CG-UCI based procedures” are enabled or disabled for unlicensed using a new RRC parameter, and “CG-DFI based procedures” are enabled or disabled for unlicensed using cg-RetransmissionTimer-r16.

- Note: if cg-RetransmissionTimer-r16 is configured, “CG-UCI based procedures” should also be enabled.

In RAN1#104-e, it was agreed to support PUSCH repetition Type B.

Agreement:

- PUSCH repetition Type B is supported for unlicensed band operation when using NR IIoT Rel-16 based CG

  - FFS whether/how to enhance

One major advantage of Rel-16 NR-U CG is that it supports the flexible start of the transmission based on LBT outcome, which is a very useful feature for unlicensed band. It also allows the UE to transmit multiple TBs within one period. Therefore, we think both should also be supported for PUSCH repetition Type B. Naturally this requires CG-UCI being enabled.

Figure 2 Illustration of PUSCH repetition Type B enhancements

Proposal 2-2: For PUSCH repetition Type B enhancements on unlicensed spectrum, support the flexible start of the transmission and multiple TBs within a period when CG-UCI is enabled.

- An additional parameter is configured for each CG configuration to indicate the total number of consecutive transmission occasions within a period.

Another issue that should be considered is the orphan symbol handling. In Rel-16 URLLC, if an actual repetition consists of a single symbol, it is not transmitted. On unlicensed spectrum, if the orphan symbol is not transmitted, the UE would need to perform another LBT when continuing the transmission of the next repetition. This behavior can be modified to avoid the LBT.

Proposal 2-3: For PUSCH repetition Type B enhancements on unlicensed spectrum, orphan symbol(s) are transmitted if they are between two actual repetitions that are transmitted.

Further CG enhancements

When multiple CGs are configured, each CG is configured with its own resource allocation with separate periodicity and offset, depending on the applications. It is very likely that some CGs will overlap in time at some occasions (e.g. CG 1 and CG2 as shown in Figure 2). With the existing design, assuming each CG is configured to carry the expected payload for each flow, the UE would not be able to transmit the traffic from multiple flows using any one of the CGs.

One possible way to get around the issue is to configure an additional CG (CG3 as shown in Figure 2), which covers the overlapping occasions with a larger resource allocation to carry the traffic from both flows. Then in the overlapping occasions, the UE can choose to use CG3 instead of CG1 or CG2. However, this may require more resources to be configured/reserved for CGs, which is not resource efficient. In addition, as the number of CG configurations increase, the overlapping situation may become more complicated, and the configuration of additional CGs to handle the overlapping occasions also becomes more complicated. The additional CGs that can be configured may also be limited by the number of CGs a UE can support.

Figure 3 The overlapping of multiple CG configurations

The framework of NR-U CG design provides a very good foundation to address such an issue. With CG-UCI being transmitted by the UE together with CG, the UE can have some freedom adapting the transmission parameters for the traffic flows, and then provide these parameters to the gNB as part of CG-UCI to avoid blind detection on data transmission at the gNB. The most straightforward example is MCS. For example, we have two CG configurations for an audio flow and a video flow, respectively, that overlaps at certain occasions. At these occasions, the UE can choose the CG with more resources (the one for video flow), and choose a slightly higher MCS level that can accommodate the traffic for both flows. With the higher MCS level, the UE can also adjust transmit power accordingly to ensure decoding performance. Some configurations can be provided by the gNB to control the range that a UE can adapt, e.g. the set of allowed MCS values.

The idea of allowing the UE to autonomously choose MCS for CG is not new, and it has been discussed before in both NR-U and URLLC. There were two main concerns:

- gNB complexity. The concern was that it may cause too much additional gNB complexity in decoding. However, as all the UE transmission parameters are carried in CG-UCI, there is no need for blind detection/decoding at the gNB. The gNB could further restrict the MCS values the UE can choose from.

- Reliability of UCI. This was raised as a strong concern in URLLC discussion. However, for unlicensed spectrum, CG-UCI is already present to address the LBT issue. As long as the payload size increase is not significant, the beta offset can be configured properly to ensure the UCI performance.

Supporting autonomous MCS adaptation at the UE is an efficient way to address the CG overlapping issue. It can additionally address the drawback of CG that gNB cannot adapt the MCS dynamically, by allowing the UE to adapt by itself based on the RF condition change. This further improves the efficiency. Other than MCS, other parameters may also be considered for adaptation such as the transmission duration.

Proposal 2-4: Consider enhanced CG-UCI on unlicensed spectrum to allow the UE to autonomously adapt certain transmission parameters such as MCS.

# Conclusion

In this contribution, we discussed the UE-initiated COT for FBE and potential configured grant enhancements for unlicensed spectrum, and proposed the following:

Proposal 1-1: No additional value is introduced for UE FFP periodicity. UE FFP offset is defined relative to the even radio frame boundary.

Proposal 1-2: UE-initiated COT is considered enabled once the FFP periodicity and offset are configured. Introduce a RRC parameter to disable UE-initiated COT for P-CSI and/or SRS.

- FFS whether to introduce a RRC parameter to disable UE-initiated COT for each CG configuration, which overrides the per-UE configuration for this CG if configured.

Proposal 1-3: When a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE, adopt Alt-b, i.e., the UE assumes that the configured UL transmission corresponds to UE-initiated COT.

Proposal 1-4: When a configured UL transmission overlaps with the UE FFP’s idle period,

- If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the UL transmission corresponds to gNB-initiated COT using Cat-2 LBT.

- Otherwise, it is not transmitted.

Proposal 1-5: For a scheduled UL transmission, it is supported that the scheduling DCI indicates whether the UE initiates its own COT or shares gNB’s COT (Alt-a), and the corresponding LBT scheme (Cat-1 or Cat-2 with proper CP extension). If such indication is not included in the scheduling DCI, the UE follows the same rules as a configured UL transmission.

Proposal 2-1: Adopt Option 2-a, i.e., “CG-UCI based procedures” are enabled or disabled for unlicensed using a new RRC parameter, and “CG-DFI based procedures” are enabled or disabled for unlicensed using cg-RetransmissionTimer-r16.

- Note: if cg-RetransmissionTimer-r16 is configured, “CG-UCI based procedures” should also be enabled.

Proposal 2-2: For PUSCH repetition Type B enhancements on unlicensed spectrum, support the flexible start of the transmission and multiple TBs within a period when CG-UCI is enabled.

- An additional parameter is configured for each CG configuration to indicate the total number of consecutive transmission occasions within a period.

Proposal 2-3: For PUSCH repetition Type B enhancements on unlicensed spectrum, orphan symbol(s) are transmitted if they are between two actual repetitions that are transmitted.

Proposal 2-4: Consider enhanced CG-UCI on unlicensed spectrum to allow the UE to autonomously adapt certain transmission parameters such as MCS.

# Reference

- RP-210854, Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR, Nokia, Nokia Shanghai Bell, RAN#91e, March 2021.

- ETSI EN 301 893 V2.1.1, 5 GHz RLAN; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU.

# Appendix A: Previous Agreements

## RAN1#102-e

Agreements:

- For semi-static channel access mode,

- If sensing is needed, it is performed immediately before the configured/scheduled transmission opportunity.

- For operation with semi-static channel access, the Rel-16 random starting offsets for UL configured grants with Full BW allocation when UE initiates a COT, is not supported.

Agreements:

- For semi-static channel access mode,

  - When gNB operates as an initiating device

    - The gNB is not allowed to transmit during the idle period of any FFP associated with the gNB in which the gNB initates a COT

  - When a UE operates as an initiating device

    - The UE is not allowed to transmit during the idle period of any FFP associated with the UE in which the UE initates a COT

  - When a UE shares a COT initiated by the gNB during an FFP associated with the gNB

    - The UE is not allowed to transmit during the idle period of that FFP in which the UE shares the COT initiated by the gNB

  - When the gNB shares a COT initiated by a UE during an FFP associated with the UE

    - The gNB is not allowed to transmit during the idle period of that the FFP in which the gNB shares the COT initiated by the UE

  - FFS whether/how to support additional restrictions to the idle period

Agreements:

- For semi-static channel access mode, support using the transmission of any scheduled/configured UL channel/signal to initiate a COT by a UE in RRC_CONNECTED mode

  - FFS the case when the UE is IDLE/INACTIVE mode

Agreements:

- A UE initiates a COT in an FFP associated with the UE, if the UE transmits a UL transmission burst starting at the beginning of the FFP and ending at any symbol before the FFP’s idle period after a successful CCA of 9us immediately before the UL transmission burst.

Agreements:

- Conditions on the channel access procedures with respect to sensing duration and transmission gap for UE-initiated COT with UE-to-gNB COT sharing is similar as those for gNB initiated COT and gNB-to-UE COT sharing in Rel-16 by exchanging UE and gNB roles.

Agreements:

- UE-to-gNB COT sharing in semi-static channel access mode is supported.

  - The gNB determines a COT in an FFP associated to a UE, that is initiated by the UE, if the gNB detects a UL transmission from the UE starting from the beginning of the FFP and ending before the idle period of the FFP.

    - FFS details

  - When the gNB determines a UE has initiated a COT in an FFP associated to the UE, the gNB can transmit within the FFP and before the idle period corresponding to the FFP.

    - FFS whether/how UE to gNB COT sharing when the gap is >16us

Agreements:

For semi-static channel access mode,

o    Start of FFP for UE-initiated COT can be different from the start of FFP for gNB-initiated COT.

o    FFS: FFP Periodicity for UE-initiated COT can be different from the FFP periodicity for gNB-initiated COT.

Agreements:

- For semi-static channel access mode,

- FFP parameters for UE-initiated COT can be provided to the UE by at least dedicated RRC signaling.

  - FFS on to be provided by SIB-1

- FFS whether the UE FFP periodicity is explicitly configured, or implicitly determined based on other higher layer parameters

Agreements:

- At least for FBE, configuration of (cg-RetransmissionTimer) should not be mandated when configured grant Type 1 or Type 2 are configured on unlicensed spectrum.

Conclusion:

Further study and decide how to harmonize the CG features for Rel-16 URLLC and Rel-16 NR-U. Table 1 in R1-2005376 can be used as a starting point for the corresponding discussion and decision.

## RAN1#103-e

Agreements:

- In semi-static channel access mode, a single FFP (periodicity and offset) is associated to an initiating device (gNB or UE) at a given time which can be used for the purpose of channel occupancy. The FFP configuration that is used for initiating channel occupancy purposes, is such that it shall not be changed for at least 200ms

Conclusion:

- For operation on unlicensed channels and irrespective of the adopted LBT mechanism (LBE or FBE), all transmissions in DL and UL are controlled by gNB similarly to licensed channels, and potential collisions or blocking are controlled/mitigated by gNB.

Agreements:

  - UE-to-gNB COT sharing in semi-static channel access mode with a gap > 16us is supported

Conclusion:

If a device X at a given time is initiating a COT, the applicable FFP for the device X is the FFP associated with X.

If a device X at a given time is sharing a COT initiated by a device Y, the applicable FFP for the device X  is the FFP associated with Y.

Note 1: One of the devices X and Y is a UE and the other is its serving gNB.

Note 2: Whether or not there is additional restriction on idle period is still FFS.

Update on 11/11

Agreements:

- The gNB configures a UE to initiate semi-static CO in an unlicensed channel(s) only if the gNB configures the UE also with the higher layer parameters of the gNB’s initiating semi-static CO in the same channel(s).

  - Note: UE initiated FBE configuration is configured per serving cell

Agreements:

In semi-static channel access mode,  FFP Period for UE-initiated COT is separately provided from FFP period for gNB-initiated COT.

o    Note: Any value for the period, shall be at least 1ms and at most 10ms.

o    Note: Aim for low complexity operation to handle gNB and UE COT interactions

Agreements:

In semi-static channel access mode, a UE should be able to determine whether a scheduled UL transmission should be transmitted according to shared gNB COT or UE-initiated COT.

- UE determines the initiator of a COT based on at least one of the following alternatives:

  - Alt 1: Introduce additional bit field in the scheduling DCI

  - Alt 2: Based on ChannelAccess-CPext field in DCI

  - Alt. 3: Based on a predetermined rule(s)

  - Alt. 4: Based on RRC signalling

  - Alt. 5: Based on MAC CE

  - FFS other alternatives

- FFS on overriding possibility and/or the assumption

- Note: A scheduled UL transmission cannot be transmitted according to both shared gNB COT and UE-initiated COT.

Agreements:

In semi-static channel access mode:

- When a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE, down-select one of the following:

  - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT

  - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT.

  - Alt-c: The UE assumption on whether the configured UL transmission is allowed to correspond to UE-initiated COT is based on gNB configuration.

- When a configured UL transmission starts after a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE:

  - If the UE has already initiated the UE FFP, then UE assumes that the configured UL transmission corresponds to UE-initiated COT

  - Otherwise, If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and if the UE has already determined that gNB has initiated that gNB FFP, then UE assumes that the configured UL transmission corresponds to gNB-initiated COT.

- FFS on other conditions for determining the corresponding UE or gNB initiated COT

- Note: A configured UL transmission cannot be transmitted according to both shared gNB COT and UE-initiated COT.

Agreements:

Down-select one of the following options (target RAN1#104-e):

- Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16.

- Option 2-a: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and cg-RetransmissionTimer-r16, respectively.

- Option 2-b: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and new parameter Y, respectively, where X and Y are different from cg-RetransmissionTimer-r16.

- Option 3: CG-UCI based procedures are supported for unlicensed. CG-DFI based procedures are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16

- Note: Procedures based on CG-UCI rely on UE including CG-UCI in CG PUSCH at least as in Rel-16 where the values of the respective fields of CG-UCI are decided by UE.

- Note: Procedures based on CG-DFI rely on automatic re-transmission on CG configuration and reception of CG downlink feedback information (DFI) in DCI for re-transmissions.

## RAN1#104-e

Agreement:

- PUSCH repetition Type B is supported for unlicensed band operation when using NR IIoT Rel-16 based CG

  - FFS whether/how to enhance

Agreement:

- In semi-static channel access mode, UE FFP periodicity is chosen from the following set of values in ms: {1, 2, 2.5, 4, 5,10}.

  - FFS on other values

Agreement:

- In semi-static channel access mode:

- An FFP period for UE-initiated COT is configured as the same, integer multiple of, or inter-factor of the FFP period configured for gNB-initiated COT

- FFP period for UE-initiated COT can be configured independently from FFP period of gNB-initiated COT, if the UE indicates the corresponding capability

- FFP offset for UE-initiated COT is the starting point of first UE FFP relative to the radio frame X boundary.

  - The offset value range is 0 ≤ offset ＜FFP period of UE-initiated COT

    - FFS on X (e.g. X=0, or X= even index number)

Agreement:

In semi-static channel access mode when a UE can operate as initiating device,

- Select one of the following alternatives to determine whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT:

- Alt-a: Determination based on the content in the scheduling DCI

  - FFS on whether the corresponding field(s) can be absent in DCI

    - If absent, determination based on the rules applied for configured UL transmissions is applied

  - FFS whether/how to handle the case when the gNB schedules an UL transmission in the next gNB’s FFP period

  - Alt-b: Determination based on the rules applied for a configured UL transmission

Agreement:

In semi-static channel access mode when a UE can operate as UE-initiated COT,

- Select one of the following alternatives to determine whether a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, is based on UE-initiated COT or sharing a gNB-initiated COT:

  - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT

  - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT.

Agreement:

- In semi-static channel access mode, sharing a UE initiated COT through the gNB to other intra-cell UEs for UL transmissions, is not supported.