3GPP TSG RAN WG1 #104b-e		R1-2103127

e-Meeting, April 12th – 20th, 2021

Source: 	Apple Inc.

Title:                     On efficient SCell Activation/Deactivation

Agenda item:	8.13.3

Document for:	Discussion and Decision

# 1 Introduction

In RAN1 104-e meeting, the following was agreed for efficient activation/deactivation mechanism for one SCG and SCells [1]:

Working Assumption For efficient SCell activation with assistance of temporary RS, a SSB of the to-be-activated SCell can be indicated as a QCL source for the temporary RS in case of known SCell - FFS: QCL type - FFS: the case of unknown SCell - FFS: other QCL source, e.g., the SSB/P-TRS of another active cell R1-2101933 Summary#2 of efficient SCell activation/de-activation mechanism of NR CA Moderator (Huawei) Agreement For efficient activation of SCells, down select at least one option from below: - Option 1a: MAC CE(s) contained in a single PDSCH to trigger both SCell activation and corresponding temporary RS(s) - Details FFS including timeline design for receiving temporary RS - Option 1b: A single DCI to trigger both SCell activation and corresponding temporary RS(s) - Details FFS including potential impact on SCell activation related procedures and, e.g. timeline design for SCell activation and for receiving temporary RS - FFS: The same DCI for SCell deactivation - Option 2: A Rel-15/16 SCell activation MAC-CE to trigger SCell activation and a Rel-15/16 DCI to trigger corresponding temporary RS(s) with enhancement of timeline - Details FFS including timeline design for receiving a DCI trigger of temporary RS, and for receiving temporary RS - Note: Companies are encouraged to provide complete solutions for fast SCell activation. - Note: the previous agreement on the definitions of Alt 1 and Alt 2 is still effective

In this contribution, we present the analysis on triggering commands candidates i.e. DCI vs. MAC-CE signaling and discuss certain open issues regarding the timeline, QCL information as well as the BWP location.

# 2. Discussion

For both LTE and Rel-15/Rel-16 NR, MAC CE signaling is used to activate/deactivate the SCell(s) to provide the increased capability to handle the instantaneous traffic volume variation. The required delay was defined in [2] as sum of the following three components as depicted in FIG.1 below:

- THARQ: The timing between DL data transmission and acknowledgement as specified in TS 38.213

- TActivation_time: The SCell activation delay in millisecond. Table 1 summarizes the TActivation_time values required for different cases in FR1 depending on the SCell status and measurement cycle configuration.

Table 1: TActivation_time values for FR1

| Case | Description |  | TActivation_time |
| --- | --- | --- | --- |
| 1 | If the SCell is known | The SCell measurement cycle <=160ms | TFirstSSB+5ms |
| 2 |  | The SCell measurement cycle >160ms | TFirstSSB_MAX+Trs+5ms |
| 3 | If the SCell is unknown and Ês/Iot ≥ -2dB |  | TFirstSSB_MAX+TSMTC_MAX+2Trs+5ms |

- TCSI-reporting: The delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331

Figure 1: MAC-CE based SCell Activation Delay in Rel-15/16 NR

The key distinction between FR1 and FR2 system is that the beamforming operation was used in both gNB and UE. Correspondingly, delay caused by necessary beam management operations was added for SCell activation delay in FR2. According to the section 8.3.2 in [2], for both FR1 and FR2, the longest delay occurs for the case of unknown at the UE side, e.g. the target SCell is in a band pair that is operated with independent QCL assumptions or beam management compared to PCell/PSCell or other active SCells on the same FR2 band .

In RAN1 102 e-meeting, TRS was agreed as temporary RS for SCell activation, which at least provides the functionalities of ACG setting and T/F tracking procedure to reduce the latency. It was further agreed in RAN1 103 e-meeting as working assumption that the existing Rel-15/16 TRS structure is reused for temporary RS for the efficient SCell activation purpose. One LS was sent to RAN4 to request the number of TRS burst/symbols to fulfil the targeted functionalities i.e., ACG settling and T/F tracking.

In the RAN1 #103 e-meeting, signalings to trigger A-TRS has been discussed. Two alternatives were agreed for further study.  Alt.1 allows an integrated command for both TRS triggering and SCell activation; While, Alt.2 intends to additionally allow separate triggering for TRS and SCell activation operation in addition to Alt.1. Compared to Alt.1, the advantage of Alt.2 is unclear for us. On the contrary, Alt.2 results in a larger signaling overhead due to separate commands. In addition, it increases the probability of missing one of the TRS commands or activation command, causing either increased activation latency or wastage of TRS transmission. Regarding the flexibility of A-TRS triggering for SCell activation procedure, it should be noted that Alt.1 provides same flexibility as Alt.2 as long as separate IEs are used for A-TRS triggering and SCell Activation in a single integrated command. As one example, one state for A-TRS triggering is reserved to indicate that A-TRS is not triggered and correspondingly UE needs to fallback to use SSB for AGC settling and T/F tracking.

Proposal 1: Introduce a single triggering signaling for the A-TRS triggering and SCell activation that is transmitted on an activated cell.

Commonly for both Alt.1 and Alt.2, one FFS aspect is how to design the triggering signaling. Various design examples were identified in the RAN1 103 e-meeting, including PDSCH with two separate MAC CEs, enhanced DCI, a combination of PDSCH and DCI, etc.

First, the reliability of activation/deactivation is very important as missing activation command would result in data transmission loss at MAC layer and PDU retransmission at RLC layer. Considering this fact, we assume HARQ-ACK is needed for both two approaches in the following analysis.

- One of claimed advantage of DCI-based approach is to reduce latency. However, referring to the break down timeline in FIG.1, the latency reduction of DCI-based approach over MAC-CE based approach is unclear given the fact that HARQ-ACK feedback is commonly required for both. Note that the HARQ-ACK timing difference between PDCCH and PDSCH is marginal especially if PDSCH with a short duration is used for MAC-CE transmission, which is fully under network control. As one example, the N1 value for PDCCH (e.g., SPS PDSCH release) and N1 value for PDSCH is almost identical e.g. N1 = 10 for DCI-based and N1=8 for MAC-CE-based approach assuming u=0.

- In addition, the reduced latency by the new DCI format may not bring meaningful gain considering the conservative CQI setting at the start of newly activate CC and the ‘slow-start’ characteristic of typical TCP-based applications on the mobile devices.

- Furthermore, the activation/deactivation procedure is so far operated based on MAC-CE command and detailed procedure was defined by RAN2 and maintained in TS 38.321. Compared to DCI-based approach, keeping legacy MAC-CE signaling based approach allows for keeping the current functions splitting, minimize the specification impact and standard efforts. Also, support of more than one signaling mechanisms for a single function (i.e., activation/deactivation) unnecessarily complicates gNB schedulers to manage different time gaps for different releases of UEs.

- Moreover, the SCell dormancy operation has been introduced in Rel-16 for activated SCell to handle bursty traffic, balancing between the latency performance and power consumption. Correspondingly, the SCell activation operation is expected to be used only at the starting of bursty traffic session. Hence, the latency reduction by L1 signaling is of less importance.

Having said that, our preference is to introduce a new MAC CE to trigger SCell activation/deactivation together with TRS triggering.

Proposal 2: Prefer to introduce a new MAC CE signal (i.e., Opt.1a) for SCell activation/deactivation and triggering A-TRS transmission as well as A-CSI-RS transmission for known SCells.

On the TCSI-reporting, the value depends on the periodicity configuration of P-CSI-RS or SP-CSI-RS used for CSI reporting. It was widely observed that 20ms/40ms periodicity is typically configured in NR system, which makes TCSI-reporting dominant for SCell activation procedure. As one example, if we count the additional delay from periodic report, there could still be up to 25ms delay even with A-TRS to expedite TO/FO/AGC refinement.

Observation 1: The CSI measurement and reporting are one of dominant factors for the latency of SCell activation procedure.

To solve this problem for any known SCell, the same MAC CE can be used to trigger subsequent A-TRS as well as A-CSI-RS transmission on the SCell so as to minimize the SCell activation latency. The CSI reporting delay can be significantly reduced if UE receives an aperiodic CSI-RS after slot n+k1+3Nslotsubframe with assuming same QCLed as A-TRS. The triggered A-CSI reporting can be transmitted over the PUSCH, which is allocated by the new activation/deactivation MAC CE command.

Proposal 3: The A-TRS is served as the QCL source for the subsequent aperiodic CSI-RS triggered by the same activation/deactivation MAC CE command.

Another FFS aspect is w.r.t the timeline of A-TRS transmission. Assuming HARQ-ACK feedback on PUCCH is supported for triggering command, it is beneficial to define the A-TRS offset relative to the last symbol of PUCCH so as to reduce the RRC signaling overhead. Considering the SCell activation time instances maybe varied for different UEs, it is desirable to allow indicating the A-TRS offset in triggering command such that a single A-TRS transmission can be utilized by multiple UEs even with different SCell activation instances over time, as illustrated in FIG.2.

Figure 2: The timeline of A-TRS transmission

Proposal 4: The timing of A-TRS transmission is defined relative to the PUCCH transmission that carries the HARQ-ACK for triggering command.

Proposal 5: The offset value of TRS transmission is indicated in triggering command.

# 3. Conclusion

In this contribution, we have presented our views with the following proposal and observations:

Observation 1: The CSI measurement and reporting are one of dominant factors for the latency of SCell activation procedure.

Proposal 1: Introduce a single triggering signaling for the A-TRS triggering and SCell activation that is transmitted on an activated cell.

Proposal 2: Prefer to introduce a new MAC CE signal (i.e., Opt.1a) for SCell activation/deactivation and triggering A-TRS transmission as well as A-CSI-RS transmission for known SCells.

Proposal 3: The A-TRS is served as the QCL source for the subsequent aperiodic CSI-RS triggered by the same activation/deactivation MAC CE command.

Proposal 4: The timing of A-TRS transmission is defined relative to the PUCCH transmission that carries the HARQ-ACK for triggering command.

Proposal 5: The offset value of TRS transmission is indicated in triggering command.

# References

- RAN1 103 e-meeting Chairman notes

- TS 38.133 		Requirements for support of radio resource management