3GPP TSG RAN WG1 #104bis-e		     R1-2103138

e-Meeting, Apr. 12th – Apr. 20th, 2021

Agenda item:	5.1

Source: 	Qualcomm Incorporated

Title: 	Discussion on UL switching

Document for:	Discussion/Decision

# Introduction

RAN plenary approved the work item R17 NR FR1 which includes the R17 UL Tx switching. The WID lists the scope as the following [1].

- 2) Specify UE requirements to enable Tx switching between different cases across carriers based on SUL and NR inter-band uplink CA for UE supporting maximum two concurrent transmissions - Specify UE requirements to enable Tx switching between cases - The scenarios include - For Tx switching based on SUL band combination, or uplink CA band combination Number of Tx chains in WID (carrier 1 + carrier 2). Case 2; 0T+2T. Case 3; 2T+0T. - For Tx switching based on uplink CA band combination Number of Tx chains in WID (carrier 1 + carrier 2). Case 1; 1T+1T. Case 2; 0T+2T. Case 3; 2T+0T. - Specify the following RAN4 requirements for above scenarios - Length of switching period - Time mask RF requirements - Uplink interruption and downlink interruption (RRM) requirements, if needed - Minimize the impacts on RAN1 - Update RAN1 uplink switching for carrier aggregation and supplementary uplink - Minimize the impacts on RAN2 - Update the RRC signaling to indicate the switching period location and length - Update the UE capabilities - Specify UE requirements to enable Tx switching between cases, where 1 carrier on band A and 2 contiguous aggregated carriers on band B, and band A is for SUL or non-SUL and band B is a non-SUL band - The scenarios include - For Tx switching based on SUL band combination, or uplink CA band combination Number of Tx chains in WID (band A + band B). Case 1; 1T+1T. Case 2; 0T+2T. And Number of Tx chains in WID (band A + band B). Case 2; 0T+2T. Case 3; 2T+0T. - For Tx switching based on uplink CA band combination Number of Tx chains in WID (band A + band B). Case 1; 1T+1T. Case 2; 0T+2T. Case 3; 2T+0T. - Specify the following RAN4 requirements for above scenarios - Length of switching period - Time mask RF requirements - Uplink interruption and downlink interruption (RRM) requirements, if needed - Minimize the impacts on RAN1 - Update RAN1 uplink switching for carrier aggregation and supplementary uplink - Minimize the impacts on RAN2 - Update the RRC signaling to indicate the switching period location and length - Update the UE capabilities Note 1: Only addressing the case of co-located and synchronized network deployment for the two UL carriers. Note 2: Only addressing the case of single TAG for the two UL carriers for SUL and for UL CA. Note 3: The UE is configured with two different uplink carrier frequencies.

In this paper, we provide views on the issues need to be addressed in R17. The basic principle from our side is trying to leverage as much as possible from R16 normative work and only make necessary changes.

# New state machine and switching options

The first objective of R17 UL Tx switching is to introduce the new state (case 3) on top of 2 cases in R16 UL Tx switching feature. Inheriting from R16 UL Tx switching, two separate tables are listed in the WID for SUL and inter band UL CA option 1, and inter band UL CA option 2, while the difference is simultaneous transmission on two carriers are not allowed for SUL & inter band UL CA option 1.

Table 1 switching cases of SUL and CA option 1

|  | Number of Tx chains in WID (carrier 1 + carrier 2) |
| --- | --- |
| Case 2 | 0T+2T |
| Case 3 | 2T+0T |

Table 2 switching cases of CA option 2

|  | Number of Tx chains in WID (carrier 1 + carrier 2) |
| --- | --- |
| Case 1 | 1T+1T |
| Case 2 | 0T+2T |
| Case 3 | 2T+0T |

In R16, to reduce the switching behaviors RAN1 defined memory-based state machine to map the configured antenna ports states and the cases of Tx chains as in table 3.

Table 3 Rel-16 mapping of configured transmissions to cases for the memory-based option

|  | Number of Tx chains in WID (carrier 1 + carrier 2) | Number of antenna ports for UL transmission (carrier 1 + carrier 2) |
| --- | --- | --- |
| Case 1 | 1T+1T | 1P+0P, 1P+1P, 0P+1P, 0P+0P |
| Case 2 | 0T+2T | 0P+2P, 0P+1P, 0P+0P |

To maintain current spec structure and backward compatibility, it’s a natural extension that R17 UL Tx switching still use this memory-based framework and define the states for case 3. Based on this principle, we list our proposed mapping rule of antenna ports state of case 3 in table 4 and 5 namely.

Table 4 is for SUL and CA option 1, which doesn’t allow simultaneous transmission. The only overlapped configured transmission is 0P+0P which means if no transmission is configured and UE can stay at its current state. Table 5 allows simultaneous transmission on both carriers and more overlapped configured transmission are possible – e.g. both 0P+1P and 1P+0P belong to more than 1 case.

Table 4 Mapping of configured transmissions to cases – SUL and CA option 1

|  | SUL (carrier 1 is SUL), and CA option 1 |
| --- | --- |
| Case 2 | 0P+2P, 0P+1P, 0P+0P |
| Case 3 | 2P+0P, 1P+0P, 0P+0P |

Table 5 Mapping of configured transmissions to cases – CA option 2

|  | CA option 2 |
| --- | --- |
| Case 1 | 1P+1P, 0P+1P, 1P+0P, 0P+0P |
| Case 2 | 0P+2P, 0P+1P, 0P+0P |
| Case 3 | 2P+0P, 1P+0P, 0P+0P |

Proposal 1 Adopt memory-based mapping rule of configured transmissions to cases in table 4 for SUL & CA option 1 and table 5 for CA option 2.

# Intra band contiguous CA

The second objective of R17 UL Tx switching is to cover the case that one of the bands is configured with intra-band CA with 2 carriers.

The WID includes 3 tables for SUL, CA option 1 and option 2 for this sub-feature as below. During the scenario description, the WID assumes 1 carrier on band A and 2 contiguous aggregated carriers on band B, and band A is for SUL or non-SUL (CA) and band B is always a non-SUL band (NUL or CA).

With extensive discussion in R16, we understand there are two interpretation of case 1 “1T+1T” from RAN1 perspective.

  - SUL and CA option 1: no simultaneous UL Tx on both carriers and this is effectively defined as 1T + 0T when switching between case 1 and case 2 as in Table 6a. It could be naturally extended to 0T+1T when switching between case 1 and 3 as in Table 7 if the case is in the scope.

  - CA option 2: simultaneous UL Tx on both carriers and case 1 is 1P + 1P which is aligned with CA without UL Tx switching.

Table 6 scenarios for intra band CA and SUL of R17 UL Tx switching (band A – 1 CC, band B – 2CC).

      - For Tx switching based on SUL band combination, or uplink CA band combination

Table 6a - Case 1 and Case 2

|  | Number of Tx chains in WID (band A + band B) |
| --- | --- |
| Case 1 | 1T+1T |
| Case 2 | 0T+2T |

And

Table 6b – Case 2 and Case 3

|  | Number of Tx chains in WID (band A + band B) |
| --- | --- |
| Case 2 | 0T+2T |
| Case 3 | 2T+0T |

      - For Tx switching based on uplink CA band combination

Table 6c Case 1, Case 2 and Case 3

|  | Number of Tx chains in WID (band A + band B) |
| --- | --- |
| Case 1 | 1T+1T |
| Case 2 | 0T+2T |
| Case 3 | 2T+0T |

Following Rel.16, Case 1 in the Table 6a would be considered as 1T+0T for CA option 1 and SUL, and 1T+1T for CA option 2. The WID doesn’t include the scenario switching between “0T+1T of case 1” and case 3 for CA option 1 and SUL which is listed in Table 7.

Table 7 scenario for intra band CA option 1 and SUL, switching between case 1 and case 3

|  | Number of Tx chains in WID (band A + band B) |
| --- | --- |
| Case 1 | 0T+1T |
| Case 3 | 2T+0T |

Some companies view might be that table 6a already cover this case, but our understanding that table 6a and table 7 are different. The difference is Case 2 (0T+2T) is 0P + yP + zP where y, z = (0, 1, 2) and Case 3 is xP + 0P + 0P where x = (0, 1, 2). Before we get necessary consensus, it will be good to assume these two cases might be with different standard impact.  Therefore, we want to the WG to clarify whether this switching scenario is allowed or not.

For the above question, our understanding is intra band CA of one band is the natural extension of single CC for each band, the scenario in Table 7 should be part of the R17 scope.

Proposal 2: Clarify whether R17 includes the switching scenario for intra band CA option 1 and SUL where one-port is supported in the band with intra-band CA and two-port is supported in the band without intra-band CA.

To extend the configured transmission to different cases of intra band CA, we make the following proposals to cover SUL CA option 1 and SUL (Table 8a and 8b), and CA option 2 (Table 9). In short, we propose UE jointly checks the configuration of CC2 and CC3 and use the maximum ports number among the scheduling for CC2 and CC3 on band B to decide whether to switch or not.

In addition, if companies can reach the consensus on proposal 2 that the scenario in Table 7 should be included in the scope. We also list the proposal in Table 8c for CA option 1 and SUL.

Proposal 3: UE jointly checks the configuration of CC2 and CC3 and uses the maximum ports number among the scheduling for CC2 and CC3 on band B to decide whether to switch or not.

Proposal 4: For intra band CA, adopt memory-based mapping rule of configured transmissions to cases in table 8a and 8b for SUL & CA option 1 and table 9 for CA option 2.

Table 8 Mapping of configured transmissions to cases for intra band CA – SUL and CA option 1

Table 8a switching between Case 1 and Case 2

|  | SUL (carrier 1 is SUL), and CA option 1 SUL or CC1 on band A, CC2 and CC3 on band B |
| --- | --- |
| Case 1 | 1P+0P+0P |
| Case 2 | 0P+yP+zP, where y = (0, 1, 2) and z= (0, 1, 2) |

Table 8b switching between Case 2 and Case 3

|  | SUL (carrier 1 is SUL), and CA option 1 SUL or CC1 on band A, CC2 and CC3 on band B |
| --- | --- |
| Case 2 | 0P+yP+zP, where y = (0, 1, 2) and z= (0, 1, 2) |
| Case 3 | xP+0P+0P, where x = (0, 1, 2) |

Table 8c switching between Case 1 and Case 3

|  | SUL (carrier 1 is SUL), and CA option 1 SUL or CC1 on band A, CC2 and CC3 on band B |
| --- | --- |
| Case 1 | 0P+yP+zP, where y = (0, 1), z = (0, 1) |
| Case 3 | xP+0P+0P, where x = (0, 1, 2) |

Table 9 Mapping of configured transmissions to cases for intra band CA – CA option 2

|  | CA option 2 CC1 on band A, CC2 and CC3 on band B |
| --- | --- |
| Case 1 | xP+yP+zP, where x = (0, 1), y = (0, 1), z = (0, 1) |
| Case 2 | 0P+yP+zP, where y = (0, 1, 2), z = (0, 1, 2) |
| Case 3 | xP+0P+0P, where x = (0, 1, 2) |

# Categorization of 1Tx vs. 2Tx

In R16 UL Tx switching, we had some discussion, but no decision was made at that time due to diverged proposals.

Hereby we would like to update our proposal to accommodate R17 scope.

  - DCI format 0_1 defines PUSCH precoding with TPMI and mapping matrix is defined for different antenna port combination in TS38.211. TPMI = [1 1] is two-port transmission of CC2 (TDD) which should be with no confusion. For TPMI = [1 0] cases, we would propose consider both as one-port, while TPMI = [0 1] should be considered as two-port transmission.

  - PUCCH and PRACH are one port transmission from RAN1 perspective even though two Tx chain could be used transparently to increase the UL reliability. However, we need to decide which case it belongs to. For simplicity, we propose to put them into Case 1.

  - For a Configured Grant (CG) PUSCH, UE is with flexibility to initial Tx without DCI. To avoid ambiguity between network and UE, we propose that a UE uses 2-port for CG PUSCH in Case 2 and 1-port for CG PUSCH in Case 1.

Based on the above discussion, we have the following proposals to categorize 1 Tx or 2 Tx.

Proposal 5: Use the following rule to decide the Tx number(s) on a certain carrier,

- 2 Tx is used for these UL transmissions:  PUSCH with TPMI=[1201 , 1211], 2-port SRS, 2-port configured grant PUSCH, SRS carrier switching on the paired carrier

- 1 Tx is used for these UL transmissions:  PUCCH, SR, PRACH, PUSCH scheduled by DCI 0_0, single port configured grant PUSCH, PUSCH with TPMI=1210

# Others

In R16, we discussed and defined 3 other important parts: validity period, observation period and PUSCH preparation time. The agreements are listed below. Our view is to leverage R16 agreements unless issues are foreseen with R17 scenarios.

Proposal 6: In R17, on validity period, observation period and PUSCH preparation time, leverage R16 agreement unless issues are foreseen with R17 scenarios.

Table 10 consideration on validity period, observation period and PUSCH preparation time

|  | Agreement of R16 | R17 considerations |
| --- | --- | --- |
| Validity period | Per slot | Leverage R16 agreement unless issues were foreseen with R17 scenarios. |
| Observation period | No switching if the time between scheduling and expected switching is less than UE processing procedure time |  |
| PUSCH preparation time N2 | Original value + Tswitch of the lower SCS CC |  |

# Conclusions

We shared the views on new switching options, intra band contiguous CA, Categorization of 1Tx and 2Tx, and other issue for inter-band CA, and SUL. Based on the discussion, we have the following proposals:

Proposal 1 Adopt memory-based mapping rule of configured transmissions to cases in table 4 for SUL & CA option 1 and table 5 for CA option 2.

Proposal 2: Clarify whether R17 includes the switching scenario for intra band CA option 1 and SUL where one port is supported in the band with intra-band CA and two port is supported in the band without intra-band CA.

Proposal 3: UE jointly checks the configuration of CC2 and CC3 and uses the maximum ports number among the scheduling for CC2 and CC3 on band B to decide whether to switch or not.

Proposal 4: For intra band CA, adopt memory-based mapping rule of configured transmissions to cases in table 8a and 8b for SUL & CA option 1 and table 9 for CA option 2.

Proposal 5: Use the following rule to decide the Tx number(s) on a certain carrier,

- 2 Tx is used for these UL transmissions:  PUSCH with TPMI=[1201 , 1211], 2-port SRS, 2-port configured grant PUSCH, SRS carrier switching on the paired carrier

- 1 Tx is used for these UL transmissions:  PUCCH, SR, PRACH, PUSCH scheduled by DCI 0_0, single port configured grant PUSCH, PUSCH with TPMI=1210

Proposal 6: In R17, on validity period, observation period and PUSCH preparation time, leverage R16 agreement unless issues are foreseen with R17 scenarios.

# Reference

[1] RP-210899, revised WID: RF requirements enhancement for NR frequency range 1 (FR1), Mar. 2021