3GPP TSG-RAN WG1 #104bis-e	R1-2103162

e-Meeting, April 12th – April 20th, 2021

Agenda item:    8.2.6

Source:              Qualcomm Incorporated

Title:                  Channel Access Mechanisms for NR in 52.6 to 71GHz band

Document for:  Discussion and Decision

# Introduction:

In this paper we discuss a collection of potential design enhancements based on the agreements and convergences reached by RAN1-104e. The relevant agreements and guidelines for different considerations are captured in the section that discusses them.

# LBT/Sensing Bandwidth considerations

By meeting RAN1#104e, RAN1 reached the following agreements and listed for-further-study considerations related to LBT Bandwidth.

Agreement:

The baseline ED threshold can be computed as

EDT=-80 dBm+10*log10PmaxPout+10*log10(Operating Channel BW in MHz)

Where Pout is RF output power (EIRP) and Pmax is the RF output power limit, Pout≤Pmax.

- FFS: Further adjustment on ED threshold based on the sensing beam and the transmission beam (further adjustment should not violate EDT requirements as per regulations)

- FFS: If Pout is max output EIRP of the device or instantaneous output EIRP

- FFS definition of Operating Channel BW

- FFS: Whether ED threshold for NR-U and NR-U coexistence scenarios (eg, at regulation level) can be appropriately relaxed compared with the threshold of coexistence between NR-U and Wi-Fi.

- FFS: EDT when the COT has time varying transmission beams and varying EIRP

Agreement:

For LBT for single carrier transmission, consider the following alternatives

- Alt SC.1. gNB/UE performs LBT over the channel bandwidth (or BWP bandwidth)

- Alt SC.2. gNB/UE performs LBT over the transmission bandwidth (from the lowest RB to the highest RB used for the transmission)

- Alt SC.3. Define a unit of LBT bandwidth and gNB/UE performs LBT in all the LBT units (to be transmitted in) in the channel bandwidth

For LBT for multi-carrier transmission in intra-band CA, consider the following alternatives

- Alt CA.1. gNB/UE performs multiple LBT, one for each channel bandwidth separately

- Alt CA.2. gNB/UE performs single LBT over all CCs

- Alt CA.3. gNB/UE performs multiple LBT, one for each CC over the transmission bandwidth (from the lowest RB in to the highest RB used for the transmission in the CC)

- Alt CA.4. gNB/UE performs LBT over the transmission bandwidth over all CCs (from the lowest RB in the lowest CC to the highest RB in the highest CC used for the transmission)

- Alt CA.5. Define a unit of LBT bandwidth and gNB/UE performs LBT in all the LBT units (to be transmitted in) in the channel bandwidth in each CC

Note: supporting more than one alternative for at least multi-carrier transmission in intra-band CA is not precluded.

All 3 options converged by RAN1 have their merits and demerits. The key considerations for determining of the bandwidth used for sensing are flexibility of operation and simplicity of specification. Given the wide and heterogenous bandwidth and open channelization available to be used in the 60 GHz band it is expected that the channel access procedure should allow opportunistic transmissions in unoccupied sections of the deployed bandwidth. That indicates a favorable case for performing sensing on multiple units of LBT bandwidth. On the other hand given the wide bandwidth, determining a single number for LBT bandwidth is harder to justify – larger values losing opportunistic spectrum usage, while smaller values would imply enforcing large number of separate sensing  operations in a wide channel deployment.

To continue to support the flexibility and wide range of deployment bandwidths offered by NR, and yet also allow opportunistic transmissions in unoccupied sections of the deployed bandwidth, we propose that the LBT Bandwidth unit be left to implementation with small requirements imposed. The main requirement, as commonly understood, is that the LBT bandwidth should cover the transmission bandwidth.

Leaving LBT bandwidth to implementation needs to address how COT sharing will be performed, especially if the node sharing the COT needs to know the bandwidth used to perform LBT by the initiating node. The transmission bandwidth inclusion requirement dictates the following proposal.

Proposal 1: For single carrier LBT, support both Alt SC.1 and Alt SC.3 as implementation choices, as long as the aggregated LBT bandwidth covers the transmission bandwidth. FFS how to indicate the aggregated LBT bandwidth from the COT initiating node to the COT sharing node.

The multi-carrier transmissions should be compatible with the design choice made for single carrier operation.

Proposal 2:  For multi-carrier transmission in intra-band CA, support Alt-CA.1,  Alt-CA-2, and Alt CA.5 as implementation choices, as long as the aggregated LBT bandwidth covers the transmission bandwidth.

Having left to implementation, the number of LBT bandwidth units a UE can separately sense, should be a capability.

Proposal 3: Consider specifying the maximum number of LBT-Bandwidth units a UE can sense as a UE capability.

# Mismatched and Directional Sensing for Channel Access

By meeting RAN1#104e, RAN1 did not reach a consensus but listed for-further-study the multiple considerations related to mismatched and directional sensing. The general majority view is that the 3GPP specification defines the relative relationship between all applicable sensing beams and the transmission beam, at least sensing beam “covers” the transmission beam. But it was left for further study on how to define the relationship, the meaning of ‘covering’ transmission beam and whether there is a RAN1 specification impact, and in absence of such an impact whether it can be left to RAN4 to introduce a testing requirement.

In this section, we use a simulation-based approach to study the impact of sensing beam choices (omni vs directional) and ED threshold used on channel access.

The main goal of the study is to establish that the directionality and corresponding associated beamforming gain of the sensing beam used has a significant impact on (1) the sensing footprint of a contending node (the set of interferers/transmitters sensed) (2) the energy levels computed and (3) the relationship with the interference footprint (the set of nodes affected by the contending transmitter’s transmission). Based on (1), (2) and (3) and the need for one design with one sensing beam to sense for another transmission beam, we argue that it is useful for the network to explicitly allow different Energy Detection thresholds than as captured by regulation text.

First, to better appreciate the need for specifying a good notion of directional sensing, it is useful to understand the behavior of directional sensing. We carried out simulation studies working with two extremes of directional sensing namely, one with omni-directional sensing at the LBT sensing device and other is fully directional sensing where the transmission beam is used for data transmission as well as for sensing with full beamforming.  The assumptions for the discussion are captured below. The key ideas are best seen in scenarios with higher directivity.

Table 1. Simulation assumptions for Directional vs Omni sensing analysis

| Layout | InH Open office model – with 2 operators, 12gNB/operator in 120m x 50m |
| --- | --- |
| {BS, UE} Antenna Configuration | Higher Directivity at UE: (Mg,Ng,M,N,P) = (1,1,4,8,2) with (0.5 dv, 0.5 dH), (Mg,Ng,M,N,P) = (1,1,4,4,2) with (0.5 dv, 0.5 dH) with 2 such panels at the UE. |
| Energy Sensing Beams | - Omni directional (b) Directional matched to transmission beam |
| EIRP | gNB – 40 dBm, UE 25 dBm |
| BW/SCS | 200MHz, 960KHz |

Figure 1. Nodes (N1,U1) and (N2,U2) belong to different cells. Figure on the left depicts directional transmission and sensing beams at nodes N1, U2, different than transmission beams (omni sensing, S1,S2 ≠ B1,B2). Figure on the right depicts directional transmission and directional sensing at N1,U2 using matched/corresponding beams (Sd1,Sd2 =B1,B2). Beams used at N2 and U1 are not depicted.

The simulation does not model traffic but consists of recording of received powers at various nodes when data transmissions at various other nodes along their links are occurring. Consider the case depicted in Figure 1 where two links N1-U1 and N2-U2 are from different cells in the network. When UE U2 is performing a uplink transmission to its serving cell N2, it creates an interference at N1 if there is an uplink transmission on U1-N1 link. It also contributes to energy sensed at node N1 along its sensing beam S1 if N1 is performing energy sensing. Similarly, When Node N1 is transmitting to its UE U1 on the downlink, it creates an interference to DL reception at U2 if one is ongoing. It also contributes to energy sensed at U2 along its sensing beam S2 it U2 is performing energy sensing. The energy level sensed and the interference level observed are identical if the sensed beam is matched to (same as) transmission beam (or corresponding reception beam). By recording the received powers at nodes using their sensing beams, one can obtain an understanding their sensing footprint. When the sensing beams are matched/corresponding to their transmission beams, the same set of measurements can depict true interference footprint of their interferers. A collective look at the two sets of data can provide understanding of the relationship between the two.

Figures 2 and 3 depict scatter-plots of received powers by sensing beams at various gNB-UE pairs of the kind N1,U2, i.e. those belonging to different cells and transmitting to the serving links in their own cells.  When UE transmits UL data, the gNB records the received power (X coordinate) and when gNB transmits DL data, the UE records the received power (Y coordinate).   In both figures all transmissions from gNB and UE are directional, i.e. Beam B1 is used for DL transmissions and B2 is used for UL transmissions.

In Figure 2 the two charts depicts Directional sensing (left, X axis) vs Omni Sensing (right, X axis) at the set of  gNBs.  In Figure 2 the UE sensing beam is directional (i.e. Y axis depicts the gNB Interference footprint – i.e. true interference created by gNB).   The shift of points to the left under omni sensing shows reduced beamforming gain of the sensing beam. But the presence of points below the magenta  line depicts Omni sensing magnifies UEs that are not in the interference footprint . These points under Omni sensing at gNB shows considerably higher ‘unnecessary’ contributor UEs to energy at the gNB.

In Figure 3, the two charts depicts Directional sensing (left, Y axis) vs Omni Sensing(right, Y axis) at the set of UEs. All gNB and UE Transmissions are directional.  The gNB sensing beam is directional (i.e. the X axis depicts true interfernce caused by UE at the gNB). The downward pull of  on the points under Omni sensing at UE shows  reduced beamforming gain,  but the presence of point above the green line indicates gNBs that get amplified under omni sensing while not falling in the UE’s interference footprint (low x coordinates).  There are considerably higher ‘unnecessary’ contributor gNB transmissions to energy at the UE under omni sensing.

Similar phenomenon is depicted in Figure 4 for UE-UE sensing for UEs belonging to different cells. The chart on the left is a scatter plot of directionally sensed energy and energy sensed using an ideal istropic omni antenna.  The CDF of the difference of the two energies. Given that the UEs are typically pointing to different directions, although the peak difference approaches the beamforming gain (12 dB) the median remains closer to zero. Further, it is seen that omni sensing at UE may amplify the sensed energy from other UEs considerably.

Figure 2. Directional vs Omni Sensing at gNB. Left: Directional Sensing at gNB (X axis),  Right: Omni Sensing at gNB (X axis). All gNB and UE transmissions are directional. The UE sensing beam is directional (i.e. Y axis depicts gNB Interference footprint – i.e. true interference created by gNB).

Figure 3. Directional vs Omni Sensing at UE: Left:  Directional Sensing at UE (Y axis), Right: Omni Sensing at UE (Y axis).  All gNB and UE Transmissions are directional.  The gNB sensing beam is directional (i.e. the X axis depicts true interference caused by UE at the gNB).

Figure 4.  The figure captures energy sensed over symmetric links betweenUE-UE belonging to different cells in an Indoor Hotspot deployment with antenna configuration (Mg,Ng,M,N,P) = (1,1,4,8,2) with (0.5 dv, 0.5 dH), at gNB and  (Mg,Ng,M,N,P) = (1,1,4,4,2) with (0.5 dv, 0.5 dH) at the UE. There are two such panels at the UE.

The key observations from the simulations are as follows.

Observation 1:   In general, the baseline energy detection thresholds are generous for any significant LBT based backoff at this layout.

This may be the case for the layout under consideration but the behavior of directional vs omnidirectional sensing points to possible design directions for denser layouts and stuck situations.

Observation 2: For the same interference caused, the measured values under directional and omni-directional sensing are starkly different due to increased beamforming gain under directional sensing

Observation 3: Directional sensing matching transmission beam provides a tighter match between sensing and interference footprint.

Observation 4: Especially for UEs, omni-directional sensing may pick up ‘spurious’ energy from transmissions that do not fall in the interference footprint.  Directional sensing naturally avoids those transmissions.

Therefore, the energy measurements performed with sensing beam have to be recalibrated to continue to be valid for EDT comparison for a different transmission beam.

To make opportunistic use of available spectrum and spatial/directional separation of links, a provision for directional sensing beam is necessary. As a consequence, RAN1 should work towards identifying relationship between directional sensing beams and transmission beams when they are different. At a minimum, ED adjustment before comparison with ED threshold based on the sensing and transmission beams can be identified in RAN1.

Proposal 4: Consider use of ED adjustment when sensing and transmission beams are different before comparison with ED Threshold.

# Multi-Beam LBT Considerations

By meeting RAN1#104e, RAN1 reached the following agreements and listed for-further-study the following considerations related to Muti-Beam COT and LBT.

Agreement:

For a COT with MU-MIMO (SDM) transmission, further consider the follow alternatives (down-select or support both)

- Alt 1: Single LBT sensing at the start of the COT with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold

- Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT

Agreement:

Within a COT with TDM of beams with beam switching, down-select one or more of the following LBT operations

- Alt 1: Single LBT sensing with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold

- FFS: Details on the definition of "cover"

- Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT

- Alt 3: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT with additional requirement on Cat 2 LBT before beam switch

For SDM transmissions, independent per-beam sensing and channel access decision provides the maximum flexibility to nodes contending for the medium.  On the other hand, a single sensing beam valid to be used with multiple beams with potentially different beam gains is attractive for simplicity and beam selection flexibility so long as the chosen beams can be used with the sensing beam. For SDM transmission, we propose supporting both Alt 1 and Alt 2 in the specification.

Proposal 5:  For SDM transmission, support both single LBT sensing with wide beam covers all beams used in the COT and independent per beam sensing.

For TDM transmissions, both Alt 1 and Alt 2 have merit so long as ED Threshold used for single beam sensing and for independent per beam sensing are compatible.  Alt 2 is a generalization of Alt 1, so long as the sensing beams are compatible with the transmission beams in the sense of section 3.

Proposal 6: For a COT with TDM of beams, support both Alt 1 (single LBT sensing with wide beam covers all beams) and ALT 2 (independent LBT sensing to be performed at the start of the COT).

Cat 2 LBT on the transmitter side for TDM COT has benefits when the duration between beam switches is larger.

Further, a Cat 2 LBT sensing before beams which may be combined with relaxing of the ED Threshold and sensing duration used to win the COT at the start.  The key consideration is that the Cat 2 LBT procedure, if defined be optional.

In particular, the decision related to Mult-Beam TDM COT have to be consistent with what is being considered for SSB transmission.

Proposal 7: If any Cat 2 LBT procedure before beam switch is considered, let it be optional/configured and not mandatory.

# Cat2 LBT Considerations

By meeting RAN1#104e, RAN1 reached the following agreements and listed for-further-study the following considerations related to CAT 2 LBT.

Agreement:

For Cat 2 LBT, down-select from the following alternatives

- Alt 1: Do not introduce Cat 2 LBT for 60GHz unlicensed band operation

- Alt 2: Introduce Cat 2 LBT for 60GHz unlicensed band operation

Agreement:

If Cat 2 LBT is introduced, the following use cases can be further studied:

- Resume transmission after a gap Y:  Cat 2 LBT may be used to resume transmission by the initiating device within the COT after a gap Y (FFS the value of Y)

- COT sharing: Cat 2 LBT may be used before transmission by a responding node sharing a COT

- Multi-Beam LBT:  Cat 2 LBT may be used before switching to a new transmission beam (not used in earlier part of the COT) in a COT with TDM beams, or resume a previously used transmission beam after a gap Z (FFS the value of Z)

- Rx-Assistance:  Cat 2 LBT may be used for sensing at the receiver as a responding device for Rx-Assistance measurements and associated signaling

Other use cases not precluded.

FFS if Cat 2 LBT is mandated for each use case or not.

Cat2 LBT, also called one-shot LBT (Type 2A, 2B for Rel 16) is not required by regulation and hence the principal question is whether defining such a measurement/sensing and tying it to medium access procedures brings benefits.

The agreement lists 4 use-cases where a Cat2 LBT type sensing can be used as part of the medium access procedures. There is precedence for the procedure in NRU for large gaps within a COT and COT sharing. Rx-Assistance is an important use-case where such a sensing can provide faster feedback than L1-RSRP/L1-SINR reporting/decoding, including right at the start of the COT.  Another important use case not discussed in the list is Multi-Channel medium access with LBT success tied to primary channel. The secondary channel may perform in that case a Cat 2 LBT instead of a full independent eCCA with associated variability of the medium access time. We consider (1) defining a Cat 2 LBT procedure is useful (2) it need not be a mandatory part of any medium access procedure and can be an optional/configurable feature, potentially supported via a UE Capability.

Note that as part of the eCCA procedure, sensing requirements for a 5us slot and a 8 us deferral period are already needed to be identified. It is relatively straightforward to adopt them to define a Cat 2 LBT procedure.

Proposal 8:  Consider defining Cat 2 LBT as a sensing/measurement. Consider the use of such Cat 2 LBT sensing as an optional/configured and triggered component of LBT procedures in all the 4 use-cases above and for Multi-channel medium access.

# COT Sharing Considerations

By meeting RAN1#104e, RAN1 reached the following agreements and listed for-further-study the following considerations related to COT Sharing between initiating and responding device.

Agreement:

On maximum gap within a COT to allow COT sharing without LBT, down-select from

- Alt 1. No maximum gap defined. A later transmission can share the COT without LBT with any gap within the maximum COT duration

- Alt 2. Define a maximum gap X, such that a later transmission can share the COT without LBT only if the later transmission starts within X from the end of the earlier transmission

- FFS: Value for X

- Alt 3. Define a maximum gap Y, such that a later transmission can share the COT without LBT only if the later transmission starts within Y from the end of the earlier transmission. If the later transmission starts after Y from the end of the earlier transmission, an one-shot LBT is needed to share the COT

- FFS: Value for Y

- FFS:  How to define the one-shot LBT

The current regulation does not require the responding device to perform CCA before sharing the COT. Since per our previous agreement, the gap between the initiating device transmission and responding device transmission is counted towards the MCOT as well, we do not need to limit the duration of the gap between the transmissions when sharing a COT. The implementation should try to minimize the gap to make more efficient use of the resources. But if due to implementation limitations, a gap is necessary, it should be still allowed. Therefore, we believe Alt 1 (no maximum gap defined) should be the baseline.

On the other hand, due to beamforming nature of the transmissions, the interference level seen at the initiating device and responding device can be very different. It is helpful to support a mechanism to let the responding device to check the channel before sharing the COT, as a optional feature. To support this functionality, it is useful to introduce a Cat 2 LBT like channel sensing with fixed channel sensing duration and low channel sensing overhead to check the availability of the channel.

Proposal 9:  Support Alt 1 (no maximum gap defined), but if Cat 2 LBT define, optionally allow the initiating device to trigger the responding device to use Cat 2 LBT to sense the channel before starting COT sharing transmissions.

# Class A Rx-Assistance Considerations

By meeting RAN1#104e, RAN1 reached the following agreements and listed for-further-study the following considerations related to Rx Assistance for Channel Access.

Agreement:

For receiver to provide assistance, channel sensing and reporting need to be performed. The following set of tools can be considered for further discussion

- Alt 1. Legacy RSSI measurement and reporting with possible enhancements

- Alt 2. AP-CSI report with possible enhancements

- Alt 3. LBT at receiver

- Alt 3.1 eCCA

- Alt 3.2 Cat2 LBT

There are multiple considerations for introducing LBT for Rx-Assistance. It is well understood that for this 60gHz band the mismatch between energy sensed at the transmission side LBT and actual interference conditions seen at the receiver is large. The principal question is whether there are benefits of a new procedure involving LBT sensing at the receiver over already supported measurement reporting in NR, namely CSI and L1-SINR.

Proposal 10: Any LBT based Rx-Assistance procedure should be made optional/configurable on a per UE link basis.

There are three main components to an LBT based Rx-Assistance procedure.

- Rx-Assistance measurement or sensing at the receiver: The UE in this case may perform energy sensing or specialized form of sensing such as energy sensing using specified/pre-agreed beam and parameters.

- Conditional Rx-Assistance transmission at the receiver: This is the most important new component. In the case of UE as a receiver, an LBT based procedure would require a conditional transmission (Rx-Assistance transmission) from the UE to indicate the outcome of the Rx-Assistance sensing. The initiator/transmitter in this case will first detect the presence or absence of the Rx-Assistance transmission.

- Subsequent action under absence of conditional Rx-Assistance transmission:  Under failed detection of Rx-Assistance transmission from the receiver, the transmitter can undertake an action making use of the new input.  This need not correspond to classical LBT failure action, namely abandonment of COT.

Figure 5 indicates possible procedures for receiver-assistance based on sensing where the transmission from the UE is conditioned on the success or failure of the adjoining LBT sensing procedure. By introducing such mechanism, the channel access can adapt to jammer activity and especially useful when there is persistent interferer.

Figure 5. Procedures with Rx-Assistance for DL transmissions based on LBT sensing at the UE.  The transmission from the UE is conditional on the success /failure of LBT procedure preceding its opportunity. The figure on the left represents eCCA performed at the UE. The figure on the right has a Type 2 /Cat 2 LBT performed at the UE CCA.

We see the following ways introducing an optional/configurable LBT on the receiver side can benefit.

- A faster feedback via conditional transmission and presence detection: We consider that sensing at the receiver followed by presence or absence of transmission at the receiver can be a faster alternative to full CSI or L1-SINR reporting. Consider receiver assistance for DL transmissions. CSI including L1-SINR reporting today involve decoding of the report and consequent delay. A conditional transmission from the UE based on CCA can provide a coarse but much quicker feedback than any long-term measurement of interference. L1-SINR can have multi-slot delay. Minimum requirement currently corresponds to 1 slot of 120KHz SCS, or roughly 125 us.  Aperiodic CSI report has larger delay requirements, and periodic CSI cannot be tied to success of LBT procedure on gNB side. The Rx-Assistance feedback may involve just DTX detection – providing faster input.

- Need not be tied to silencing on LBT Failure: The subsequent action under detection of absence of Rx-Assistance signal need not be mandated.  Multiple useful actions exist that can benefit the deployment considerably.

  - Example: On Rx-Assistance DTX, the gNB/network chooses to switch the beam used to serve the UE.

  - Example: On Rx-Assistance DTX, the gNB/network chooses to schedule another UE

The sensing/measurement for Rx-Assistance can take the form of a pre-determined/known duration of sensing – which forms another use-case for defining an optional Cat 2 LBT at the UE.  The Rx-Assistance sensing may happen at the beginning or in the middle of the COT.

Over-all, it can be argued that Rx-assistance is beneficial given the mismatch between sensing at Tx and true interference conditions at Rx. The three techniques in the earlier agreement, namely enhancement to CSI reporting, enhancement to RSSI reporting, and LBT and conditional transmission at the UE, can help. But there is a difference in latency of the information. RSSI is a good compromise between complexity and latency and the magnitude of changes need for support in the specifications. LBT based conditional transmission is even faster but may require larger effort in specifications.

In consideration of the above, we have the following proposals.

Proposal 11: Support enhanced RSSI reporting for Rx-Assistance, enhancements include at least L1-RSSI reporting.

Proposal 12:  Study further LBT sensing at the receiver with a conditional response from the receiver for Rx-Assistance.

- Consider the use of CAT 2 LBT for LBT-sensing for Rx-Assistance.

# Other Considerations

## Short Control Signaling Provisions

We support contention exempt transmissions for all control signalling under the duty cycle restrictions imposed by regulations, including those over PUSCH and PDSCH transmission not carrying user-plane data.

Proposal 13:  Under the restrictions of duty cycle for short control signaling, allow SS/PBCH, PDCCH, CSI-RS and PRS for contention exempt transmission

Proposal 14:  Under the restrictions of duty cycle for short control signaling, allow PRACH, msg1, msg3, msgA, SRS, PUCCH and PUSCH without user plane data for contention exempt transmission

## Energy sensing considerations

By meeting RAN1#104e, RAN1 reached the following agreements and listed for-further-study the following considerations related to energy sensing for LBT.

Agreement:

For energy measurement in 8us deferral period, down-select from the following:

- Alt 1. Two energy measurements are required

- Alt 2. One measurement is required

- Alt 3. Extend the 8us to 10us and perform two measurements, one in each 5us segment

For energy measurement in 5us observation slot, perform single measurement

- FFS minimum duration of the measurement

- FFS location of the measurement

The key considerations for energy sensing are turn-around time, missing detection of a strong jammer due to asynchronous observation slots, and computational simplicity. With the regulation permitting large gaps in channel occupancy within a COT and for COT sharing the strong jammer consideration is weaker. We prefer Alt 1 or Alt 3.  Further, given a wide variation in the deployment bandwidths, there is a trade-off in minimum duration of sensing and reliability, which is a function of bandwidth.  A reliable sensing decision can be obtained in a shorter duration in wider bandwidths. Therefore, the minimum sensing duration requirement can be relaxed for wider bandwidth sensing.

Proposal 15: Consider the use of two energy measurements for deferral period and for contention slot.

Proposal 16: Consider deployment bandwidth dependence on the minimum sensing duration requirement.

## Multi-Channel LBT considerations

By meeting RAN1#104e, RAN1 reached the following agreements and listed for-further-study the following considerations related to Multi-Channel LBT.

Agreement:

Define Type A and Type B multi-channel channel access as:

- Type A: Perform independent eCCA for each channel

- Type B: Identify a primary channel and perform eCCA on the primary channel, while perform Cat 2 LBT for other channels in the last observation slot

Down-selection between

- Alt1: Support Type A multi-channel channel access only

- Alt2: Support both Type A and Type B multi-channel channel access.

Note: How eCCA is performed on each channel, and the BW of the channels over which eCCAs are performed are separately discussed

The mode of operation under Alt 2, namely, additional support of eCCA on primary channel and a Cat 2 LBT for other channels in a last observation slot permits tying the medium access on multiple channels together, allowing increased predictability and helps in scheduling, cross-channel use of control signaling and data transmissions tied to the COT.

Proposal 17:  Support Alt 2 for Multi-Channel LBT. For Type B multi-channel access, introduce Cat 2 LBT for non-primary channels.

## LBT and SSB Transmission considerations

By meeting RAN1#104e, RAN1 reached the following agreements and listed for-further-study the following considerations related to channel access for SSB transmission.

Agreement:

- SSB transmission with LBT is supported, at least when the conditions for contention exempt short control signalling based SSB transmission is not met

- Note the channel access for SSB with LBT may not be different from a normal COT with multiple beams

- FFS: If any difference from a multi-beam COT LBT needs to be introduced

Channel access for SSB, as discussed may not be different from a normal COT with multiple beams.  The key consideration is to facilitate medium access for SSB transmissions when SSB transmissions do not fall in the short control signaling budget for the gNB.

Proposal 18: SSB burst transmission could be regarded as a Multi-Beam TDM COT, with support for both pre-burst single LBT with wide sensing and per beam independent LBT performed at the start of the COT.

# No-LBT Operation Considerations

As discussed during the study item phase, uncoordinated deployments may have beam collision situations where a node may find itself in a stuck situation for an extended period. When no-LBT mode is used, where there are no sensing requirements, it is difficult to (1) identify a stuck situation (2) react to it to assist individual nodes.

The identification of stuck situations may be done via measurements done on a slower time scale, also dubbed Long Term Sensing.  Long Term Sensing as a way of detecting damaging beam collisions needs to be facilitated in No-LBT deployments.

Proposal 19:  Support provision for sensing and measurement gaps for discovery of aggressors and victims in a No-LBT deployment

Long Term Sensing can also provide an input to a fall-back mechanism that switches from No-LBT mode to LBT mode.  But if a network working in No-LBT mode happens to be an aggressor to another network working in an LBT mode, simple Long Terming Sensing is not sufficient if the LBT mode system is completely blocked from transmission due to persistent LBT failure. Some candidate techniques to support better coexistence considered earlier includes Dynamic Frequency Selection (DFS), Automatic Transmit Power Control (ATPC) and placing duty cycle limits. A key drawback of DFS type techniques is that a whole cell has to move to a new frequency in order to assist a single node in stuck situation. Duty cycle limitation is useful but has a negative impact to the peak user throughput.

A possible candidate for consideration is an ‘away-time’ operation where a node is restricted not to occupy the channel for a specified duration of time, after occupying the channel. The away-time after the channel occupancy increases as a function of the channel occupancy. The advantage of such techniques is that they can be designed so that the peak rate is not affected for bursts of reasonably long sessions.

The away-time can provide chance for LBT based system to access the channel, so it can be detected by the Long-Term Sensing mechanism used by the no-LBT system. The away-time needs to be long enough for the LBT system to acquire the channel. The away-time may not mean complete silencing. If the notion is used for sectorized/beam-based channel access, merely switching away to another beam can serve as away-time for the original beam.

Figure 6. An optional silencing/away time *after* COT allows good neighbor coexistence without peak rate penalty

Proposal 20:  For No-LBT deployments, consider specification of optional good neighbor procedures, such as away time, to break persistent beam collisions for better coexistence.

# Conclusions

We have the following observations and proposals:

Proposal 1: For single carrier LBT, support both Alt SC.1 and Alt SC.3 as implementation choices, as long as the aggregated LBT bandwidth covers the transmission bandwidth. FFS how to indicate the aggregated LBT bandwidth from the COT initiating node to the COT sharing node.

Proposal 2:  For multi-carrier transmission in intra-band CA, support Alt-CA.1,  Alt-CA-2, and Alt CA.5 as implementation choices, as long as the aggregated LBT bandwidth covers the transmission bandwidth.

Proposal 3: Consider specifying the maximum number of LBT-Bandwidth units a UE can sense as a UE capability.

Observation 1:   In general, the baseline energy detection thresholds are generous for any significant LBT based backoff at this layout.

Observation 2: For the same interference caused, the measured values under directional and omni-directional sensing are starkly different due to increased beamforming gain under directional sensing

Observation 3: Directional sensing matching transmission beam provides a tighter match between sensing and interference footprint.

Observation 4: Especially for UEs, omni-directional sensing may pick up ‘spurious’ energy from transmissions that do not fall in the interference footprint.  Directional sensing naturally avoids those transmissions.

Proposal 4: Consider use of ED adjustment when sensing and transmission beams are different before comparison with ED Threshold.

Proposal 5:  For SDM transmission, support both single LBT sensing with wide beam covers all beams used in the COT and independent per beam sensing.

Proposal 6: For a COT with TDM of beams, support both Alt 1 (single LBT sensing with wide beam covers all beams) and ALT 2 (independent LBT sensing to be performed at the start of the COT).

Proposal 7: If any Cat 2 LBT procedure before beam switch is considered, let it be optional/configured and not mandatory.

Proposal 8:  Consider defining Cat 2 LBT as a sensing/measurement. Consider the use of such Cat 2 LBT sensing as an optional/configured and triggered component of LBT procedures in all the 4 use-cases above and for Multi-channel medium access.

Proposal 9:  Support Alt 1 (no maximum gap defined), but if Cat 2 LBT define, optionally allow the initiating device to trigger the responding device to use Cat 2 LBT to sense the channel before starting COT sharing transmissions.

Proposal 10: Any LBT based Rx-Assistance procedure should be made optional/configurable on a per UE link basis.

Proposal 11: Support enhanced RSSI reporting for Rx-Assistance, enhancements include at least L1-RSSI reporting.

Proposal 12:  Study further LBT sensing at the receiver with a conditional response from the receiver for Rx-Assistance.

- Consider the use of CAT 2 LBT for LBT-sensing for Rx-Assistance.

Proposal 13:  Under the restrictions of duty cycle for short control signaling, allow SS/PBCH, PDCCH, CSI-RS and PRS for contention exempt transmission

Proposal 14:  Under the restrictions of duty cycle for short control signaling, allow PRACH, msg1, msg3, msgA, SRS, PUCCH and PUSCH without user plane data for contention exempt transmission

Proposal 15: Consider the use of two energy measurements for deferral period and for contention slot.

Proposal 16: Consider deployment bandwidth dependence on the minimum sensing duration requirement.

Proposal 17:  Support Alt 2 for Multi-Channel LBT. For Type B multi-channel access, introduce Cat 2 LBT for non-primary channels.

Proposal 18: SSB burst transmission could be regarded as a Multi-Beam TDM COT, with support for both pre-burst single LBT with wide sensing and per beam independent LBT performed at the start of the COT.

Proposal 19:  Support provision for sensing and measurement gaps for discovery of aggressors and victims in a No-LBT deployment

Proposal 20:  For No-LBT deployments, consider specification of optional good neighbor procedures, such as away time, to break persistent beam collisions for better coexistence.

Please consider the following proposals.

# References

- Chairperson’s notes: RAN1-104e, Agenda 8.2.6

- RP-202925, Revised WID:  Extending current NR operation to 71 GHz

- ETSI EN 302 567, Harmonised European standard for Multi-Gigabit radio equipment in 60GHz, v2.2.0 (2020-12)