3GPP TSG RAN WG1 #104b-e			R1-2103168

e-Meeting, April 12th – 20th, 2021

Agenda item:	8.4.1

Source: 	Qualcomm Incorporated

Title: 	Enhancements on Timing Relationship for NTN

Document for:	Discussion/Decision

# Introduction

In RAN1 103e and 104e, the following agreements on timing relationships for NTN were made [1][2]:

Agreement:

Introduce K_offset (may or may not be the same as the K_offset value in other timing relationships) to enhance the timing relationship of HARQ-ACK on PUCCH to MsgB.

Agreement:

- For K_offset configured in system information and used in initial access, at least a cell specific K_offset configuration, which is used in all beams of a cell, should be supported.

- FFS: Beam specific K_offset configured in system information and used in initial access.

Working Assumption:

K_offset can be applied to indicate the first transmission opportunity of PUSCH in Configured Grant Type 2 in the same way as K_offset is applied to the transmission timing of DCI scheduled PUSCH.

Conclusion:

The agreement made at RAN1#102-e about introducing K_offset in the transmission timing of RAR grant scheduled PUSCH is also applicable to fallbackRAR scheduled PUSCH.

Agreement:

Denote by K_mac a scheduling offset other than K_offset:

- If downlink and uplink frame timing are aligned at gNB:

  - For UE action and assumption on downlink configuration indicated by a MAC-CE command in PDSCH, K_mac is not needed.

  - For UE action and assumption on uplink configuration indicated by a MAC-CE command in PDSCH, K_mac is not needed.

- If downlink and uplink frame timing are not aligned at gNB:

  - For UE action and assumption on downlink configuration indicated by a MAC-CE command in PDSCH, K_mac is needed.

  - For UE action and assumption on uplink configuration indicated by a MAC-CE command in PDSCH, K_mac is not needed.

- Note: This does not preclude identifying exceptional MAC CE timing relationship(s) that may or may not require K_mac.

Agreement:

Confirm the following working assumption:

K_offset can be applied to indicate the first transmission opportunity of PUSCH in Configured Grant Type 2 in the same way as K_offset is applied to the transmission timing of DCI scheduled PUSCH.

Agreement:

Update of K_offset after initial access is supported

Agreement:

For unpaired spectrum, extend the value range of K1 from (0..15) to (0..31)

FFS: Whether there is an impact on the size of the PDSCH-to-HARQ_feedback timing indicator field in DCI.

Working assumption:

Introduce K_offset to enhance the adjustment of uplink transmission timing upon the reception of a corresponding timing advance command.

In addition, the following agreement for UL TA calculation made in RAN1 103e is also related to timing relationships.

Agreement:

- In NTN, the network may broadcast

- A common timing offset value

  - FFS details of the common timing offset

- FFS: A common timing drift rate

- Before Msg1/MsgA transmission, the NR NTN UE in idle/inactive mode calculates its TA as follows:

TA= NTA+NTA, offset[+X]×Tc[+X]

Where:

NTA is derived from the User specific TA self-estimation

X is derived at least from the common timing offset value if broadcasted by the network. The granularity of X and whether X is indicated as a Timing Advance or as a Timing Offset value [unit] are FFS. Upon resolving the FFS, one of the X in the equation will be removed.

- NTA, offset depends on band and LTE/NR coexistence and is specified in TS 38.213 section 4.2.

- Tc is specified in TS 38.211 section 4.1.

- Note: UE will not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A.

In addition, an agreement was made on timing relationship for RAR window in RAN2#111e

Agreement:

From RAN2 perspective, an offset is applied to the start of ra-ResponseWindow in NTN for both LEO and GEO scenarios.

In this document, we discuss remaining details and additional enhancements for timing relationships.

# Discussions

Several scheduling offsets have been discussed. They are:

- K_offset1: To ensure that an UL transmission or report is causal.  Consequently, the associated Koffset must be greater than or equal to TA. This offset has been agreed and applies to the following timing relationships

  - The transmission timing of DCI scheduled PUSCH (including CSI on PUSCH) and the first PUSCH opportunity in Configured Grant Type 2.

  - The transmission timing of RAR grant scheduled PUSCH.

  - The transmission timing of HARQ-ACK on PUCCH.

  - The CSI reference resource timing.

  - The transmission timing of aperiodic SRS.

- K_offset2: This is essentially for the same purpose as K_offset1 and applies to HARQ_ACK on PUCCH in response to MsgB.

- K_offset3: This is to compensate the delay of an UL slot with respect to the DL slot of the same count at gNB. If downlink and uplink frame timing is aligned at the satellite, the associated Koffset must be greater than or equal to the feeder link RTD. This applies to at least the following timing relationship:

  - Application time of MAC-CE commands with DL configuration as illustrated in Figure 1.

  - The start of ra-Response window relative to the PRACH occasion.

Figure 1. Koffset for MAC-CE commands with DL configuration: the associated HARQ-ACK arriving at satellite in slot n will arrive at gNB in slot n+2*df with 2df as the feeder link RTD.

In addition, it has been agreed for a common timing offset value named X for PRACH TA calculation. This common timing offset is intended to compensate the feeder link RTD although the exact amount of feeder link delay to be compensated is up to Network implementation. As such, DL and UL timing alignment should not be assumed at gNB and signalling of the scheduling offset for MAC-CE commands should be supported.

For maximal flexibility of network implementation and reduced range of the scheduling offset values, two offset values can be signalled: one for the service link and one for the feeder link as the following.

Proposal 1:

- The following two offset values are signalled in system information:

  - Offset_1

  - Offset_2

- Offset_2=0 if not signalled.

- The scheduling offset calculated as K_offset=Offset_1+Offset_2 is for the following timing relationships:

  - The transmission timing of DCI scheduled PUSCH (including CSI on PUSCH) and the first PUSCH opportunity in Configured Grant Type 2.

  - The transmission timing of RAR grant scheduled PUSCH.

  - The transmission timing of HARQ-ACK on PUCCH.

  - The CSI reference resource timing.

  - The transmission timing of aperiodic SRS.

- The scheduling offset for MAC-CE commands with DL configuration is calculated as K_mac=Offset_2.

- FFS: Detailed signalling and granularity of offset_1 and offset_2.

- FFS: Beam specific and UE specific Offset_1.

When the same value of Koffset is applied to all UEs in a cell, Koffset must be larger than or equal to the maximal RTD of the cell. Doing so, all the UEs in a cell are subject to the maximal scheduling or action delay when DL-UL interaction is involved.  As shown as the alternative 1 in Figure 1, all UEs, near or far from the satellite, experience the same overall scheduling plus transmission delay. The unnecessary scheduling delay imposed on UEs near the satellite due to a common Koffset  per cell can be significant when the size of the cell is large. Additionally, this will create new buffering requirements at the UEs due to the increased delay between receiving a DL message and transmitting the corresponding UL response. In such case, UE specific Koffset can be configured so that UEs near the satellite can have smaller UL scheduling and transmission delay as shown as the alternative 2 in Figure 1.

Figure 2. UL scheduling and transmission delay with cell/beam specific Koffset vs UE specific Koffset.

One way to support UE specific Koffset is to let UE report its timing advance (TA) including both UE autonomous timing advance and accumulated TA commands, e.g., in message 3, and to use the reported TA rounded up to a whole number in slots or ms as the new Koffset. For LEO satellites, the RTD can change as satellites moves. To support minimized scheduling delay with varying RTD, periodic report of TA and updating of koffset can be supported.

To enable UE specific Koffset, we propose the following mechanisms.

Proposal 2 Support UE specific TA report by MAC-CE

- FFS: details of signaling

Proposal 3: Support Koffset update by MAC-CE.

Proposal 4: Support configuration of periodic TA report by RRC.

# Conclusions

In this contribution, we have discussed some remaining issues for timing relationships in NTN with following proposals:

Proposal 1:

- The following two offset values are signalled in system information:

  - Offset_1

  - Offset_2

- Offset_2=0 if not broadcasted.

- The scheduling offset calculated as K_offset=Offset_1+Offset_2 is for the following timing relationships:

  - The transmission timing of DCI scheduled PUSCH (including CSI on PUSCH) and the first PUSCH opportunity in Configured Grant Type 2.

  - The transmission timing of RAR grant scheduled PUSCH.

  - The transmission timing of HARQ-ACK on PUCCH.

  - The CSI reference resource timing.

  - The transmission timing of aperiodic SRS.

- The scheduling offset for MAC-CE commands with DL configuration is calculated as K_mac=Offset_2.

- FFS: Detailed signalling and granularity of offset_1 and offset_2.

- FFS: Beam specific and UE specific Offset_1.

Proposal 2 Support UE specific TA report by MAC-CE.

- FFS: details of signaling

Proposal 3: Support Koffset update by MAC-CE.

Proposal 4: Support configuration of periodic TA report by RRC.

# References

- Chairman’s note, 3gpp RAN1 103e.

- Chairman’s note, 3gpp RAN1 104e.